3GPP TSG-RAN WG1 #112bis-e	R1-2303555

e-meeting, April 17th – 26th, 2022

Agenda Item:	9.5.4

Source:	Ericsson

Title:	Bandwidth aggregation for positioning measurements

Document for:	Discussion, Decision

# Introduction

In the Rel-18 positioning WID [1], the following objectives related to bandwidth aggregation for positioning measurements are included:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4].

  - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2].

    - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

  - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

In this paper, we provide Ericsson’s views on bandwidth aggregation for positioning measurements.

# Bandwidth aggregation of PRS

## Conditions for coherently combining DL PRSs

In RAN1#112, the following agreement was made:

Agreement

To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs:

- In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies

  - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin

  - The same QCL

- The same number of symbols, symbol location within one slot, repetition factor,

- FFS: the same periodicity and slot offset

- FFS muting pattern

- The same numerology, i.e. the same CP and SCS

- The same or different bandwidths

- The same comb size

- FFS: The same number of PRS resource sets and resources for a TRP

- The same power per subcarrier

- FFS: the same NR-DL-PRS-SFN0-Offset

- Aggregated PFLs are configured on the same aligned numerology grid

- FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A)

- Phase continuity between aggregated PFLs

Regarding the first FFS, for condition on gNB Tx TEGs, it should be noted that the Tx TEG IDs are locally defined within each PFL.  Hence, in order to indicate that PRSs in two or three different aggregated PFLs have Tx timing error difference with a certain margin, it is sufficient for the gNB to indicate gNB Tx TEG ID for the PRS in one of the two or three aggregated PFLs.

The gNB can indicate to LMF one gNB Tx TEG ID for the PRS in just one of the two or three PFLs that can be aggregated. It is understood that the same Tx TEG ID applies to all aggregated PFLs.

As for the UE Rx TEG, we note that the UE will perform a joint measurement across the PRSs in two or three different aggregated PFLs.  Hence, in order to indicate that a joint measurement across PRSs in two or three different aggregated PFLs have Rx timing error difference within a certain margin, the UE indicates to LMF one UE Rx TEG ID for the joint measurement performed across the PRSs in two or three different aggregated PFLs.

In order to indicate that a joint measurement across PRSs in two or three different aggregated PFLs have Rx timing error difference within a certain margin, the UE indicates to LMF one UE Rx TEG ID for the joint measurement performed across the PRSs in two or three different aggregated PFLs.

Regarding the FFSs on the periodicity, slot offset, muting pattern, and NR-DL-PRS-SFN0-Offset, we think it makes sense to have the same periodicity, slot offset, muting pattern and NR-DL-PRS-SFN0-Offset for the PRSs in two or three different aggregated PFLs.  If these conditions are not met, the measurement periodicity may be limited by the largest periodicity among the PRSs in the two or three different aggregated PFLs which may unnecessarily delay the measurement.  Hence, we propose the following:

For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, the same periodicity, same slot offset, same muting pattern and NR-DL-PRS-SFN0-Offset are assumed.

Regarding the FFS on the same number of PRS resource sets and resources for a TRP, we think this is an unnecessary restriction. It should be noted that the already agreed conditions of aggregated PRSs being on the same symbol on same slot can only be satisfied on a resource level (specially in FR2).  It is sufficient that the aggregated PRSs are linked at PRS resource level.

For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, do not support the constraints on same number of PRS resource sets and resources for a TRP across the aggregated PFLs.

Regarding the FFS on how to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones, our understanding is that this can be left to network implementation. Hence, it is sufficient to agree that the UE expects the PRS pattern across the aggregated bandwidths to be contiguous.

Aspects related maintaining contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones can be left to network implementation.

For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, the UE expects the PRS pattern across the aggregated bandwidths to be contiguous.

## Indication of PRSs to be aggregated

In RAN1#112, the following agreement was reached:

Agreement

For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked.

- FFS whether the link is for all TRPs or per TRP basis

- FFS whether the link is per PRS resource set basis or per PRS resource basis.

DL PRS resources are organized in a tree structure with the positioning frequency layer at the top, then TRPs, then PRS resource sets and finally PRS resources. All resources in a frequency layer have the same bandwidth and frequency allocations.  In order to signal PRS aggregation, the following updates are thus necessary:

- Identify which PRS resources across PFLs can be linked to create an aggregated PRS transmission with wide bandwidth

- Identify which PFLs in a TRP participate with the aggregation.

Hence, the linking of PRS resources across two or three aggregated PFLs should be performed on a TRP basis since all TRPs may not be able to transmit DL PRSs across PFLs coherently.

For PRS bandwidth aggregation across PFLs, support linking of PRS resources from two or three aggregated PFLs on a per TRP basis.

The DL PRS resources that can be aggregated by UE to perform combined positioning measurement is indicated by LMF to the UE. For example, LMF may include an PRS aggregation ID as a part of DL PRS resource configuration.  When two DL PRS resources are configured with the same PRS aggregation ID, the UE can aggregate them to process together for positioning measurement. In this sense the PRS aggregation ID represents a group of DL PRS resources that can be aggregated and coherently/jointly processed for positioning measurements. If the PRS resource configuration does not include an PRS aggregation ID, the PRS resource may not be combined with another PRS resource.

LMF indicates to the UE which PRS resources (i.e., on a PRS resource basis) that can be aggregated by including an aggregation ID as part of DL PRS resource configuration

- Two or more DL PRS resources configured with the same aggregation ID can be aggregated and coherently/jointly processed by the UE

- a PRS resource that does not have an aggregation ID is not allowed to be aggregated with another PRS resource

In NR Rel-16, the LMF sends requests to NG-RAN node regarding information on TRPs hosted by the NG-RAN node via the ‘TRP INFORMATION REQUEST’ message.  In response, the NG-RAN node may provide the ‘TRP INFORMATION RESPONSE’ which may contain information related to one or more TRPs hosted by the NG-RAN node.  The ‘TRP Information’ information element which is part of the ‘TRP INFORMATION RESPONSE’ includes the PRS configuration.

Whether one or more DL PRS resources can be coherently transmitted by the TRP needs to be indicated to the LMF as it is the TRP that knows whether coherency can be maintained for the one or more DL PRS transmissions in these resources.  A simple approach is to include a PRS aggregation ID at the PRS Resource level as part of ‘TRP INFORMATION RESPONSE’.  The PRS aggregation ID is optional, and if this parameter is not included for a particular PRS resource, then it implies that this PRS resource cannot be aggregated with other PRS resources.

NG-RAN node indicates to the LMF which PRS resources (i.e., on a per PRS resource basis) from which TRPs can be aggregated by including an aggregation ID as part of ‘TRP INFORMATION RESPONSE’

- Two or more DL PRS resources configured with the same aggregation ID can be aggregated and coherently/jointly processed

- a PRS resource that does not have an aggregation ID is not allowed to be aggregated with another PRS resource

## Joint measurement and reporting based on aggregated PRSs

In RAN1#112, the following agreement was reached:

Agreement

Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods

- In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs

  - FFS: RSRP, RSRPP

- FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement

- FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs

- FFS RSTD reference configuration or report should be enhanced

We support both RSRP and RSRPP measurement reporting for the PRS resources across aggregated PFLs.

In NR Rel-18, support one RSRP measurement and one RSRPP measurement per path for the PRS resources across aggregated PFLs in a measurement report element.

Regarding the issue of whether PFLs are aggregated for the PRS measurement, we do not think this is needed.  In our view, the UE should aggregate PRS resources according to the linking information indicated in assistance data.  The report can however indicate which PRS resources are aggregated (i.e., which PRS resource pairs or triplets are aggregated can be indicated in the report).  Furthermore, the same RSTD reference should be used for the aggregated PFLs.

In NR Rel-18, the UE can aggregate PRS resources for a PRS joint measurement according to the linking information indicated in assistance data.

In NR Rel-18, the UE in a measurement report indicates which PRS resources are aggregated (i.e., which PRS resource pairs or triplets are aggregated can be indicated in the report).

In NR Rel-18, support the same RSTD reference for the aggregated PFLs.

# Bandwidth aggregation of SRS

## Conditions for coherently combining SRSs

In RAN1#112, the following agreement was made:

Agreement

To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers

- In the same slot, in same symbols, from the same antenna, this implies

  - FFS: The same gNB Rx TEG and the same UE Tx TEG

  - The same spatial relation

- The same startPosition, nrofSymbols

- FFS: periodicityAndOffset, and slotOffset

- The same numerology, i.e. the same CP and SCS

- The same or different bandwidths

- The same comb size

- FFS: The same number of SRS resource sets and resources

- The same Tx PSD (power per subcarrier)

  - FFS whether to need the same pathloss RS, Po and alpha

  - Note: the Tx PSD is not captured in RAN1 specifications

- FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones

- Phase continuity between aggregated SRS in different carriers

Regarding the first FFS, for condition on UE Tx TEGs, it should be noted that the Tx TEG IDs are locally defined within each carrier.  Hence, in order to indicate that SRSs in two or three different aggregated carriers have Tx timing error difference with a certain margin, it is sufficient for the UE to indicate UE Tx TEG ID for the SRS in one of the two or three aggregated carriers.

The UE can indicate to gNB one UE Tx TEG ID for the SRS in just one of the two or three carriers that can be aggregated. It is understood that the same Tx TEG ID applies to all aggregated carriers.

As for the gNB Rx TEG, we note that the gNB will perform a joint measurement across the SRSs in two or three different aggregated carriers.  Hence, in order to indicate that a joint measurement across SRSs in two or three different aggregated carriers have Rx timing error difference within a certain margin, the gNB indicates to LMF one gNB Rx TEG ID for the joint measurement performed across the SRSs in two or three different aggregated carriers.

In order to indicate that a joint measurement across SRSs in two or three different aggregated carriers have Rx timing error difference within a certain margin, the gNB indicates to LMF one gNB Rx TEG ID for the joint measurement performed across the SRSs in two or three different aggregated carriers.

Regarding the FFSs on the periodicity and slot offset, we think it makes sense to have the same periodicity and slot offset for the SRSs in two or three different aggregated carriers.  If these conditions are not met, the measurement periodicity may be limited by the largest periodicity among the SRSs in the two or three different aggregated carriers which may unnecessarily delay the measurement.  Hence, we propose the following:

For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, the same periodicity and same slot offset are assumed.

Regarding the FFS on the same number of SRS resource sets and resources across multiple carriers, we think this is an unnecessary restriction. It is sufficient that the aggregated SRSs are linked at SRS resource level.

For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, do not support the constraints on same number of SRS resource sets and resources for a UE across the aggregated carriers.

Regarding the FFS on how to maintain contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones, our understanding is that this can be left to implementation. Hence, it is sufficient to agree that the UE expects the SRS pattern across the aggregated bandwidths to be contiguous.

Aspects related maintaining contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones can be left to implementation.

For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, the UE expects the SRS pattern across the aggregated bandwidths to be contiguous.

## Indication of SRSs to be aggregated

In RAN1#112, the following agreement was reached:

Agreement

For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked

- SRS resources are per BWP per carrier configuration

- FFS whether the link is per SRS resource set basis or per SRS resource basis.

On the FFS above, we think the linking per SRS resource basis gives more flexibility.  Hence, we propose the following:

For SRS bandwidth aggregation across carriers, support linking of SRS resources on a per SRS resource basis.

## Joint measurement and reporting based on aggregated SRSs

In RAN1#112, the following agreement was reached:

Agreement

Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods

- Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers

  - FFS: RSRP or RSRPP

- FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement

- Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

We support both RSRP and RSRPP measurement reporting for the SRS resources across aggregated carriers.

In NR Rel-18, support one RSRP measurement and one RSRPP measurement per path for the SRS resources across aggregated carriers in a measurement report element.

Regarding the issue of whether carriers are aggregated for the SRS measurement, we do not think this is needed.  In our view, the gNB should aggregate SRS resources according to the linking information.  The report can however indicate which SRS resources are aggregated (i.e., which SRS resource pairs or triplets are aggregated can be indicated in the report).

In NR Rel-18, the gNB can aggregate SRS resources for a SRS joint measurement according to the linking information.

In NR Rel-18, the gNB in a measurement report indicates which SRS resources are aggregated (i.e., which SRS resource pairs or triplets are aggregated can be indicated in the report).

# Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1	The gNB can indicate to LMF one gNB Tx TEG ID for the PRS in just one of the two or three PFLs that can be aggregated. It is understood that the same Tx TEG ID applies to all aggregated PFLs.

Proposal 2	In order to indicate that a joint measurement across PRSs in two or three different aggregated PFLs have Rx timing error difference within a certain margin, the UE indicates to LMF one UE Rx TEG ID for the joint measurement performed across the PRSs in two or three different aggregated PFLs.

Proposal 3	For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, the same periodicity, same slot offset, same muting pattern and NR-DL-PRS-SFN0-Offset are assumed.

Proposal 4	For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, do not support the constraints on same number of PRS resource sets and resources for a TRP across the aggregated PFLs.

Proposal 5	For the feature of PRS bandwidth aggregation between PRSs in two or three different PFLs, the UE expects the PRS pattern across the aggregated bandwidths to be contiguous.

Proposal 6	For PRS bandwidth aggregation across PFLs, support linking of PRS resources from two or three aggregated PFLs on a per TRP basis.

Proposal 7	LMF indicates to the UE which PRS resources (i.e., on a PRS resource basis) that can be aggregated by including an aggregation ID as part of DL PRS resource configuration

	Two or more DL PRS resources configured with the same aggregation ID can be aggregated and coherently/jointly processed by the UE

	a PRS resource that does not have an aggregation ID is not allowed to be aggregated with another PRS resource

Proposal 8	NG-RAN node indicates to the LMF which PRS resources (i.e., on a per PRS resource basis) from which TRPs can be aggregated by including an aggregation ID as part of ‘TRP INFORMATION RESPONSE’

	Two or more DL PRS resources configured with the same aggregation ID can be aggregated and coherently/jointly processed

	a PRS resource that does not have an aggregation ID is not allowed to be aggregated with another PRS resource

Proposal 9	In NR Rel-18, support one RSRP measurement and one RSRPP measurement per path for the PRS resources across aggregated PFLs in a measurement report element.

Proposal 10	In NR Rel-18, the UE can aggregate PRS resources for a PRS joint measurement according to the linking information indicated in assistance data.

Proposal 11	In NR Rel-18, the UE in a measurement report indicates which PRS resources are aggregated (i.e., which PRS resource pairs or triplets are aggregated can be indicated in the report).

Proposal 12	In NR Rel-18, support the same RSTD reference for the aggregated PFLs.

Proposal 13	The UE can indicate to gNB one UE Tx TEG ID for the SRS in just one of the two or three carriers that can be aggregated. It is understood that the same Tx TEG ID applies to all aggregated carriers.

Proposal 14	In order to indicate that a joint measurement across SRSs in two or three different aggregated carriers have Rx timing error difference within a certain margin, the gNB indicates to LMF one gNB Rx TEG ID for the joint measurement performed across the SRSs in two or three different aggregated carriers.

Proposal 15	For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, the same periodicity and same slot offset are assumed.

Proposal 16	For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, do not support the constraints on same number of SRS resource sets and resources for a UE across the aggregated carriers.

Proposal 17	For the feature of SRS bandwidth aggregation between SRSs in two or three different carriers, the UE expects the SRS pattern across the aggregated bandwidths to be contiguous.

Proposal 18	For SRS bandwidth aggregation across carriers, support linking of SRS resources on a per SRS resource basis.

Proposal 19	In NR Rel-18, support one RSRP measurement and one RSRPP measurement per path for the SRS resources across aggregated carriers in a measurement report element.

Proposal 20	In NR Rel-18, the gNB can aggregate SRS resources for a SRS joint measurement according to the linking information.

Proposal 21	In NR Rel-18, the gNB in a measurement report indicates which SRS resources are aggregated (i.e., which SRS resource pairs or triplets are aggregated can be indicated in the report).

# References

RP 223549, New WID on Expanded and Improved NR Positioning