3GPP TSG RAN WG1 #112bis-e			R1-2303564

e-Meeting, April 17th – April 26th, 2023

Agenda item:	7.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Type1 HARQ-ACK CB issue with more than one PDSCH per slot

Document for:	Discussion/Decision

# Type1 HARQ-ACK CB generation with multiple PDSCHs per slot

In RAN1 #112, the issue in Rel-15 specification related to Type1 HARQ-ACK codebook generation with respect to the condition of “more than one PDSCH per slot” was discussed, and the following was captured in RAN1 Chair’s notes for possible agreement in RAN1#112bis-e:

Companies to check the following proposal for possible agreement in RAN1#112bis-e:

Support to introduce one RRC parameter in Rel-17 related to the condition of “receiving more than one unicast PDSCH per slot” in Type1 HARQ-ACK CB generation.

- A UE capability is introduced which indicates UE supports receiving this new RRC parameter.

- The new RRC parameter if configured, is per CC:

  - If the new RRC parameter is not configured, the legacy text in 38.213 Section 9.1.2.1 applies

  - If the new RRC parameter is configured and indicates no possibility of receiving more than one PDSCH per slot, the procedures of the “if” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

  - If the new RRC parameter is configured and indicates the possibility of receiving more than one PDSCH per slot, the procedures of the “else” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

Sent an LS to RAN2 for introducing the new RRC parameter and new UE capability in Rel-17.

if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and ,

;

;

The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot;

else

Set  to the cardinality of

Set  to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of

while

Set

while

if  for start OFDM symbol index  for row

; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row

;

;

else

;

end if

end while

;

Set  to the smallest last OFDM symbol index among all rows of ;

end while

end if

The issues with the existing specification are explained in detail in our previous contribution (R1-2301383) and are summarized below:

- Issue 1: There is an ambiguity wrt “capability to receive more than one unicast PDSCH per slot” is referring to which UE capability exactly.

- Issue 2: There is an ambiguity wrt “capability to receiving more than one unicast PDSCH per slot” should be assumed for which CC/band in a CA configuration.

- Issue 3: Network has no way to enable / disable the capability of receiving more than one PDSCH per slot, i.e., even when the network does not intend to use this capability, this issue results in unnecessary large Type1 HARQ-ACK CB size.

As discussed in RAN1 #112, a solution to address the issues above is needed given both the ambiguity issues as well as the fact that this functionality is not controllable by the network. A consensus was almost reached to address the issue by simply introducing a new RRC configuration starting from Rel-17. However, more time was requested to further check the impact to Rel-17 MBS feature. Based on the discussions below, it turns out that this fix, i.e., introducing a new RRC parameter, can even help to address some additional ambiguity issue for all cases of Rel-17 MBS feature, Rel-16 multi-DCI based multi-TRP feature, and Rel-16 feature to generate two HARQ-ACK codebooks for two priorities. Also, there is no need for additional changes (on top of the proposal above to introduce one new RRC parameter) for these features.

Discussions related to Rel-17 MBS: In Rel-17 specification, the condition of the “if/else” condition in Type1 HARQ-ACK codebook pseudo-code is “if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot”. At the same time, this pseudo-code is run one or multiple times separately depending on other configurations:

- The pseudo-code is run one time if fdmed-Reception-Multicast is not configured and type1-Codebook-Generation-Mode = ‘mode2’ is configured.

- The pseudo-code is run two times if fdmed-Reception-Multicast is configured.

- The pseudo-code is run three times if fdmed-Reception-Multicast is not configured and type1-Codebook-Generation-Mode = ‘mode1’ is configured.

However, it is not clear whether the “if” condition “if the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot” is across both unicast and multicast PDSCHs in a slot or is based on a subset of them, and whether for different modes as discussed above (for different numbers that the pseudo-code is run), this condition should be interpreted separately.

With a new RRC parameter, gNB can enable or disable the “else” condition depending on other RRC configurations that determine how Type1 HARQ-ACK codebook is constructed as discussed above. For example, if fdmed-Reception-Multicast is configured and UE can receive max of 1+1 PDSCH per slot for unicast + multicast, gNB can disable the new RRC parameter so that the “if” condition is applied for both sub-codebooks.

Even though further optimizations (e.g., to introduce separate new RRC parameters for unicast and multicast, or to introduce three separate new RRC parameters for the case that pseudo-code is run three times) is possible, they are not within the scope of this maintenance issue. In other words, even with just one new RRC parameter irrespective of unicast or multicast, the ambiguity issues are resolved, and this already gives the network enough controllability.

Discussions related to Rel-16 multi-DCI based mTRP feature: In Rel-16, for both cases of separate and joint feedback for multi-DCI based mTRP, the Type 1 HARQ-ACK codebook generation pseudo-code is run two times separately and independently for the two coresetPoolIndex values. However, UE at least supports two PDSCHs per slot in this case (at least one PDSCH per coresetPoolIndex value per slot), which means the “if” condition never applies and always the “else” condition is applicable in the pseudo-code. This is clearly not correct if UE indicates “1” as part of capability maxNumberUnicastPDSCH-PerPool-r16=1. With a new RRC parameter, gNB can enable or disable the “if/else” conditions correctly.

Discussions related to Rel-16 feature to generate two HARQ-ACK codebooks for two priorities: In Rel-16, when pdsch-HARQ-ACK-CodebookList is configured, UE generates two HARQ-ACK codebooks separately for the two priorities. Hence, the Type 1 HARQ-ACK codebook generation pseudo-code is run two times separately and independently for the two priorities. When the network intends to transmit max of 1+1 PDSCHs per slot for first priority + second priority, the “if” condition should be assumed in the pseudo-code. However, based on the existing specifications, always the “else” condition is assumed. With a new RRC parameter, gNB can enable or disable the “if/else” conditions accordingly.

Observation 1: With introducing a new RRC parameter to control the “if/else” conditions in Type 1 HARQ-ACK codebook generation pseudo-code, not only the Rel-15 issues are addressed, but also additional ambiguity and inefficiency issues for the following cases can be handled:

- Type1 HARQ-ACK CB for Rel-17 MBS.

- Type1 HARQ-ACK CB for Rel-16 multi-DCI based multi-TRP

- Type1 HARQ-ACK CB for Rel-16 feature to generate two HARQ-ACK codebooks for two priorities

Lastly, we think the following TP can implement this fix in RAN1 specification in 38.213 for Rel-17 specification:

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

Proposal 1: Support to introduce one RRC parameter in Rel-17 related to the condition of “receiving more than one unicast PDSCH per slot” in Type1 HARQ-ACK CB generation.

- A UE capability is introduced which indicates UE supports receiving this new RRC parameter.

- The new RRC parameter if configured, is per CC:

  - If the new RRC parameter is not configured, the legacy text in 38.213 Section 9.1.2.1 applies

  - If the new RRC parameter is configured and indicates no possibility of receiving more than one PDSCH per slot, the procedures of the “if” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

  - If the new RRC parameter is configured and indicates the possibility of receiving more than one PDSCH per slot, the procedures of the “else” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

Send an LS to RAN2 for introducing the new RRC parameter and new UE capability in Rel-17.

Adopt the following TP to implement this in 38.213 for Rel-17 specification:

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

# Conclusion

In this contribution, we observed / proposed the following:

Observation 1: With introducing a new RRC parameter to control the “if/else” conditions in Type 1 HARQ-ACK codebook generation pseudo-code, not only the Rel-15 issues are addressed, but also additional ambiguity and inefficiency issues for the following cases can be handled:

- Type1 HARQ-ACK CB for Rel-17 MBS.

- Type1 HARQ-ACK CB for Rel-16 multi-DCI based multi-TRP

- Type1 HARQ-ACK CB for Rel-16 feature to generate two HARQ-ACK codebooks for two priorities

Proposal 1: Support to introduce one RRC parameter in Rel-17 related to the condition of “receiving more than one unicast PDSCH per slot” in Type1 HARQ-ACK CB generation.

- A UE capability is introduced which indicates UE supports receiving this new RRC parameter.

- The new RRC parameter if configured, is per CC:

  - If the new RRC parameter is not configured, the legacy text in 38.213 Section 9.1.2.1 applies

  - If the new RRC parameter is configured and indicates no possibility of receiving more than one PDSCH per slot, the procedures of the “if” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

  - If the new RRC parameter is configured and indicates the possibility of receiving more than one PDSCH per slot, the procedures of the “else” condition in 38.213 Section 9.1.2.1 are applicable for Type1 HARQ-ACK CB generation.

Send an LS to RAN2 for introducing the new RRC parameter and new UE capability in Rel-17.

Adopt the following TP to implement this in 38.213 for Rel-17 specification:

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>

if the UE is not provided with [multiPDSCHperSlot-Type1CB] and the UE does not indicate a capability to receive more than one unicast PDSCH or multicast PDSCH per slot and R≠∅, or if the UE is provided with [multiPDSCHperSlot-Type1CB] =‘disabled’ and R≠∅,

MA,c=MA,c∪j;

j=j+1;

else

Set CR to the cardinality of R

<unchanged text omitted>

end if

<unchanged text omitted>