3GPP TSG RAN WG1 Meeting #112bis-e    	            R1-2303565

E-meeting, April 17th – 26th, 2023

Source:	Qualcomm Incorporated

Title:	Clarification on A-CSI-RS triggering offset

Agenda Item:	7.2

Document for: 	Discussion and Decision

# Background

TS 38.331 specifies three RRC parameters for aperiodic CSI-RS triggering offset, aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16, and aperiodiTriggeringOffset-r17, as follows.

NZP-CSI-RS-ResourceSet ::= SEQUENCE { […] aperiodicTriggeringOffset INTEGER(0..6) OPTIONAL, -- Need S […] aperiodicTriggeringOffset-r16 INTEGER(0..31) OPTIONAL -- Need S […] aperiodicTriggeringOffset-r17 INTEGER (0..124) OPTIONAL, -- Need S […] } aperiodicTriggeringOffset, aperiodicTriggeringOffset-r16, aperiodicTriggeringOffset-r17 Offset X between the slot containing the DCI that triggers a set of aperiodic NZP CSI-RS resources and the slot in which the CSI-RS resource set is transmitted. For aperiodicTriggeringOffset, the value 0 corresponds to 0 slots, value 1 corresponds to 1 slot, value 2 corresponds to 2 slots, value 3 corresponds to 3 slots, value 4 corresponds to 4 slots, value 5 corresponds to 16 slots, value 6 corresponds to 24 slots. For aperiodicTriggeringOffset-r16 and aperiodicTriggeringOffset-r17, the value indicates the number of slots. aperiodicTriggeringOffset-r17 is applicable to SCS 480 kHz and 960 kHz, and only the values of integer multiples of 4 are valid, i.e. 0, 4, 8, and so on. The network configures only one of the fields. When neither field is included, the UE applies the value 0..

The RAN1 specification for aperiodic CSI-RS triggering with an offset is specified in TS38.214 are as follows.

##### 5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology […] When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots for μCSIRS≤3 or {0, 4, 8, 12, …, 60, 64, 96} slots for μCSIRS=5 and μCSIRS=6, where μCSIRS is the subcarrier spacing configurations for CSI-RS. […] ##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies […] Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16 or aperiodicTriggeringOffset-r17, including the case that the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots for μCSIRS≤3 or {0, 4, 8, …, 124} slots for μCSIRS=5 and μCSIRS=6 when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for μCSIRS≤3 or {0, 4, 8, 12, …, 60, 64, 96} slots for μCSIRS=5 and μCSIRS=6 when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where […]

There are multiple issues relevant to the above specifications. In this contribution, we clarify the issues and propose solution options.

# Issue 1: Extended value range for aperiodic CSI-RS triggering offset requires UE to support crossSlotScheduling-r16

One of the use-cases of extended value range for aperiodic CSI-RS triggering offset in Rel-16 was indeed cross-slot scheduling for UE power saving. RAN1 agreed to support extended value range at RAN1#100bis meeting as follows.

Agreements: […] - Aperiodic CSI-RS triggering offset value range is extended from {0, 1, 2, 3, 4, 16, 24} to {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} […]

The other use-case of extended value range for aperiodic CSI-RS triggering offset in Rel-16 was aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies, which is based on the RAN1#100 agreements.

R1-1913003 FL Summary on A-CSI RS triggering with different SCS PDCCH Nokia R1-1913390 Agreements: When µPDCCH < µCSI-RS, X{0, 1, …, 31}

As seen above, aperiodic CSI-RS triggering when the triggering PDCCH and the CSI-RS have different numerologies is only for the case with µPDCCH < µCSI-RS. Besides, it is observed that the value range for this is beyond the values agreed for UE power saving.

However, TS 38.306 specifies a single UE capability to address the extended value range for aperiodic CSI-RS triggering offset. That is crossSlotScheduling-r16 and specified as follows.

| crossSlotScheduling-r16 Indicates whether UE supports dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset. Support of this feature is reported for licensed and unlicensed bands, respectively. When this field is reported, either of non-SharedSpectrumChAccess-r16 or sharedSpectrumChAccess-r16 shall be reported, at least. | UE | No | No | No |
| --- | --- | --- | --- | --- |

It is unclear what “the extended value range for aperiodic CSI-RS triggering offset” means. Assuming this is anything beyond the values {0, 1, 2, 3, 4, 16, 24} supported by aperiodicTriggeringOffset in Rel-15, we see the following issues.

- Use of aperiodicTriggeringOffset-r16 for aperiodic CSI-RS triggering indicating beyond {0, 1, 2, 3, 4, 16, 24} requires a UE to support crossSlotScheduling-r16. This effectively enforces the UE to support all other components of crossSlotScheduling-r16, as well as extended value range for aperiodic CSI-RS triggering offset for any pair of numerologies for PDCCH and the CSI-RS on any bands/BCs that the UE supports (note: crossSlotScheduling-r16 is per-UE capability with a differentiation between shared spectrum access and non-shared spectrum access).

- Same issue for aperiodicTriggeringOffset-r17 which is only for 480kHz/960kHz and hence is only for FR2-2. It does not make sense if the Rel-16 UE capability crossSlotScheduling-r16 indicates support of extended value range for aperiodic CSI-RS triggering offset for FR2-2. In this sense, there is currently no appropriate UE capability indicating support of extended value range that can be provided by aperiodicTriggeringOffset-r17.

Observation 1:

- Current specifications require UE to support crossSlotScheduling-r16 for extended value range for aperiodic CSI-RS triggering offset

  - Even for the purpose of cross-carrier A-CSI-RS triggering with different SCS using extended value range for aperiodic CSI-RS triggering offset

- Extended range value for aperiodic CSI-RS triggering offset for cross-carrier A-CSI-RS triggering with PDCCH of lower SCS to A-CSI-RS of higher SCS is more than what was agreed for Rel-16 cross-slot scheduling

  - Necessary value range for cross-carrier A-CSI-RS triggering with PDCCH of lower SCS to A-CSI-RS of higher SCS: {0, 1, …, 31}

  - Agreed value range for Rel-16 cross-slot scheduling: {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24}

- It is unclear whether/how crossSlotScheduling-r16 is required to be supported for extended value range for aperiodic CSI-RS triggering offset for FR2-2

# Issue 2: R16 parameter (or R17 parameter) seems applicable to the UEs that do not support the extended value range for aperiodic CSI-RS offset

The possible values for aperiodicTriggeringOffset-r16 with µPDCCH >= µCSIRS and with µPDCCH < µCSIRS for μCSIRS≤3 are clear from the spec. However, it is not clear for which UE the parameter aperiodicTriggeringOffset-r16 is configurable. For example, it is possible to interpret the spec such that a UE not supporting extended value range for aperiodic CSI-RS triggering offset (e.g., a UE not indicating support of crossSlotScheduling-r16) can be provided the aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 as long as the value is from {0, 1, 2, 3, 4, 16, 24}. Similarly, one may read the spec such that a UE not supporting FR2-2 can be provided the aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 as long as the value is from the value range the UE supports (e.g., {0, 1, 2, 3, 4, 16, 24} or {0, 1, …, 31}.

Observation 2:

- The range applicable for each case (same SCS, low-to-high SCS, high-to-low SCS) is clear from the spec. However, it is unclear which parameter is applicable to which type of UEs.

# Proposal

To resolve the above two issues, we propose to introduce new Rel-17 UE capability that indicates supported value range, RRC parameter, and {μPDCCH, μCSIRS}. This clearly resolves the issue 1. In addition, the issue 2 is also resolved without changes in RAN1 specifications or legacy UE features.

Proposal:

- Introduce new Rel-17 UE capability for “extended value range for aperiodic CSI-RS triggering offset for  μCSIRS≤3”

  - Per-FS

  - UE indicates support for one or multiple from the following:

    - Value range {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μPDCCH=μCSIRS≤3

    - Value range {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μCSIRS<μPDCCH≤3

    - Value range {0, 1, …, 31} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μPDCCH<μCSIRS≤3

- Introduce new Rel-17 UE capability for “extended value range for aperiodic CSI-RS triggering offset for  μCSIRS=5 or μCSIRS=6”

  - Per-FS

  - UE indicates support for one or multiple from the following:

    - Value range {0, 4, 8, 12, …, 60, 64, 96} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μPDCCH=μCSIRS and μCSIRS=5 or  μCSIRS=6

    - Value range {0, 4, 8, 12, …, 60, 64, 96} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μCSIRS<μPDCCH and μCSIRS=5 or  μCSIRS=6

    - Value range {0, 4, 8, …, 124} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μPDCCH<μCSIRS and μCSIRS=5 or  μCSIRS=6

# Conclusion

In this contribution, we point out the issues regarding extended value range for aperiodic CSI-RS triggering offset. Following are the observations.

Observation 1:

- Current specifications require UE to support crossSlotScheduling-r16 for extended value range for aperiodic CSI-RS triggering offset

  - Even for the purpose of cross-carrier A-CSI-RS triggering with different SCS using extended value range for aperiodic CSI-RS triggering offset

- Extended range value for aperiodic CSI-RS triggering offset for cross-carrier A-CSI-RS triggering with PDCCH of lower SCS to A-CSI-RS of higher SCS is more than what was agreed for Rel-16 cross-slot scheduling

  - Necessary value range for cross-carrier A-CSI-RS triggering with PDCCH of lower SCS to A-CSI-RS of higher SCS: {0, 1, …, 31}

  - Agreed value range for Rel-16 cross-slot scheduling: {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24}

- It is unclear whether/how crossSlotScheduling-r16 is required to be supported for extended value range for aperiodic CSI-RS triggering offset for FR2-2

Observation 2:

- The range applicable for each case (same SCS, low-to-high SCS, high-to-low SCS) is clear from the spec. However, it is unclear which parameter is applicable to which type of UEs.

To resolve the issues, we propose the following.

Proposal:

- Introduce new Rel-17 UE capability for “extended value range for aperiodic CSI-RS triggering offset for  μCSIRS≤3”

  - Per-FS

  - UE indicates support for one or multiple from the following:

    - Value range {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μPDCCH=μCSIRS≤3

    - Value range {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μCSIRS<μPDCCH≤3

    - Value range {0, 1, …, 31} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r16 for μPDCCH<μCSIRS≤3

- Introduce new Rel-17 UE capability for “extended value range for aperiodic CSI-RS triggering offset for  μCSIRS=5 or μCSIRS=6”

  - Per-FS

  - UE indicates support for one or multiple from the following:

    - Value range {0, 4, 8, 12, …, 60, 64, 96} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μPDCCH=μCSIRS and μCSIRS=5 or  μCSIRS=6

    - Value range {0, 4, 8, 12, …, 60, 64, 96} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μCSIRS<μPDCCH and μCSIRS=5 or  μCSIRS=6

    - Value range {0, 4, 8, …, 124} for aperiodic CSI-RS triggering offset by aperiodicTriggeringOffset-r17 for μPDCCH<μCSIRS and μCSIRS=5 or  μCSIRS=6

# Reference

- 3GPP TS 38.331

- 3GPP TS 38.214