3GPP TSG RAN WG1 #112bis-e			R1-2303567

e-Meeting, April 17th – April 26th, 2023

Agenda item:	7.2 (NR_enh_IIOT)

Source: 	Qualcomm Incorporated

Title: 	Clarification on CBG based Type 3 HARQ-ACK codebook including HARQ-ACK processes with different max number of CBGs

Document for:	Discussion/Decision

# Introduction

In this contribution, an issue of undefined UE behavior for Type 3 and enhanced Type 3 HARQ-ACK codebook is discussed with a few proposed solutions for RAN1 to consider.

# CBG based Type-3/eType-3 HARQ-ACK codebook including inter PHY priorities HARQ-ACK processes

## Motivation

CBG based HARQ-ACK feedback is a legacy feature in Rel-15. When a UE is configured with a max number of CBGs in a CC / serving cell (by RRC param codeBlockGroupTransmission inside which maxCodeBlockGroupsPerTransportBlock can be set to 2,4,6, or 8 as the max number of CBGs per TB, denoted by NHARQ-ACK,cCBG/TB,max in CC c), the UE reports CBG-based HARQ-Ack (one bit A/N for each CBG of a given TB). In Rel-16, because two PHY priorities were introduced, two different max number of CBGs per TB can be configured (by RRC param pdsch-CodeBlockGroupTransmissionList-r16), which are denoted as NHARQ-ACK,c,1CBG/TB,max and NHARQ-ACK,c,2CBG/TB,max, one for each priority.

Type-3 HARQ-ACK book is introduced in Rel-16 and is enhanced to eType-3 HARQ-ACK codebook in Rel-17. In Rel-17, type-3/eType-3 can support HARQ-ACK processes with two different PHY priorities. As we know, with type-3 HARQ-ACK codebook, once it is triggered, the feedback for all 16 HARQ process IDs (regardless of PHY priorities) are included in the codebook. For eType 3 HARQ-ACK codebook, the feedback only includes a subset of configured HARQ process ID, while still regardless their PHY priorities.

Therefore, one issue could occur in the following scenario, as illustrated in Fig 1.

- Type-3/eType-3 codebook is configured to report CBG-based HARQ-ACK (by RRC parameter pdsch-HARQ-ACK-OneShotFeedbackCBG-r16)

- Two HARQ processes (with different PHY priorities) are configured with different number of CBGs per TB (by RRC parameter pdsch-CodeBlockGroupTransmissionList-r16)

- Type-3/eType-3 codebook includes the two HARQ processes (with different PHY priorities)

The problem in this scenario is that, when UE generate the CBG based type-3 or eType-3 HARQ-ACK codebook, how many CBGs UE should report per TB?

Fig 1: CBG based Type 3/eType3 HARQ-ACK codebook including inter PHY priorities HARQ-ACK processes

## Solutions

A few solutions can be considered for this issue.

- Option 1 (max of the two): Number of HARQ-Ack bits for each TB of a HARQ process for that CC in the type-3/eType-3 HARQ-ACK codebook consists of max{NHARQ-ACK,c,1CBG/TB,max , NHARQ-ACK,c,2CBG/TB,max} bits. In this case, if actual number of CBGs for a given TB of a HARQ process in a CC is less than the number determined above, UE generates a NACK for each of the last remaining positions in the codebook.

- Option 2: The priority indicator field of the DCI that triggers type-3/eType 3 HARQ-ACK codebook determines the max number of CBGs and the number of HARQ-Ack bits for each TB of a HARQ process number for that CC. For example, in Fig 1, if the DCI trigger high priority type-3 codebook, max of 2 CBGs are assumed; otherwise, max of 8 CBGs are assumed.

  - If a TB is scheduled originally with a larger number of CBGs than the max number of CBGs determined as above for type-3/eType-3 HARQ-ACK codebook, UE generates HARQ-ACK bit for a new CBG that consists of two or more original CBGs by the binary AND operation of the HARQ-ACK information bits corresponding the original CBGs.

- Option 3: if the max number of CBGs for the two priorities are different, or if only one is CBG based while the other is TB based, the UE is only expected to be configured with TB level A/N reporting for type-3/eType 3 codebook (does not expect to be configured with pdsch-HARQ-ACK-OneShotFeedbackCBG-r16)

Among the three options, option 1 is the simplest with some additional HARQ-ACK feedback overhead. Option 2 is more complicated because of the AND operation to merge HARQ-ACK feedback for multiple CBGs. Option 3 put restriction on gNB scheduling. We slightly prefer option 1 and propose resolve the issue with option 1.

Proposal 1: When Type-3/eType-3 codebook is configured to report CBG-based HARQ-ACK which includes HARQ processes with two different max number of CBGs NHARQ-ACK,c,1CBG/TB,max and NHARQ-ACK,c,2CBG/TB,max,  max{NHARQ-ACK,c,1CBG/TB,max , NHARQ-ACK,c,2CBG/TB,max} is assumed to generate the CBG based Type-3/eType-3 codebook.

# Conclusion

In this contribution, we propose the following:

Proposal 1: When Type-3/eType-3 codebook is configured to report CBG-based HARQ-ACK which includes HARQ processes with two different max number of CBGs NHARQ-ACK,c,1CBG/TB,max and NHARQ-ACK,c,2CBG/TB,max,  max{NHARQ-ACK,c,1CBG/TB,max , NHARQ-ACK,c,2CBG/TB,max} is assumed to generate the CBG based Type-3/eType-3 codebook.