3GPP TSG RAN WG1 #112bis 			R1-2303570

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on feedback for first PDSCH afer multicast SPS activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current spec is not clear whether to apply the HARQ-ACK feedback to the first PDSCH after the multicast SPS activation. Without ACK/NACK feedback for the first PDSCH after the activation of multicast SPS, network cannot know whether each UE can receive the following SPS PDSCH(s) and UE may have missed the whole SPS multicast transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Network can configure harq-feedbackEnablingforSPSactive for the first PDSCH after the multicast SPS activation for TN, no need to be together with downlinkHARQ-FeedbackDisabled for NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When downlinkHARQ-FeedbackDisabled is not configured, the harq-feedbackEnablingforSPSactive cannot be configured for the first PDSCH after multicast SPS activation, and the UE may have missed the whole SPS multicast transmission associated with a G-CS-RNTI with disabled feedback. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2, 9.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 9.1.2	Type-1 HARQ-ACK codebook determination

## *** Unchanged text is omitted ***

If a UE is provided downlinkHARQ-FeedbackDisabled indicating disabled HARQ-ACK information for a HARQ process associated with a transport block in PDSCH reception occasion m on serving cell c, the UE reports a NACK value for a HARQ-ACK information bit corresponding to the transport block in a Type-1 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived in clause 9.1.2.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE reports NACK values for HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-1 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.2.1.

If the UE is provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception.

## *** Unchanged text is omitted ***

### 9.1.3	Type-2 HARQ-ACK codebook determination

## *** Unchanged text is omitted ***

If a UE is provided downlinkHARQ-FeedbackDisabled indicating disabled HARQ-ACK information for a HARQ process associated with a transport block for PDCCH monitoring occasion m or for SPS PDSCH receptions on serving cell c, the UE does not multiplex a HARQ-ACK information bit corresponding to the transport block in a Type-2 HARQ-ACK codebook and does not consider the transport block as received in the determination of Nm,creceived or of NSPS,c in clause 9.1.3.1. If the UE is also provided PDSCH-CodeBlockGroupTransmission, the UE does not multiplex HARQ-ACK information bits corresponding to CBGs of the transport block in the Type-2 HARQ-ACK codebook and does not consider the CBGs as received in the determination of Nm,creceived,CBG in clause 9.1.3.1.

If the UE is provided harq-feedbackEnablingforSPSactive = 'enabled', the UE considers a HARQ process associated with a transport block in a first SPS PDSCH reception, after an activation of SPS PDSCH receptions, to have enabled HARQ-ACK information and the UE provides a HARQ-ACK information bit according to a decoding outcome for the transport block in the first SPS PDSCH reception.

## *** Unchanged text is omitted ***