3GPP TSG RAN WG1 #112bis-e			R1-2303574

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.1.2

Source: 	Qualcomm Incorporated

Title: 	Supporting two TAs for multi-DCI based mTRP

Document for:	Discussion/Decision

# Introduction

The Rel-18 WID on NR MIMO evolution for downlink and uplink is approved [1], which includes the following objective:

- Study, and if justified, specify the following

  - Two TAs for UL multi-DCI for multi-TRP operation

  - …

For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In Rel-16/17 mTRP, there can be only one single TA per CC, and it is further assumed that the received signals from different TRPs are within the CP at the UE side. In this contribution, we discuss framework and possible enhancements to enable two TAs in a CC that is configured with multi-DCI based mTRP operation in Section 2, address the impact on random access procedures in Section 3, and discuss switching between intra-cell and inter-cell mTRP operations in Section 4.

# Framework, signaling, and other details for two TAs

Regarding the association of a TAG with UL channels / signals, the following was agreed in RAN1 #112:

Agreement

For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following:

Associate TAG to TCI-state

- Associate TAG ID with UL/joint TCI state

- For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized

- A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG

- Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs

FFS: on how to handle association when Rel-15/16 spatial relation framework is used for

- PUCCH

- DG/CG Type 1/Type 2 PUSCH

- AP/SP/P SRS

Despite a lot of discussions, the following aspects are still not clear from the agreement above:

- Issue 1: Whether the association between TAG ID and UL/joint TCI state is based on explicit configuration as part of TCI state or whether the association defined in sub-agenda 9.1.1.1 for unified TCI is to be used.

- Issue 2: Exact definition of the baseline UE in the agreement above.

- Issue 3: Whether to confirm the working assumption on the additional UE capability or not.

- Issue 4: The FFS for the case of Rel-15/16 spatial relation framework

The issues listed above are somewhat coupled together. Starting with Issue 4, in our view, given the agreement above, it makes sense to limit the applicability of two TA enhancements for multi-DCI based multi-TRP to the unified TCI only. Otherwise, “Option 2” is again needed, and it does not make sense to have different frameworks in the spec for associating UL signals / channels to a TAG ID.

Proposal 1: For multi-DCI based multi-TRP operation with two TAs, only support unified TCI framework.

- This reverts the earlier agreement wrt applicability to “UL beam indication via spatial relation”.

Then, at least for the case of “baseline UE” in the agreement, using the association defined in sub-agenda 9.1.1.1 can address both issues 1 and 2 above. That is, the UL channels/signals that apply the first indicated TCI state are associated with the first TAG, and the UL channels/signals that apply the second indicated TCI state are associated with the second TAG. An alternative to this would be explicit configuration of a TAG ID as part of TCI state configuration, but in that case, the restrictions of the “baseline UE” should be clearly and unambiguously defined. Regarding issue 4, the use case may require more clarification as it does not seem very logical to us to support DPS with different TAs within one coresetPoolIndex, but do not support DPS with different TAs in the absence of coresetPoolIndex configurations (i.e., w/o multi-DCI).

Proposal 2: For the case of “baseline UE”, the association TAG ID and UL/joint TCI state is based on the association defined in sub-agenda 9.1.1.1 for unified TCI, i.e., the UL channels/signals that apply the first indicated TCI state are associated with the first TAG, and the UL channels/signals that apply the second indicated TCI state are associated with the second TAG.

In RAN1 #110-bis-e, the following was agreed with respect to reference timing of DL reception for TA:

Agreement

For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG

- baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length

- as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length

  - FFS: the maximum Rx timing difference (could be up to RAN4)

  - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

One remaining issue is how to determine the two DL reference timing. In legacy, the reception of the first detected path (in time) in a reference CC is considered, where the reference CC is SpCell for PTAG, and any of the CCs in the same TAG for STAG. Assuming unified TCI, similar rule can be followed in the case of multi-DCI except that reception of the first detected path (in time) in the reference CC is based on the DL-RS of the two indicated TCI states corresponding to the two TAGs.

Proposal 3: For multi-DCI multi-TRP operation with two TAs, the two DL reference timings associated with the two TAGs are based on “the first detected path (in time) in the reference CC” corresponding to the two indicated TCI states.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, down-select at least one of the following in RAN1#112bis-e:

- Alt 1:  Introducing a time gap X between two UL transmissions associated with two different TA values

  - E.g., X symbols in the slot(s) corresponding to the two UL transmission remain unused

  - FFS: How X is determined

- Alt 2:  Reduce the overlapping duration of one of the two UL transmissions

- Alt 3:  Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap

- Other alternatives are not precluded

TBD: how to capture the downselected alternative(s) in the specifications in case specification impact is deemed needed.

Regarding overlap between two UL transmissions due to different TAs, the following was agreed in RAN1 #112:

In legacy, UCI multiplexing procedures and dropping rules are based on logical time. Given that all UL channels in a CC have the same TA, there is no overlap in actual time after resolving the overlap in logical time. However, this is no longer the case with two TAGs in the same CC. For example, if UL channel 1 and UL channel 2 do not overlaps in logical time (w/o considering TA) but they overlap in actual time due to different TA values.

Note that the issue above exists also in legacy but only across different slots and only due to a new TA command. The issue is addressed by reducing the duration of a later slot after applying a TA command. This is captured in 38.213 as “If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot”. In comparison, in the case of multi-TA in one CC:

- UE may be able to transmit two different UL signals/channels in a CC simultaneously,

- The overlap is not only across different slots but can be also within a given slot,

- The overlap is not only occasionally due to a new TA command but can be also frequent due to the fact that different UL signals/channels are associated with different TAs.

It is important to note that scheduling restrictions can solve the issue as leaving a symbol gap can be enough since MTTD is less than half a symbol or less than one symbol for most sub-carrier scaping (34.6usec in FR1 and 8.5usec in FR2 based on the RAN4 reply LS). It should be noted that for Rel-16 TDM based multi-DCI with non-ideal backhaul, TRPs need to coordinate semi-statically to ensure that UL signals / channels do not overlap logically. Now, in addition to that, TRPs needs to leave a symbol margin as part of that semi-static coordination to ensure that UL signals / channels do not overlap physically.

Alternatively, if the issue is addressed by UE dropping rule (Alt2), we can have two cases

- Alt 2-1: If the dropped part is from the beginning of the later UL (legacy rule), DMRS is lost if UL transmission has front-loaded DMRS (typical case), and hence, the whole UL is lost.

- Alt 2-2: If the dropped part is from the ending of the earlier UL (which is different than legacy rule), coding rate is impacted (as the last symbol is lost)

In both cases, at least one symbol is lost anyways (unless if the overlapping part is very small). Furthermore, in Alt 2-1, the whole UL channel may be lost if the DMRS is lost. Then, it seems that the scheduling restriction (leaving a symbol gap) is a better approach.

Overall, we think that scheduling restriction may be the easiest approach w/o resulting in losing the whole UL transmission (e.g., which is the case if UE drops part of the front-loaded DMRS symbol, i.e., Alt2-1 above).

Regarding Alt1 versus Alt3 in the agreement above, we think Alt3 should be enough for a scheduling restriction as specification does not need to define “X symbols”. Furthermore, it seems ambiguous whether the gap in Alt3 is based on the logical time (before applying the TA) or actual time (after applying the TA). For Alt3, network needs to ensure the overlapping never happens, which is applicable only to two UL channels / signals that are actually transmitted (i.e., after legacy UCI multiplexing / UL dropping / limitations for UE transmission).

Proposal 4: For multi-DCI based multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, support Alt3 (Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap).

- This applies after UE determines the two UL channels/signals are to be transmitted, i.e., after legacy UCI multiplexing / UL dropping / limitations for UE transmission as described in clause 11.1 of 38.213.

Given that multiple TAGs are also used in legacy UL-CA operation (up to 4 TAGs can be configured for different CCs), some consistency for TAG configurations is needed across different CCs. Among multiple CCs, some may be configured with two coresetPoolIndex values (with multi-DCI based mTRP) while others may be configured with a single value of coresetPoolIndex. For example, for the purpose of HARQ-Ack (separate and joint), there is relationship between coresetPoolIndex value(s) of different CCs for the purpose of HARQ-Ack reporting and codebook construction. In the case of multi-TA in a CC, TAGs configured for different CCs need to be consistent across coresetPoolIndex values of different CCs. Figure 1 illustrates a few examples for consistent and inconsistent configurations, where it is assumed that UE can support up to 4 TAGs overall.  As it can be seen, the top-right and bottom-right examples are not consistent as a same TAG is used in the two CCs but are associated with different coresetPoolIndex values across the two CCs.

Figure 1: Consistency of TA configurations across different CCs.

Proposal 5: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

# Impact on random access procedures

In legacy systems for connected mode UEs, PDCCH order can be used to trigger contention-free random access to acquire UL timing when it goes out-of-sync. The PDCCH order for this purpose can be used either for the PCell (for PTAG) or SCell (for STAG). In either case, UE transmits the PRACH in the cell for which the UL timing re-sync is required in response to the PDCCH order, and then RAR PDCCH/PDSCH is received on the PCell. The RAR PDSCH indicates the absolute TA value (12 bits) for the cell in which the PRACH is transmitted on. This is illustrated in Figure 2.

Figure 2: CFRA triggered by PDCCH order to acquire UL timing for PCell / SCell.

In RAN1 #111, CFRA triggered by PDCCH order was agreed for both intra-cell and inter-cell multi-DCI based mTRP with two TAGs:

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases.

For intra-cell multi-DCI based mTRP, the following alternatives were agreed in RAN1 #110-bis-e:

Agreement

For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111):

- Alt 1:  indicate TAG ID as part of TA command in RAR

- Alt 2:  indicate TAG ID as part of PDCCH order

- Alt 3:  divide SSBs into two groups, one for each TRP.    If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 4:  divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP.

- Alt 5:  divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP

- Alt 6:   TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order

- Alt 7:  Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order

Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

Regarding Alts 3, 4, and 5, such division is not necessary for intra-cell case given that association of SSBs / RACH resources / preambles with the two TRPs is up to the network and is transparent to the UE so far. In fact, the existing CFRA-based PDCCH order is able to trigger PRACH toward either of TRPs for intra-cell mTRP, and the main remaining issue is for the UE to know the absolute TA command in RAR corresponds to which of the two TAGs. Between Alt 6 and Alt 7, Alt 6 does not require additional grouping given that coresetPoolIndex already provides this functionality. Between Alt 2 and Alt 6, Alt2 is preferred given that it is more flexible (it can support cross-TRP PDCCH order even for intra-cell), and it allows for a unified solution between inter-cell and intra-cell cases.

On the other hand, the problem with Alts 2, 6, and 7 is that they are not applicable to UE-initiated CBRA. Given that RAN1 has already agreed that the decision related to UE-initiated CBRA is up to RAN2, selecting any of the Alts 2, 6, or 7 means potential misalignment between RAN1 and RAN2. Even w/o enhancing UE-initiated CBRA, the UE may select an SSB from either the first TRP or the second TRP for CBRA in which case Alt1 is necessary to avoid the confusion that TA command should be applied to which of the two TAGs unless if UE always falls back to single-TAG operation after any UE-initiated CBRA. This fallback behaviour may require some spec impacts and should be discussed in case Alts 2, 6, or 7 are agreed.

Given the discussions above, our first preference is Alt1. Our second preference is Alt2 (to have a unified solution with inter-cell), but in that case, a fallback behaviour as discussed above is needed.

Proposal 6: For CFRA-based PDCCH order in case of intra-cell multi-DCI based multi-TRP operation with two TA enhancement:

- First preference: Support to indicate TAG ID (first TAG or second TAG) as part of TA command in RAR (Alt1)

- Second preference:

  - Support to indicate TAG ID (first TAG or second TAG) as part of PDCCH order (Alt2).

  - UE always falls back to single-TAG operation after any UE-initiated CBRA.

    - FFS: Details.

For inter-cell multi-DCI based mTRP, the following were agreed in RAN1 #110-bis-e and RAN1 #112, respectively:

Agreement

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order

- FFS:  Explicit indication or implicit indication through PDCCH order

Agreement

Confirm the following working assumption:

For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, one additional PRACH configuration is supported for each configured additional PCI

- the additional PRACH configuration is used in a RACH procedure triggered by a PDCCH order for the corresponding configured additional PCI

For determining which PRACH configuration to be used in the PRACH procedure triggered by PDCCH order, explicit indication in the PDCCH order DCI payload is needed given that 7 additional PCIs may be configured. It should be noted that there are already 10 reserved bits in PDCCH order DCI, which can be used for this purpose.

Proposal 7: For CFRA-based PDCCH order in case of inter-cell multi-DCI based multi-TRP operation with two TA enhancement, support PDCCH order DCI payload to indicate a RACH configuration corresponding to the serving cell PCI or one of the configured additional PCIs (explicit indication).

Agreement

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111):

- Alt 1: PDCCH scheduling RAR will always be received from serving cell  there is no need for additional type 1 CSS configuration per additional PCI

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported  additional type 1 CSS configuration per additional PCI needs to be supported

Conclusion

For inter-cell multi-DCI based multi-TRP operation with two TA enhancement, there is no consensus to introduce additional type 1 CSS configuration per additional PCI.

Regarding PDCCH scheduling RAR in case of inter-cell multi-DCI based multi-TRP, the following were agreed  in RAN1 #110-bis-e and RAN1 #112, respectively:

The aspect above (PDCCH scheduling RAR) should be discussed separately for the active additional PCI and inactive additional PCIs (given that in Rel-17 inter-cell mTRP, up to 7 additional PCIs can be RRC-configured, and only one of them can be active). This is because when a PCI is not active, UE does not transmit any UL channel/signal toward that TRP until TCI state activation MAC-CE indicates a TCI state associated with that PCI. Hence, the indication of absolute TA in RAR PDSCH may not be necessary, but it may still be possible to trigger CFRA by PDCCH order (i.e., w/o the subsequent RACH procedures) for the inactive PCI to acquire the timing and get prepared before that PCI is activated. More specifically, we note the following aspects:

- If the multi-DCI based mTRP is defined in a SCell, the PDCCH scheduling RAR (Type 1 CSS) can be only transmitted in the PCell. PCell may not even be configured with multi-DCI based mTRP.

- For an inactive additional PCI, RAR cannot be transmitted from that PCI anyway. Even when RAR is transmitted from an active TRP (e.g., serving PCI), UE should not apply the absolute TA command of RAR since that PCI is still inactive. Hence, the following possibilities may be considered for inactive additional PCI:

  - Possibility 1: RAR is not transmitted to the UE (the random-access ends after PRACH transmission). The TRP associated with the inactive PCI can determine the TA based on measuring the PRACH, and can send TA commands to adjust the timing once it is activated.

  - Possibility 2: RAR is transmitted to the UE (e.g., from the serving cell in PCell) but the UE does not apply it. Instead, UE saves the absolute TA command and applies it after a TCI state associated with that PCI becomes active.

- For an active additional PCI, both Alt1 and Alt2 are possible at least for the PCell. However, in Rel-17 inter-cell mTRP, Type1 CSS cannot be from an additional PCI. Given the conclusion above, our understanding is that Type-1 CSS still needs to come from the serving PCI of PCell. Then, the following two aspects need to be addressed:

  - a) whether additional delay for starting the MSG2 response window would be needed to accommodate the non-ideal backhaul between two TRPs, and

  - b) whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is coming from additional PCI but Type1 CSS is coming from serving PCI.

Hence, we propose the following:

Proposal 8: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If PDCCH order triggers PRACH transmission toward an active additional PCI in the CC, PDCCH scheduling RAR is received from serving cell PCI of the SpCell.

  - FFS: Whether additional delay for starting the RAR response window is needed to accommodate the non-ideal backhaul.

  - FFS: When the CC is SpCell, whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is from additional PCI but Type1 CSS is coming from serving PCI.

- If PDCCH order triggers PRACH transmission toward an inactive additional PCI in the CC, study the following possibilities:

  - Possibility 1: RAR is not transmitted to the UE.

  - Possibility 2: PDCCH scheduling RAR is received from serving cell PCI of the SpCell (same as active additional PCI), but the UE does not apply it until after a TCI state associated with the PCI becomes active.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI.

- FFS: for intra-cell Multi-DCI

- FFS: whether there are any restrictions needed

- FFS: if cross TRP RACH triggering is an optional feature

In RAN1 #111, the following was agreed with respect to cross-TRP PDCCH order:

One remaining issue is the PRACH power control in case of cross-TRP PDCCH order. In existing spec, PL-RS and referenceSignalPower for PRACH power control are based on the DL-RS that the DMRS of PDCCH order is QCLed with. However, this rule is not applicable to cross-TRP PDCCH order. In order to understand how the issue should be resolved, RAN1 first needs to decide for which scenarios the cross-TRP PDCCH order should be allowed in case of inter-cell mTRP.

Let’s call this legacy power control rule for PRACH in case of CFRA-based PDCCH order as “Rule 1”. Let’s also call the QCL rule between PDCCH order and RAR in case of CFRA-based PDCCH order in PCell as “Rule 2”, and further assume that PDCCH scheduling RAR is received from serving cell PCI of the SpCell as discussed above. Furthermore, let’s assume coresetPoolIndex value 0 (TRP0) is associated with the serving cell PCI (PCI0), and coresetPoolIndex value 1 (TRP1) is associated with PCI1 (active additional PCI). Hence, RAR is always from TRP0. Considering the sequence of PDCCH order  PRACH  RAR, we have the following cases for the three different scenarios of triggering PRACH toward serving cell PCI (PCI0), active additional PCI (PCI1), and inactive additional PCI (PCI2):

| PRACH toward | PDCCH order PRACH RAR | Rule 1 | Rule 2 |
| --- | --- | --- | --- |
| PCI0 (serving PCI) | Case 1: TRP0 PCI0 TRP0 | Ok | Ok |
|  | Case 2: TRP1 PCI0 TRP0 | Violated | Violated |
| PCI1 (active additional PCI) | Case 1: TRP0 PCI1 TRP0 | Violated | Ok |
|  | Case 2: TRP1 PCI1 TRP0 | Ok | Violated |
| PCI2 (inactive additional PCI) | Case 1: TRP0 PCI2 TRP0 | Violated | Ok |
|  | Case 2: TRP1 PCI2 TRP0 | Violated | Violated |

As it can be seen from the table above:

- When PRACH is triggered for serving cell PCI, cross-TRP PDCCH order is not needed, and Case 1 is enough, which does not violate Rule 1 or Rule 2.

- When PRACH is triggered for active additional PCI, both Case 1 and Case 2 can be supported:

  - Case 1 corresponds to cross-TRP PDCCH order and has the benefit that Rule 2 is not violated. However, Rule 1 is violated and an alternative rule is needed.

  - Case 2 corresponds to same-TRP PDCCH order and should be supported as agreed before.

- When PRACH is triggered for inactive additional PCI, Case 1 can be enough. This assumes that RAR is transmitted. Otherwise, Rule 2 becomes irrelevant and Case 1 and Case 2 become similar. Note that whether RAR is transmitted for this scenario is a separate discussion (as discussed in detail before). In any case, Rule 1 is violated, and an alternative rule is needed.

Hence, cross-TRP PDCCH order should be supported for both active additional PCI and inactive additional PCI. For the case of active additional PCI and in Case 1 above, the PRACH power control can be based on the DL-RS of either the first indicated TCI state or the second indicated TCI state that is associated with the active additional PCI. For the case of inactive additional PCI, the PRACH power control can be based on the SSB index that is indicated by PDCCH order DCI. However, note that this assumes that the UE has already measured that SSB index from that inactive additional PCI. Hence, it should be configured for L1-RSRP report. In general, for the inactive additional PCI, the same solution as in Rel-18 mobility (LTM) can be followed.

Proposal 9: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If the serving cell PCI is indicated by PDCCH order DCI, the legacy procedure is applied for PRACH power control (cross-TRP PDCCH order is not allowed).

- If the active additional PCI is indicated by PDCCH order DCI:

  - If the PDCCH order DCI is from the active additional PCI (same-TRP PDCCH order): The legacy procedure is applied for PRACH power control.

  - If the PDCCH order DCI is from the serving PCI (cross-TRP PDCCH order): The PRACH power control is based on the DL-RS of the indicated TCI state that is associated with the active additional PCI.

- If an inactive additional PCI is indicated by the PDCCH order DCI (only cross-TRP PDCCH order is possible), the PRACH power control is based on the SSB index that is indicated by the PDCCH order DCI, and the UE expects the SSB index to be configured for L1-RSRP report.

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support enhancements related to indicating TAG ID via absolute TA command:

- FFS: whether the indication is implicit or explicit

- Detailed indication schemes are FFS

- This does not preclude indication of two TAG IDs (if supported)

- Note: This applies at least to MSGB in case of C-RNTI

In 2-step RACH, the “Absolute Timing Advance Command MAC CE” can be included in a PDSCH scheduled by a PDCCH in response to MSGA transmission (in case of C-RNTI). This MAC-CE currently has two octets with 12 bits for TA command and 4 reserved bits. In RAN1 #110-bis-e, the following was agreed:

Our understanding is that the agreement above is specific to “Absolute Timing Advance Command MAC CE” for SpCell (given that 2-step RACH can be only configured for the SpCell). Hence, we propose the following:

Proposal 10: For SpCell configured with two TAGs, for MSGB in case of C-RNTI, support using a reserved bit of the “Absolute Timing Advance Command MAC CE” to indicate whether the TA command corresponds to the first TAG or the second TAG.

# Switching between intra-cell and inter-cell mTRP

In Rel-17 inter-cell mTRP, up to 7 additional PCIs can be configured while one of them may be activated based on TCI state activation MAC-CE. The MAC-CE can also switch between intra-cell and inter-cell mTRP based on whether both coresetPoolIndex values have activated TCI states associated with the serving cell PCI, or one coresetPoolIndex value has activated TCI states associated with serving cell PCI and the other coresetPoolIndex value has activated TCI states associated with an additional PCI.

At the same time, the configuration of two TAGs for a CC configured with multi-DCI based mTRP and additional PCIs is RRC-based. Then, depending on the deployment scenario, network should be able to operate in one of the following assumptions:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

  - With this assumption, CFRA-based PDCCH order should be able to acquire TA for serving cell PCI associated with coresetPoolIndex value 0, serving cell PCI associated with coresetPoolIndex value 1, additional PCI #1, additional PCI #2, …, additional PCI #7 (up to 9 possibilities).

  - Note that only two TAs out of the 9 possibilities above are active at any given time (which correspond to the two maintained TAGs) depending on intra-cell mTRP or inter-cell mTRP as well as which additional PCI is active.

- Assumption 2: UE maintains only one TAG if all active TCI states across both coresetPoolIndex values are associated with the serving cell PCI.

  - With this assumption, one TAG is associated with the serving cell PCI, and another TAG is associate with an additional PCI. Hence, UE switches between single-TAG and two-TAGs depending on whether the UE is in intra-cell or inter-cell mTRP mode.

Proposal 11: If a CC is RRC-configured with two TAGs and one or more additional PCIs with multi-DCI based mTRP operation, study the following assumptions when TCI state activation MAC-CE(s) result in switching between intra-cell and inter-cell multi-DCI based mTRP:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

- Assumption 2: UE maintains only one TAG when MAC-CE switches the operation from inter-cell mTRP to intra-cell mTRP.

# Conclusion

In this contribution, we proposed:

Proposal 1: For multi-DCI based multi-TRP operation with two TAs, only support unified TCI framework.

- This reverts the earlier agreement wrt applicability to “UL beam indication via spatial relation”.

Proposal 2: For the case of “baseline UE”, the association TAG ID and UL/joint TCI state is based on the association defined in sub-agenda 9.1.1.1 for unified TCI, i.e., the UL channels/signals that apply the first indicated TCI state are associated with the first TAG, and the UL channels/signals that apply the second indicated TCI state are associated with the second TAG.

Proposal 3: For multi-DCI multi-TRP operation with two TAs, the two DL reference timings associated with the two TAGs are based on “the first detected path (in time) in the reference CC” corresponding to the two indicated TCI states.

Proposal 4: For multi-DCI based multi-TRP operation with two TA enhancement, for the case when the UE does not support UL STxMP transmission, support Alt3 (Scheduling restriction is applied such that the UE does not expect the two UL transmissions to overlap).

- This applies after UE determines the two UL channels/signals are to be transmitted, i.e., after legacy UCI multiplexing / UL dropping / limitations for UE transmission as described in clause 11.1 of 38.213.

Proposal 5: Study necessary conditions to ensure consistency of TA configurations across different CCs. Use case for each configuration needs to be justified.

Proposal 6: For CFRA-based PDCCH order in case of intra-cell multi-DCI based multi-TRP operation with two TA enhancement:

- First preference: Support to indicate TAG ID (first TAG or second TAG) as part of TA command in RAR (Alt1)

- Second preference:

  - Support to indicate TAG ID (first TAG or second TAG) as part of PDCCH order (Alt2).

  - UE always falls back to single-TAG operation after any UE-initiated CBRA.

    - FFS: Details.

Proposal 7: For CFRA-based PDCCH order in case of inter-cell multi-DCI based multi-TRP operation with two TA enhancement, support PDCCH order DCI payload to indicate a RACH configuration corresponding to the serving cell PCI or one of the configured additional PCIs (explicit indication).

Proposal 8: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If PDCCH order triggers PRACH transmission toward an active additional PCI in the CC, PDCCH scheduling RAR is received from serving cell PCI of the SpCell.

  - FFS: Whether additional delay for starting the RAR response window is needed to accommodate the non-ideal backhaul.

  - FFS: When the CC is SpCell, whether the existing QCL relationship in SpCell between PDCCH order DCI and RAR PDCCH/PDSCH needs to be relaxed when PDCCH order is from additional PCI but Type1 CSS is coming from serving PCI.

- If PDCCH order triggers PRACH transmission toward an inactive additional PCI in the CC, study the following possibilities:

  - Possibility 1: RAR is not transmitted to the UE.

  - Possibility 2: PDCCH scheduling RAR is received from serving cell PCI of the SpCell (same as active additional PCI), but the UE does not apply it until after a TCI state associated with the PCI becomes active.

Proposal 9: For inter-cell multi-DCI based multi-TRP operation with two TAGs in a CC:

- If the serving cell PCI is indicated by PDCCH order DCI, the legacy procedure is applied for PRACH power control (cross-TRP PDCCH order is not allowed).

- If the active additional PCI is indicated by PDCCH order DCI:

  - If the PDCCH order DCI is from the active additional PCI (same-TRP PDCCH order): The legacy procedure is applied for PRACH power control.

  - If the PDCCH order DCI is from the serving PCI (cross-TRP PDCCH order): The PRACH power control is based on the DL-RS of the indicated TCI state that is associated with the active additional PCI.

- If an inactive additional PCI is indicated by the PDCCH order DCI (only cross-TRP PDCCH order is possible), the PRACH power control is based on the SSB index that is indicated by the PDCCH order DCI, and the UE expects the SSB index to be configured for L1-RSRP report.

Proposal 10: For SpCell configured with two TAGs, for MSGB in case of C-RNTI, support using a reserved bit of the “Absolute Timing Advance Command MAC CE” to indicate whether the TA command corresponds to the first TAG or the second TAG.

Proposal 11: If a CC is RRC-configured with two TAGs and one or more additional PCIs with multi-DCI based mTRP operation, study the following assumptions when TCI state activation MAC-CE(s) result in switching between intra-cell and inter-cell multi-DCI based mTRP:

- Assumption 1: UE maintains two TAGs irrespective of intra-cell or inter-cell mTRP.

- Assumption 2: UE maintains only one TAG when MAC-CE switches the operation from inter-cell mTRP to intra-cell mTRP.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung.