3GPP TSG RAN WG1 #112bis-e		R1-2303582

April 17th – 26th, 2023

Agenda item:	9.2.2.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation on AI/ML for CSI feedback enhancement

Document for:		Discussion and Decision

# Introduction

In this contribution, we present evaluation results of the benefits of AI/ML-based CSI feedback using two-sided model.

# System-level evaluation for CSI compression

In this section, we show performance improvements of ML-based CSI feedback over eType2 for full buffer and bursty traffic. We plot the mean and edge throughputs (for full buffer) and mean and edge user experiences (for bursty traffic) against the number of bits required on the UL for PMI feedback, based on UE mean requested rank. For Release 16 eType2, we have used the maximum number of non-zero coefficients (K0 for rank 1, 2K0 for rank > 1) to compute the required PMI bits on the UL.

Table 1 shows the simulation assumptions used in the evaluation.

Table 1: System Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Scenario | Dense Urban |
| Bandwidth | 20 MHz |
| Sub-carrier Spacing, Sub-band size | 30 KHz, 4 RBs/Sub-band (13 SB/Bandwidth) |
| Antenna setup and port layouts at gNB | 32 ports: (8,8,2,1,1,2,8) (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4RX: (1,2,2,1,1,1,2) (dH,dV) = (0.5, 0.5)λ |
| Channel Estimation | Realistic |
| Interference Estimation | Realistic |
| eType2 parameter combinations | PC 1 through 8 |

Figure 1 below shows the mean UE throughput vs the UL PMI overhead required for eType2 as well as for ML-based CSI feedback. The overhead gain from ML-based CSI feedback is around 50%, at a mean user throughput of around 14.3 Mbps. Figure 2 shows the cell-edge UE throughput vs the UL PMI overhead required. The overhead gain from ML-based CSI feedback is around 32% at an edge user throughput of around 2.6 Mbps.

Figure 1: Mean UE throughput vs UL PMI overhead

Figure 2: Edge UE throughput vs UL PMI overhead

For bursty traffic, we show the performance gains from ML-based CSI feedback at three values of resource utilization: approximately 10%, 40% and 80% in the curves below. Figure 3 and Figure 4 shows the mean and cell-edge user experience for Release 16 eType2 and ML-based CSI feedback around 10% resource utilization. Figure 5 and Figure 6 show the same for 40%, and Figure 7 and Figure 8 show the same for 80% resource utilizations.

At 10% RU, we see a 63% improvement in UL overhead at a mean user experience of around 150 Mbps. At 55% improvement in UL overhead, the edge user experience is around 80 Mbps. At 40% RU, we observe a 60% improvement in UL overhead at a mean user experience of 88 Mbps. At an edge user experience of around 38 Mbps, the gain in UL overhead is 51%. At 80% RU, we observe a 60% improvement in UL overhead at a mean user experience of around 45 Mbps, and 70% improvement in UL overhead at an edge user experience of around 11 Mbps.

Figure 3: Mean User Experience vs UL PMI OH (~10%RU)

Figure 4: Edge User Experience vs UL PMI OH (~10%RU)

Figure 5: Mean User Experience vs UL PMI OH (~40%RU)

Figure 6: Edge User Experience vs UL PMI OH (~40%RU)

Figure 7: Mean User Experience vs UL PMI OH (~80%RU)

Figure 8: Edge User Experience vs UL PMI OH (~80%RU)

AI/ML based CSI compression gives UL overhead gains of around 50% for mean throughputs (at a mean throughput around 14.3 Mbps) and 32% for cell-edge throughputs (around 2.6 Mbps).

For Bursty Traffic,

- At 10% RU, we see a 63% improvement in UL overhead at a mean user experience of around 150 Mbps. At 55% improvement in UL overhead, the edge user experience is around 80 Mbps.

- At 40% RU, we observe a 60% improvement in UL overhead at a mean user experience of 88 Mbps. At an edge user experience of around 38 Mbps, the gain in UL overhead is 51%.

- At 80% RU, we observe a 60% improvement in UL overhead at a mean user experience of around 45 Mbps, and 70% improvement in UL overhead at an edge user experience of around 11 Mbps.

# Discussion on the result templates

The following is a portion of the table that was accepted as a working assumption in RAN1 #112 ([5]) for companies to use to report results for CSI compression.

| SGCS of benchmark, [layer 1] | X: <=80bits |
| --- | --- |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 2] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 3] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| SGCS of benchmark, [layer 4] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Gain for SGCS, [layer 1] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Gain for SGCS, [layer 2] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Gain for SGCS, [layer 3] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |
| Gain for SGCS, [layer 4] | X: <=80bits |
|  | Y: 100bits-140bits |
|  | Z: >=230bits |

When reporting results in this section of the table, there may be two approaches:

- Option 1: Compute SGCS for each layer assuming all layers up to the maximum rank are compressed using the payload

- Option 2: Compute SGCS for layer k only from channel samples when the rank is at least k

Option 2 would require rank selection to be implemented and may require SNR and interference modelling. It further implies that the CSI feedback overhead for a sample of layer k would depend on the rank associated with that sample. For example, if we compare a sample of layer 2 from a rank 2 scenario and a sample of layer 2 from a rank 3 scenario, the per-layer overhead would be different. Therefore, option 2 is complicated, and option 1 is preferable.

We propose the following related to the reporting of results in this table.

When providing results for the SGCS of the benchmark and the gain for SGCS for each layer, compute SGCS and the CSI feedback overhead assuming Max Rank for every UE for both the benchmark and AI/ML-based CSI feedback.

The range of overhead for the third bin has been defined as Z: >= 230 bits. However, when the maximum rank is 4, for eType-II, even the highest parameter combination (PC6) does not result in a per-layer overhead that could be associated to this range.

Using the maximum number of non-zero coefficients, the total PMI overhead for Release 16 eType II for parameter combination (PC) 6 is 557 bits. For Rank 4, this would imply the per-layer overhead is below 140 bits. Hence it would not be possible to fill the “Z” bin for this case using the current template.

The definition of the bins (i.e., X, Y, Z ranges) for the result templates should consider whether a particular overhead is possible for the benchmark case for a given Max Rank.

- Hence, the bin-identifiers (X, Y and Z) should be specified for each Max Rank value separately.

- Alternatively, replace X/Y/Z with the overhead for eType II parameter combinations. For each Max Rank value, the overhead for Release 16 eType2 Benchmark could be computed for PC 1 through 6.

Gain from AI/ML-based CSI feedback can also be measured in terms of CSI feedback overhead. As shown in observations above (Figure 1 through Figure 8), we observe double digit gains in CSI feedback overhead for both full buffer and FTP traffic. Consider for instance, the bin at X = 60 bits. At this overhead, assume that the benchmark scheme gives a UPT of T Mbps. To achieve this UPT, usage of AI/ML-based CSI feedback would require fewer bits as CSI feedback overhead. We propose that this reduction in CSI feedback overhead could be reported by companies in the spreadsheet for each bin. A snippet of the table is shown below.

For each bin that identifies the benchmark CSI feedback overhead, companies should report the reduction in CSI feedback overhead from the usage of AI/ML-based CSI feedback over the benchmark CSI feedback method.

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) (Mean UPT based measurement) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

# Model performance monitoring

It has been agreed to study various performance monitoring methods, including monitoring based on inference accuracy, system performance and data distribution (input or output).  In this section, we elaborate our views on the necessity and feasibility of these options.

The goal of model monitoring is to detect and address the cases where the model performance is not adequate to ensure good user experience. As discussed in [4], performance issues for AI/ML models may arise from several sources:

- Bad training/validation dataset

- Bad model design/training

- Imperfect model selection and switching

- Training and target platform differences

- Data distribution shift

- Unexpected events

The above issues arise at different steps of the AI/ML LCM pipeline and at different frequencies, and therefore they demand different solutions.

As discussed in [4], most of the issues that cause inadequate model performance can be addressed using testing, validation and monitoring solutions that happen offline during the model development stage. If a model has been developed in this manner, then real-time monitoring of the model after it has been activated for use should not be necessary. If there is a shift in the data distribution, that can be detected using non-real-time monitoring of KPIs at a training server or at the UE / NW. Based on this discussion, the necessity for specifying real-time model monitoring is not clear.

Real-time performance monitoring that incurs overhead and/or additional processing complexity is unnecessary.

Besides, real-time monitoring to detect and address an unexpected model performance issue may not be feasible, as it comes with a cost of increased overhead and processing complexity.

For two-sided CSI feedback, the target CSI and output CSI are generated at UE side and NW side separately. To enable monitoring based on inference accuracy, the UE could either run NW-side decoder or provide the target CSI to the NW-side.

The option of UE reporting the target CSI to the NW-side over the air-interface can only provide a delayed view of the model performance to the NW due to the processing and reporting delays. Moreover, to prevent false alarms due to outlier samples (even within the training distribution), sufficient averaging may be required, and this may result in additional latency. Such latency can prevent timely detection and response.

Also, to detect a performance issue quickly, frequent periodic reports would be needed, but frequent triggers can result in large overhead. This may significantly reduce the overhead reduction benefit of AI/ML-based CSI compression. On demand triggering based on a performance degradation event would anyway result in a delayed detection after the issue has happened.

In contrast, an alternate approach can be considered where the monitoring happens on the UE-side based on the input channel samples. Such an approach does not incur the overhead associated with reporting target CSI. The option of UE running the NW-side model may be feasible only for UEs that can handle the added complexity. In the following sections, we discuss two such schemes along with evaluation results.

Model monitoring based on ground-truth provided by UE to the network requires large signalling overhead and may be sensitive to large latency.

For model performance monitoring, specification change for reporting the target CSI with high resolution from UE to network requires clear justification as it incurs additional overhead and may not be necessary.

## Monitoring based on intermediate KPI prediction at UE-side

In this section, we develop a proxy at the UE side which takes the latent information generated inside the UE-side model as input and outputs a predicted intermediate KPI (i.e., SGCS) directly. The total number of weights in the proxy model is around 13k. The block diagram is shown in Figure 9.

Figure 9: Proxy model on UE-side to estimate intermediate KPI

Figure 9 illustrates the comparison between the predicted SGCS compared and the actual SGCS value resulted by the reconstructed CSI at NW side. For illustration purpose, we produce the results by randomly selecting 100 samples from total 171000 samples in the testing set. It can be seen that the predicted SGCS value and actual SGCS value are very close. Figure further shows that the CDF of the SGCS prediction and the CDF of the actual SGCS almost overlap. The average gap between the predicted SGCS and actual SGCS is around ± 0.019.

Figure 10:    Left: predicted SGCS vs. actual SGCS;    Right: SGCS CDF

The proxy-based model monitoring can be designed to monitor UE-side model only or designed to monitor UE-side and NW-side model jointly, depending on whether the SGCS ground-truth label for training the proxy model is generated by UE autonomously or provided by NW.

- For training the proxy model for the purpose of monitoring UE-side model only, UE may develop a private decoder which generates a reconstructed CSI, and the SGCS label is computed using this reconstructed CSI and its ground-truth. With this SGCS label and its associated input sample, UE is able to develop the proxy model. The reference decoder can be developed in the first step of UE-first Type3 training or developed together with the UE-side model in the last step of NW-first Type3 training.

- For training the proxy model for the purpose of monitoring both UE-side and NW-side model jointly, the training would need to use SGCS labels derived from the reconstructed CSI output by the actual CSI decoder deployed at the NW side. The NW-side can provide the reconstructed CSI output or SGCS labels along with the necessary information exchange in the Type3 training.

Model monitoring using a proxy model that outputs the intermediate KPI directly shows an accurate inference accuracy prediction.

Study specification impact of methods that directly outputs intermediate KPI at the UE side.

## Input-based monitoring

Thus, monitoring based on system performance and data distribution are more interesting. System-performance-based approach (e.g., throughput, BLER) is generic for all kinds of CSI feedback including the legacy codebook-based approach, it may have no specification impact. For data-distribution-based approach, UE calculates the target CSI (or input) based on the input sample measured in inference phase, and then compares it with the counterpart of the training set.

An example of data-distribution-based (input-based) approach is elaborated as following. Training samples are partitioned into two groups, the participation can be based on clustering methods, data statistics (e.g., angle, delay spread, doppler) or the meta-information provided in data-collection phase. One can see from Figure 10 that the distribution of the distance of a sample from samples belonging to an unmatched group is biased from the distribution of the distance to samples belonging to its matched group. Therefore, it is feasible for UE to differentiate samples based on distance or probability assessment.

Figure 11: Distribution of sample distances from two groups in the training data

To show the relationship between the monitoring metric and inference performance, we first divide the testing samples to matched samples and unmatched samples where matched samples are those belong to group 1 and unmatched samples are those do not belong to group 1. Then, we further divide matched samples and unmatched samples based on the distance to group 1. Hence, there are 4 parts in the testing data: 1) matched samples and within a distance, 2) matched samples and beyond a distance, 3) unmatched and within a distance to group 1, and 4) unmatched samples and beyond a distance to group 1.

In Figure 11, we plot the inference accuracy using the model trained by training data group 1. We can see that matched/unmatched samples within the threshold outperform those beyond the threshold. This result implies a good relationship between the distance and inference accuracy – longer distance yields lower accuracy than shorter distance. Besides, matched samples within the threshold (part 1) and unmatched samples within the threshold (part 3) achieve similar performance. This result implies that inferencing with unmatched samples yields acceptable performance if the unmatched samples are within a certain distance to the training data. The same exercise has been applied to matched/unmatched samples for group 2 and similar observations are obtained.

Figure 12: Inference accuracy of testing data based on matching with training data and the distance to the training data

Model monitoring based on metrics derived by comparison between input samples at inference and training samples can have strong relationship with the inference accuracy. As a result, input-based monitoring appears promising.

Study specification impact of input-based model monitoring on the UE-side by comparing input samples at inference time to the training samples.

# Localized models

In this section, we present results on the approach of model development that involves developing localized models, i.e., models developed specifically for use within a local region.

For this evaluation, we consider two cases for the training dataset –

- Case 1 (global data training):

  - UE-side encoder and NW-side decoder are trained using a dataset from the full dense urban layout.

- Case 2 (local data training):

  - UE-side encoder and NW-side decoder are trained using data from UEs that are local to a small region within the layout (Indoor UEs from a 25 m. by 25 m. region were used for this purpose).

Due to spatial consistency, the channel samples collected from a local region are expected to be more correlated and hence more compressible as compared to the overall data from the entire layout.

Figure 12 shows the SGCS (expressed as a loss metric in dB scale) for layer 1 for the two cases described above, when those models are used for inference on the local dataset. The result is shown for different model sizes.

Figure 13: Performance of localized model

The result shows that the SGCS improves by using the localized model as compared to the model trained on the global dataset. Alternately, the same performance can be achieved using smaller models using the localized model approach.

Localized models, which are developed specifically for use within a local region, provide better performance compared to global models of comparable size trained using data from the entire layout.

Localized models provide similar performance with a smaller model size compared to global models.

Based on this discussion, we have the following proposal:

Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

# Impact of quantization awareness for training

The following agreement was made during RAN1#110b-e, in the Other Aspects agenda:

Agreement In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB: - Alignment of the quantization/dequantization method and the feedback message size between Network and UE

The following was agreed in RAN1-111 related to the evaluation of quantization-aware training:

Agreement For the evaluation of quantization aware/non-aware training, the following cases are considered and reported by companies: - Case 1: Quantization non-aware training, where the float-format variables are directly passed from CSI generation part to CSI reconstruction part during the training - Fixed/pre-configured quantization method/parameters is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2: Quantization aware training, where quantization/dequantization is involved in the training process - Case 2-1: Fixed/pre-configured quantization method/parameters are applied during the training phase; the same quantization codebook is applied for the inference phase - Companies to report the design of the fixed/pre-configured quantization method/parameters, e.g., quantization resolution, vector quantization codebook, etc. - Case 2-2: The quantization method/parameters are updated in together with the AI/ML models during the training; when training is finished, the final quantization codebook is applied for the inference phase - Companies to report how to update the quantization method/parameters during the training - Note: the above cases apply for training Type 1/2/3 - Others are not precluded.

In this section, we present our evaluation results on this aspect. The SGCS metric is shown for different variations on the training approach and quantization method. The payload size was set to 128 bits. The results are based on joint training of the UE-side and NW-side models. For case 2-2, the UE-side model includes the quantization method and the NW-side model includes the dequantization method, and they are jointly trained.

Table 2: Results on quantization-aware/non-aware training

| NN Train type | Case | Quantization |  | SGCS |  |
| --- | --- | --- | --- | --- | --- |
|  |  | method | training/config | Linear | dB |
| Jointly training Enc-Dec (Quant non-aware) | 1 | fixed SQ | uniform (parameter config 1) | 0.648 | -4.54 |
| Jointly training Enc-SQ_fixed-Dec | 2-1 | fixed SQ | uniform (parameter config 1) | 0.796 | -6.91 |
| Jointly training Enc-SQ_fixed-Dec | 2-1 | fixed SQ | uniform (parameter config 2) | 0.738 | -5.83 |
| Jointly training Enc-SQ-Dec | 2-2 | trained SQ | joint with Enc/Dec | 0.798 | -6.95 |
| Jointly training Enc-VQ-Dec | 2-2 | trained VQ | joint with Enc/Dec | 0.828 | -7.65 |

Quantization non-aware training (case-1) leads to noticeable performance degradation compared with quantization aware training (case-2).

For quantization aware training, fixed or pre-configured quantization (case 2-1) is more sensitive to quantization’s parameters/configuration compared with trainable quantization (case 2-2). That is, quantization’s parameters/configuration in case 2-1 need to be carefully chosen to align with statistical distribution of latent vector (z), otherwise performance is degraded.

Trainable quantization offers more flexibility and better performance compared to fixed quantization, e.g., trainable vector quantization can improve the performance.

Quantization method should be considered a part of the UE-side model and dequantization method should be considered a part of the NW-side model. The quantization method should be aligned for good performance, but there is no need for separate specification support to align the quantization method.

# Impact of device variations on performance

In practice, the data corresponding to different types of devices may have different characteristics. The source of such differences could be from device construction, RF aspects, implementation differences across vendors or device models or chipsets, etc. Training data may be obtained from one type of device to develop models. If such models are used for inference on another type of device, then the discrepancy in the data distribution between training and inference can impact the performance of the model.

In this section, we present evaluation results on this aspect and discuss how the performance can be improved.

Table 3: Impact of mismatch between training and inference data distribution due to device variations

|  | SGCS (Training on device type A data) |
| --- | --- |
| Device Type A | 0.839 |
| Device Type B | 0.810 |
| Device Type C | 0.627 |
| Device Type D | 0.798 |

Table 3 shows the average SGCS for 4 types of devices. Data from device type A was used for training. The value shown in the table is the SGCS when the data from the corresponding device type was used for inference. The differences modelled among the 4 cases are only related to device-specific aspects such as channel estimation, antenna imbalance, SVD implementation, etc. Other aspects such as scenario and configuration are identical across the different cases. The result shows that the performance of the two-sided model can vary considerably due to device variations.

The performance of AI/ML-based CSI compression using two-sided model can vary considerably if there is a discrepancy between the training data and inference data due to device-side variations.

The above observation implies the need of new models to be developed via the data corresponding to the new devices (e.g., device types B, C, D). However, if the network-side models have already been deployed, it would be preferable if the new types of devices can be accommodated without a need to deploy new models on the network side, i.e., in a backward compatible manner.

To this end, a gradient-exchange based Type 3 training can be conducted. In this approach, NW-side entity keeps the NW-side model frozen but opens an API for the UE-side training entity to compute the gradients. With the gradients provided via the API, the UE-side is able to train the new UE-side models for the new devices with their own corresponding data matched to that device type. This method allows simple operation at NW-side (no model training needed) and does not need data collection from new devices to the NW-side.

Table 4 shows the average SGCS for device type D after gradient-exchange based Type 3 training. The second column presents the average SGCS of testing using NN designed with data of device type 1. The third column presents the average SGCS achieved by Type 3 training (with decoder frozen as for device type 1). The fourth column presents the average SGCS achieved by training the encoder-decoder pair with mixed dataset, which acts as an upper-bound for both device type A and D. The result shows that gradient-exchange based Type 3 training can enhance the performance of device type D and its performance is comparable with device type A.

Table 4: SGCS of various device types under gradient-exchange based NW-first Type 3 training

| SGCS | Training dataset: from device type A (encA-decA) | Training dataset: from device type D but with dec frozen (encD-decA) | Training dataset: mixed dataset from device type A and D (encA&D-decA&D) |
| --- | --- | --- | --- |
| Inference: Device Type A | 0.839 | - | 0.848 |
| Inference: Device Type D | 0.798 | 0.83 | 0.842 |

Gradient-exchange based Type 3 training enables the training of encoders for new devices in a scalable and backward compatible manner without requiring updates of the NW-side models and avoids data distribution mismatch issues.

# Impact of ground truth quantization

The following agreements were made related to the evaluation of the impact of ground truth quantization for the CSI compression sub-use-case:

Agreement For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, Float32 is adopted as the baseline/upper-bound of performance comparison. Agreement For the evaluation of the high resolution quantization of the ground-truth CSI in the CSI compression, if R16 Type II-like method is considered, companies to report the R16 Type II parameters with specified or new/larger values to achieve higher resolution of the ground-truth CSI labels, e.g., L, pv, β, reference amplitude, differential amplitude, phase, etc.

In this section, we present results for the ground truth quantization study comparing the SGCS metric for the rank 1 case for the dense urban scenario. For the baseline, we use the training dataset in floating point representation. We then construct another dataset where each training sample is based on the representation of the corresponding original precoding vector using R16 Type II codebook with parameter combination 8.

Table 5 shows the SGCS achieved for different CSI feedback payload size settings.

Table 5: SGCS for different types of training dataset quantization

| Payload size (bits) | Avg. SGCS for ML-CSI feedback, trained using floating point training dataset | Avg. SGCS for ML-CSI feedback, trained using R16 Type II PC8 based training dataset |
| --- | --- | --- |
| 32 | 0.668 | 0.672 |
| 64 | 0.735 | 0.726 |
| 128 | 0.786 | 0.774 |
| 512 | 0.950 | 0.940 |

The SGCS achieved with a training dataset quantized using R16 Type II PC 8 is very close to the SGCS achieved using the ideal (floating point) training dataset for the rank 1 case with dense urban scenario.

During data collection phase, the UE first receives and processes CSI-RS samples. The training process typically happens offline at a server and does not require tight latency guarantees. The data collected by the UE can therefore be delivered to the training entity over-the-top without burdening the control resources that are needed for control signalling for ongoing communications. If a training entity on the NW-side requires data for training, the same process can be followed.

Alternatively, the NW-side can construct a dataset based on the existing R16 Type II CSI feedback. Our results in Table 5 show that this approach can also provide good performance similar to using the original unquantized dataset. This can be explained by the fact that for good performance, we only require that the dataset be sufficiently large and diverse to cover the distribution of the CSI realizations in the scenario of interest. Moreover, the CSI derived by the UE is already a noisy version based on the estimated channel, and it is not clear why that needs to be conveyed with high resolution for training purposes.

Considering these aspects, the need to specify new values for R16 Type II parameters to achieve higher resolution of the ground-truth CSI labels is unclear.

For training data collection, specifying new/larger values of R16 Type II parameters to achieve higher resolution of the ground truth CSI needs clear justification.

# Comparison of offline training scenarios

In RAN1#110, the following was agreed regarding AI/ML model training:

Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, repectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW Other collaboration types are not excluded.

In this section, we compare the performance of models trained under these different types of offline training.

- Type 1 training:

  - Joint training of the two-sided model at a single side/entity (at the same time in a single training session)

Figure 14: Type 1 offline training

- Type 2 training:

  - Joint training of the two-sided model at network side and UE side, respectively (at the same time in a single training session)

Figure 15: Type 2 offline training

- Type 3 training:

  - Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively (in different training sessions, with collaboration outside the training process to ensure compatibility of the two-sided models)

Figure 16: Type 3 offline sequential training starting with UE side training

Figure 17: Type 3 offline sequential training starting with NW side training

## Single UE-side model, single NW-side model

We first consider the case of training the UE-side and NW-side model of a two-sided model involving one UE vendor and one NW vendor.

### Type 2 training

Since Type 1 training is managed by a single entity, the UE-side model structure and NW-side model structure can be chosen in a matched way – for example – both models can be chosen to have a transformer structure. With Type 2 training, if the two model structures are the same, the performance is expected to be essentially the same as Type 1 training.

However, with Type 2 training, the UE-side and NW-side model structures may not be disclosed but instead kept proprietary. This raises the question of whether it is feasible to achieve good end-to-end ML model performance of the two-sided model if the two structures are not matched – for example – if one side uses a convolutional neural network (CNN) while the other side selects a transformer-based architecture. We present results comparing these scenarios.

Figure 18: Type 2 offline training is feasible

The result shows that the distributed offline training scenario results in an SGCS value of 0.825 even though the UE-side and NW-side model structures were mismatched (transformer and CNN). The Type 1 training which uses matched structure (CNN) for both UE-side and NW-side model achieves an SGCS of 0.829.

Type 2 offline training of the UE-side model and NW-side model is feasible even if the ML model structure of the UE-side and NW-side models are not matched.

### Type 3 training

In this section, we present results on Type 3 (separate) training with vector quantization.

As was concluded in RAN1#110bis-e, we have two flavors of separate training (type 3)

- UE-first type3 training: UE-Enc is trained first and UE server shares training dataset with gNB server for training the gNB-Dec

- gNB-first type3 training: gNB-Dec is trained first and gNB server shares training dataset with UE server for training the UE-Enc

Note that VQ can be trained with UE-Enc or gNB-Dec. The following results assume that

- For UE-first type3 training, VQ is trained with UE-Enc and UE server shares (zq, Vtarget) dataset with gNB server

- For gNB-first type3 training, VQ is trained with gNB-Enc and gNB server shares (zq, Vtarget) dataset with UE server

### UE-first separate training

We focus on offline sequential UE-first training where the UE-side training entity trains the UE-side model first and generates a training dataset which is shared with NW-side for training the NW-side model. The objective is to compress precoder vector ‘Vtarget’ to a quantized latent representation ‘zq’ which is used to generate the reconstructed precoder vector ‘Vout’. There are multiple approaches for training the UE-side model and generating the separate training dataset. In particular, we have

- Approach 1: UE server trains the encoder without quantization and shares the dataset (z, Vtarget)

  - Vector quantizer (VQ) is trained with the decoder at NW-side training entity

- Approach 2: UE server trains the encoder with quantization

  - UE server shares the dataset (ze, Vtarget), where ze is the input to VQ

    - VQ is trained with the decoder at NW-side training entity

  - UE server shares the dataset (zq, Vtarget), where zq is the output of VQ

Table 6 illustrates the performance of Type 3 training for different approaches in training a CNN encoder and a CNN decoder with vector quantization. We observe that training UE encoder without quantization may lead to some performance loss when compared with encoder training with quantization. For approach 2, we observe that separate training based on (ze, Vtarget) or (zq, Vtarget) leads to almost same performance.

Table 6: Comparison of different approaches for separate training with VQ for a single UE

| Enc | Dec | Separate training dataset | NN Params |  | SGCS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | z-dim | Quant | Linear | dB |
| CNN | CNN | (ze, Vtarget) | 64 | VQ with total payload 128 bits | 0.7794 | -6.564 |
|  |  | (zq, Vtarget) |  |  | 0.7788 | -6.552 |
|  |  | (z, Vtarget) |  |  | 0.768 | -6.345 |

Training UE encoder without quantization and generating the separate training based on this encoder may lead to some performance degradation compared to encoder training with quantization.

### Comparing different approaches to separate training

In this section, we present results comparing different approach to separate offline training of single UE-encoder and single gNB-decoder with vector quantization (VQ). In these results, we assume that encoder is a CNN and the decoder is a transformer NN.

An alternative type3 training is based on activation/gradient exchange (type3-alt), where either UE-Enc or gNB-Dec is trained first and then the other is trained based on activation/gradient exchange, e.g., UE-Enc training with a frozen Dec (no update of Dec weights). In particular, we have the following two flavors:

- gNB-first type3-alt training: gNB-Dec is trained first and then UE-Enc is trained with a frozen gNB-Dec

  - This approach may require exchange of activation (latent vector), gradient and corresponding target for loss function computation

- UE-first type3-alt training: UE-Enc is trained first and then gNB-Dec is trained with a frozen UE-Enc

  - This approach may require exchange of activation (latent vector) and corresponding input

Note that the difference between this type3-alt and type 3 training is that instead of using supervised learning based on separate training dataset (e.g., (zq, Vtarget) dataset), the encoder or decoder is trained using the end-to-end loss between Vtarget and the output of the decoder.

Table 7: Comparison of type1, type3, and type3-alt training for CNN-Enc and TF-Dec

| Enc | Dec | Train type | SGCS |  |
| --- | --- | --- | --- | --- |
|  |  |  | Linear | dB |
| CNN | TF | Type1: Joint baseline with VQ | 0.7854 | -6.684 |
| CNN+VQ | TF | Type3: UE-first with (zq, Vtarget) | 0.7898 | -6.774 |
| CNN | VQ+TF | Type3: gNB-first with (zq, Vtarget) | 0.7693 | -6.37 |
| CNN+VQ | TF | Type3-alt: UE-first (frozen Enc) | 0.7851 | -6.678 |
| CNN | VQ+TF | Type3-alt: gNB-first (frozen Dec) | 0.7914 | -6.807 |

Table 7 shows the SGCS performance comparison of two flavors of type3 training with type3-alt and type1 training baseline for a payload size of 128 bits. For UE-first training, we observe that type1, type3 and type3-alt approaches have almost same performance, as in all these approaches gNB-Dec is trained by minimizing loss in V-space. On the other hand, for gNB-first training, type3 training performs worse than type1 and type3-alt, as in type3 gNB-Dec is trained by minimizing the loss in the latent space.

UE-first type3 training with dataset or activation exchange can achieve the same performance of Type1 training.

gNB-first type3 training with dataset exchange performs worse compared to type1 and type3 with activation/gradient exchange (type3-alt), since type3-alt training is based on end-to-end loss minimization in contrast to latent space loss minimization which is used in type3.

Next, we show results on the comparison between UE-first type3 training and joint training for rank >1 assuming TF-TF architecture and with a payload size of 128 bits.

| Enc | Dec | Train type | Layer idx | SGCS (type1) |  | SGCS (type3) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Linear | dB | Linear | dB |
| TF | TF | NN with VQ (128 bits) | 0 | 0.8062 | -7.13 | 0.8077 | -7.16 |
|  |  |  | 1 | 0.6781 | -4.92 | 0.6778 | -4.92 |
|  |  |  | 2 | 0.5794 | -3.76 | 0.5796 | -3.76 |
|  |  |  | 3 | 0.4993 | -3.00 | 0.5004 | -3.01 |

Joint and sequential training (e.g., UE-first type3) training achieves similar SGCS performance for rank >1.

## Generalization to multiple vendor scenario

We next discuss the aspect of generalization to a scenario with multiple models corresponding to different vendors. Specifically, we consider the case where a gNB interacts with multiple UE-side models. One option could be to develop a different NW-side model corresponding to each UE-side model. However, this comes with the need to switch among the models. To address this, we consider the problem of training a common NW-side model that is compatible with multiple UE-side models.

### Type 2 training

In Type 2 offline training approach, the common NW-side model and UE-side models will be trained at different entities at the same time. The training entity of the NW-side model will interact with the training entities of each of the UE-side models during the training session to exchange training related information. The setup is shown for the case of 3 UE-side models in Figure 18:

Figure 19: Type 2 offline training of a single NW-side model and multiple UE-side models

We evaluated the ML-model performance for the case where the UE-side model structures are chosen to be transformer-based for the first two models and CNN-based for the third model. The resulting SGCS performance for the three UE-side models when working with a common NW-side model (transformer-based) is shown below in comparison to the case where a separate NW-side model was trained corresponding to each of the UE-side models. Figure 19 shows that the performance of the models is not affected by using a common NW-side model.

Figure 20: Type 2 training: effect of common NW-side model for multiple UE-side models

It is feasible to use Type 2 offline training to train a common NW-side model together with separate UE-side models without any performance impact when compared to training a separate NW-side model for each UE-side model.

### Type 3 (separate) training results for multiple UE vendors

In this section, we present results on Type 3 (separate) training in a multi-vendor setup with vector quantization. We consider a shared NW-side model which is trained based on datasets collected from multiple UE vendors. There are different ways to design the shared NW decoder. In this simulation, we assume that UE encoder is trained with a VQ block and NW trains or stores UE-specific VQ codebooks. We consider a shared NW decoder and personalization is achieved via the UE-specific VQ codebook. In particular, we consider a shared transformer-based decoder at NW-side, a transformer-based encoder at UE-vendor 1, and a CNN-based encoder at UE-vendor 2. Each UE-side encoder is trained separately. Then, each UE-side generates a training dataset which is shared with NW-side, e.g., UE-vendor 1 generates (ze, Vtarget)UE1 or (zq, Vtarget)UE1 and UE-vendor 2 generates (ze, Vtarget)UE2 or (zq, Vtarget)UE2.

Table 8 shows the performance of separate training with VQ for two UE-vendors and the corresponding joint training baseline of one UE-vendor encoder and NW decoder. Results show that separate training with a shared decoder achieves same performance as joint training baselines (i.e., Type 1 training).

Table 8: Results on multi-vendor separate training with VQ

| UE-Enc | gNB-Dec | UE-vendor | NN Params |  | Train type | SGCS |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | z-dim | Quant |  | Linear | dB |
| TF | TF |  | 64 | VQ with total payload 128 bits | Joint 1-to-1 baseline | 0.8062 | -7.126 |
| CNN | TF |  |  |  |  | 0.7854 | -6.684 |
| TF | TF common | UE-vendor 1 |  |  | Separate (ze, Vtarget) | 0.8049 | -7.097 |
| CNN |  | UE-vendor 2 |  |  |  | 0.7881 | -6.739 |

Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training.

# Generalization studies

The following agreements were made in RAN1#110 related to the aspect of generalization of the ML model [2]:

Agreement The following cases are considered for verifying the generalization performance of an AI/ML model over various scenarios/configurations as a starting point: - Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A - Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B - Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g., Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B. - Note: Companies to report the ratio for dataset mixing - Note: number of the multiple scenarios/configurations can be larger than two - FFS the detailed set of scenarios/configurations - FFS other cases for generalization verification, e.g., - Case 2A: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model is updated based on a fine-tuning dataset different than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B. After that, the AI/ML model is tested on a different dataset than Scenario#A/Configuration#A, e.g., subject to Scenario#B/Configuration#B, Scenario#A/Configuration#B. Agreement For CSI enhancement evaluations, to verify the generalization performance of an AI/ML model over various scenarios, the set of scenarios are considered focusing on one or more of the following aspects as a starting point: - Various deployment scenarios (e.g., UMa, UMi, InH) - Various outdoor/indoor UE distributions for UMa/UMi (e.g., 10:0, 8:2, 5:5, 2:8, 0:10) - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Other aspects of scenarios are not precluded, e.g., various antenna spacing, various antenna virtualization (TxRU mapping), various ISDs, various UE speeds, etc. - Companies to report the selected scenarios for generalization verification Agreement For CSI enhancement evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations (e.g., which may potentially lead to different dimensions of model input/output), the set of configurations are considered focusing on one or more of the following aspects as a starting point: - Various bandwidths (e.g., 10MHz, 20MHz) and/or frequency granularities, (e.g., size of subband) - Various sizes of CSI feedback payloads, FFS candidate payload number - Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports) - Other aspects of configurations are not precluded, e.g., various numerologies, various rank numbers/layers, etc. - Companies to report the selected configurations for generalization verification - Companies are encouraged to report the method to achieve generalization over various configurations to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

In this section, we present results and discussion on the generalization performance of ML models for the CSI compression sub-use case.

## Generalization to different scenarios

In this section, we present results on our study on the generalization performance of the two-sided AI/ML model for CSI compression across indoor and outdoor scenarios. Consider the following scenarios:

- Scenario A: Dense urban layout, indoor UEs

- Scenario B: Dense urban layout, outdoor UEs

We present results for the following 3 cases that are aligned with the agreement above:

- Case 1:

  - (A à A): The AI/ML model is trained based on a training dataset from Scenario A, and then the AI/ML model performs inference on a dataset from different UEs in the same Scenario A.

  - (B à B): The AI/ML model is trained based on a training dataset from Scenario B, and then the AI/ML model performs inference on a dataset from different UEs in the same Scenario B.

- Case 2:

  - (A à B): The AI/ML model is trained based on a training dataset from Scenario A, and then the AI/ML model performs inference on a dataset from UEs in a different Scenario B.

  - (B à A): The AI/ML model is trained based on a training dataset from Scenario B, and then the AI/ML model performs inference on a dataset from UEs in a different Scenario A.

- Case 3: The AI/ML model is trained based on a training dataset constructed by mixing the datasets from Scenarios A and B (mixing ratio: 80% indoor, 20% outdoor).

  - (A-B mix à A): the AI/ML model performs inference on a dataset from UEs in scenario A.

  - (A-B mix à B): the AI/ML model performs inference on a dataset from UEs in scenario B.

The following table presents the SGCS values for each case above for rank 1 (single layer):

Table 9: Generalization across indoor and outdoor scenarios

| Case | Train à Test A = Dense urban indoor B = Dense urban outdoor | SGCS |
| --- | --- | --- |
| 1 | A à A | 0.771 |
|  | B à B | 0.897 |
| 2 | A à B | 0.903 |
|  | B à A | 0.696 |
| 3 | A-B mix à A | 0.780 |
|  | A-B mix à B | 0.912 |

Based on the results, the ML model that is trained on a mix of datasets from both scenarios is able to generalize well across both scenarios. During inference, in each of the two scenarios considered (indoor and outdoor), the performance of this model is comparable to the performance of a model that was trained only on the same scenario used for inference.

Training on a dataset constructed by mixing the datasets of multiple scenarios enables the same ML model to perform well during inference in each of the scenarios.

## Generalization to variable configurations

In this section, we present results on generalization ability of AI models to support variable payload, subband and antenna configurations. In NR, the number of subbands can be ranging from 1 to 19, and the subbands can be contiguous or non-contiguous. Also, in Type I, Type II and eType II codebooks, there are 13 antenna configurations with respect to number of antenna (CSI-RS) ports and the layout (N1 array in first dimension, N2 array in second dimension). In legacy PMI reporting, these variable configurations are supported by generating respective spatial compression matrix and frequency compression matrix (or by reporting PMI subband-by-subband in Type I/II codebook). In AI-based CSI feedback, the most straightforward way is to train specific AI model using dataset generated based on the specific configuration. In this sense, a huge amount of model needs to be trained and stored in server or device, and frequent switching among models maybe required per configuration and/or triggering. Thus, it is meaningful to study generalization ability of AI models to support variable configurations.

Training with mixed dataset

To achieve a good generalization ability, in the training phase, we mixed the dataset generated under variable configurations and fed them into the AI model, so that AI model would see sufficient variations in the training.

Exercise 1: Variable subband configurations with same payload

In this example, we consider a transformer-based AI model, and 2 training options.

- NN0: The first option is only training the AI model using data sample with full subband configuration. This NN is considered as a baseline.

- NN1: The second option is training the AI model using random subband patterns in addition to the full subband case. Arbitrary patterns are considered, e.g., randomly select N3 subbands from total 12 subbands where N3 ranging between 3 and 11.

As shown in Figure 20, 3 cases are considered in testing, full-subband configuration (SB 0-11), second half of the total subband (SB 6-11), and random patterns. One can see that NN1 achieves robust performance across all testing cases. For the full subband configuration testing, NN2 even outperforms the specifically trained NN0. This implies that training mixed datasets could benefit each other.

Figure 21: Results of Exercise 1

Exercise 2: Variable subband configurations with variable payload

In this example, we also consider a transformer-based AI model, and 4 training options.

- NN0: The first option is only training the AI model using data sample with full subband configuration. This NN is considered as a baseline.

- NN1: The second option is trained with the same data set as NN0. The difference is that two payload configurations are considered (i.e., encoder output dimension = 32 and 64) and are trained at the same time.

- NN2: The third option is training using contiguous patterns. The number of subbands are randomly generated between 3 and 12. In this case, if Nsb > 6, we consider encoder output dimension = 64, while 32 is considerd if Nsb <=6.

- NN3: The fourth option is similar to the third option except that arbitrary subband pattern is considered in the training. Still two payload configurations are applied per number of subbands.

Results of 5 testing cases are presented in Figure 21, i.e., full-subband configuration, contiguous subband configurations with Nsb > 6 and Nsb <= 6, arbitrary patterns with Nsb > 6 and Nsb <= 6. For full-subband case, one can see that NN1 (dimension=64) outperforms the baseline NN0 (dimension=64) because of the concurrent training of two payloads (dimension=64 and dimension = 32). The performance can be further improved by introducing contiguous patterns (NN2) and random patterns (NN3).

Besides, training using arbitrary pattern (NN3) yields good results for all cases. Training with contiguous pattern (NN2) yields good results for all contiguous case. Moreover, smaller subband configuration (Nsb <= 6) yields similar performance as the larger subband configuration (Nsb > 6) saving half payload.

Figure 22: Testing results of Exercise 2

Training with mixed variable subband configurations achieve robust performance across all possible subband configurations including arbitrary number of subbands and arbitrary subband patterns.

Training with mixed variable subband configurations outperforms specific training with specific subband configuration.

Smaller number of subbands can achieve comparable results to the larger number of subbands with half of reporting payload.

Exercise 3: Variable gNB antenna configurations

In this example, we consider a transformer-based AI model, and 2 training options.

- NN0: The first option is training specialized AI model using data sample with 2x8, 4x4 and 2x4 antenna configuration specifically. This set of NNs are considered as a baseline.

- NN1: The second option is training a common AI model using mixed data set of 2x8, 4x4 and 2x4 antenna configurations.

Results of 2 testing case are presented in Figure 22.  Similar observation can be drawn as Exercise 1 that training with mixed datasets benefit each other and outperforms the specific training case (NN0).

Figure 23: Testing results of Exercise 3

Training with mixed antenna configuration achieves robust performance across all antenna configurations in the training.

Training with mixed antenna configuration outperforms specific training with specific antenna configuration.

For the evaluation of generalization of AI model to variable configurations, consider the following in data set generation:

- For subband generalization, generate N>=1 random patterns (either contiguous or non-contiguous) for each data sample in the training set. The full subband pattern can be used in addition.

- For antenna configuration generalization, mix data sample generated based on M antenna configuration with equal proportion.

- Same configuration in the testing set and training set

# Conclusions

In this document, we have discussed various evaluation results for the AI/ML-based CSI feedback enhancement use case. We made the following observations:

Observation 1:	AI/ML based CSI compression gives UL overhead gains of around 50% for mean throughputs (at a mean throughput around 14.3 Mbps) and 32% for cell-edge throughputs (around 2.6 Mbps).

Observation 2:	For Bursty Traffic,

- At 10% RU, we see a 63% improvement in UL overhead at a mean user experience of around 150 Mbps. At 55% improvement in UL overhead, the edge user experience is around 80 Mbps.

- At 40% RU, we observe a 60% improvement in UL overhead at a mean user experience of 88 Mbps. At an edge user experience of around 38 Mbps, the gain in UL overhead is 51%.

- At 80% RU, we observe a 60% improvement in UL overhead at a mean user experience of around 45 Mbps, and 70% improvement in UL overhead at an edge user experience of around 11 Mbps.

Observation 3:	Using the maximum number of non-zero coefficients, the total PMI overhead for Release 16 eType II for parameter combination (PC) 6 is 557 bits. For Rank 4, this would imply the per-layer overhead is below 140 bits. Hence it would not be possible to fill the “Z” bin for this case using the current template.

Observation 4:	Real-time performance monitoring that incurs overhead and/or additional processing complexity is unnecessary.

Observation 5:	Model monitoring based on ground-truth provided by UE to the network requires large signalling overhead and may be sensitive to large latency.

Observation 6:	Model monitoring using a proxy model that outputs the intermediate KPI directly shows an accurate inference accuracy prediction.

Observation 7:	Model monitoring based on metrics derived by comparison between input samples at inference and training samples can have strong relationship with the inference accuracy. As a result, input-based monitoring appears promising.

Observation 8:	Localized models, which are developed specifically for use within a local region, provide better performance compared to global models of comparable size trained using data from the entire layout.

Observation 9:	Localized models provide similar performance with a smaller model size compared to global models.

Observation 10:	Quantization non-aware training (case-1) leads to noticeable performance degradation compared with quantization aware training (case-2).

Observation 11:	For quantization aware training, fixed or pre-configured quantization (case 2-1) is more sensitive to quantization’s parameters/configuration compared with trainable quantization (case 2-2). That is, quantization’s parameters/configuration in case 2-1 need to be carefully chosen to align with statistical distribution of latent vector (z), otherwise performance is degraded.

Observation 12:	Trainable quantization offers more flexibility and better performance compared to fixed quantization, e.g., trainable vector quantization can improve the performance.

Observation 13:	The performance of AI/ML-based CSI compression using two-sided model can vary considerably if there is a discrepancy between the training data and inference data due to device-side variations.

Observation 14:	Gradient-exchange based Type 3 training enables the training of encoders for new devices in a scalable and backward compatible manner without requiring updates of the NW-side models and avoids data distribution mismatch issues.

Observation 15:	The SGCS achieved with a training dataset quantized using R16 Type II PC 8 is very close to the SGCS achieved using the ideal (floating point) training dataset for the rank 1 case with dense urban scenario.

Observation 16:	Type 2 offline training of the UE-side model and NW-side model is feasible even if the ML model structure of the UE-side and NW-side models are not matched.

Observation 17:	Training UE encoder without quantization and generating the separate training based on this encoder may lead to some performance degradation compared to encoder training with quantization.

Observation 18:	UE-first type3 training with dataset or activation exchange can achieve the same performance of Type1 training.

Observation 19:	gNB-first type3 training with dataset exchange performs worse compared to type1 and type3 with activation/gradient exchange (type3-alt), since type3-alt training is based on end-to-end loss minimization in contrast to latent space loss minimization which is used in type3.

Observation 20:	Joint and sequential training (e.g., UE-first type3) training achieves similar SGCS performance for rank >1.

Observation 21:	It is feasible to use Type 2 offline training to train a common NW-side model together with separate UE-side models without any performance impact when compared to training a separate NW-side model for each UE-side model.

Observation 22:	Separate training with VQ for multiple vendors achieves almost the same performance as Type 1 training.

Observation 23:	Training on a dataset constructed by mixing the datasets of multiple scenarios enables the same ML model to perform well during inference in each of the scenarios.

Observation 24:	Training with mixed variable subband configurations achieve robust performance across all possible subband configurations including arbitrary number of subbands and arbitrary subband patterns.

Observation 25:	Training with mixed variable subband configurations outperforms specific training with specific subband configuration.

Observation 26:	Smaller number of subbands can achieve comparable results to the larger number of subbands with half of reporting payload.

Observation 27:	Training with mixed antenna configuration achieves robust performance across all antenna configurations in the training.

Observation 28:	Training with mixed antenna configuration outperforms specific training with specific antenna configuration.

We have the following proposals:

Proposal 1:	When providing results for the SGCS of the benchmark and the gain for SGCS for each layer, compute SGCS and the CSI feedback overhead assuming Max Rank for every UE for both the benchmark and AI/ML-based CSI feedback.

Proposal 2:	The definition of the bins (i.e., X, Y, Z ranges) for the result templates should consider whether a particular overhead is possible for the benchmark case for a given Max Rank.

- Hence, the bin-identifiers (X, Y and Z) should be specified for each Max Rank value separately.

- Alternatively, replace X/Y/Z with the overhead for eType II parameter combinations. For each Max Rank value, the overhead for Release 16 eType2 Benchmark could be computed for PC 1 through 6.

Proposal 3:	For each bin that identifies the benchmark CSI feedback overhead, companies should report the reduction in CSI feedback overhead from the usage of AI/ML-based CSI feedback over the benchmark CSI feedback method.

| CSI feedback reduction (%) (for a given CSI feedback overhead in the benchmark scheme) (Mean UPT based measurement) | [X*Max rank value], RU<=39% |
| --- | --- |
|  | [Y*Max rank value], RU<=39% |
|  | [Z*Max rank value], RU<=39% |
|  | [X*Max rank value], RU 40%-69% |
|  | [Y*Max rank value], RU 40%-69% |
|  | [Z*Max rank value], RU 40%-69% |
|  | [X*Max rank value], RU >=70% |
|  | [Y*Max rank value], RU >=70% |
|  | [Z*Max rank value], RU >=70% |

Proposal 4:	For model performance monitoring, specification change for reporting the target CSI with high resolution from UE to network requires clear justification as it incurs additional overhead and may not be necessary.

Proposal 5:	Study specification impact of methods that directly outputs intermediate KPI at the UE side.

Proposal 6:	Study specification impact of input-based model monitoring on the UE-side by comparing input samples at inference time to the training samples.

Proposal 7:	Study the performance improvement associated with localized models (models developed specifically for use within a local region) compared to global models.

Proposal 8:	Quantization method should be considered a part of the UE-side model and dequantization method should be considered a part of the NW-side model. The quantization method should be aligned for good performance, but there is no need for separate specification support to align the quantization method.

Proposal 9:	For training data collection, specifying new/larger values of R16 Type II parameters to achieve higher resolution of the ground truth CSI needs clear justification.

Proposal 10:	For the evaluation of generalization of AI model to variable configurations, consider the following in data set generation:

- For subband generalization, generate N>=1 random patterns (either contiguous or non-contiguous) for each data sample in the training set. The full subband pattern can be used in addition.

- For antenna configuration generalization, mix data sample generated based on M antenna configuration with equal proportion.

- Same configuration in the testing set and training set

# References

- RAN1 Chair’s notes, RAN1 #109-e.

- RAN1 Chair’s notes, RAN1 #110.

- R1-2303583, “Other aspects on AI/ML for CSI feedback enhancement”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #112bis-e, April 2023.

- R1-2303581, “General aspects of AI/ML framework”, Qualcomm Incorporated, 3GPP TSG RAN WG1 #112bis-e, April 2023.

- RAN1 Chair’s notes, RAN1 #112.