3GPP TSG-RAN WG1 Meeting #112bis-e                                                                                                             	                             R1- 2303585

Online, April 17th – 26th, 2023

Agenda item:	9.2.3.2

Source: 	Qualcomm Incorporated

Title: 	Other aspects on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

At RAN #94, a new study on artificial intelligence/machine learning for NR air interface has been approved [1] with the following goals briefly summarized as below.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on:

- Initial set of use cases includes:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

- The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

- PHY layer aspects, e.g., (RAN1)

- Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

- Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

Some progress has been made in RAN1 meetings so far, towards achieving the SI objectives. In this contribution, we discuss various aspects of the above-mentioned goals for the beam management use case.

# Common aspects related to spatial and temporal beam prediction

Because some of the specification aspects related to the two agreed use case cases (BM-Case1 and BM-Case2) are identical, we first consider the spec impacts that are common to both use cases in this section, and then consider aspects specific to each use case in later sections. We consider different phases of ML workflow in the following sections and discuss the corresponding specification impact related to each phase.

## Spec impact for model development and training phase

Model development and training strategies is a multi-faceted problem that require extensive testing and tuning. As elaborated in [2], on-device models today and in the near future need offline engineering for model development. This includes model development, training, quantization, compiling the model to hardware primitives with power, area, and latency consideration, target-chip-specific run-time binary image generation, and going through full UE testing. This is like today’s non-ML implementations that go through similar offline development and extensive UE testing, and ML algorithms will not be exceptions. Various options can be discussed in RAN1 in terms of different levels of network control, but offline development and training should be the focus to guarantee a concrete outcome that can lead to specification work in the potential Rel-19 WI. In RAN1 #110 meeting, it is agreed that both alternatives of AI/ML model training at NW side and AI/ML model training at UE side should be supported, but the issue of online versus offline training is considered as a separate discussion. The following proposal addresses the training procedure for beam prediction (one-sided AI/ML) use cases.

In RAN1 #110b-e an FL proposal was discussed in which it was proposed to start with the assumption of offline training for the agenda item 9.2.3.2 and defer discussions of online training to wait for the conclusion/agreement of Agenda item 9.2.1 whether online training is supported or not, but eventually there was no consensus. Our view is that we can separate the discussion to UE-side and NW-side AI/ML models. At least for UE-side AI/ML models, we can focus and agree on offline training scenario, and the discussion on NW-side AI/ML models can wait for the conclusion/agreement of agenda item 9.2.1. There was a similar separation already in RAN1 #110b-e for data collection, model monitoring, etc., and we believe a similar separation here would facilitate the progress of this aspect. With this being said, we have the following proposal:

#### Proposal 1

For the sub use case BM-Case1 and BM-Case2 and for UE-side AI/ML models, Agenda item 9.2.3.2 should focus on offline training scenario, in which the development and training of the AI/ML model happens offline without the need to involve 3gpp signaling

Prospect of Assistance Information

Let us consider the following agreement:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

One of the aspects in the above agreement that needs to be elaborated and studied is the “association/mapping of beams within Set A and beams within Set B”. This can be considered as assistance information from gNB to UE to help UE in making predictions. For instance, consider spatial domain beam prediction and particularly consider the case in which Set B is composed of wide beams and Set A is composed of narrow beams. For the input of UE-side AI/ML models, we can have the following options:

- L1-RSRP measurements corresponding to Set B

- L1-RSRP measurements corresponding to Set B and association/mapping of beams within Set A and beams within Set B

If the UE has this extra information of the association/mapping of beams within Set A and beams within Set B, this assistance information can be leveraged for the purpose of enhancing the beam prediction performance.

Additionally, in light of the following conclusion, indication of information about association/mapping of beams within Set A and beams within Set B partly curbs the necessity of sending explicit beam shape information for beams within Set A and beams within Set B and is in line with the spirit of the following conclusion which encompasses the indication of relative beam shape-related information.

Conclusion (RAN1 #112) Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement

The following proposal states some examples we can have for association/mapping of beams in Set A and beams in Set B:

#### Proposal 2

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following aspects related to association/mapping of beams within Set A and beams within Set B which is indicated from NW to UE:

- QCL relation between beams within Set A and beams within Set B

- Beams within Set B are superposition and/or linear combination of beams within Set A (e.g., for wide-to-narrow beam prediction)

- Relative beam pointing angles of beams within Set A and beams within Set B

- FFS: other options

Type of Beam Prediction

- Let us consider the agreements related to types of beam prediction so far:

Agreement (RAN1 #110) For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact

Conclusion (RAN1 #112) For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized.

- Beam pair prediction comes with numerous practical challenges, some of them stemming from the fact that DL Rx beam changes at a much faster pace compared to DL Tx beam and this makes the prediction task much more challenging. Let us consider UE-side and NW-side beam pair prediction. For UE-side beam pair prediction, the most relevant part in terms of specification impact is the predicted DL TX beam and the distinction in terms of specification impact with DL Tx beam prediction is minimal, if any. On the other hand, NW-side beam pair prediction has been discussed in the past few meetings, but there have been serious feasibility concerns that have not been addressed, even at a conceptual level.

- In the evaluations across companies, many of these practical challenges are not modelled, and even so, the performance of beam pair prediction (e.g., in terms of agreed KPIs including top-K beam prediction accuracy) is generally worse (in some cases much worse) than DL Tx beam prediction. With this being said, based on all the observations we have had for the evaluations related to beam pair prediction [3], we believe DL Tx beam prediction should be prioritized in the context of Rel-18 SI.

#### Proposal 3

For BM-Case1 and BM-Case2 prioritize the study of DL Tx beam prediction.

## Spec impact for inference phase

Prediction of information about beams in temporal and spatial domains may lead to different signalling implications based on the node in which the prediction task is carried out. In the following, the signalling aspects related to gNB-side beam prediction is discussed.

NW-side beam prediction

Working Assumption

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following L1 beam reporting enhancement for AI/ML model inference

- UE to report the measurement results of more than 4 beams in one reporting instance

- Other L1 reporting enhancements can be considered

The following WA was made in RAN1 110b-e which is about the signalling aspects related to NW-side AI/ML models:

Other than UE reporting a greater number of beams, reporting other types of information from UE to NW may help with the prediction performance of NW-side AI/ML models. For example, spatial variation of RSRPs (i.e., concentration or lack of concentration of RSRP among a few beams) can trigger NW to ask for L1-report of larger number of beams or be content with more compact L1-report (with smaller number of reported beams). Another example is that reporting temporal variation of beams can inform NW to better optimize the parameters of its temporal beam prediction model. The following proposal discusses a few options for such UE report.

#### Proposal 4

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following L1 beam reporting enhancement for AI/ML model inference

- Report of temporal and/or spatial variance/variations of L1-RSRP/L1-SINR measurements for beams

At least for BM-Case2, the report of temporal variance of L1-RSRP/L1-SINR measurements can be utilized at the input of NW-side AI/ML model. Let us consider a given gNB beam, e.g., beam ID 1. As the beam measurement and reporting periodicity may be different, report of temporal variation of L1-RSRP measurements gives gNB more granular information about the temporal variance of L1-RSRP measurements in between reporting instances. Additionally, the measurements related to beam ID 1 may not be reported in some of the previous reporting instances. This is more of an assistance information from UE to help NW with AI/ML inference. NW may be able to make more informed predictions based on whether the measurements for a given beam have been more or less static in the past measurement instances.

Additionally, report of spatial variance across beams per beam management instance could also provide useful information at the NW side as only a subset of beam measurements is reported to NW and this can be considered as some kind of assistance information, i.e., variance of L1-RSRP measurements for beams from Set B.

In the following two sections we discussed the signaling aspects specific to BM-Case1 and BM-Case2, respectively.

# Specific aspects related to spatial beam prediction

We consider BM-Case1 with a UE-side AI/ML model. Let us review the agreement from RAN1 110 regarding potential outputs of AI/ML models:

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and  other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

- FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s))

- Note1: It is up to companies to provide other alternative(s)

- Note2: Beam ID is only used for discussion purpose

- Note3: All the outputs are “nominal” and only for discussion purpose

- Note4: Values of N is up to each company.

- Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side.

- Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

Alt.3 in the above agreement declares “Tx and/or Rx Beam angle(s)” as potential outputs of AI/ML models. Regarding the definition of “beam angles”, beam pointing angles refer to the direction of peak beam gain. This concept is not new to 3gpp, and there is already the terminology of “beam boresight direction” defined in 37.355 in which the azimuth and elevation angle of the boresight directions for PRS are defined as “dl-PRS-Azimuth” and “dl-PRS-Elevation”. As an example, if UE predicts an SD beam from Set A in terms of beam angle, based on measurements of Set B of beams from NW, that may be considered as a beam angle as an output of UE-side AI/ML model. As mentioned above, beam boresight direction is the formal terminology adopted in 3gpp for beam pointing angles.

#### Proposal 5

For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

- Information about NW DL TX beam angles from target prediction beam set (Set A)

- UE may predict best beam angles from target prediction beam set (Set A) by measuring measurement beam set being input to AI/ML model (Set B) of DL TX beams

- FFS: details of beam angle, e.g., beam boresight direction

The Set A/Set B terminology were agreed so that companies have common understanding of the terminology, primarily for evaluation purpose. That is why we are using the terms “target prediction beam set” and “measurement beam set being input to AI/ML model” in the above proposal.

# Specific aspects related to temporal prediction

We consider BM-Case2 with a UE-side AI/ML model. A sub use case of temporal beam prediction is beam blockage prediction. Let us assume that a UE can predict blockage based on the history of beam measurements (potentially along with other auxiliary inputs to the AI/ML model such as position information, information from sensors, e.g., camera, etc.). One of the ways that this UE capability could be useful is that UE can proactively indicate to gNB that a blockage is imminent, and the gNB can take this information into account and proactively switch the downlink beam to a secondary beam. The existing methods for beam failure detection and recovery are reactive in nature, in which the blockage event is detected first, and then the beam failure recovery procedure is initiated.

#### Proposal 6

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

Predicted beam blockage/failure

# Discussion on UE features and functionality prospects for BM use case

Following along the discussion in [2] regarding “overall framework of features, functionalities and models”, we discuss some of the aspects specific to the beam management use case in this section. As compared to the legacy features, one fundamental distinction of AI-ML-enabled features is the need to support dynamic capabilities in addition to static capabilities. This occurs mainly because there may be supported functionalities and/or trained models for some cells, scenarios, configurations but not for others. The purpose of this section is to discuss some prospects related to these aspects for the beam management use case.

A given feature (or a configuration of a feature or a functionality) can be supported by a given UE for a specific cell, e.g., cell-A. On the other hand, the same feature (or a configuration of a feature or a functionality) for the same UE may not be supported for cell-B. That’s why, UE needs to dynamically indicate the supported feature in the case of handover when UE moves to cell-B. An example for “a configuration” could be Set A/Set B configurations for beam prediction.

Suppose that three Features (or Feature Groups), denoted as F1, F2, and F3, have been defined for the AI/ML beam management sub-use-case. A UE may support multiple features/functionalities of this beam management use case denoted by F1, F2, F3 in a cell-specific manner.  For example, suppose that the UE supports F1 and F2 for cell-A, F3 for cell-B, and does not support AI/ML-based beam prediction for cell-C. This implies that in the case of handover from cell A to cell B, the UE needs to communicate the supported features/functionalities with NW dynamically to indicate that F1 and F2 are no more supported, whereas F3 is. Likewise, each time the UE enters cell-C it should notify NW that there is no support for F1 and F2 so that NW may deactivate this feature/functionality.

Another aspect related to the beam management use case is the prospect of assistance information. For instance, let us consider the following agreement:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused).

The “association/mapping of beams within Set A and beams within Set B” can be considers as an example of assistance information from NW to UE to help UE improve prediction accuracy.

As another example, suppose that there is one Feature, denoted as F1, that has been defined for the sub-use-case. Suppose that cell-A uses a certain beam codebook that is provided as an assistance information (e.g., as a component of the Feature F1) in the form of Set A and Set B relationship. Suppose that cell-B uses a different beam codebook that results in a different relationship between Set A and Set B. Cell-B also provides it via an assistance information (e.g., as a component of the Feature F1). Suppose that UE supports the Set A/B relationship of cell-A but not of cell-B. This implies that in the case of handover from cell A to cell B, the UE needs to communicate that UE no longer supports the Feature F1 so that NW may deactivate F1. Likewise, in case of handover from cell B to cell A, the UE needs to communicate that UE now supports the Feature F1 so that NW may activate F1.

To summarize, AI/ML enabled features can be reported by both static and dynamic UE capabilities. Although it is clear from legacy framework that UEs report the supported features with UE capability reporting for static capabilities, it should be studied how to handle dynamic capability of AI/ML enabled features. To this aim, the key consideration is that NW and UE should be on the same page regarding whether a given AI/ML-enabled Feature is supported on the given cell/configuration/scenario.

# Model performance monitoring

We consider UE-side AI/ML models and discuss the corresponding signalling aspects.

UE-side AI/ML Models

The on-device models trained for the purpose of spatial and temporal beam prediction may be subject to performance degradation in certain scenarios, deployments, or use cases, when they get deployed in the field. One main reason for performance degradation could be the fact that UE-side AI/ML models get deployed in an environment that they had not encountered during training. There needs to be a mechanism defined in Spec through which the performance of on-device AI/ML models for beam prediction can be monitored. Along these lines, the following agreement was made in RAN1 110b-e which includes different variations of UE-side model monitoring:

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/deactivation/switching/fallback operation

In light of the following agreement:

Agreement (RAN1 #112) For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indication/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

As an example of “configuration/signalling from gNB to UE for performance monitoring”, we consider dedicated reference signals from gNB to UE to help UE in comparing predicted elements from Set A with actual measurements from Set A. The predicted elements could refer to predicted top-K beams from Set A and/or predicted L1-RSRP of the top-K beams from Set A.

#### Proposal 7

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following signalling aspects related to configuration/signalling from gNB to UE for performance monitoring:

Dedicated RS from gNB to UE for performance monitoring

# Conclusions

In this paper, we discussed signalling aspects related to beam prediction use case and made the following proposals:

Proposal 1: For the sub use case BM-Case1 and BM-Case2 and for UE-side AI/ML models, Agenda item 9.2.3.2 should focus on offline training scenario, in which the development and training of the AI/ML model happens offline without the need to involve 3gpp signaling

Proposal 2: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following aspects related to association/mapping of beams within Set A and beams within Set B which is indicated from NW to UE:

- QCL relation between beams within Set A and beams within Set B

- Beams within Set B are superposition and/or linear combination of beams within Set A (e.g., for wide-to-narrow beam prediction)

- Relative beam pointing angles of beams within Set A and beams within Set B

- FFS: other options

Proposal 3: For BM-Case1 and BM-Case2 prioritize DL Tx beam prediction.

Proposal 4: For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following L1 beam reporting enhancement for AI/ML model inference

- Report of temporal and/or spatial variance/variations of L1-RSRP/L1-SINR measurements for beams

Proposal 5: For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

- Information about NW DL TX beam angles from target prediction beam set (Set A)

- UE may predict best beam angles from target prediction beam set (Set A) by measuring measurement beam set being input to AI/ML model (Set B) of DL TX beams

- FFS: details of beam angle, e.g., beam boresight direction

Proposal 6: For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signalling to report the following information of AI/ML model inference to NW:

- Predicted beam blockage/failure

Proposal 7: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following signalling aspects related to configuration/signaling from gNB to UE for performance monitoring:

Dedicated RS from gNB to UE for performance monitoring

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RAN Plenary

- R1-2303581, “General aspects of AI-ML framework”, Qualcomm Incorporated.

- R1-2303584, “Evaluation on AI/ML for beam management”, Qualcomm Incorporated.

- R2-2203315, “Introduction of R17 Positioning Enhancements in LPP”, 3GPP TSG-RAN WG2 #117e, Feb. 2022.