3GPP TSG RAN WG1 Meeting #112bis-e    	            R1-2303594

E-meeting, April 17th – 26th, 2023

Source:	Qualcomm Incorporated

Title:	Enhanced sidelink operation on FR2 licensed spectrum

Agenda Item:	9.4.3

Document for: 	Discussion and Decision

# Background

At the RAN#98 meeting, it was agreed that enhanced sidelink operation on FR2 licensed spectrum will continue in Rel-18. The objective is following [1]:

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In this contribution, we provide our views on beam management aspects of enhanced sidelink operation on FR2 licensed spectrum.

# Periodic RS based SL beam management

## General aspect

### 2.1.1	SL beam management framework

SL beam management include following three aspects:

- Initial beam-pairing (IBP):

  - IBP is a procedure to make a pair of beams between UEs that have not established unicast-link / PC5-RRC

  - Beam-sweeping is necessary to find an appropriate beam-pair

- Beam maintenance (BM):

  - BM is to measure and refine a pair of beams between UEs that have established unicast-link / PC5-RRC

  - Beam-sweeping is necessary to enable beam-refinement

- Beam failure recovery (BFR):

  - BFR is to perform beam failure detection and candidate beam identification/indication to recover from the beam failure condition between UEs that have established unicast-link(s) / PC5-RRC.

  - Beam-sweeping is necessary to measure candidate beams and select the one for recovery

Overall, it is clear that beam-sweeping is essential for all IBP, BM, and BFR. In Uu, a BS transmits SSB (and additionally CSI-RS) in periodic manner for beam management (including initial/random-access, BM, and BFR). For SL beam management, the analogy, i.e. periodic SL RS transmissions for beam-sweeping, should be enabled. For SL, currently we only have slot-based transmissions. Since the SL RS measurement for each beam can be done in a quite short period of time, e.g., 1-symbol transmission of a reference signal per beam in theory (FFS: AGC), using slot-level transmission of existing channel/signal (e.g., PSCCH/PSSCH) for each beam for beam-sweeping is too expensive. RAN1 should study enabling periodic SL RS based beam-sweeping for IBP, BM, and BFR, in an efficient manner.

Fig. 1	Periodic Tx beam-sweeping

Observation 1:

- For FR2-SL initial beam-pairing (IBP), beam maintenance (BM), and beam-failure recovery (BFR), SL RS transmissions over multiple beams and monitoring them are necessary.

  - The RS can be [1] symbol transmission per beam in theory

    - FFS: Impact from AGC

  - The RS should be transmitted/monitored in periodic manner

### 2.1.2	Device types and scenarios

SL beam management should be able to be enabled for various UE implementations/types and/or sidelink deployments/services. For example:

- One type of UE may want to use only a limited number of ‘wide’ beams (e.g., one beam per panel) for SL beam management (Fig. 1 (a)). The set of wide beams is common for all the SL beam management procedures for one or multiple of other UEs.

- Another type of UE may want to use a set of ‘wide’ beams (e.g., one beam per panel) and another set of ‘narrow’ beams (e.g., multiple beams per panel) (Fig. 1(b)). The set of wide beams is used for procedures that is not dedicated to a particular target UE (e.g., IBP). The other set of narrow beams is configured to be used for high data rate communication with a particular paired UE.

(a) 						(b)

Fig. 2	Example UE types with different beam management approaches

Observation 2:

- Different UE types may use different sets of beams for SL beam management, e.g.:

  - One Type of UEs may use common set of (wide) beams for all SL beam management procedures including initial beam-pairing, beam maintenance, and beam failure recovery

  - Another type of UEs may use multiple sets of beams for different SL beam management procedure

    - E.g., one set of (wide) beams for initial beam-pairing for other UEs to measure, and another set of (narrow) beams for high data rate communication with a paired UE

With the above two observations, we propose to enable periodic RS based beam-sweeping for SL beam management in an efficient manner.

Proposal 1:

- Enable periodic SL RS based beam-sweeping for SL beam management, including initial beam-pairing (IBP), beam maintenance (BM), and beam-failure recovery (BFR)

  - Unified framework shall be able to support variety of scenarios and UE types

## Initial beam-pairing (IBP)

### 2.2.1	Need for IBP before unicast-link / dedicated configuration is available

As explained earlier, IBP should be for a pair of UEs that has not established unicast-link / PC5-RRC. To complete unicast-link / PC5-RRC establishment, two UEs must exchange some messages (e.g., direct communication request message that carries information regarding application-layer and ProSe services for unicast-link establishment, or discovery message as specified in TS 23.304) by using PSSCHs. In general, one of the UEs initiates/triggers the message exchange and the other UE may or may not proceed the procedure, depending on whether the other UE receiving the message is a target UE or is interested in the communication. After the unicast-link establishment or direct discovery procedure, the UEs must exchange messages for UE capability, PC5-RRC establishments, etc. It does not make sense to assume no beam-forming for those message exchanges.

One may say that these message exchanges require only low data rate and hence beam-forming gain is not essential. It is true that selecting the best beam-pair among all the possible combinations of beams between the UEs may not be necessary until actual unicast data communication starts. However:

- For FR2, a handheld UE typically equips with multiple antenna panels to achieve spherical coverage. At the RAN1#111 meeting, RAN1 indeed agreed to assume two antenna panels with panel bearing angle of 180 degrees at each UE as part of evaluation assumption. Selecting a wrong antenna panel that does not have direct/indirect paths toward a target UE may cause significant SNR degradation. We analyze the impact of antenna panel selection according to the model presented in [2] as Fig. 3. The plot with “1 panel” is the case where a UE has a single antenna panel and that is used for any angle. “2 panels” is the case where a UE has two antenna panels and the best panel is used for each angle. It can be observed that the case with single antenna panel degrades the antenna gain compared to two antenna panels by about 2.5 dB at CDF from 20% to 40%.

Fig. 3	CDFs of gains with 1 panel and with 2 panels based on the modeling and methodology in [2]

- With an appropriate antenna panel selection for a target UE, a UE can form a wider or narrower beam. Using a wider beam enables beam-pairing without beam-sweeping by the panel, but it may lose array gain. In order to see the impact, we plot the CDFs of array gains of the hemisphere for the following two cases in Fig. 4. It is clear that Case 2 is not only offering higher beam forming gain at a particular direction, but also enabling larger beam forming gain over the coverage of the antenna panel. For most of the CDF region, the gain of Case 2 over Case 1 is about 3.0 – 3.5dB.

  - Case 1: single fixed wide beam from the panel is used. To model this, a single antenna element with the antenna pattern defined in TR37.885 Table 6.1.4-6 is assumed.

  - Case 2: multiple beams can be formed from the panel using (2 x 2) antenna elements and one of the beams that has the largest gain is used at each angle. For each antenna element, the pattern in TR37.885 Table 6.1.4-6 is assumed.

For reference, 2D cut of the patterns are plotted in Fig. A1 in the annex.

Fig. 4	CDFs of gains in Case 1 (fixed wide beam) and Case 2 (beam forming) based on the RAN1 agreed model

To demonstrate the benefit, a simple link-budget analysis has been conducted for UMi and InH. The assumptions for the link-budget analysis are presented in the Annex. Assuming that very low MCS index is usable for the message exchanges until PC5-RRC establishment, PSCCH carrying SCI-1 would be a bottleneck channel for communication range. Taking SCI-1 carried by PSCCH with 3 OFDM symbols as an example, the required SNRs for CDL-A DS 30ns and 100ns are -0.23dB and -2.92dB, respectively, according to Table A-5. For the case without IBP, according to the analyses above, the joint loss due to wrong antenna panel selection and beam-mismatch could be 2.5 + 3.5 = 6dB per UE for a UE pair. With this assumption, the maximum communication distances for message exchanges before unicast-link establishment can be estimated as in Table 1.

Table 1.	Max distance for enabling unicast-link establishment without and with IBP

| Scenario | LOS/NLOS | w/o IBP (6dB loss at each UE) | w/ IBP |
| --- | --- | --- | --- |
| UMi with CDL-A DS 100ns | LOS | 80m | 305m |
|  | NLOS | 20m | 40m |
| InH with CDL-A DS 30ns | LOS | 145m | >> 150m |
|  | NLOS | 15m | 30m |

As seen from the above, the distance is quite different depending on whether IBP is done or not.

Observation 3:

- Without initial beam-pairing, communication range for message exchanges between a pair of UEs for unicast-link establishment and other procedures is quite limited.

WID describes that the study considers sidelink unicast communication only. However, this does not mean the study shall be limited to the case where unicast-link has been established. Since IBP is technically essential for unicast-link communication over FR2-SL, enabling IBP before unicast-link establishment should be part of the study.

Proposal 2:

- Clarify that initial beam-pairing for SL beam management is for a pair of UEs that has not yet established unicast-link / PC5-RRC, so that the initial beam-pairing is part of the study.

### 2.2.2	Use of ‘SSB – PRACH association’ concept for initial beam-pairing

Before unicast-link / PC5-RRC is established, UEs for a unicast-link communication do not have an idea on appropriate beam-pair to communicate each other. Initial beam-pairing (IBP) must be a procedure that enables beam-pairing between UEs before the UEs know they are a pair for a unicast-link communication each other. Suppose one UE (UE1) transmits RSs for beam-sweeping in periodic manner. Another UE (UE2) can determine its beam towards UE1 using the RSs for beam-sweeping transmitted by UE1. However, UE1 does not know whether there is UE2, and/or which UE1’s Tx beam is preferred for UE2. In order to let UE1 identifying these, a transmission/response from UE2 to UE1 is necessary.

For Uu initial-access/random-access, this was done by using SSB – PRACH association as follows.

- BS transmits multiple SSBs with different beams periodically for beam-sweeping

- A UE transmits PRACH on the resource associated with the SSB for the preferred BS’s Tx beam

- BS receives the PRACH from the UE and identifies the BS’s Tx beam preferred for the UE that has transmitted PRACH

The concept of ‘SSB – PRACH association’ can be applied to SL beam management, as follows:

- A UE (UE1) transmits multiple RSs with different beams periodically for beam-sweeping

  - The RSs are SL RSs (e.g., SL CSI-RS, S-SSS) or its modified format

    - FFS: Impact from AGC

- Another UE (UE2) transmits ‘beam-pairing response’ on the resource associated with the RS for the preferred UE1’s Tx beam.

  - The ‘beam-pairing response’ is an SL RS/channel (e.g., PSFCH) or its modified format

    - FFS: Impact from AGC

- UE1 receives the ‘beam-pairing response’ from UE2 and identify the UE1’s Tx beam preferred for UE2

Fig. 4	Initial beam-pairing (IBP) enabled by using analogy from ‘Uu SSB – PRACH association’

Observation 4:

- Initial beam-pairing can be based on the concept of ‘Uu SSB – PRACH association’.

We propose to study initial beam-pairing, based on the concept of ‘SSB – PRACH association’ using the SL RS beam-sweeping and ‘beam-pairing response’.

Proposal 3:

- For initial beam-pairing, consider re-using the concept of ‘SSB – PRACH association’ from Uu

  - UE1 transmits RSs over multiple beams (beam-sweeping) in periodic manner.

    - FFS: structure of the RSs for beam-sweeping

  - A set of resources for ‘beam-pairing response’ that are associated with UE1’s Tx beams are presented.

    - FFS: structure of the ‘beam-pairing response’

  - UE2 transmits ‘beam-pairing response’ on the resource that is associated with the preferred UE1’s Tx beam.

  - UE1 identifies the UE2’s response and the preferred Tx beam for the UE2 based on the resource that UE1 detects the ‘beam-pairing response’ from UE2

### 2.2.3	Integration of IBP into unicast-link establishment

Next step is to consider integration of IBP with unicast-link establishment. Below, we consider two cases.

Case 1: UE1 transmits RSs for beam-sweeping and UE1 initiates unicast-link establishment with UE2

UE1 can identify the preferred UE1’s Tx beam only if/after UE1 receives the ‘beam-pairing response’ from UE2. Assuming that there are only UE1 and UE2 in the field for simplicity, the combined procedure of IBP and unicast-link establishment can be illustrated as in Fig. 6.

Fig. 6	IBP before unicast-link establishment for Case 1

The overhead (number of OFDM symbols) necessary for IBP and first message exchange (initiating message and response message to the initiating message) can be denoted as

NW×MW×RSW+NW×RESPIBP+InitMsg+RespMsg,					(1)

where NW and MW are the number of beams for IBP at UE1 and UE2, respectively. RSW is the number of OFDM symbols for the RS per beam transmitted by UE1, RESPIBP is the number of OFDM symbols for ‘beam-pairing response’ associated with a particular Tx beam of UE1. For ‘beam-pairing response’, (Nw x RESPIBP) symbols are necessary in total if the number of Tx beams of UE1 is Nw. InitMsg and RespMsg represent the number of symbols for the transmission/reception of initiating message and its response message, respectively.

Note that IBP can also be enabled by transmitting initiating message itself (Nw x Mw) times instead of transmitting RSs for beam-sweeping. Further, it is possible to use the response message to the initiating message as ‘beam-pairing response’. For this option, the corresponding overhead in terms of the number of OFDM symbols is given by

NW×MW×InitMsg+NW×RespMsg.							(2)

Assuming both UEs have the same number of beams for IBP (i.e., Nw = Mw), the required overhead derived from Eqs. (1) and (2) are plotted in Fig. 7. Here, RSw = 2 and RESPIBP = 2 are assumed, and InitMsg = RespMsg = 14 is assumed. With a given number of candidate beam pairs for UE1 and UE2 (Nw x Mw), , it is clear that RS-based approach (i.e., (1)) requires much smaller overhead than MSG-based approach (i.e., (2)).

Fig. 7	Overhead comparison b/w MSG-based and RS-based IBP

Case 1 is a bit tricky scenario in a sense that UE2 transmits ‘beam-pairing response’ while UE1 initiates unicast-link establishment. UE2 should be able to know when/whether UE1 would initiate it. Following options can be considered:

Opt. 1: Aperiodic indication/trigger from UE1 that indicates UE1 would initiate unicast-link establishment

This is an analogy from Uu paging mechanism – UE1 transmits an aperiodic indication/trigger to indicate unicast-link establishment procedure. Since UE1 does not know to which beam direction UE2 is located when it transmits the aperiodic indication/trigger, the indication/trigger must be transmitted over the Nw Tx beams. UE2 transmits a ‘beam-pairing response’ using the resource associated with the preferred UE1’s Tx beam. After receiving the ‘beam-pairing response’, UE1 transmits initiating message for unicast-link establishment using the indicated Tx beam.

Fig. 8	IBP with unicast-link establishment based on analogy from ‘Uu paging’ Option 1

The overhead with this Opt. 1 can be derived as

NW×MW×RSW+NW×Trigger+NW×RESPIBP+InitMsg+ResMsg,			(3)

Where Trigger is the number of symbols for the aperiodic indication/trigger per beam.

Opt. 2: Beam-pairing response is after initiating message for the unicast-link establishment

A variation of analogy from Uu paging mechanism can be: UE1 transmits initiating message of unicast-link establishment over the possible Nw Tx beams. UE2 transmits ‘beam-pairing response’ if UE2 receives the message and would like to continue the unicast-link establishment procedure with UE1. Based on the resource for the beam-pairing response transmitted by UE2, initial beam-pair can be established.

Fig. 9	IBP with unicast-link establishment based on analogy from ‘Uu paging’ Option 2

Similar to Eqs. (1) and (3), the overhead of this Opt. 2 can be derived as follows.

Nw×Mw×RSw+Nw×InitMsg+Nw×RESPIBP+ResMsg				(4)

The overhead calculated using Eqs. (3) and (4) are plotted in Fig. 8. For Opt.1, Trigger = 2 is assumed. MSG-based (Eq. (2)) and RS-based (Eq. (1)) are also plotted in Fig. 10 (identical to the plots in Fig. 7) for reference. It is observed that both options 1 and 2 work well – the required overhead is still much smaller than MSG-based (Eq. (2)).

Fig. 10	Overhead comparison of options for RS-based IBP and MSG-based IBP

Observation 5:

- Initial beam-pairing based on RSs for beam-sweeping requires much smaller overhead compared to initial beam-pairing using messages of unicast-link establishment.

- For the case where UE1 transmits RSs for beam-sweeping and UE1 initiates unicast-link establishment with UE2 that monitors the RSs from UE1 (Case 1):

  - UE2 should be able to know when/whether UE1 initiates unicast-link establishment.

  - Similar to Uu paging, UE1 can indicate/trigger unicast-link establishment.

Case 2: UE1 transmits RSs for beam-sweeping and UE2 initiates unicast-link establishment with UE1

Same as for Case 1, it should be noted that UE1 can identify the preferred UE1’s Tx beam only if/after UE1 receives the ‘beam-pairing response’ from UE2. Unlike Case 1, UE2 can indicate the preferred UE1’s beam using the ‘beam-pairing response’ only when UE2 would like to initiate unicast-link establishment with UE1. The procedure is quite similar to 2-step RACH for Uu. In other words, Case 2 can, by nature, resolve the issue identified for Case 1.

Fig. 11	IBP before unicast-link establishment for Case 2 based on analogy from ‘Uu 2-step RACH’

Observation 6:

- Initial beam-pairing for the case where UE1 transmits RSs for beam-sweeping and UE2 that monitors the RSs from UE1 initiates unicast-link establishment with UE1 (Case 2):

  - Procedure for Case 1 can mostly be re-used

  - Unlike Case 1, initial beam-pairing can be triggered only when UE2 would like to initiate unicast-link establishment with UE1.

Based on the analysis in 2.2.3 and the observations 5 and 6, we propose the following:

Proposal 4:

- Study detailed procedures of initial beam-pairing using RSs for beam-sweeping and ‘beam-pairing response’

  - At least following should be taken into account:

    - Overhead

    - Delay until initial beam-pairing

  - Both of the following cases should be considered:

    - Case 1: UE1 transmits RSs for beam-sweeping and UE1 initiates unicast-link establishment with UE2 that monitors the RSs from UE1

    - Case 2: UE1 transmits RSs for beam-sweeping and UE2 that monitors the RSs from UE1 initiates unicast-link establishment with UE1

## Beam maintenance (BM)

### 2.3.1	Beam measurement and report

Unlike IBP, BM is only between UEs that have established unicast-link. Assuming Tx/Rx beam-correspondence applies for both UEs, either UE1 or UE2 can transmit the periodic RSs for beam-maintenance. A UE monitoring the periodic RS for beam-maintenance can be configured semi-statically or triggered dynamically to report L1-RSRP/SINR measurement of the beams from the paired UE (see Fig. 12). Current SL supports SL CSI feedback using SL MAC-CE via PSSCH. Same mechanism can be considered for L1-RSRP/SINR measurement report, i.e., using SL MAC-CE via PSSCH.

Fig. 12	SL RS beam measurement and report

As long as half-duplex constraint is met, it would also be possible to configure periodic RS transmission for beam-maintenance from both UEs in a pair (see Fig. 13). Since this is after unicast-link / PC5-RRC establishment being completed, it is fully up to the pair of UEs on which/how to transmit RSs over multiple beams.

Fig. 13	SL RS beam measurement and report (bi-directional case)

Proposal 5:

- Enable beam maintenance based on periodic RSs for multiple beams, similar to Uu beam maintenance

  - RSs for beam maintenance are transmitted by one UE (UE1) or by both UEs for a pair of UEs

  - L1-RSRP/SINR report can be transmitted in the same way as SL CSI report (via SL MAC-CE)

## Beam failure recovery (BFR)

### 2.4.1	Procedure aspect

At the RAN1#112 meeting, following agreement was made:

Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.

There was a discussion on whether the overall design/procedure of BFR should be studied in RAN1 or RAN2. The above agreement looks recommending RAN1 to focus only on the condition(s) for beam-failure detection from the physical layer point of view. However, the overall design/procedure of BFR highly depends on the SL beam management framework. Therefore, it is our understanding that the overall framework discussion for BFR should first take place in RAN1. This is how Uu BFR was studied in Rel-15.

Proposal 6:

- Clarify that RAN1 will discuss overall BFR procedure first.

### 2.4.2	Beam failure detection and candidate beam identification

First of all, it should be noted that Uu BFR can use only SSB or periodic CSI-RS as the RS for BFR as specified in TS38.213 Clause 6. As per WID objective describes, SL BFR should be based on Uu which uses periodic RS transmissions unless there is any substantial reason.

Observation 7:

- For Uu BFR, RSs for BFR have to be periodic RS transmissions.

- Following Uu BFR, SL BFR should be based on periodic RS transmissions

Same as for BM, BFR is only between UEs that have established unicast-link. Assuming Tx/Rx beam-correspondence applies for both UEs, either UE1 or UE2 can transmit the periodic RSs for BFR. A UE is configured with monitoring periodic RS transmissions for beam failure detection and candidate beam identification. Once a BFD condition is met, the UE triggers the BFR procedure. In Uu, the BFR trigger is based on the hypothetical PDCCH BLER derived from  measured RSRPs of the BFD-RS; similar BFR trigger should be used for FR2-SL.

Proposal 7:

- For FR2-SL, beam failure detection re-uses the concept of Uu BFR procedure:

  - A UE is configured to monitor periodic transmissions of BFD-RS and CBD-RSs

  - If a UE detect BFD condition is met, the UE triggers the BFR procedure.

  - BFD condition can be similar to the one for Uu BFR

    - Based on the hypothetical PSCCH BLER derived from the measured RSRPs of the RSs for BFR

### 2.4.3	Beam failure recovery request

In Uu BFR, once a UE detects beam failure, the UE may request BFR to the BS. This is based on PRACH transmission on a resource associated with the selected periodic RS for BFR that is transmitted using the preferred BS’s Tx beam. This mechanism can be re-used for BFR for FR2-SL.

Fig. 14	Beam failure recovery for UEs communicating with UE1

The signal for BFR request can re-use the structure of ‘beam-pairing response’ for initial beam-pairing. With this, IBP and BFR follows almost the same procedure – the main difference is that IBP uses a set of resources for ‘beam-pairing response’ that is not UE/unicast-link dedicated, while BFR configures a set of resources for ‘BFR request’ in UE/unicast-link specific manner.

Proposal 8:

- For FR2-SL, BFR request re-uses the concept of Uu BFR request:

  - Each UE is configured with resources for BFR request associated with UE1’s Tx beams

    - FFS: structure of the ‘BFR request’

  - Once the UE detects beam failure, the UE transmits a BFR request using the resource associated with the UE1’s preferred Tx beam (= the resource associated with the preferred CBD-RS)

  - For different UE, different set of resources for BFR request can be reserved.

## Further details of the periodic RS for SL beam management

As presented earlier, RS transmissions over multiple beam for IBP, BM, and/or BFR should be periodic. Unlike legacy SL CSI-RS, such periodic RS transmission does not require associated PSCCH/PSSCH in principle. Enabling such ‘stand-alone’ periodic SL RS for beam management enables sweeping larger number of beams in a slot in efficient manner.

On the other hand, associating RSs for SL beam management with PSCCH/PSSCH is still useful for some scenarios, e.g., aperiodically triggered RS transmissions over multiple beams. For example, a UE may have SL data to transmit and it would like to let the paired UE measuring particular beams in aperiodic manner for fast beam refinement. Both options should be considered in the study.

(a) RS without associated PSCCH/PSSCH			(b) RS with associated PSCCH/PSSCH

Fig. 15	Example structures of SL RS for SL beam management

Another important question is the waveform/signal structure of the SL RS for beam management, including the number of symbols per beam, RE mapping, sequence generation and its scrambling, etc. These have to be resolved once SL beam management framework is solid.

Proposal 9:

- Study following two options for RS transmissions for SL beam management

  - Option 1: Stand-alone transmission (RSs not associated with PSCCH/PSSCH)

  - Option 2: Non-stand-alone transmission (RSs associated with PSCCH/PSSCH)

- Further discuss the structure of RS for SL beam management:

  - Number of OFDM symbols per RS (per beam)

    - Including impact from AGC

  - RS waveform, sequence generation, etc

    - E.g., reusing SL CSI-RS (or S-SSS)

# Beam indication and switching schemes

## 3.1	Basic Tx/Rx beam determination

For Uu, Rx spatial filter relations for DL receptions and Tx spatial filter relations for UL transmissions are specified. For DL receptions, Rx spatial filter is defined as QCL Type-D and each DL reception can have the same QCL Type-D relation with another DL reception. Relation of QCL Type-D for multiple DL receptions can form a QCL chain. Typically, the source for Rx spatial filter of a QCL chain is an SSB in the same cell or in a different cell (or a CSI-RS if SSB is not available). For UL transmissions, Tx spatial filter is defined as spatial relation information. Each UL transmission can have the same spatial relation information with another UL transmission (SRS) or with a DL reception (SSB or CSI-RS) in the same cell or in a different cell. For Unified-TCI framework for Uu beam management, a UE can be configured to determine Tx/Rx spatial filter for PDCCH/PDSCH/CSI-RS receptions and for PUSCH/PUCCH/SRS transmissions by a joint DL/UL TCI-state or separate DL-TCI state / UL-TCI state.

For SL, as presented in Section 2, periodic RS for SL beam management is necessary. For a UE monitoring the periodic RS transmitted by another UE for SL beam management, the periodic RS should be able to be the source of Tx/Rx spatial filter for PSCCH/PSSCH/PSFCH transmissions/receptions for the UE. This is quite straightforward analogy from Uu Unified-TCI framework. The periodic RS for SL beam management can be either the one configured for unicast communication with the target UE, or the one used for IBP.

For SL, resource pool is configured separately for Tx and Rx, where Tx resource pool includes configurations for PSCCH/PSSCH Tx and for associated PSFCH Rx, while Rx resource pool includes configurations for PSCCH/PSSCH Rx and for associated PSFCH Tx. Basic operation should be that one RS for SL beam management can be the source of Tx/Rx spatial filter for all these transmissions/receptions for a given paired UE.

For SL, a UE may rather transmit periodic RS for SL beam management towards a paired UE. Unlike Uu, it should also be possible for a UE transmitting periodic RS for SL beam management to use the transmitted RS as the source of Tx/Rx spatial filter for these transmissions/receptions for the paired UE.

Fig. 16	Basic Tx/Rx spatial filter for transmission/reception for a paired UE

Proposal 10:

- Re-use the concept of Tx/Rx spatial filter relations from Uu that uses SSB or CSI-RS as the source for Tx/Rx spatial filter for DL receptions and UL transmissions

  - Received periodic RS for SL beam management from a paired UE is used as the source for:

    - Rx spatial filter for PSCCH/PSSCH reception and Tx spatial filter for its associated PSFCH transmission for the paired UE, and;

    - Tx spatial filter for PSCCH/PSSCH transmission and Rx spatial filter for its associated PSFCH reception for the paired UE

- In addition, study the following:

  - Transmitted periodic RS for SL beam management for a paired UE is used as the source for:

    - Rx spatial filter for PSCCH/PSSCH reception and Tx spatial filter for its associated PSFCH transmission for the paired UE, and;

    - Tx spatial filter for PSCCH/PSSCH transmission and Rx spatial filter for its associated PSFCH reception for the paired UE

## 3.2	Beam indication and switching schemes

As presented in Section 3.1, the RS used as the source for beam determination for PSCCH/PSSCH/PSFCH for a paired UE can be selected through initial beam-pairing or dedicated configuration. In addition, based on the measurement for multiple RSs for BM and BFR for a pair of UEs in unicast communication, there must be ways to select/switch the beam based on dynamic indication.

As for dynamic beam indication and switching schemes, at least following should be considered:

- A UE switches its beam (Tx/Rx spatial filter) by itself towards a paired UE (Case 1 of Fig. 17)

- One of the UEs indicates/switches beam (Tx/Rx spatial filter) of both UEs in the pair (Case 2 of Fig. 17)

- A UE detects beam failure for a paired UE and switch its beam (Tx/Rx spatial filter) toward the paired UE based on the selected candidate beam for beam failure recovery (as presented in Section 2.4)

For the above 1st and 2nd cases, dynamic signalling is necessary to indicate/inform the beam switch. Similar to Uu Unified-TCI framework which uses a DCI format for the indication, beam switch indication/information for SL can be enabled by using a SCI format. Note that a UE receiving a SCI format requires a certain time gap until it completes the process of the received SCI format and switch the beam accordingly. The necessary time gap should be part of the study.

Fig. 17	Beam switch indication schemes

Proposal 11:

- Beam indication and switching schemes for FR2-SL should include at least following:

  - Case 1: A UE switches its beam (Tx/Rx spatial filter) by itself towards a paired UE

  - Case 2: One of the UEs indicates/switches beam (Tx/Rx spatial filter) of both UEs in the pair

  - Case 3: A UE detects beam failure for a paired UE and switch its beam (Tx/Rx spatial filter) toward the paired UE based on the selected candidate beam for beam failure recovery

- Consider re-using the concept of Uu Unified-TCI framework

  - For a paired UE, one or multiple spatial filter(s) for transmissions to/receptions from the paired UE can be configured/active

  - If one spatial filter is configured/active for a paired UE, the spatial filter is used for transmissions to /receptions from the paired UE

  - If multiple spatial filter are configured/active for a paired UE, one of the spatial filter is used for transmissions to/receptions from the paired UE, where:

    - The spatial filter can be switched/selected according to the indication by a SCI format with a certain time gap (FFS the gap)

# Mode-1 or in-coverage

## 4.1	BS-involvement for SL beam management

For in-coverage case, SL transmission occurs only on UL slots. On UL slots, there must be uplink in the same cell or in neighboring cells. gNBs would want to control the interference from SL transmissions to UL receptions. More specifically, gNB may want to fix or limit the available SL beam(s) for SL transmissions from a SL UE for better Uu interference management. For IAB/NCR, it is possible for gNB to control the beams of IAB-DU/MT and NCR-backhaul/C-link/access-link. Similar mechanism should be studied for SL beam management in in-coverage scenario.

Proposal 12:

- Study BS-involvement for SL beam management for a UE in the coverage of the BS

  - Solutions of Tx/Rx beam control for IAB/NCR can be starting points

## 4.2	Uu + SL beam management at a UE

A UE in a coverage of a BS may have both Uu link and SL unicast-link on the same carrier/band. The UE would need to manage Uu beams and SL beams accordingly. Basically, a UE should be able to use up to one beam (across Uu beams and SL beams) for transmission/reception at a time on the carrier/band even in this scenario. However, maintaining two sets of configured/active beams for Uu and SL at a UE may cause a certain operational impact or configuration restriction. RAN1 should carefully investigate this aspect for in-coverage scenario.

Proposal 13:

- Investigate potential impact when/if a UE has to maintain two set of configured/active beams for Uu beam management and SL beam management on the same carrier/band

  - Note: A UE is, by default, assumed to be able to use one beam (even across Uu beams and SL beams) for transmission/reception at a time on the carrier/band

# Mode-2 or out-of-coverage

## 5.1	Resource sensing/selection

In mode 2, each UE performs resource sensing and selection. For legacy SL, the mode 2 resource sensing/selection is based on SCI format 1 and PSCCH/PSSCH DMRSs. For FR2-SL, all PSCCH/PSSCH transmissions are supposed to be beam-formed. In addition, resource sensing/selection may also be based on a particular beam at the sensing UE. This may impact on reliability of mode 2 resource sensing/selection procedure.

An example is illustrated in Fig. 2. Here, UE1 transmits PSCCH/PSSCH to UE2, and UE3 perform sensing it. All the three UEs use particular beams for Tx/Rx/sensing. In Case 1, UE3 senses UE1’s transmission using a sensing beam that direct to UE1 and therefore, the UE3 takes this sensing result into account for resource selection. On the other hand, in Case 2, UE3 uses sensing beam which is not directed to UE1. In this case, UE3 may consider the resource is available as a result of sensing and may transmit on the resource using the beam corresponding to the sensing beam – causing resource collision for UE2’s reception.

Fig. 17 Example of mode 2 resource sensing when beam-forming is used

Considering that the resource collision is a problem always at a receiving UE, the sensing UE should be able to identify the potential resource collision at the receiving UE. This may not be possible as long as the sensing UE monitors transmission from the transmitting UE. Ideally, the receiving UE should be able to announce/inform that it has a reception from the other UE and the sensing UE can identify that announcement/information. Firstly, RAN1 should study whether the current mode 2 resource sensing/selection properly works when beam-forming are performed for FR2-SL and discuss if any enhancements are necessary. If a significant performance degradation is observed, such solution should be considered.

Proposal 14:

- Study the performance impact of beam-forming operation in mode 2 resource sensing/selection

  - If necessary, consider a methodology of announcement from a receiving UE so that sensing UE can identify the potential resource collision

## 5.2	Inter-UE coordination

Inter-UE coordination enables UEs to exchange preferred/non-preferred resources. However, similar issue as for resource sensing/selection explained in 5.1 exists. For example, legacy IUC request cannot indicate for which beam the IUC info is requested. Also, legacy IUC info cannot indicate which beam the preferred/non-preferred resources in the IUC info is associated. It should be studied whether/how to take into account beam(s) for preferred/non-preferred resources for FR2-SL inter-UE coordination.

An example is illustrated in Fig. 8. UE1 and UE2 has a unicast-link and there are two candidate beam-pairs to use for the unicast communication. Suppose UE2 receives IUC request from UE1 and then UE2 informs preferred/non-preferred resource to UE1. If UE2 may sense transmission from UE3 by using the beam in the beam-pair 2 but not by using the beam in the beam-pair 1, UE2’s preferred/non-preferred resources for beam-pair 1 and beam-pair 2 can be different.

To address the issue, it would be beneficial to incorporate information of Tx/Rx beam(s) that are associated with the preferred/non-preferred resources indicated in the IUC messages. The study should include investigation of incorporating beam information on inter-UE coordination schemes.

Fig. 18	Example scenario: UE2 senses interference from UE3 on the beam for pair 2 but not on the beam for pair 1

Proposal 15:

- Study inter-UE coordination in the presence of beam-forming at the UEs.

  - Investigate the benefit and methodology of incorporating Tx/Rx beam information with the preferred/non-preferred resources for inter-UE coordination.

# Beam correspondence

According to TR 38.912, Uu Tx/Rx beam correspondence was captured as follows for the study:

Tx/Rx beam correspondence at TRP and UE are considered as follows: - Tx/Rx beam correspondence at TRP holds if at least one of the following is satisfied: - TRP is able to determine a TRP Rx beam for the UL reception based on UE’s DL measurement on TRP’s one or more Tx beams - TRP is able to determine a TRP Tx beam for the DL transmission based on TRP’s UL measurement on TRP’s one or more Rx beams - Tx/Rx beam correspondence at UE holds if at least one of the following is satisfied: - UE is able to determine a Tx beam for the UL transmission based on the UE’s measurement on the UE’s one or more Rx beams. - UE is able to determine a Rx beam for the DL reception based on TRP’s indication based on UL measurement on UE’s one or more Tx beams.

Later on, the UE capabilies for Uu Tx/Rx beam correspondence were defined in TS 38.306 as beamCorrespondenceWithoutUL-BeamSweeping, beamCorrespondenceSSB-based-r16, and beamCorrespondenceCSI-RS-based-r16. Necessary requirements corresponding to these capabilities were specified in RAN4 as well.

For FR2-SL, it is necessary to consider SL Tx/Rx beam correspondence. Following the descriptions in TR 38.912, SL Tx/Rx beam correspondence for UE-A that communicates with UE-B can be characterized by one or multiple of the following:

- UE-A has the ability to determine:

  - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams or

  - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

Details should be for further study.

We consider that the Rel-18 FR2-SL study should prioritize the case where SL Tx/Rx beam correspondence applies at each UE.

Proposal 15:

- Consider SL Tx/Rx beam correspondence with the following as the starting point of the condition(s) that SL Tx/Rx beam correspondence applies:

  - UE-A has the ability to determine:

    - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams

    - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

  - For Rel-18 FR2-SL study, prioritize the case where the SL Tx/Rx beam correspondence applies at each UE

# Conclusion

In this contribution, we discussed overall framework of SL beam management for FR2.

We propose to study periodic RS for SL beam management including initial beam-pairing, bean maintenance, and beam failure recovery.

Proposal 1:

- Enable periodic SL RS based beam-sweeping for SL beam management, including initial beam-pairing (IBP), beam maintenance (BM), and beam-failure recovery (BFR)

  - Unified framework shall be able to support variety of scenarios and UE types

For initial beam-pairing, our proposals are following.

Proposal 2:

- Clarify that initial beam-pairing for SL beam management is for a pair of UEs that has not yet established unicast-link / PC5-RRC, so that the initial beam-pairing is part of the study.

Proposal 3:

- For initial beam-pairing, consider re-using the concept of ‘SSB – PRACH association’ from Uu

  - UE1 transmits RSs over multiple beams (beam-sweeping) in periodic manner.

    - FFS: structure of the RSs for beam-sweeping

  - A set of resources for ‘beam-pairing response’ that are associated with UE1’s Tx beams are presented.

    - FFS: structure of the ‘beam-pairing response’

  - UE2 transmits ‘beam-pairing response’ on the resource that is associated with the preferred UE1’s Tx beam.

  - UE1 identifies the UE2’s response and the preferred Tx beam for the UE2 based on the resource that UE1 detects the ‘beam-pairing response’ from UE2

Proposal 4:

- Study detailed procedures of initial beam-pairing using RSs for beam-sweeping and ‘beam-pairing response’

  - At least following should be taken into account:

    - Overhead

    - Delay until initial beam-pairing

  - Both of the following cases should be considered:

    - Case 1: UE1 transmits RSs for beam-sweeping and UE1 initiates unicast-link establishment with UE2 that monitors the RSs from UE1

    - Case 2: UE1 transmits RSs for beam-sweeping and UE2 that monitors the RSs from UE1 initiates unicast-link establishment with UE1

For beam maintenance, our proposals are following.

Proposal 5:

- Enable beam maintenance based on periodic RSs for multiple beams, similar to Uu beam maintenance

  - RSs for beam maintenance are transmitted by one UE (UE1) or by both UEs for a pair of UEs

  - L1-RSRP/SINR report can be transmitted in the same way as SL CSI report (via SL MAC-CE)

For beam failure-recovery, our proposals are following.

Proposal 6:

- Clarify that RAN1 will discuss overall BFR procedure first.

Proposal 7:

- For FR2-SL, beam failure detection re-uses the concept of Uu BFR procedure:

  - A UE is configured to monitor periodic transmissions of BFD-RS and CBD-RSs

  - If a UE detect BFD condition is met, the UE triggers the BFR procedure.

  - BFD condition can be similar to the one for Uu BFR

    - Based on the hypothetical PSCCH BLER derived from the measured RSRPs of the RSs for BFR

Proposal 8:

- For FR2-SL, BFR request re-uses the concept of Uu BFR request:

  - Each UE is configured with resources for BFR request associated with UE1’s Tx beams

    - FFS: structure of the ‘BFR request’

  - Once the UE detects beam failure, the UE transmits a BFR request using the resource associated with the UE1’s preferred Tx beam (= the resource associated with the preferred CBD-RS)

  - For different UE, different set of resources for BFR request can be reserved.

Additional proposal regarding RS transmissions for SL beam management is following.

Proposal 9:

- Study following two options for RS transmissions for SL beam management

  - Option 1: Stand-alone transmission (RSs not associated with PSCCH/PSSCH)

  - Option 2: Non-stand-alone transmission (RSs associated with PSCCH/PSSCH)

- Further discuss the structure of RS for SL beam management:

  - Number of OFDM symbols per RS (per beam)

    - Including impact from AGC

  - RS waveform, sequence generation, etc

    - E.g., reusing SL CSI-RS (or S-SSS)

For beam indication and switching schemes, we propose the following.

Proposal 10:

- Re-use the concept of Tx/Rx spatial filter relations from Uu that uses SSB or CSI-RS as the source for Tx/Rx spatial filter for DL receptions and UL transmissions

  - Received periodic RS for SL beam management from a paired UE is used as the source for:

    - Rx spatial filter for PSCCH/PSSCH reception and Tx spatial filter for its associated PSFCH transmission for the paired UE, and;

    - Tx spatial filter for PSCCH/PSSCH transmission and Rx spatial filter for its associated PSFCH reception for the paired UE

- In addition, study the following:

  - Transmitted periodic RS for SL beam management for a paired UE is used as the source for:

    - Rx spatial filter for PSCCH/PSSCH reception and Tx spatial filter for its associated PSFCH transmission for the paired UE, and;

    - Tx spatial filter for PSCCH/PSSCH transmission and Rx spatial filter for its associated PSFCH reception for the paired UE

Proposal 11:

- Beam indication and switching schemes for FR2-SL should include at least following:

  - Case 1: A UE switches its beam (Tx/Rx spatial filter) by itself towards a paired UE

  - Case 2: One of the UEs indicates/switches beam (Tx/Rx spatial filter) of both UEs in the pair

  - Case 3: A UE detects beam failure for a paired UE and switch its beam (Tx/Rx spatial filter) toward the paired UE based on the selected candidate beam for beam failure recovery

- Consider re-using the concept of Uu Unified-TCI framework

  - For a paired UE, one or multiple spatial filter(s) for transmissions to/receptions from the paired UE can be configured/active

  - If one spatial filter is configured/active for a paired UE, the spatial filter is used for transmissions to /receptions from the paired UE

  - If multiple spatial filter are configured/active for a paired UE, one of the spatial filter is used for transmissions to/receptions from the paired UE, where:

    - The spatial filter can be switched/selected according to the indication by a SCI format with a certain time gap (FFS the gap)

For mode-1 or in-coverage scenario, we propose the following.

Proposal 12:

- Study BS-involvement for SL beam management for a UE in the coverage of the BS

  - Solutions of Tx/Rx beam control for IAB/NCR can be starting points

Proposal 13:

- Investigate potential impact when/if a UE has to maintain two set of configured/active beams for Uu beam management and SL beam management on the same carrier/band

  - Note: A UE is, by default, assumed to be able to use one beam (even across Uu beams and SL beams) for transmission/reception at a time on the carrier/band

For mode-2 or out-of-coverage scenario, we propose the following.

Proposal 14:

- Study the performance impact of beam-forming operation in mode 2 resource sensing/selection

  - If necessary, consider a methodology of announcement from a receiving UE so that sensing UE can identify the potential resource collision

Proposal 15:

- Study inter-UE coordination in the presence of beam-forming at the UEs.

  - Investigate the benefit and methodology of incorporating Tx/Rx beam information with the preferred/non-preferred resources for inter-UE coordination.

Lastly, we propose Tx/Rx beam-correspondence as follows.

Proposal 15:

- Consider SL Tx/Rx beam correspondence with the following as the starting point of the condition(s) that SL Tx/Rx beam correspondence applies:

  - UE-A has the ability to determine:

    - its Tx beam for SL transmission based on UE-A’s SL measurement on one or more UE-A’s Rx beams

    - its Rx beam for SL reception based on UE-B’s SL measurement on one or more UE-A’s Tx beams

  - For Rel-18 FR2-SL study, prioritize the case where the SL Tx/Rx beam correspondence applies at each UE

# Reference

- RP-222806, “WID revision: NR sidelink evolution”, RAN #98-e.

- ‘Antenna Placement and Performance Tradeoffs With Hand Blockage in Millimeter Wave Systems,’ Vasanthan Raghavan, et. al., IEEE Trans. Comm., Vol. 67, Issue 4, April 2019

# Annex

Fig. A1	2D cut of beam forming gains for Case 1 and Case 2

Table A-1.	Assumptions for link-budget analysis

| Parameter | Value |
| --- | --- |
| Transmit power | 23 dBm |
| Tx antenna element gain | 5 dBi |
| Carrier frequency | 30 GHz |
| Path-loss calculation | UMi-LOS, InH-LOS, UMi-NLOS, InH-NLOS from TR 38.901 |
| Bandwidth | 28.8 MHz (20 RBs with SCS 120kHz) |
| Noise figure | 13 dB |
| Noise floor | -174 dBm/Hz |
| Body loss | 8 dB |
| Rx antenna element gain | 5 dBi |

Table A-2.	Link-level simulation parameters for PSSCH BLER

| Parameter | Value |
| --- | --- |
| FFT size | 2048 |
| SCS | 120 kHz |
| Slot duration | 125 us (14 OFDM symbols) |
| DMRS configuration | 3 DMRS symbols |
| MCS table | 256 QAM |
| UE moving speed | 3 km/h |
| Channel model | CDL-A, DS 100 ns for UMi CDL-A, DS 30 ns for InH |
| Antenna configurations | Both Tx and Rx: 4 elements per polarization, 2-pol |
| PSSCH bandwidth | 20 RBs with SCS 120 kHz |
| Target BLER | 10 % |

Table A-3.	Link-level simulation parameters for SCI-1 BLER

| Parameter | Value |
| --- | --- |
| SCI-1 payload | 32 bits |
| CRC length | 24 bits |
| PSCCH bandwidth | 10 RBs with SCS 120 kHz |
| Target BLER | 1 % |
| Note: other parameters are same as for link-level simulation for PSSCH BLER |  |

Table A-4.	Link-level simulation parameters for SCI-2 BLER

| Parameter | Value |
| --- | --- |
| SCI-2 payload | 48 bits |
| CRC length | 24 bits |
| Bandwidth | Function of MCS for PSSCH |
| Target BLER | 1 % |
| Note: other parameters are same as for link-level simulation for SCI-1 BLER |  |

Table A-5.	Required SNRs for SCI-1, SCI-2, PSSCH

| Channel | Required SNR (dB) |  |
| --- | --- | --- |
|  | CDL-A DS 30 ns | CDL-A DS 100 ns |
| SCI-1 (3 OFDM symbols) | -0.23 | -2.92 |
| SCI-1 (2 OFDM symbols) | 2.00 | -0.51 |
| SCI-2 (MCS index 0) | -4.88 | -6.56 |
| SCI-2 (MCS index 9) | 6.45 | 2.47 |
| SCI-2 (MCS index 27) | 8.25 | 6.44 |
| PSSCH (MCS index 0) | -7.00 | -7.51 |
| PSSCH (MCS index 9) | 6.69 | 6.46 |
| PSSCH (MCS index 27) | 24.65 | 25.17 |