3GPP TSG RAN WG1 #112-bis		R1- 2303597

Electronic Meeting, April 17th – 26th, 2023

Agenda item:	9.5.1.3

Source: 	Qualcomm Incorporated

Title: 	Resource Allocation for SL-PRS

Document for:		Discussion and Decision

# Introduction

A work item has been agreed for positioning in RAN #98-e with the following objective for sidelink positioning [1]:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we discuss the design of the dedicated and shared resource pools as well as how to perform Scheme 1 and Scheme 2 resource allocation in each.

Channels in the Dedicated Resource Pool

The dedicated resource pool does not contain legacy UEs and there is no backward compatibility constraint. Therefore, the design could be tailored to maximize resource utilization as we present in this section.

RAN1 agreed that at least SL-PRS is included in the dedicate resource pool with options the only PSCCH is included, and both PSCCH and PSSCH are included. RAN1 also agreed that SCI can be used for reserving or indicating SL-PRS resources:

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication - Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Given the agreement to use SCI for reserving or indicating SL-PRS resources and the lack of standalone control transmissions in sidelink, SL-PRS transmissions should be accompanied by PSCCH in the dedicated resources. If there are cases where only higher-layer reservations are used, then SL-PRS could be transmitted without PSCCH.

Both SCI-2 and SL-SCH require the presence of PSSCH DMRS. Hence, including either significantly reduces the number of symbols available for SL-PRS in a slot, negating the primary benefit of having a dedicated resource pool. Based on the above, we propose to only include PSCCH and SL-PRS in the dedicated resource pool (Option 2). Figure 1 illustrates the corresponding slot structure.

Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool.

Figure 1 Slot structure in a dedicated resource pool.

# Resource Allocation Procedure

## Resource Definition

In the shared resource pool, supported by disallowing multiplexing SL-PRS from different UEs on different comb offsets, the existing resource definition can be used: a resource is a set of contiguous sub-channels in a slot as defined in [6].

Proposal 2: In a shared resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot, i.e. the existing definition for a sidelink resource is reused.

In the dedicated resource pool, the resource definition should be updated to incorporate the comb offset used, accommodating multiplexing of multiple UEs on different comb offsets.

Proposal 3: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size and comb offset.

When TDM is supported by the UE and enabled in the dedicated resource pool, the resource definition should account for the fact that the UE can use a portion of the available sidelink slots for its SL-PRS transmission. The TDM partitioning should be semi-static and not dynamic; otherwise, collisions and AGC issues would arise.

Proposal 4: Sub-slot TDM SL-PRS resource partitioning, when supported by the UE and enabled for the dedicated resource pool, is semi-static.

## Scheme 1 Resource Allocation

RAN1 agreed to no longer consider a UE receiving SL-PRS resources from the LMF, leaving gNB as the only source of signaling:

Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

For the above to work, there needs to be signaling from the LMF to the gNB where the LMF provides the gNB with the requisite SL-PRS properties. The gNB would then provide the UE with a grant containing the resource allocation and respond to the LMF with allocation and configuration information. In this setting, the target UE of the SL-PRS transmission, would receive the SL-PRS configuration from the transmitter UE via SLPP messaging, then would search for and decode PSCCH to locate the resources used for SL-PRS. The last part is similar to how SL data reception operates in Mode 1 resource allocation.

We note that the existing DCI format and RRC configuration suffice to provide SL-PRS resource grants to the transmitter UE and a new DCI format is not needed. This existing signaling includes DCI 3-0 grants from as well as sidelink configured grants Type 1 and Type 2.

Proposal 5: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information.

Proposal 6: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation.

Proposal 7: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging.

Proposal 8: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications.

## Scheme 2 Resource Allocation

RAN1 agreed to support both sensing-based resource selection and random selection. The sensing-based resource allocation procedure from Rel-16 is used in both the dedicated and shared resource pool and to the Rel-17 sensing procedure at least in the shared resource pool:

Agreement Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication). - Working assumption: Sensing-based and random selection can be allowed in the same resource pool - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool - FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17. Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

Rel-17 introduced partial sensing to save UE power compared to full sensing, while providing better performance than random selection. This feature is important regardless of whether the UE is using a dedicated or shared resource pool. Some UE types could be task with only performing periodic SL-PRS transmissions. Such UEs would benefit greatly from partial sensing without impact on performance.

Proposal 9: Partial sensing procedure from Rel-17 is supported in the dedicated resource pool.

In sidelink communications, higher layers provide a packet delay budget. Lower layers will use the packet delay budget to determine a resource selection window and then report available resources to MAC. Since SL-PRS are not a packet and do not have an associated packet delay budget, a replacement is needed to determine which resources lower layers use to report resource availability to higher layers. This can be done by higher layers directly indicating the resource selection window to lower layers.

Proposal 10: Higher layers directly indicate the SL-PRS resource selection window to lower layers.

In terms of which RS to use for sensing RSRP measurements, Rel-16 NR Sidelink introduced the ability to (pre-)configure between PSCCH DMRS and PSSCH DMRS. PSCCH DMRS was included as a method to ensure forward compatibility with changes to PSSCH or PSSCH DMRS. Consequently PSCCH DMRS can be used for sensing operations to select SL-PRS resources and no change to the sensing procedure is necessary.

Proposal 11: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources.

The Rel-17 inter-UE coordination framework cannot be reused for a dedicated resource pool due to lack of cross resource pool indication. This is not a simple issue of missing fields in signaling since the same resource pool could have different indices at different UEs, for example if the UEs have a different number of resource pools or the UEs are configured by different networks. For now, we propose to postpone inter-UE coordination discussions to focus on essential aspects of system functionality.

Proposal 12: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete.

Triggering, Activation/Deactivation, and Configuration of SL-PRS

## Triggering

In sidelink Mode 2 resource allocation, higher layers trigger the resource selection procedure in the physical layer, e.g. upon packet arrival, as shown in Figure 2. Also, higher layers trigger PHY to transmit CSI-RS. Similarly, higher layers determine which sensing type to use from among the (pre-)configured options. In general, triggering starts in the UE’s higher layers, not in lower layers due to the lack of centralized control and scheduling. Higher layers can coordinate with other UEs, e.g. at the protocol or application layer, and exert control within the UE to meet application requirements and maintain system performance, tasks that would have been performed by the network in other cases.

Figure 2 Sidelink transmission triggering and flow in Mode 2 RA

Observation 1: Operations are generally being triggered by the UE’s own higher layers in sidelink when using Mode 2 RA.

One possible option discussed in RAN1 is to use lower-layer signaling, e.g. SCI or SL MAC-CE, from another UE to trigger SL-PRS transmission. Removing higher layers from the loop in this manner brings up a number of issues. For example, there is no security or authentication mechanism associated with SCI or SL MAC-CE and using them to trigger another UEs behavior could result in security issues that are out of RAN1’s expertise. As mentioned earlier, higher layers ensure system performance based on target application, this knowledge is absent in lower layers and using lower-layer triggers results in loss of this functionality. Lastly, triggering by the UE’s own higher layers does not imply that a positioning session cannot be initiated by another UE, only that such operation occurs at the higher layers and then the UE’s PHY receives the trigger from its own higher layers.

Observation 2: Triggering SL-PRS by higher layers offers enhanced security and knowledge of scenario requirements compared to triggering by lower layers.

Observation 3: The origin of an SL-PRS transmission trigger can be another UE, the network, LMF, or the UE itself when the UE’s PHY receives an SL-PRS transmissions trigger from its own higher layers.

Using higher-layer triggering simplifies implementation and specification work. It is also consistent with RAN2’s agreement to wait for SA2’s work on triggering:

Agreement: RAN2 wait for SA2 on the triggering of the positioning procedures from upper layers

Proposal 13: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers only (i.e. Option 1 for Scheme 2).

This corresponds to Option 1 in the RAN1 agreement for Scheme 2.

Agreement In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options: - Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers. - Note: this also includes higher layer triggering from another UE - Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

The issue of requests for SL-PRS transmission was discussed in RAN1 110bis-e and RAN1 111. The core of the discussion was whether a request is a higher-layer or lower-layer message and procedure. Positioning is a higher layer operation and requesting that another UE transmit PRS requires higher layer involvement to validate the source of the request and to prevent denial of service and other attacks that could overload the system. General QoS concerns aside from security also need to be addressed, for example, reducing non-essential SL-PRS traffic in case of an emergency to leave more resources for emergency vehicles and first responders. Further, defining a lower-layer mechanism for requests requires dedicated signaling as was done in Rel-17 inter-UE coordination, which is not necessary in the positioning case since the UEs higher layers will need to decide whether to accept a request or not and there will not be a benefit from the lower latency of L1 signaling.

Proposal 14: Requests for transmission of SL-PRS are introduced as higher-layer messages and no explicit support is needed in lower layers.

## Activation/Deactivation

Activation and deactivation starts and stops a periodic or semi-persistent process. In sidelink, there is no distinction between periodic and semi-persistent processes and they both are considered periodic processes. In Mode 1 resource allocation, periodic transmissions are achieved by configured grant type 1 and type 2, which are RRC and DCI activated, respectively. In Mode 2, there is no external activation or deactivation of periodic transmissions and the decision whether to use a periodic transmission or an aperiodic one is up to the UE’s higher layers. In Uu positioning, the LMF starts/stops periodic PRS transmissions ondemand at the gNB.

For Scheme 2 SL positioning operation, activation/deactivation by higher layers is a straightforward solution that follows existing sidelink design. Like triggering, this would also address issues related to security, scenario requirements, and inter-UE interaction.

Scheme 1 SL positioning operation can reuse the higher-layer activation/deactivation design from Scheme 2 to provide a unified framework, minimizing the differences between the two schemes. It could become part of the higher-layer triggering procedure as well.

Higher-layer activation and deactivation of SL-PRS periodic transmission is a suitable, unified solution for both Scheme 1 and Scheme 2 positioning operation. Hence, we propose to adopt it:

Proposal 15: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only.

## Configuration

In sidelink, there are three methods for providing configuration to a UE: RRC pre-configuration, RRC configuration from the network, and PC5-RRC configuration from another UE. The first two are interchangeable from a PHY point of view and referred to as (pre-)configuration in RAN1 agreements and specifications. PC5-RRC configuration is limited to unicast and hence introduced unnecessary limitations to positioning operations. The limitations of using PC5-RRC as the sole configuration method and transport as discussed in detail in our RAN2 contribution.

Observation 4: PC5-RRC configuration is limited to unicast and could only be applied for that cast type. Hence it is not suitable as the sole method for SL-PRS configuration.

Proposal 16: Higher layer signaling (SLPP) is used for configuration of SL-PRS.

The Rel-17 inter-UE coordination framework cannot be used for the dedicated resource pool due to lack of cross resource singaling ability. This issue goes beyond lack of fields in signaling since the same resource pool could have different indices at different UEs, for example if the UEs have a different number of resource pools or the UEs are configured by different networks.

Reservations and Signaling Details

RAN1 agreed to use at least SCI for indicating SL-PRS reservations or resources:

Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

In addition to indicating SL-PRS resources in SCI-1, we propose to also include them in higher-layer messages (SLPP) that can be sent on a sidelink communication resource pool prior to the SL-PRS transmission when possible. This allows the receiver to anticipate when SL-PRS would arrive. In addition to SL-PRS time and frequency resources, higher layer message would include positioning method, periodicity, comb size and offset, scrambling seed parameter, cast type, QoS, and identification information. This also reduces the size of required L1 messages.

Proposal 17: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources.

In a dedicated resource pool and as discussed earlier, an SCI-1 would accompany SL-PRS in the same transmission. This would be a new SCI-1 format that contains SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index.

Proposal 18: In a dedicated resource pool, a new SCI-1 format is introduced and includes SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index.

In the shared resource pool, and to maintain backward compatibility, part of this information would be included in a new SCI-2 format.

Proposal 19: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity.

Proposal 20: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index.

# Congestion Control

Congestion control is an essential item required for proper functionality of a sidelink system. Without it, the system cannot adapt to changes in load. In a shared resource pool, the existing congestion control mechanism can be reused with minimal changes, e.g. including SL-PRS slots in the RSSI measurements. Similarly, in a dedicated resource pool, the existing mechanism can be reused, potentially with minor updates to measurements.

Proposal 21: Congestion control is introduced for transmissions with SL-PRS and the existing congestion mechanism from sidelink communications is reused with updates if necessary.

# Conclusions

Observation 1: Operations are generally being triggered by the UE’s own higher layers in sidelink when using Mode 2 RA.

Observation 2: Triggering SL-PRS by higher layers offers enhanced security and knowledge of scenario requirements compared to triggering by lower layers.

Observation 3: The origin of an SL-PRS transmission trigger can be another UE, the network, LMF, or the UE itself when the UE’s PHY receives an SL-PRS transmissions trigger from its own higher layers.

Observation 4: PC5-RRC configuration is limited to unicast and could only be applied for that cast type. Hence it is not suitable as the sole method for SL-PRS configuration.

Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool.

Proposal 2: In a shared resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot, i.e. the existing definition for a sidelink resource is reused.

Proposal 3: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size and comb offset.

Proposal 4: Sub-slot TDM SL-PRS resource partitioning, when supported by the UE and enabled for the dedicated resource pool, is semi-static.

Proposal 5: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information.

Proposal 6: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation.

Proposal 7: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging.

Proposal 8: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications.

Proposal 9: Partial sensing procedure from Rel-17 is supported in the dedicated resource pool.

Proposal 10: Higher layers directly indicate the SL-PRS resource selection window to lower layers.

Proposal 11: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources.

Proposal 12: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete.

Proposal 13: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers only (i.e. Option 1 for Scheme 2).

Proposal 14: Requests for transmission of SL-PRS are introduced as higher-layer messages and no explicit support is needed in lower layers.

Proposal 15: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only.

Proposal 16: Higher layer signaling (SLPP) is used for configuration of SL-PRS.

Proposal 17: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources.

Proposal 18: In a dedicated resource pool, a new SCI-1 format is introduced and includes SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index.

Proposal 19: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity.

Proposal 20: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index.

Proposal 21: Congestion control is introduced for transmissions with SL-PRS and the existing congestion mechanism from sidelink communications is reused with updates if necessary.

- References

- RP-223549, “New WID on Expanded and Improved NR Positioning,” Intel Corporation, CATT, Ericsson. RAN #98-e.

- Chair’s notes, RAN1 #109-e, Electronic meeting.

- Chair’s notes, RAN1 #110, Toulouse.

- Chair’s notes, RAN1 #111, Toulouse.

- R1- 2301417, “Reference Signal Design for SL Positioning,” Qualcomm Inc. RAN1 #112, Athens.

- 3GPP TS 38.214, “Physical layer procedures for data (Release 17),” 3GPP.