3GPP TSG RAN WG1 #112bis-e																	       R1-2303598

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.5.2

Source: 	Qualcomm Incorporated

Title: 	NR Carrier Phase Positioning

Document for:		Discussion and Decision

# Introduction

A work item has been agreed for positioning in RAN #98-e with the following objective for carrier phase positioning [1]:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In RAN1#112, the following agreements were made with regards to this objective:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

PRU operation is being standardized in Rel-18 and is important for proper operation of both DL and UL carrier phase positioning, given the difficulties of achieving phase synchronization of multiple TRPs. In RAN1#112, the following conclusions and agreements were made regarding PRU operation:

Conclusion Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact. Agreement The draft LS reply in R1-2302145 is endorsed. Final LS in R1-2302146.

In this contribution, we discuss carrier phase measurements and associated signaling; and aspects signaling enhancements to enable PRU operation

# Carrier phase measurements and reporting

## Carrier phase measurement definition

As seen in Section 1, the agreements in the last meeting have made a good beginning at defining the carrier phase measurements for both DL and UL, with some details still FFS for both. However, the UL definition (NR UL RSCP) is somewhat more solidified than the DL one because there are two possible DL definitions – DL RSCP and DL RSCPD, and it is still not agreed as to whether only RSCP, only RSCPD, or both will be captured in TS38.215.

To decide on this question, we begin by noting that there is a direct analogy to corresponding timing measurements that are already defined for TDOA: The UL TDOA timing measurement is RTOA, whereas the DL TDOA timing measurement is DL RSTD which is the difference of two DL TOAs, and there is no direct report of the individual DL TOAs. If we were to extend this analogy, it would imply defining only DL RSCPD and not DL RSCP. However, the following points should be noted:

- In both the cases of DL TDOA and carrier phase, the differential report is fundamentally a difference of two quantities, and if those quantities were reported separately, it would have been equivalent to or potentially more informative than reporting only the difference.

- A formulation for reporting TOAs would further have to grapple with the issue of what should be a valid range for the TOAs, and whether it should depend on other quantities such as the subcarrier spacing, given that the TOA may likely be measured relative to the DL OFDM symbol timing. However in case of carrier phase, the phase is always over a fixed range of [0, 2pi], eliminating this complexity.

- Further, differential reporting has the added complexity of keeping track of the reference TRP for the differences, and associated signaling. While this has been handled for the DL RSTD definition for DL TDOA, further introducing it for the DL RSCPD for DL carrier phase imposes a lot of further unnecessary complexity when it has to be reported together with legacy reports. When reporting together with UE Rx-Tx measurement, we have to add a notion of reference TRP where none exists in the legacy report. When reporting together with the DL RSTD, there is the potential for more than 1 reference TRP (one for RSTD and one for carrier phase).  Note that reporting of carrier phase together with legacy reports has already been agreed.

- Also note that reporting of additional measurements – specifically, RSRPs, together with the ‘main’ measurement, is already present in legacy reporting of both RSTD and UE Rx-Tx. Since these additional reports are not differential, adding a further phase report which is not differential will reduce the specification complexity.

- Finally note that even if UE reports RSCP, it is only the difference between RSCPs from different TRPs that is actually usable at the LMF for position calculation. Thus the RAN4 requirements should not be defined on RSCP itself, but on differences or possibly on double-differences of RSCPs. The details can be worked out in RAN4, but it is clear that given those differences can be derived from the reported RSCP, an explicit reporting of RSCPD is not needed in order to complete the RAN4 work.

In summary, DL RSCP carries all the information that DL RSCPD would carry, and is much easier to specify UE reporting. Hence we propose:

Proposal 1: Define only DL RSCP and do not define DL RSCPD in TS38.215. UE reports do not include RSCPD.

  - RAN1 understanding is that there will not be RAN4 requirements on absolute accuracy of RSCP. RAN4 requirements are defined on differences of RSCPs, and the detailed definitions of the differencing operations is upto RAN4

Another FFS in all the definitions agreed at the last meeting is on the definition of the reference point for the measurements. While the definition has to eventually be captured in RAN1 specification TS38.215, we note that the definition of the reference point has previously been left to RAN4 (e.g., during Rel-16, for the reference point in FR2). When a new measurement is pretty similar to an existing measurement (e.g., as with Rel-17 DL RSRPP and Rel-16 DL RSRP), RAN1 may easily conclude that the same reference point definition as in the existing measurement should be reused. The phase measurement is however quite new. Our view is that it should still be possible to reuse the reference point definitions for Rel-16/Rel-17 measurements, with the same understanding mentioned in Proposal 1, i.e., that the absolute phases do not mean anything, only the differences are relevant to positioning calculations and the requirements are defined only on the differences. However, there is some likelihood that this could get contentious and ultimately unresolvable in RAN1 discussions, in which case it is preferrable not to waste RAN1 time on it and to leave it to RAN4 instead.

Proposal 2: The reference points for DL and UL RSCP measurements should be the same as those for DL RSTD and UL RTOA respectively

  - If this is not easily agreeable in RAN1; it should be left to RAN4 to define the reference points.

Another point for further discussion from the previous agreements is whether the RSCP will be defined for the i-th path for any i or only for i=1, i.e., for the first path. Note that when additional path reports were defined in Rel-17, there were simulation results providing justification for their use. The same approach should be used here.

Proposal 3: Do not adopt RSCP definition for the i-th path for i>1 unless clear benefits from this are demonstrated

It is also FFS whether/how the measurement timing is defined. Note that existing NR-TimeStamp IE already allows slot-level granularity, and existing reporting allows indication of DL PRS resource measured, which allows a sufficiently fine granularity for the measurement timing.

Proposal 4: Measurement timing can be inferred from the existing reporting of NR-TimeStamp and PRS resource measured.

## Reporting for carrier phase positioning

Regarding reporting related to integer ambiguity, at the last meeting, it was agreed to further consider the following options:

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

Here, Option 4 can be viewed as an analogue or extension of the ExpectedRSTD and ExpectedRSTDuncertainty IEs in the Rel-16 assistance data for UE-based positioning. Just as those IEs helped the UE to narrow down its PRS detection time-range, the integer ambiguity range helps the UE to narrow down its integer ambiguity resolution search range. LMF can provide this based on its rough estimate of UE position, similar to how it provides these legacy IEs. So this option should be supported

Option 3 asks for UE/TRP to report such a search range. There is no analogous report in legacy Rel-17, hence this requires more clarification as to how the UE can be expected to know or come up with the search range to be provided. If UE has some prior knowledge/estimate of its own position (e.g. through other RATs), then it seems more direct to use existing LPP reporting to allow the UE to directly report that position, rather than to come up with the expected search range.

Proposal 5: Support Option 4 “Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data”. For Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”, clarify how the UE/TRP can compute such an estimate.

Regarding reporting for multiple subcarriers, at the last meeting it was agreed to consider the following further options:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

Regarding Option 2, as already explained in the ‘NOTE’, the phase differentials are related to time-of-arrival, as has also been observed in many previous meetings. Thus, there is no good motivation to introduce it. Regarding Option 1, we note that using all available subcarriers (the entire PRS/SRS BW) allows more precisely isolating the timing of the first path, and hence, a more accurate report of the phase of the first path. This is preferable to reporting multiple less-accurate phases each based on only a portion of the available bandwidth. These less-accurate phases could in theory be combined to produce a phase with improved accuracy, following the techniques referred to as ‘lane combination’ in GNSS context. However, there is an important distinction between these cases: In the GNSS case, there is no option for coherent processing of the ‘lanes’, hence they have to be ‘combined’ in the carrier-phase domain. In NR, the ‘lanes’ are being created by subdividing a group of contiguous subcarriers that could easily be processed coherently instead.

Proposal 6: Do not support Options 1 and 2 of the RAN1#112 agreement as listed below:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

It is already agreed that carrier phase measurements can be reported with the legacy reports. On the question of whether they can also be reported all by themselves, we note the following:

- As already agreed, at least the phase of the 1st path will be reported, which implies the first path has to be extracted by the receiver, and hence the timing of the first path is also readily available from the same measurement. Reporting it can only help (as explained next), and there is no compelling reason not to report it.

- The legacy timing-based positioning reports provide a natural way for the LMF to obtain a coarse positioning estimate which can the be used to obtain a more accurate carrier-phase-based positioning estimate with limited complexity of integer ambiguity search achieved from using the coarse position.

- Further as seen in Section 2.1, the addition of DL RSCP measurement to the existing UE reports is fairly straightforward, and so again there is no compelling need for the additional work needed to define a new standalone report. Based on these, we have:

Proposal 7: UE doesn’t report standalone RSCP measurement report, instead RSCP is reported along with the measurements of DL TDOA or DL RTT positioning methods.

Proposal 8: The granularity of the phase reporting should be fine enough so that the error it produces is much smaller than the UE’s positioning error.

In Rel-17, the concept of TEGs was introduced, in order to allow indication from a transmitter that two different transmissions experienced similar transmit group delays, and similarly to allow indication from a receiver that two different received signals experienced similar receive group delays. The group delays may have been different for a variety of implementation specific reasons, such as different transmit/receive chains being used, or variation of group-delay with time/temperature, etc. The details of the implementation are abstracted away and the information relevant to positioning is captured in the reports of the TEG-IDs. In the case of carrier phase positioning, a similar approach can be used, using the concept of a Phase Error group (PEG), which represents a set of signals transmitted (for Tx-PEG) or received (for Rx-PEG) while maintaining relative phase coherence (to within some margin of error, e.g., related to frequency offset errors etc). This is even more important for carrier phase as compared to the case of TEGs, because while the group-delays may be generally expected not to vary too dramatically, the phase coherence may be completely lost for a variety of implementation-specific reasons such as uplink/downlink switching and power-level changes.

Proposal 9: Study PEG-ID reporting scheme, an extension of TEG-ID reporting concept for phase reports. Measurements/transmissions with same PEG-ID are phase-coherent.

- Consider allowing a larger number of PEG-IDs than TEG-IDs.

## Assistance data for Carrier phase, and role of PRUs

Analogous to the RTDinfo IE for UE-based DL TDOA, a UE computing UE-based position using carrier phase requires knowledge of the phase offsets between the TRPs. Due to difficulties in keeping the TRPs perfectly phase-synchronized relative to each other, these phase offsets may vary with time much more rapidly than the time-synchronization offsets vary. Thus, PRUs play an essential role in carrier phase positioning, namely to estimate these offsets and report them to the LMF. Since the offsets may vary rapidly across time (e.g., across slot or occasion index), and also across PRS resources, the LMF should provide the offsets to the UE with more time-granularity than allowed in current RTDinfo IE.

Proposal 10: Assistance data for UE-based carrier phase positioning includes phase differences between the PRS resources of the TRPs, including timing (e.g., PRS occasion) of the PRS resources whose phase difference is being reported.

# Reporting enhancements for simultaneous UE & PRU measurements

## Requesting UE to measure specific PRS resources from specific TRPs

The assistance data already contains TRPs and PRSResourceSets arranged in priority order, either by their ordering in the assistance data IE, or by an explicit vector of indices into the list of assistance data elements provided. From TS38.214,

Within a positioning frequency layer, the DL PRS resources are sorted in the decreasing order of priority for measurement to be performed by the UE, with the reference indicated by nr-DL-PRS-ReferenceInfo being the highest priority for measurement, and the following priority is assumed: - Up to 64 NR-SelectedDL-PRS-IndexPerTRP of the frequency layer are sorted according to priority if nr-SelectedDL-PRS-IndexListPerFreq is provided, or up to 64 NR-DL-PRS-AssistanceDataPerTRP of the frequency layer are sorted according to priority otherwise; - Up to 2 DL-SelectedPRS-ResourceSetIndex per dl-PRS-ID of the frequency layer are sorted according to priority if dl-SelectedPRS-ResourceSetIndexList is provided, or up to 2 NR-DL-PRS-ResourceSet per dl-PRS-ID of the frequency layer are sorted according to priority otherwise

Thus, we could attempt to force UE and PRU to measure the same set of resources by configuring both their assistance data to have those resources to be the highest priority for both the UE and PRU. However, this approach has the following limitations:

- The priority is currently only a suggestion and specification does not prevent the UE from deviating from it.

- There is no relative prioritization across the PFLs and across PRSResources within a PRSResourceSet.

- A PRU may serve multiple UEs, and the PRSResources it needs to measure for different UEs may be different. Hence, its assistance data may need frequent updating to change the priority order depending on which UE is being served. This may be further complicated when assistance data is broadcast to multiple UEs

These limitations can be readily circumvented by defining new signaling, which defines a duration of time during which specific PRSResources are to be measured and reported. This signaling may be carried in the RequestLocationInformation IE instead of in assistance data. The duration may be configured using a measurement window which may be periodic, and multiple such windows may be defined each with their own associated resources, thus avoiding the need to frequently update the assistance data.

Proposal 11: Subject to UE capability, include in RequestLocationInformation, a specific list of PRS resources which UE is required to measure, together with a window duration during which that list is valid.

A similar signaling can be defined for the gNBs as well for UL carrier phase; the LMF should be able to tell the gNBs to configure and measure specific SRS resources, so that (a) multiple gNBs will measure exactly the same SRS from the UE, and (b) the SRS from the UE and the PRU are nearby in time or FDMed at exactly the same time.

Proposal 12:  Introduce signaling for LMF to be able to inform the TRPs to measure specific SRS resources in a specific time-window.

Proposal 13:  Introduce signaling for LMF to be able to request from the serving TRP of a specific slot-offset to be used for the SRS for Positioning configuration (i.e., enhance the “Requested SRS Transmission Characteristics” signaling from the LMF to the serving gNB).

# Conclusion

We have discussed the standardization support needed for carrier phase positioning feature, including further details of definition of measurements and signaling for reporting and for UE-PRU timing-alignment and proposed the following:

Proposal 1: Define only DL RSCP and do not define DL RSCPD in TS38.215. UE reports do not include RSCPD.

  - RAN1 understanding is that there will not be RAN4 requirements on absolute accuracy of RSCP. RAN4 requirements are defined on differences of RSCPs, and the detailed definitions of the differencing operations is upto RAN4

Proposal 2: The reference points for DL and UL RSCP measurements should be the same as those for DL RSTD and UL RTOA respectively

  - If this is not easily agreeable in RAN1; it should be left to RAN4 to define the reference points.

Proposal 3: Do not adopt RSCP definition for the i-th path for i>1 unless clear benefits from this are demonstrated

Proposal 4: Measurement timing can be inferred from the existing reporting of NR-TimeStamp and PRS resource measured.

Proposal 5: Support Option 4 “Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data”. For Option 3 “Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF”, clarify how the UE/TRP can compute such an estimate.

Proposal 6: Do not support Options 1 and 2 of the RAN1#112 agreement as listed below:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier

  - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

Proposal 7: UE doesn’t report standalone RSCP measurement report, instead RSCP is reported along with the measurements of DL TDOA or DL RTT positioning methods.

Proposal 8: The granularity of the phase reporting should be fine enough so that the error it produces is much smaller than the UE’s positioning error.

Proposal 9: Study PEG-ID reporting scheme, an extension of TEG-ID reporting concept for phase reports. Measurements/transmissions with same PEG-ID are phase-coherent.

- Consider allowing a larger number of PEG-IDs than TEG-IDs.

Proposal 10: Assistance data for UE-based carrier phase positioning includes phase differences between the PRS resources of the TRPs, including timing (e.g., PRS occasion) of the PRS resources whose phase difference is being reported.

Proposal 11: Subject to UE capability, include in RequestLocationInformation, a specific list of PRS resources which UE is required to measure, together with a window duration during which that list is valid.

Proposal 12:  Introduce signaling for LMF to be able to inform the TRPs to measure specific SRS resources in a specific time-window.

Proposal 13:  Introduce signaling for LMF to be able to request from the serving TRP of a specific slot-offset to be used for the SRS for Positioning configuration (i.e., enhance the “Requested SRS Transmission Characteristics” signaling from the LMF to the serving gNB).

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning,” Intel Corporation, CATT, Ericsson. RAN #98-e