3GPP TSG RAN WG1 #112bis-e														R1-2303599

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.5.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on LPHAP Positioning

Document for:		Discussion and Decision

# Introduction

In RAN1#112, the following agreements were made with regards to this objective:

Agreement The draft LS in R1-2301991 is endorsed. Final LS in R1-2302074. Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

In this paper, we provide our initial views from RAN1 perspective with regards to this objective.

# SRS for positioning configurations in multiple cells

The following agreement was reached with regards to common parameters for the SRS for positioning configuration:

Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell

With regards to the sequenceID, we believe it makes sense to leave it up to network implementation whether to use the same sequenceId for all SRS transmissions of  a UE within the cells of an area. In short, if a UE is pre-configured with the same sequenceId to be used for all the cells, then the SRS capacity might be limited especially if the area has many cells. Specifically, in this case, the sequenceIDs need to be planned across several cells, a restriction that may be too limiting given the small number of available SRS sequences. For example, imagine we have 10 cells in an area, and 10 UEs per cell in an area. Initially, imagine a network which tries to preserve the SRS capacity and determines that, since the UEs are well distributed across multiple cells, it can assign the same sequenceID to multiple UEs to save SRS capacity. Then, assume a worst-case scenario in which these 100 UEs all move towards the same single cell in this area, but the network is not aware about it (since the UEs are in RRC inactive state and the UEs’ movement is not being tracked). In this case, the SRSs from the UEs will start colliding as they are geographically close to each other and still use the same sequence. To avoid that, the network will have to assign different sequenceId to each  UE at the beginning of the configuration, even if initially the UEs do not collide. However, to do so, i.e. to commit a single sequenceId for each UE across multiple cells, might not be always possible if that area has a large set of TRPs inside; e.g., cell1 may have already assigned a first subset, cell2 may have already assigned a second subset, and the sequences that remain unassigned across both cells may be small). Based on the above, we believe it is better to let the network decide whether to assign the same or different sequenceIds to the UE no matter which is the camping cell in the area.

Proposal 1: For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the sequenceId  to be allowed to be configured differently for cells in an area.

# Consideration on RSRP-change Threshold

The following agreement was reached during the previous meeting:

Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold

With regards to the RSRP change threshold for the TA validity, we propose to use as the “reference” for the computation of the RSRP change threshold the measurement that the UE did on the latest cell that has resulted to a valid TA. This doesn’t necessarily mean the cell from which the UE got an explicit signaling of a TA value, as we describe in the procedure described in Section 3 (and the corresponding proposal). For this Section we make the following proposal:

Proposal 2: For the RSRP change threshold, the reference RS should be:

- obtained from the cell the UE has determined the latest valid TA, where determining a valid TA could happen according to the ways described in Proposal 3.

# UL Timing for area-specific SRS

The following agreement was reached during the previous meeting:

Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

We believe that a solution should try to use multiple of the above options for the following reasons:

- If the DL reference timing changes by a small amount (e.g. it could just be due to measurement errors), there is no need of adjusting the TA value unnecessarily and derive a new “valid TA value”. Similarly, if such DL reference timing changes are due to the UE changing the SSB that it measures from a single cell, there is no need for the UE to be asked to autonomously adjust the TA.

- However, if the DL reference timing changes by a large amount, due to for example, the previous camping cell and the new camping cell having a large Transmission time difference, it may be appropriate for the UE to adjust the TA to compensate for that DL timing difference and try, up to some extend and up to UE implementaton, to keep the UL timing constant. Having UL timing constant during multiple SRS instances would enable TRPs (and the LMF) to average the RTOA/gNBRx-Tx measurements across multiple instances.

- The network can control whether the UE is keeping a constant TA or is adjusting the TA, by adjusting a corresponding threshold.

- Obviously, if the TA timer has expired, the TA has been rendered invalid.

- Finally, if the UE performs RACH, e.g. starts a new SDT and is configured with a regular SRS in RRC_INACTIVE, the UE will receive a new TA value from the network, and this should be the new valid TA for all the signals.

Based on the above, we make the following proposal:

Proposal 3: For the determination of a valid TA to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following:

- If the TA timer has expired, then the TA renders invalid, otherwise,

- If the UE received a new TA value through explicit signaling from the network, this TA value is the new valid TA, otherwise,

- If the  received downlink reference timing has changed by less than a threshold, the UE maintains the same valid TA, otherwise,

- If the received downlink reference timing has changed by more than the threshold,  the UE may autonomously, and up to UE implementation, adjust the TA to obtain a new valid TA.

  - FFS: Whether this clause applies for the case of cell-reselection only

  - Up to RAN4 to discuss UE requirements with regards to these UE autonomous TA adjustments

# Spatial Relation and pathloss reference for area-specific SRS

With regards to the spatial relation and pathloss reference, the following 2 agreements were reached:

Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

We believe that it makes sense to allow the network to decide whether to configure a a spatial relation / pathloss reference to a UE, since a different UE behavior may be expected in each case:

- First, when the spatial relation is absent, a UE could always use the QCL Type D of the strongest SSB from the camping cell as a default mode. This could include the scenario of a UE having a pseudo-omni beam to try to receive that given SSB and any other in other directions.

- Second, if the spatial relation is provided in an SRS resource, we think that the UE, since it is in RRC inactive and it is not being tracked by the network, it should transmit only those SRS resources whose reference can be well measured. For example, imagine a device in FR2 that is configured with 8 SRS resources with 8 different SSBs as spatial relation information. Then, as the UE moves inside the area, if it keeps transmitting the SRS resources without being able to accurately measure the SSBs, it may result to unctrollable interference, which will result to the network having to page all such UEs to reconfigure the SRS. This will miss the point of this feature, which is to save power by avoiding frequent reconfigurations. Therefore, we believe it makes sense to preconfigure the UE will multiple SRS resources, each one “pointing” to a different direction, but only if the UE can measure the reference signal from that direction, it should transmit the SRS, otherwise it should be avoid transmitting the SRS to avoid interference pollution in the area and unnecessary power consumption.

Proposal 4: For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state:

- If spatial relation information is absent from an SRS resource,

  - a UE shall transmit the SRS resource using a spatial domain transmission filter  based on the strongest SSB of the camping cell

- If the spatial relation information is provided for an SRS resource,

  - a UE shall only transmit this SRS if it can accurately measure the spatial relation reference, otherwise, the SRS transmission is dropped

Proposal 5: For the power control relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state:

- If Pathloss RS is absent from an SRS resource set,

  - a UE shall transmit the SRS resources of the cell based on the strongest SSB of the camping cell

- If Pathloss RS is provided for an SRS resource set,

  - a UE shall only transmit the SRS resources of the set if it can accurately measure the provided pathloss RS, otherwise, the SRS transmission is dropped

Proposal 6: Support updating the spatial relation information, or the pathloss RS using an SRS activation message received independently from the pre-configuration signaling.

# SRS for positioning activation/request procedure(s)

With regards to the SRS for positioning activation/request procedure(s), we suggest to wait for RAN2 progress first before dedicating time in RAN1.

Proposal 7: From RAN1 perspective, the procedure for SRS for positioning activation/request can be left for RAN2 to discuss and decide.

# Conclusions

With regards to the topic of LPHAP positioning we make the following proposals:

Proposal 1: For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, support the sequenceId  to be allowed to be configured differently for cells in an area.

Proposal 2: For the RSRP change threshold, the reference RS should be:

- obtained from the cell the UE has determined the latest valid TA, where determining a valid TA could happen according to the ways described in Proposal 3.

Proposal 3: For the determination of a valid TA to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support the following:

- If the TA timer has expired, then the TA renders invalid, otherwise,

- If the UE received a new TA value through explicit signaling from the network, this TA value is the new valid TA, otherwise,

- If the  received downlink reference timing has changed by less than a threshold, the UE maintains the same valid TA, otherwise,

- If the received downlink reference timing has changed by more than the threshold,  the UE may autonomously, and up to UE implementation, adjust the TA to obtain a new valid TA.

  - FFS: Whether this clause applies for the case of cell-reselection only

  - Up to RAN4 to discuss UE requirements with regards to these UE autonomous TA adjustments

Proposal 4: For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state:

- If spatial relation information is absent from an SRS resource,

  - a UE shall transmit the SRS resource using a spatial domain transmission filter  based on the strongest SSB of the camping cell

- If the spatial relation information is provided for an SRS resource,

  - a UE shall only transmit this SRS if it can accurately measure the spatial relation reference, otherwise, the SRS transmission is dropped

Proposal 5: For the power control relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state:

- If Pathloss RS is absent from an SRS resource set,

  - a UE shall transmit the SRS resources of the cell based on the strongest SSB of the camping cell

- If Pathloss RS is provided for an SRS resource set,

  - a UE shall only transmit the SRS resources of the set if it can accurately measure the provided pathloss RS, otherwise, the SRS transmission is dropped

Proposal 6: Support updating the spatial relation information, or the pathloss RS using an SRS activation message received independently from the pre-configuration signaling.

Proposal 7: From RAN1 perspective, the procedure for SRS for positioning activation/request can be left for RAN2 to discuss and decide.

# References

[1] RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98e, December 12-16, 2022.