3GPP TSG RAN WG1 #112bis-e			R1-2303600

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.5.4

Source: 	Qualcomm Incorporated

Title: 	Discussion on Bandwidth aggregation for Positioning

Document for:		Discussion and Decision

# Introduction

In RAN1#112, the following agreements were made with regards to this objective:

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis. Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis. Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Agreement From RAN1 perspective, support UE performs PRS measurement across multiple aggregated PFLs in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE state. Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way Agreement Study potential power control enhancement of simultaneous transmission of SRS for SRS bandwidth aggregation especially in the case when the total uplink transmission power across multiple carriers exceeds P_c,max Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

In this paper, we provide our updated views on the PRS and SRS BW aggregation subagenda.

# Background

# PRS Bandwidth Aggregation

## Common Transmission properties

With regards to the common transmission properties of multiple aggregated PRS resources, the following was agreed:

Agreement To enable PRS bandwidth aggregation between PRS in two or three different PFLs, the following conditions should be satisfied for the aggregated PRS resources from a TRP across the aggregated PFLs: - In the same slot, in same symbols, by the same TRP associated with the same ARP, from the same RF chain (i.e. the same antenna), this implies - FFS: The same gNB Tx TEG and the same UE Rx TEG, the maximum TX timing error margin - The same QCL - The same number of symbols, symbol location within one slot, repetition factor, - FFS: the same periodicity and slot offset - FFS muting pattern - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of PRS resource sets and resources for a TRP - The same power per subcarrier - FFS: the same NR-DL-PRS-SFN0-Offset - Aggregated PFLs are configured on the same aligned numerology grid - FFS: How to maintain contiguous PRS pattern across aggregated bandwidths even in the presence of guard tones (e.g, PFLs with different RE-offset configurations, PFLs with different point A) - Phase continuity between aggregated PFLs

There were a few items for further discussion in the above agreement, for which we provide the following considerations:

- With regards to the “periodicity and slot offset”, “muting pattern”, “number of PRS resource sets and resources”, “SFN0-Offset”, even though strictly speaking there is no need for these parameters to be the same, we think it should be a common understanding that aggregated measurements are expected to be performed only for instances of multiple PRS resources being simultaneously transmitted on the same symbols, as already agreed.

Proposal 1: For multiple PRS resources that are linked for aggregation, it is not necessary to limit having the same

- “periodicity and slot offset”

- “muting pattern”

- “number of PRS resource sets and resources”

- “SFN0-Offset”

as long as a UE is required to report aggregated measurements only on instances of the PRS resources which are simultaneously transmitted on the same symbols of the same slot.

Furthermore, from the receiver complexitly standpoint, it will be more suitable the aggregated PFLs to have comb-offsets amongst the PFLs  that would maintain contiguous PRS pattern across aggregated bandwidths. In the example below, there is a guard band of 6 tones, and using a comb-4 pattern for both, if the first PFL has an RE-offset of 0, the 2nd PFL should have an RE-offset of 2.

Proposal 2: For PRS BW aggregation, the UE is expected to be configured with PRS resources with RE-offset configurations that maintain a per-symbol uniformonly spaced PRS pattern across aggregated bandwidths in the presence of guard tones.

Finally, with regards to the FFS on the gNB Tx TEG and UE Rx TEG, we observe the following:

- PRS resources that are linked for the purpose of PRS aggregation are expected to be transmitted from the same RF chain, with the same QCL and with the same ARP. Therefore, the signaling that the 2 resources will be linked for aggregation purposes is stronger than a Tx TEG indication and not conflicting with Tx TEG indication. Therefore, it may still be OK that the gNB assigns different TxTEG IDs to 2 resources that have the aggregation link, since, having 2 resources on different TxTEGs does not mean that the resources do not have a small Tx Timing Error. If 2 resources are on different TEGs, it actually says nothing about their Tx Timing error.

  - For example, consider the following scenario: The gNB has {RS1, RS5, RS6} in one Tx TEG1 with a margin of Y, and {RS2, RS3, RS4} in another TEG2 with another margin Y. The network can still signal that RS1 and RS2 can be aggregated. Obviously, the “aggregation link” means that the Tx Timing Error between RS1 and RS2 is very small (up to RAN4 to decide how small), but the mere fact that RS1 in TEG1 and RS2 is in TEG2, doesn’t mean that RS1 and RS2 cannot be aggregated.

- Furthermore, with regards to UE Rx TEG, if the UE reports an Rx TEG ID for an aggregated measurement, it should be able to report an Rx TEG ID, but it doesn’t necessarily have to be the same as the Rx TEG ID reported for a legacy measurement that is derived based on one of the PRS resources. For example, imagine a UE measures a first instance of a PRS resource and derives a first non-aggregated measurement for a PRS resource, and then it measures a second instance of this PRS resource in addition to a second PRS resource that are linked together. For the 2nd measurement (the aggregated measurement), a separate Rx chain may be used and therefore it may result to a different Rx TEG.

Proposal 3: For the PRS resources linked for PRS aggregation purposes,

- Up to RAN4 to decide what, if any, should be the maximum TX timing error margin for 2 PRS resources that are linked for PRS aggregation purposes.

- From RAN1 perspective, 2 resources linked for aggregation, can be on the same or different TxTEG.

Proposal 4: For the PRS resources linked for PRS aggregation purposes, no additional constraint is needed to be agreed with regards to the Rx TEG ID used for the aggregated measurement.

## Assistance data Enhancements

With regards to the Assistance data enhancement for this feature, the following was agreed:

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis.

We support the linkage to be on a PRS resource basis and allow to be configured on a per TRP basis. That is, it may be possible that, from all the TRPs appearing the assistance data, only a subset of those, can transmit linked PRS resources for PRS aggregation measurements. In that case, the UE, will be considering only those TRPs for the aggregated measurement report, and for the rest, it will just report a legacy, per-PFL, measurement.

Proposal 5: For PRS aggregation,

- the link on a PRS resource basis from multiple PFLs, i.e., PRS resources from multiple PFLs can be linked together for the purpose of PRS measurement aggregation.

- The link shall be on a per TRP basis, i.e., it should be possible to configure multiple PFLs which have linked PRS resources from TRP1, but have PRS resources from TRP2

## Measurement Report Enhancements

With regards to the measurement report enhancements for this feature, the following was agreed:

Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced

When it comes to the UE’s measurement report, a UE should have the freedom to include either legacy non-aggregated measurements, or the new aggregated measurements inside a regular measurement report. To do so, what is needed to be added in the report is an indication of the additional, one or two, {PRS resource ID and PRS resource set ID} that were used to derive a measurement.

Proposal 6: For the reporting of measurements based on PRS aggregation

- In addition to the legacy reporting structure, a UE is expected to include one or two tuples of {PRS resource ID, PRS resource set ID} in the report which are associated with a given RSTD or UE Rx-Tx measurement.

  - This is applicable to both the first and additional RSTD, UE Rx-Tx measurements of a measurement report

## Location Request Enhancements

With regards to the location request enhancements for this feature, the following was agreed:

Agreement Support joint measurement and report for the PRS resources aggregated across the PFLs for DL-TDOA and multi-RTT positioning methods - In a measurement report element, single RSTD or single UE Rx-Tx time difference is reported for the PRS resources across aggregated PFLs - FFS: RSRP, RSRPP - FFS: In a measurement report, PFL aggregation indication is supported to indicate whether/which PFLs are aggregated for the PRS measurement - FFS whether to use PRS assistance data or use location information request message to indicate UE to perform joint measurement across aggregated PFLs - FFS RSTD reference configuration or report should be enhanced

We believe that the LMF should include in the location information request message whether the UE should perform aggregated measurements or not, and not in the assistance data. The assistance data only provides PRS-related information and configuration, and it is not related on what the UE is expected to measure. For example, imagine the case of broadcast of assistance data, where the same assistance data is being sent out to multiple UEs. If the PRS aggregation link information that appears in the AD is also used to request the UEs to perform aggregated measurements, all UEs will be under the impression that they are supposed to perform such measurements. Furthermore, even for the case of UE-specific assistance data, an LMF may want to send different requests at different time instances; one request that asks from the UE to perform the more power-consuming and accurate measurements based on aggregated PRS resources, and a second request to perform the legacy measurements. Updating the assistance data for that purpose is not an efficient solution.

Proposal 7: For PRS aggregation, use the PRS assistance data to indicate to a UE can perform measurements across aggregated PRS resources.

- Up to the UE’s implementation, e.g., according to the QoS requirements in the location request, whether eventually the UE will perform aggregation.

## On demand PRS Enhancements

With regards to the on-demand PRS enhancements, the following was agreed:

Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details

With regards to the above feature it will be useful a device or an LMF to be able to request for a suggested number of PFLs to be aggregated (i.e. 2 or 3) and how much should be the BW of the PRS in each one.

Proposal 8: For both LMF-initiated and UE-initiated on-demand PRS for BW aggregation support the request of:

- A number of PFLs to be configured on a given band for the purpose of PRS BW aggregation

- The BW to be configured for each PFL for the purpose of PRS BW aggregation

# SRS Bandwidth Aggregation

## Common Transmission Properties

For the common transmission properties of multiple aggregated SRS resources the following was agreed:

Agreement To enable SRS bandwidth aggregation between SRS in two or three carriers, the following conditions should be satisfied for the aggregated SRS resources across the aggregated carriers - In the same slot, in same symbols, from the same antenna, this implies - FFS: The same gNB Rx TEG and the same UE Tx TEG - The same spatial relation - The same startPosition, nrofSymbols - FFS: periodicityAndOffset, and slotOffset - The same numerology, i.e. the same CP and SCS - The same or different bandwidths - The same comb size - FFS: The same number of SRS resource sets and resources - The same Tx PSD (power per subcarrier) - FFS whether to need the same pathloss RS, Po and alpha - Note: the Tx PSD is not captured in RAN1 specifications - FFS: SRS with RE-offset configuration which maintains contiguous SRS pattern across aggregated bandwidths even in the presence of guard tones - Phase continuity between aggregated SRS in different carriers

With a similar argumentation as for the case of PRS aggregation, we first make the following proposal:

Proposal 9: For multiple SRS resources that are linked for aggregation, it is not necessary to limit having the same

- periodicityAndOffset, and slotOffset

- number of SRS resource sets and resources

as long as a UE is required to support coherent transimissions only on instances of the SRS resources which are simultaneously transmitted on the same symbols of the same slot.

Furthermore, similar to what was pointed out in Section 3.1, with regards to the guard band between the contiguous CCs of a PFL, we propose the following to facilitate the receive processing at the gNB side:

Proposal 10: For SRS BW aggregation, the UE is expected to be configured with SRS resources with RE-offset configurations that maintain a per-symbol uniformonly spaced SRS pattern across aggregated bandwidths in the presence of guard tones.

Finally, with regards to the FFS on the UE Tx TEG and gNB Rx TEG, we observe the following:

- Similar to the DL-PRS scenario, the signaling of 2 resources being aggregated is independent of the signaling of the TxTEG.

Proposal 11: For the SRS resources linked for SRS aggregation purposes, the UE can report same or different UE Tx TEG ID to be associated with SRS resources that are linked for aggregation purposes.

- Up to RAN4 to decide what, if any, should be the maximum TX timing error margin for 2 SRS resources that are linked for aggregation pruposes.

Proposal 12: For the SRS resources linked for SRS aggregation purposes, no additional constraint is needed to be agreed with regards to the gNB Rx TEG ID used for the aggregated measurement.

## TRP’s measurement report Enhancements

Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

Proposal 13: With regards to the SRS carrier aggregation indication, support a TRP to report multiple SRS resource IDs associated with a single measurement to indicate which SRS resources are aggregated for the measurement.

## Support of Aperiodic SRS for SRS BW aggregation

With regards to the types of SRS for positioning to be used for SRS BW aggregation, the following agreement was made:

Agreement At least support periodic positioning SRS and semi-persistent positioning SRS for bandwidth aggregation - Support single MAC CE activating positioning SRS resource sets across the linked carriers - FFS whether support aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state. Study a single DCI scheduling positioning SRS across the linked carriers, and check whether the conclusion/agreements in agenda of multi-cell PUSCH/PDSCH scheduling with a single DCI can be reused - FFS MIMO SRS can be supported for bandwidth aggregation, e.g. with UE transparent way

With regards to the above open items, the following are noted:

- There is no reason to exclude aperiodic SRS from this feature, but at the same time, there is no need to introduce a new dedicated DCI, or any changes in the already specified DCIs for aperiodic SRS triggering. The serving gNB may schedule 2 DCIs, each one on a different CC, and ask the UE to transmit simultaneous SRS for positioning in a similar way as it is already specified. Any additional enhancement will be an unnecessary optimization.

Proposal 14: Support aperiodic SRS for positioning to be used for SRS BW aggregation, without however introducing any additional/further enhancement than what is already specified for the purpose of triggering the aperiodic SRS.

## Decoupling of SRS BW aggregation from UL communication CA

With regards to the types of SRS for positioning to be used for SRS BW aggregation, the following agreement was made:

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e. SRS outside BWP and across carriers

For DL, the DL PRS aggregation and the DL CA configuration is separated, and this goes back to a  forward-compatible decision that was made in NR Rel-16 to define DL-PRS outside the BWP and the CC limitations that were constructed for communication purposes.

However, for UL, in NR Rel-16 an unfortunate decision was made to include SRS for positioning as part of the UL active BWP, and follow the regular SRS approach. This has resulted to some shortcomings with regards to the SRS for Positioning and how decoupled is from the communication features.

There has been another SRS feature that is trying to enable the UE to sound on an UL CC without having UL CA, and that is SRS for carrier switching. The UE, in this feature, it receives an SRS RRC configuration in a CC without having PUSCH/PUCCH configured in that CC. We believe that a similar framework could be considered to enable the decoupling of the SRS BW aggregation from having PUSCH/PUCCH in that CC.

Due to work load concerns, we do not think that a different solution/framework should be considered, otherwise it will result to a lot of unnecessary changes.

Proposal 15: Use the SRS carrier switching framework as a starting point for enabling SRS bandwidth aggregation across 2 CCs in intraband CA at least for periodic SRS transmissions.

# Conclusions

With regards to the topic of Bandwidth aggregation for positioning we make the following observations and proposals:

Proposal 1: For multiple PRS resources that are linked for aggregation, it is not necessary to limit having the same

- “periodicity and slot offset”

- “muting pattern”

- “number of PRS resource sets and resources”

- “SFN0-Offset”

as long as a UE is required to report aggregated measurements only on instances of the PRS resources which are simultaneously transmitted on the same symbols of the same slot.

Proposal 2: For PRS BW aggregation, the UE is expected to be configured with PRS resources with RE-offset configurations that maintain a per-symbol uniformonly spaced PRS pattern across aggregated bandwidths in the presence of guard tones.

Proposal 3: For the PRS resources linked for PRS aggregation purposes,

- Up to RAN4 to decide what, if any, should be the maximum TX timing error margin for 2 PRS resources that are linked for PRS aggregation purposes.

- From RAN1 perspective, 2 resources linked for aggregation, can be on the same or different TxTEG.

Proposal 4: For the PRS resources linked for PRS aggregation purposes, no additional constraint is needed to be agreed with regards to the Rx TEG ID used for the aggregated measurement.

Proposal 5: For PRS aggregation,

- the link on a PRS resource basis from multiple PFLs, i.e., PRS resources from multiple PFLs can be linked together for the purpose of PRS measurement aggregation.

- The link shall be on a per TRP basis, i.e., it should be possible to configure multiple PFLs which have linked PRS resources from TRP1, but have PRS resources from TRP2

Proposal 6: For the reporting of measurements based on PRS aggregation

- In addition to the legacy reporting structure, a UE is expected to include one or two tuples of {PRS resource ID, PRS resource set ID} in the report which are associated with a given RSTD or UE Rx-Tx measurement.

  - This is applicable to both the first and additional RSTD, UE Rx-Tx measurements of a measurement report

Proposal 7: For PRS aggregation, use the PRS assistance data to indicate to a UE can perform measurements across aggregated PRS resources.

- Up to the UE’s implementation, e.g., according to the QoS requirements in the location request, whether eventually the UE will perform aggregation.

Proposal 8: For both LMF-initiated and UE-initiated on-demand PRS for BW aggregation support the request of:

- A number of PFLs to be configured on a given band for the purpose of PRS BW aggregation

- The BW to be configured for each PFL for the purpose of PRS BW aggregation

Proposal 9: For multiple SRS resources that are linked for aggregation, it is not necessary to limit having the same

- periodicityAndOffset, and slotOffset

- number of SRS resource sets and resources

as long as a UE is required to support coherent transimissions only on instances of the SRS resources which are simultaneously transmitted on the same symbols of the same slot.

Proposal 10: For SRS BW aggregation, the UE is expected to be configured with SRS resources with RE-offset configurations that maintain a per-symbol uniformonly spaced SRS pattern across aggregated bandwidths in the presence of guard tones.

Proposal 11: For the SRS resources linked for SRS aggregation purposes, the UE can report same or different UE Tx TEG ID to be associated with SRS resources that are linked for aggregation purposes.

- Up to RAN4 to decide what, if any, should be the maximum TX timing error margin for 2 SRS resources that are linked for aggregation pruposes.

Proposal 12: For the SRS resources linked for SRS aggregation purposes, no additional constraint is needed to be agreed with regards to the gNB Rx TEG ID used for the aggregated measurement.

Proposal 13: With regards to the SRS carrier aggregation indication, support a TRP to report multiple SRS resource IDs associated with a single measurement to indicate which SRS resources are aggregated for the measurement.

Proposal 14: Support aperiodic SRS for positioning to be used for SRS BW aggregation, without however introducing any additional/further enhancement than what is already specified for the purpose of triggering the aperiodic SRS.

Proposal 15: Use the SRS carrier switching framework as a starting point for enabling SRS bandwidth aggregation across 2 CCs in intraband CA at least for periodic SRS transmissions.

# References

[1] RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98e, December 12-16, 2022.