3GPP TSG RAN WG1 #112bis-e																	R1-2303601

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.5.5

Source: 	Qualcomm Incorporated

Title: 	Positioning for Reduced Capabilities UEs

Document for:		Discussion and Decision

# Introduction

In RAN1#112, the following agreements were made with regards to this objective:

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Conclusion The scope for RedCap positioning includes FR1 and FR2. Agreement For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same. Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources Agreement The draft LS in R1-2302126 is endorsed with the addition of the two agreements above. Final LS in R1-2302127.

In this paper, we provide our initial views on the following topics with regards to the Redcap frequency hopping:

- Overlapped frequency-domain Hopping

- MG-based and MG-less DL PRS Hopping

- SRS for Positioning Hopping

# Overlapped Frequency-domain hopping in DL & UL

One of the main issues that may need to be addressed is related to the phase offset/mismatch that may be introduced between the hops in the case of Tx or Rx frequency hopping. This is occurring at the device due to the RF retuning that may be occurring at the Tx or Rx chain.

In the figures below, we show the performance of frequency hopping and coherently stitching between the hops for a TDD scenario with 30 KHz and 24 PRB PRS processing in each hop where, an unknown and uncompensated phase offset (θ) exists between the hops randomly chosen between the hops. We observe that a phase jump of a few degrees may have a non-negligible impact on the gains that one could expect from this feature.

With regards to FR2 Redcap devices, we first show the impact of phase shift in PRS band stitching. We assume that phase difference between two consecutive hops θi+1  -θi  follows a uniform distribution between [-θ,θ]  and 4 PRS hops, 100 MHz each, are processed coherently at the receiver. For comparison, we propose two baseline cases.

- Single hop (100MHz) without repetition. (1/4 of resource overhead with 4 PRS hops)

- Single hop (100 MHz) with 4 repeptions, non-coherent combining ( Same resource overhead )

As can be seen from the following Figure, PRS frequency hopping could improve the accuracy until the phase difference is larger than 8 deg with 2 Rx chains, in the InF-SH channel compared with single hops with repetition. Among two baselines, Single hop with repetitions show slightly better accuracy.

Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 2 Rx chains for InF-SH FR2 with 4 Hops (120 KHz SCS) and phase offset chosen uniformly in [-θ,θ] with (θ = {0, 1, 2, 4, 8, 16,32} degrees) between two consecutive hops

In the next Figure, we show the comparison between the PRS frequency hopping without overlapping tones across the hops and the PRS frequency hopping with overlapping tones across the hops, assuming the same resource overhead. We assume 4 PRS hops, 100MHz each, and the overlapping BW is 25 MHz.  Therefore, the total BW is 325 MHz with overlapping tones. With the same phase shift error range, the PRS band stitching with phase offset estimation is inferior to the stitching without phase offset estimation due to the lower total BW. With overlapping tones, receiver can estimate phase offset by using overlapping tones across the hops, thus reduce the phase offset and improve positioning accuracy. For example, the positioning accuracy of PRS hopping with overlapping tones (θ=4) is better than the one without overlap (θ=8).

Positioning Accuracy of RedCap UE (TDD Evaluation Assumptions) with 1 Rx chain for InF-SH FR2 with 4 Hops (120 KHz SCS) , with and without overlapping tones

One way to compensate for such phase offset is to use overlapping tones across the hops, assuming the hops are close-by in time and the device is not in a high mobility scenario. To demonstrate that, we show results below with 1 PRB overlap between the hops,

- for the case of DL-TDOA with 4 hops, each 20 MHz and we compare it against the best performance of 80 MHz coherent processing, and a UE that is only doing non-coherent processing. We simulate this for the UMI, 4 GHz scenario with 200m ISD. We observe that, even with 1 PRB of overlap, the UE is able to estimate well enough the phase error (which was randomly chosen in each hop in the U[-180,180] degrees) and still achieve a performance very close to the 80 MHz performance.

- for the case of M-RTT with 5 hops, each 20 MHz and we compare it against the best performance of 100 MHz coherent processing, and a UE that is only doing non-coherent processing. We simulate this for the UMI, 4 GHz scenario with 200m ISD. We observe that, even with 1 PRB of overlap, the UE is able to estimate well enough the phase error (which was randomly chosen in each hop in the U[-180,180] degrees) and still achieve a performance close to the 100 MHz performance.

The phase offset estimation algorithm that we used is the following: Let L overlapping tones be h11 h12…h1Land h21 h22…h2L are the 2 frequency domain channels. The device could compute gk=minh1k,hnkei(angleh1k-anglehnk) and estimate the phase offset as θ=anglekg(k).

Proposal 1: Support DL-PRS Rx frequency hopping and SRS for Positioning frequency hopping with overlapping tones.

- A UE cannot perform coherent DL-PRS processing or coherent SRS transmissions without overlapped hopping.

Proposal 2: Support a UE to report a capability for the amount of overlap required for DL PRS and SRS for Positioning.

Proposal 3: Support in the specification multiple overlap options between 2 frequency-adjacent hops with a minimum value of 1 PRB.

# DL PRS Frequency Hopping: MG-based and MG-less PRS processing

## 4.1 MG-based DL-PRS Frequency Hopping

The following was agreed with regards to the MG-based PRS frequency hopping:

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW

The case of MG-based DL-PRS processing could result to relatively straightforward specification impact as follows:

- A UE could first report the support of Rx frequency hopping in a band for MG-based DL-PRS processing, together with its Rx retuning time capability.

- Within a single MG instance, which can be as much as 20 msec long already in the specification, the UE is expected to perform multiple hops, and therefore, instead of only considering a retune time at the beginning and the end of the MG, we could have additional such times as shown in the figure below:

- We can support up to 4 Rx retuning instances, having in mind the case of a 20 MHz UE that wants to cover up to 100 MHz BW (5 hops).

Proposal 4: Support a MG-based DL-PRS frequency hopping approach in which the UE is expected to perform up to N Rx Retunings during a single MG instance in order to measure multiple frequency parts of a single PRS resource, with N = [4].

- FFS: the case of hopping across multiple instances of a MG

## 4.2 MG-less DL-PRS Frequency Hopping

The following was agreed with regards to the MG-less PRS frequency hopping:

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW

With regards to MG-less DL-PRS frequency hopping, we acknowledge that a first solution with rather small specification changes would be to perform DL-PRS frequency hopping by reusing the BWP switching framework. In this case, a UE can be configured with up to 4 BWPs, and as the UE switches active BWP, a different part of the same PRS instance is expected to be measured, assuming a correctly-configured PPW. However, for DCI and timer-based BWP switching mechanisms, the BWP switch delay requirements, as specified in 3GPP TS 38.133 are given in Table 2.2.3-1, are of the order of 1 msec or more. For RRC-based BWP switching, the BWP switch delay is even larger, i.e., several milliseconds. Furthermore, it requires the UE to support BWP switching with all the RRC-reconfiguration baggage this feature entails, whereas for PRS frequency hopping what is needed is just an RF retune. Based on the above we make the following observations:

Observation 1: BWP switching delay is large considering the use case of PRS frequency hopping.

Observation 2: Reusing BWP switching for enabling DL-PRS frequency hopping would couple 2 different features (BWP switching) to (PRS Frequency hopping) unnecessarily.

Observation 3: Reusing BWP switching for enabling DL-PRS frequency hopping would enable to sample only up to 4 subbands (e.g. total of 80 MHz for a 20 MHz UE).

However, a second more appropriate way to consider PRS hopping within PPW is to consider introducing retune periods before and after a PRS instance, and include those in the prioritization/collision rules that are already specified. Specifically, we make the following observations for different PPW types:

- For Type 1A and Type 1B, the UE will be either way performing a time-domain prioritization for all the symbols within a PPW, and therefore, using a subset of these symbols for retuning will not affect the overall system. As long as the PPW is long enough, a UE could do a retune within a single PPW instance and measure multiple hops in a similar way that it will be specified for MG-based hopping.

Observation 4: For PPW-based Type 1A and Type 1B PRS processing, a UE could be performing Rx frequency hops during a single instance of a PPW in a similar way as for the case of MG-based PRS frequency hopping.

- For Type 2 however, due to the additional time spent in the retune, the other channels may be affected even outside the PRS symbols. Having said that though, we can just clarify that any retune time needed before and after a PRS hop should be counted when it comes to the prioritization/collision rules. Such a behavior is actually similar to how RAN4 treats PRS collisions with other channels in RRC inactive state.

Proposal 5: Support Rx frequency hopping for MG-less PRS processing using the following principle as a starting point:

- A UE may perform Rx frequency hopping within a PPW instance under the condition that the required retune time before and after each hop is taken into account in the specified PRS prioritization/collision rules.

  - E.g. If the retune time before or after a low-priority PRS hop collides with a high-priority channel, then the UE does not perform the Rx frequency hopping.

- FFS: Details including relation to the Rx frequency hopping with the PPW types

# SRS for Positioning Frequency Hopping

## 4.1 SRS for Positioning Frequency Hopping mechanism

In RRC connected, SRS for Positioning is configured within active BWP. For the purpose of enabling SRS frequency hopping for positioning for the purpose of increased SRS bandwidth we propose the following approach:

- Define an SRS for Positioning that is associated with a component carrier (CC) and not the active BWP. In that case, the UE is expected it will need a “transmission gap”, similar to the measurement gap that we have for the DL, during which the UE is expected to perform fast switches. In this case, the UE could report switching time capabilities, similar to the switching times we hve for SRS transmission in RRC inactive.

Proposal 6: For SRS for Positioning frequency hopping:

- Define SRS for positioning associated with a CC (and not an active BWP) with each own numerology and bandwidth (e.g. similar to the SRS for Positioning of Rel-17 RRC inactive feature).

- Introduce a transmission/switching/retune gap before and after a transmission of such an SRS resource.

## 4.2 SRS for Positioning Frequency Hopping pattern

The following was agreed with regards to the SRS Hopping pattern:

Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

With regards to the SRS frequency hopping pattern, we consider the following 2 options:

- Use the current hopping formula as a starting point, but ensure that there is a configurable parameter of the amount of overlap between frequency-consecutive hops. Such a solution is shown in the Figure below for a 48-PRB hopping bandwidth

The changes needed in the specification are rather small and are shown below:

- The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+b=0BSRS(KTCMsc,bSRSnb -Moverlap⋅nb⋅b'=b+1BSRSNb')

  - where Moverlap is the number of tones of frequency overlap of 2 frequency-adjacent hops

- Use a staircase-based hopping formula as a starting point, in which the time-domain consecutive hops are sounding frequency-domain adjacent chunks of the bandwidth.

Even though the 2nd solution would ensure that time-domain consecutive hops are always overlapping, and therefore reducing the gap between the overlapped PRBs which are used for phase offset compensation, it has lower UE multiplexing capacity, compared to the 1st solution. Specifically, one could argue that, other UEs could be multiplexed as follows in the following Figure, but again in this case, for 4 out of the UEs (UEs 2,3,4,5) there exist overlapped hops that are far away in time.

Proposal 7: Study further at least the following two options for the SRS frequency hopping pattern:

- Option 1: Use the current hopping formula of SRS as a starting point and update it such that there can be frequency domain overlap over 2 hops that are adjacent in frequency.

  - Example of the update in the frequency-domain starting position k0(pi) :

k0(pi)=k0(pi)+b=0BSRS(KTCMsc,bSRSnb -Moverlap⋅nb⋅b'=b+1BSRSNb')

- Option 2: Use a staircase-like hopping formula with a parameter that controls the amount of overlap of frequency domain adjacent hops.

# Conclusions

Overall, we make the following observations and proposals:

Proposal 1: Support DL-PRS Rx frequency hopping and SRS for Positioning frequency hopping with overlapping tones.

- A UE cannot perform coherent DL-PRS processing or coherent SRS transmissions without overlapped hopping.

Proposal 2: Support a UE to report a capability for the amount of overlap required for DL PRS and SRS for Positioning.

Proposal 3: Support in the specification multiple overlap options between 2 frequency-adjacent hops with a minimum value of 1 PRB.

Proposal 4: Support a MG-based DL-PRS frequency hopping approach in which the UE is expected to perform up to N Rx Retunings during a single MG instance in order to measure multiple frequency parts of a single PRS resource, with N = [4].

- FFS: the case of hopping across multiple instances of a MG

Observation 1: BWP switching delay is large considering the use case of PRS frequency hopping.

Observation 2: Reusing BWP switching for enabling DL-PRS frequency hopping would couple 2 different features (BWP switching) to (PRS Frequency hopping) unnecessarily.

Observation 3: Reusing BWP switching for enabling DL-PRS frequency hopping would enable to sample only up to 4 subbands (e.g. total of 80 MHz for a 20 MHz UE).

Observation 4: For PPW-based Type 1A and Type 1B PRS processing, a UE could be performing Rx frequency hops during a single instance of a PPW in a similar way as for the case of MG-based PRS frequency hopping.

Proposal 5: Support Rx frequency hopping for MG-less PRS processing using the following principle as a starting point:

- A UE may perform Rx frequency hopping within a PPW instance under the condition that the required retune time before and after each hop is taken into account in the specified PRS prioritization/collision rules.

  - E.g. If the retune time before or after a low-priority PRS hop collides with a high-priority channel, then the UE does not perform the Rx frequency hopping.

- FFS: Details including relation to the Rx frequency hopping with the PPW types

Proposal 6: For SRS for Positioning frequency hopping:

- Define SRS for positioning associated with a CC (and not an active BWP) with each own numerology and bandwidth (e.g. similar to the SRS for Positioning of Rel-17 RRC inactive feature).

- Introduce a transmission/switching/retune gap before and after a transmission of such an SRS resource.

Proposal 7: Study further at least the following two options for the SRS frequency hopping pattern:

- Option 1: Use the current hopping formula of SRS as a starting point and update it such that there can be frequency domain overlap over 2 hops that are adjacent in frequency.

  - Example of the update in the frequency-domain starting position k0(pi) :

k0(pi)=k0(pi)+b=0BSRS(KTCMsc,bSRSnb -Moverlap⋅nb⋅b'=b+1BSRSNb')

- Option 2: Use a staircase-like hopping formula with a parameter that controls the amount of overlap of frequency domain adjacent hops.

# References

[1] RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, RAN#98e, December 12-16, 2022.