3GPP TSG RAN WG1 #112bis-e		                                                              R1-2303603

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.7.1

Source: 	Qualcomm Incorporated

Title: 	Techniques in spatial and power domains

Document for:	Discussion/Decision

# Introduction

3GPP Rel-18 has the following objectives for necessary enhancements on CSI and beam management related procedures to enable efficient adaptation of spatial elements and necessary enhancements on CSI related procedures to enable efficient adaptation of PDSCH/CSI-RS power offsets for network energy savings [1].

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

Since CSI related procedures are being discussed for both spatial and power domain techniques in this objective, we will first provide a recap on the legacy CSI feedback framework. Then, we will discuss our view on enhancing CSI and beam management related procedures.

On NR CSI feedback framework, it includes Reporting Settings, Resource Settings and List of trigger states. The higher layer configuration records corresponding to each of the setting types are as follows:

- Reporting Settings is configured in CSI-ReportConfig

- Resource Settings is configured in CSI-ResourceConfig

- List of trigger states is configured in CSI-AperiodicTriggerStateList or CSI-SemiPersistentOnPUSCH-TriggerStateList. Each trigger state in the list of trigger states contains a list of associated Reporting Settings.

CSI Reporting Setting

Each Reporting Setting contains the following parameters:

- Codebook configuration including codebook subset restriction indicated in the record CodebookConfig e.g.,

  - Type I single-panel codebook (CodebookType set to 'typeI-SinglePanel')

  - Type I multi-panel codebook (CodebookType set to 'typeI-MultiPanel')

  - Type II codebook without port selection (CodebookType set to 'typeII')

  - Type II codebook with port selection (CodebookType set to 'typeII-PortSelection')

- Time-domain behavior indicated by reportConfigType and can be set to

  - 'aperiodic'

  - 'semiPersistentOnPUCCH',

  - 'semiPersistentOnPUSCH', or

  - 'periodic'

- Frequency granularity for CQI and PMI

- CSI band defining the frequency subbands in which the CSI report is valid

- Measurement restriction configured by timeRestrictionForChannelMeasurements and timeRestrictionForInterferenceMeasurements, each of which controls whether the UE is allowed to perform time averaging for the relevant quantity

- CSI-related quantities to be reported by the UE such as the layer indicator (LI), L1-RSRP, CRI, SSBRI (SSB Resource Indicator), RI, PMI and CQI.

CSI Resource Settings

Each CSI Resource Setting is a collection of measurement resources of a given type and contains a list of CSI Resource Sets given by higher layer parameter csi-RS-ResourceSetList.  The following are configured for a CSI Resource Setting. There can be one, two, or three Resource Settings configured for each Reporting Setting.

- NZP CSI-RS resource for channel measurement

- NZP CSI-RS resource for interference measurement

- CSI-IM resource for interference measurement

Table 1: Possible CSI Reporting and CSI Resource Configurations

|  |  | CSI Report Configuration |  |  |
| --- | --- | --- | --- | --- |
|  |  | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| CSI Resource Configuration | Periodic CSI-RS | Supported No dynamic triggering/activation Reporting on PUCCH | Supported For reporting on PUCCH, the UE receives an activation command MAC-CE For reporting on PUSCH, the UE receives triggering on DCI | Supported Triggered by DCI Reporting on PUSCH |
|  | Semi-Persistent CSI-RS | Not Supported | Supported For reporting on PUCCH, the UE receives an activation command MAC-CE For reporting on PUSCH, the UE receives triggering on DCI | Supported Triggered by DCI Reporting on PUSCH |
|  | Aperiodic CSI-RS | Not Supported | Not Supported | Supported Triggered by DCI Reporting on PUSCH |

Table 1 summarizes the allowed combinations of CSI resource time domain behavior and CSI reporting time domain configuration. The frequency and time domain patterns of NZP CSI-RS resource configured by CSI-RS-ResourceMapping is provided in Table 2.

The UE may be configured with multiple CSI report settings. For example, for aperiodic CSI reporting or aperiodic CSI reporting, the UE may be configured with a list of trigger states. Each trigger state in the list of trigger states contains a list of associated Reporting Settings. One or more trigger states may be dynamically triggered.

Table 2 CSI-RS locations in frequency and time within a slot in 3GPP standards

| Row | Ports X | Density ρ | CDM-Type | k, l | CDM group index j | k' | l' |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | No CDM | k0,l0, k0+4,l0, k0+8,l0 | 0, 0, 0 | 0 | 0 |
| 2 | 1 | 1, 0.5 | No CDM | k0,l0 | 0 | 0 | 0 |
| 3 | 2 | 1, 0.5 | FD-CDM2 | k0,l0 | 0 | 0, 1 | 0 |
| 4 | 4 | 1 | FD-CDM2 | k0,l0, k0+2,l0 | 0, 1 | 0, 1 | 0 |
| 5 | 4 | 1 | FD-CDM2 | k0,l0, k0,l0+1 | 0, 1 | 0, 1 | 0 |
| 6 | 8 | 1 | FD-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0 | 0, 1, 2, 3 | 0, 1 | 0 |
| 7 | 8 | 1 | FD-CDM2 | k0,l0, k1,l0, k0,l0+1, k1,l0+1 | 0, 1, 2, 3 | 0, 1 | 0 |
| 8 | 8 | 1 | CDM4-(FD2, TD2) | k0,l0, k1,l0 | 0, 1 | 0, 1 | 0, 1 |
| 9 | 12 | 1 | FD-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k4,l0, k5,l0 | 0, 1, 2, 3, 4, 5 | 0, 1 | 0 |
| 10 | 12 | 1 | CDM4-(FD2, TD2) | k0,l0, k1,l0, k2,l0 | 0, 1, 2 | 0, 1 | 0, 1 |
| 11 | 16 | 1, 0.5 | FD-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1 | 0, 1, 2, 3, 4, 5, 6, 7 | 0, 1 | 0 |
| 12 | 16 | 1, 0.5 | CDM4-(FD2, TD2) | k0,l0, k1,l0, k2,l0, k3,l0 | 0, 1, 2, 3 | 0, 1 | 0, 1 |
| 13 | 24 | 1, 0.5 | FD-CDM2 | k0,l0, k1,l0, k2,l0, k0,l0+1, k1,l0+1, k2,l0+1,k0,l1, k1,l1, k2,l1, k0,l1+1, k1,l1+1, k2,l1+1 | 0, 1, 2, 3 ,4, 5, 6, 7, 8, 9, 10, 11 | 0, 1 | 0 |
| 14 | 24 | 1, 0.5 | CDM4-(FD2, TD2) | k0,l0, k1,l0, k2,l0, k0,l1, k1,l1, k2,l1 | 0, 1, 2, 3, 4, 5 | 0, 1 | 0, 1 |
| 15 | 24 | 1, 0.5 | CDM8-(FD2, TD4) | k0,l0, k1,l0, k2,l0 | 0, 1, 2 | 0, 1 | 0, 1, 2, 3 |
| 16 | 32 | 1, 0.5 | FD-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0,k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1, k0,l1, k1,l1, k2,l1, k3,l1, k0,l1+1, k1,l1+1, k2,l1+1, k3,l1+1 | 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15 | 0, 1 | 0 |
| 17 | 32 | 1, 0.5 | CDM4-(FD2, TD2) | k0,l0, k1,l0, k2,l0, k3,l0, k0,l1, k1,l1, k2,l1, k3,l1 | 0, 1, 2, 3, 4, 5, 6, 7 | 0, 1 | 0, 1 |
| 18 | 32 | 1, 0.5 | CDM8-(FD2, TD4) | k0,l0, k1,l0, k2,l0, k3,l0 | 0, 1, 2, 3 | 0,1 | 0,1, 2, 3 |

# Efficient Adaptation of Spatial Elements

Massive MIMO is one of the fundamental technologies in 5G NR for high spectral efficiency and extended coverage. For downlink transmission, massive MIMO is equipped with a large number of transceiver chains (e.g., 64 chains in FR1 are typically deployed in commercial 5G networks especially at carrier frequency of 3.5GHz or higher). Each transceiver chain may be connected to one or more power amplifiers. It has been shown that the power amplifiers consume 70%-80% of BS power. To reduce BS power consumption, a cell can turn on/off one or more power amplifiers depending on the time/frequency resource utilization in the cell. Equivalently, a cell can turn on/off one or more transceiver chains. Such on/off adaptation scheme might be possible by current standards. However, it may be further improved as being standardized in R18 under the following objective for necessary enhancements on CSI and beam management to enable efficient adaptation of spatial elements [1]. It should be also noted that 3GPP standards only specify UE behaviors with respect to antenna ports. The mapping between antenna ports and transceiver chains is left to implementation.

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

## Enhancements on CSI Procedure

From our perspectives, the dynamic adaptation of spatial elements can be realized by the following two main steps as illustrated in Figure 1:

Figure 1: Dynamic adaptation of spatial elements

- Step 1: CSF for adaptation of spatial elements

  - gNB configures multiple CSI report settings to the UE, where each CSI report setting corresponds to a CSI-RS antenna port configuration.

  - gNB turns on active CSI-RS antenna ports for CSI measurement and reporting

  - UE measures and reports CSI associated with all active CSI-RS antenna ports and/or subset(s) of active CSI-RS antenna ports.

- Step 2: PDSCH transmission at gNB

  - gNB can switch among different configurations of active spatial elements based on CSI reports from UE.

Step 1 can be implemented based on the legacy 3GPP standards. For example, to switch between 32-port CSI-RS and 24-port CSI-RS, the UE can be configured with two separate CSI report configurations in which the CSI report config 2 may include one or multiple 24-port CSI-RS resources which are a subset of 32-port CSI-RS resource configured in the CSI report config 1 as being illustrated in Table 3. Note that the antenna port indexing for a given antenna array is from UE perspective following specification in TS 38.211.

Table 3: Example of CSI report configurations for adaptation between 32-port and 24-port CSI-RS

|  | CSI report config 1 | CSI report config 2 |
| --- | --- | --- |
| CSI-RS pattern | Row 17 | Row 14 |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(6, 2) |

The key question being asked is whether adaptation of spatial elements based on the legacy 3GPP standards is efficient or not. If it is not efficient, where is inefficiency?

Observation 1: it is important to discuss whether the legacy CSI feedback framework can already enable efficient adaptation of spatial elements. If it cannot, which part of CSI feedback framework is limiting factor in enabling such adaptation.

To answer the question, we should first discuss what “efficient adaptation” really means. From our perspectives, it means that CSI-RS resource overhead for CSI is low and CSI reports associated with CSI report settings in Step 1 provide useful information for the gNB to dynamically turn on/off spatial elements. In order to achieve such useful information, the reduced CSI-RS resource associated with CSI-RS antenna configuration in an CSI report setting to be used for CSI computation should be a subset of the CSI-RS resource with all antenna ports being active. Furthermore, the reduced CSI-RS resource pattern is one of the patterns provided in Table 2 and the antenna array corresponding to the reduced CSI-RS resource pattern is a uniform linear array (which is assumption for codebook design) with supported configuration in provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively. When the resources have these properties, we call them having a nested structure. Later, we will show that CSI report configurations as being illustrated in Table 3 provides useful CSI report for the gNB to dynamically adapt between 32-port and 24-port CSI-RS. In particular, for 32-port configuration, the gNB can turn off first two columns or last two columns of the antenna array to achieve 24-port configuration.

Table 5.2.2.2.1-2 in TS 38.214: Type I single-panel codebook supported configurations of N1,N2

| Number of CSI-RS antenna ports, PCSI-RS | N1,N2 | Number of polarizations |
| --- | --- | --- |
| 4 | (2,1) | 2 |
| 8 | (2,2) | 2 |
|  | (4,1) | 2 |
| 12 | (3,2) | 2 |
|  | (6,1) | 2 |
| 16 | (4,2) | 2 |
|  | (8,1) | 2 |
| 24 | (4,3) | 2 |
|  | (6,2) | 2 |
|  | (12,1) | 2 |
| 32 | (4,4) | 2 |
|  | (8,2) | 2 |
|  | (16,1) | 2 |

Observation 2: CSI reports associated with different CSI-RS antenna port configurations should provide useful information for the gNB to dynamically adapt spatial elements. In particular, the reduced CSI-RS resource associated with a CSI report setting for CSI computation has a nested structure as follows:

- It is subset of the CSI-RS resource with all antenna ports being active,

- its resource pattern is one of the legacy patterns, and

- the antenna array corresponding to the reduced CSI-RS resource pattern is a uniform linear array with supported configuration in provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively.

We examine legacy CSI-RS resource patterns in Table 2 to understand if they have such nested structure. It has been observed that the CSI-RS resource patterns with different CDM group sizes do not have the nested structure. Furthermore, for CSI-RS resources with the same CDM group size, only a few CSI-RS resource patterns provide the nested structure. Below we provide detail analysis for some examples:

Example 1

gNB deploys 32-port CSI-RS resource pattern specified in row 17 of Table 2 with a choice of k0=0, 2, 4, 8; l0=10 and Type I single-panel codebook configuration (N1,N2)=(8, 2). Assume the gNB wants to turn off transceiver chains associated with CSI-RS antenna ports having time/frequency mapping into the 4th CDM group (i.e., j = 3) and the 8th CDM group (i.e., j = 7). The gNB requests an CSI report associated with the reduced resources in the remaining 24 ports.

- The reduced CSI-RS resource pattern is identical to the 24-port CSI-RS resource pattern specified in row 14.

- The Type I single-panel codebook configuration corresponding to the reduced CSI-RS resource pattern becomes codebook configuration (N1,N2)=(6, 2) which is one of the configurations in Table 5.2.2.2.1-2 of TS 38.214.

Therefore, the resource pattern in row 14 is nested in resource pattern in row 17. This means that the gNB can use the resource pattern in row 14 to configure 24-port CSI-RS resources to turn off antenna ports in the first two antenna columns or the last two antenna columns of the antenna array (N1,N2)=(8, 2).

Table 4: Resource pattern in Row 14 is nested in resource pattern in Row 17

|  | All CSI-RS ports are active | Reduced CSI-RS ports and resource for CQI computation | Equivalent CSI-RS port config |
| --- | --- | --- | --- |
| CSI-RS pattern | Row 17 | Row 17 | Row 14 |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(6, 2) |  |

Example 2

Similar to Example 1, gNB deploys 32-port CSI-RS resource pattern as specified in row 17 of Table 2 with a choice of k0=0, 2, 4, 8; l0=10 and Type I single-panel codebook configuration (N1,N2)=(8, 2). Now, assume the gNB wants to turn off transceiver chains associated with CSI-RS antenna ports having time/frequency mapping into the 5th – 8th CDM groups (i.e., j = 4-7). The gNB requests an CSI report associated with the reduced resources in the remaining 16 ports.

- The reduced CSI-RS resource pattern is identical to the 16-port CSI-RS resource pattern specified in row 12.

- The Type I single-panel codebook configuration corresponding to the reduced CSI-RS resource pattern becomes codebook configuration (N1,N2)=(8, 2) but with only one polarization which may be arguably not one of the configurations in Table 5.2.2.2.1-2 of TS 38.214. To make it compliant to a valid configuration, we can implement this reduced pattern with (N1,N2)=(8, 1) with two virtual polarizations as illustrated in Table 5. However, such virtual polarization implementation shall downgrade the performance of CSI-RS and PDSCH due to loss of polarization diversity. Furthermore, for the case where both polarizations are mapped into the same transceiver chains, turning off the transceiver chains is not possible. As a result, the reduced CSI-RS pattern is not efficient for adaptation of spatial elements.

Table 5: Loss of polarization diversity with reduced CSI-RS resource pattern

|  | All CSI-RS ports are active | Reduced CSI-RS ports and resource for CQI computation | Equivalent CSI-RS port config |
| --- | --- | --- | --- |
| CSI-RS pattern | Row 17 | Row 17 | Row 12 |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(8, 2) |  |

Example 3

gNB deploys 32-port CSI-RS resource pattern as specified in row 16 of Table 2 with a choice of k0=0, 2, 4, 8 and l0=8, l1=12 and Type I single-panel codebook configuration (N1,N2)=(8, 2). Assume the gNB wants to turn off transceiver chains associated with CSI-RS antenna ports having time/frequency mapping into the CDM groups with index j = 3, 7, 11, 15. The gNB requests an CSI report associated with the reduced resources in the remaining 24 ports.

- The reduced CSI-RS resource pattern is identical to the 24-port CSI-RS resource pattern specified in row 13.

- However, the antenna array corresponding to the reduced CSI-RS resource pattern becomes non-uniform linear array with Type I single panel codebook configuration. If we treat the reduced pattern with Type I multi-panel codebook configuration, the antenna array is uniform linear array with (Ng,N1,N2)=(2, 3, 2) which is not supported by Table 5.2.2.2.2-1 of TS 38.214.

Table 6: Non-uniform linear array

|  | All CSI-RS ports are active | Reduced CSI-RS ports and resource for CQI computation | Equivalent CSI-RS port config |
| --- | --- | --- | --- |
| CSI-RS pattern | Row 16 | Row 16 | Row 13 |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | Non-uniform linear array with single panel or two panels with unconfigurable (N1,N2) |  |

Observation 3: CSI-RS resources of different CDM group sizes do not have a nested structure. For resources with the same CDM group size, only a few resources have a nested structure. In particular,

- For CDM8 (rows 15 & 18), 24-port CSI-RS resource in row 15 is nested within 32-port CSI-RS resource in row 18

- For CDM4 (rows 8, 10, 12, 14 and 17),

  - 24-port CSI-RS resource in row 14 is nested within 32-port CSI-RS resource in row 17

  - CSI-RS resources in other rows with CDM4 is not nested within 32-port CSI-RS resource in row 17 (one polarization is not usable).

- For CDM2 (rows 3-7, 9, 11, 13 and 16),

  - 2-port CSI-RS resource in row 3 is nested within 32-port CSI-RS resource in row 16

  - CSI-RS resources in other rows with CDM2 is not nested within 32-port CSI-RS resource in row 16: one polarization is not usable (row 11); invalid antenna array (rows 4-7, 9 and 13)

The above observation implies that we can only perform efficient adaptation of spatial elements between 32-port and 24-port antenna array when using CDM8 or CDM4, and between 32-port and 2-port antenna array when using CDM2. This implementation is very limited to harvest the potential network energy savings gain from the dynamic adaptation of spatial elements. Therefore, from our perspectives, it is necessary to enhance CSI-RS patterns for improving network flexibility in selecting spatial elements for the adaptation. In other words, the enhancements should enable reduced CSI-RS patterns associated with subsets of CSI-RS antenna ports to have the nested structure discussed in Observation 2.

Observation 4: It is necessary to enhance CSI-RS patterns so that additional reduced CSI-RS patterns associated with subsets of CSI-RS antenna ports are nested inside the CSI-RS pattern with all CSI-RS antenna ports being active.

The following options can be further investigated to enhance CSI-RS resource patterns:

Option 1: Introduce additional CSI-RS resource patterns

Some new CSI-RS resource patterns can be introduced to improve adaptation flexibility. For example, to fix the issue observed in Example 2, we can introduce a new row for CSI-RS resource patterns as illustrated in Table 7. Possible specification impact includes adding new rows for CSI-RS locations in Table 7.4.1.5.3-1of TS 38.211.

Table 7: New resource pattern to fix issue in Example 2

|  | All CSI-RS ports are active | Reduced CSI-RS ports and resource for CQI computation | Equivalent CSI-RS port config |
| --- | --- | --- | --- |
| CSI-RS pattern | Row 17 | Row 17 | New Row |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(4, 2) |  |

Option 2: Support CSI report based on a subset of CSI-RS resources configured in an CSI report setting

In legacy CSI feedback framework, the UE is configured with one or multiple CSI report settings. For each CSI report setting, the number of CSI-RS antenna ports in the configured CSI-RS resources is equal to the number of ports in CSI-RS antenna port configuration provided in the codebook configuration. Hence, it is not possible for the UE to provide CSI feedback for a CSI-RS antenna port configuration with a smaller number of ports than the number of ports in the configured CSI-RS resource. In other words, the 3GPP standards do not allow the CSI report configuration in which the number of CSI-RS ports in CSI-RS antenna port configuration in codebook configuration is smaller than the number of ports in the configured CSI-RS resources. Having option to enable CSI report based on a subset of CSI-RS antenna ports in the CSI-RS resource configured in an CSI report setting may improve network flexibility in selecting active spatial elements for the adaptation of spatial elements. For example, to cover first missing case with CDM4 group, the UE can configure a CSI report configuration with resource setting as 32-port CSI-RS resource in row 17 while codebook configuration has (N1,N2)=(8, 1) for Type-I single panel codebook and an indication on the CSI-RS ports (e.g., ports 3001, 3003, 3005, …, 3029, 3031) to be used for CSI computation as illustrated in Table 8. Possible 3GPP spec impact to support this option at least includes indication on the CSI-RS ports to be used for CSI computation.

Table 8: Separate CSI report configs with an indication on the CSI-RS ports to be used for CSI computation

|  | CSI report config 1 | CSI report config 2 |
| --- | --- | --- |
| CSI-RS pattern (Row 17) |  |  |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(8, 1) |

Observation 5: The following options can improve efficiency in selecting spatial elements for the adaptation of spatial elements:

- Option 1: Introduce additional CSI-RS time and frequency domain patterns.

- Option 2: Support CSI report based on a subset of CSI-RS antenna ports in the CSI-RS resource configured in an CSI report setting.

Based on above discussion, from our perspectives, a spatial adaptation pattern includes a CSI-RS antenna port configuration (e.g., (N1, N2) for single panel or (Ng, N1, N2) for multi-panel) in codebook configuration and NZP CSI-RS resource(s) for channel measurement that has the same number of CSI-RS antenna ports as the number of CSI-RS antenna ports in the configured CSI-RS antenna port configuration. Furthermore, the codebook design in 3GPP standards assumes uniform linear array. Hence, an antenna array corresponding to the spatial adaptation pattern needs to be a uniform linear array with a supported configuration provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively. In addition, RI restriction and/or codebook subset restriction may be different for different spatial adaptation patterns.

Proposal 1: A spatial adaptation pattern includes a codebook configuration and reduced NZP CSI-RS resource(s) for channel measurement with the same number of antenna ports as that in the configured codebook.

- An antenna array corresponding to the spatial adaptation pattern is a uniform linear array with a supported configuration provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively.

- FFS: how to determine the reduced NZP CSI-RS resource(s).

Given definition of spatial adaptation pattern, we will discuss our view on CSI report configuration, CSI-RS resource configuration, and CSI report. In particular, RAN1 #112 made the following agreements on CSI-RS resource configuration and CSI report configuration:

Agreement For spatial element adaptation, further study the following - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern - FFS: Details on how the association is done - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns - FFS: Details on how the association is done - FFS: Details on the definition of “spatial adaptation patterns” Agreement For spatial element adaptation, further study the following - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern - FFS: Details of sub-configuration

Table 9 provides examples and analysis for 4 combinations of CSI report config and NZP CSI-RS resource set config options.

Table 9: Analysis of different combinations for CSI report config and NZP CSI-RS resource set config options

| Scheme | CSI report config. | NZP CSI-RS resource and resource set config. for channel measurement | Analysis |
| --- | --- | --- | --- |
| 1 | A2-1 | A1-1 | - This is legacy configuration and can support spatial domain adaptation. - Some limitation in CSI-RS resource configuration which may limit efficiency of spatial domain adaptation. - Such limitation may be solved by introducing new CSI-RS resource pattern(s) in the spec. as discussed in Option 1 of Observation 5. No further change is necessary. |
|  | Example CSI report config 1: - 32-port CSI-RS resource set 1 - Codebook config 1 with (N1, N2) = (8, 2) CSI report config 2: - 16-port CSI-RS resource set 2 - Codebook config 2 with (N1, N2) = (8, 1) CSI report config 3: - 8-port CSI-RS resource set 3 - Codebook config 3 with (N1, N2) = (4, 1) |  |  |
| 2 | A2-1 | A1-2 | - Improve efficiency of spatial domain adaptation by properly configuring spatial adaptation pattern to indicate subset(s) of CSI-RS antenna ports in configured CSI-RS resource set as discussed in Option 2 of Observation 5. - The CSI report configs have same CSI-RS resource set configuration. In other words, the CSI-RS resource set is shared across CSI report configs. - Hence, only spec change is to define spatial adaptation pattern and its configuration. |
|  | Example CSI report config 1: - 32-port CSI-RS resource set 1 - Spatial adaptation pattern - Codebook config 1 with (N1, N2) = (8, 2) CSI report config 2: - 32-port CSI-RS resource set 1 - Spatial adaptation pattern - Codebook config 2 with (N1, N2) = (8, 1) - 16-port NZP CSI-RS resource(s) for channel measurement, where each resource is a subset of 32-port CSI-RS resource in CSI-RS resource set 1 and corresponds to ULA with (N1, N2) in codebook 2. CSI report config 3: - 32-port CSI-RS resource set 1 - Spatial adaptation pattern - Codebook config 3 with (N1, N2) = (4, 1) - 8-port CSI-RS resource(s), where each resource is a subset of 32-port CSI-RS resource in CSI-RS resource set 1 and corresponds to ULA with (N1, N2) in codebook 3. |  |  |
| 3 | A2-2 | A1-1 | - No benefit compared to Scheme 1. - In addition, it requires spec change in defining CSI processing related requirements e.g., CPU counting, counting simultaneous active CSI-RS resources etc. - This approach should not be pursued. |
|  | Example One CSI report config with three sub-configurations: - Sub-configuration 1 is CSI report config 1 in scheme 1, - Sub-configuration 2 is CSI report config 2 in scheme 1, - Sub-configuration 3 is CSI report config 3 in scheme 1. |  |  |
| 4 | A2-2 | A1-2 | - Improve efficiency of spatial domain adaptation by properly configuring spatial adaptation pattern to indicate subset(s) of CSI-RS antenna ports in configured CSI-RS resource set as discussed in Option 2 of Observation 5. - However, no clear benefit in terms of CSI report config overhead compared to Scheme 2. It should be noted that overhead of Scheme 2 can be reduced as well e.g., e.g., one CSI report config can carry parameters common across the CSI report configs while other CSI report configs do not carry these common parameters. - Compared to Scheme 2, it requires larger spec change in CSI processing related requirements e.g., CPU counting, counting simultaneous active CSI-RS resource etc. - It should be noted that from CSI processing perspectives, although it is one CSI report config in this scheme, it is treated similarly to the case where the UE is configured with K CSI report configs (K is the number of sub-configurations that gNB requests CSI report) e.g., K = 3 in the example. |
|  | Example One CSI report config with three sub-configurations: - 32-port CSI-RS resource set - (Sub-configuration 1) Spatial adaptation pattern 1 - Codebook config 1 with (N1, N2) = (8, 2) - (Sub-configuration 2) Spatial adaptation pattern 2 - Codebook config 2 with (N1, N2) = (8, 1) - 16-port CSI-RS resource(s), where each resource is a subset of 32-port CSI-RS resource in CSI-RS resource set and corresponds to ULA with (N1, N2) in codebook 2. - (Sub-configuration 3) Spatial adaptation pattern 3 - Codebook config 3 with (N1, N2) = (4, 1) - 8-port CSI-RS resource(s), where each resource is a subset of 32-port CSI-RS resource in CSI-RS resource set and corresponds to ULA with (N1, N2) in codebook 3. |  |  |

From perspectives of 3GPP specification change and UE complexity in CSI measurement/reporting, our preference is scheme 2 that uses A2-1 for CSI report configuration and A1-2 for NZP CSI-RS resource set configuration for channel measurement.

Proposal 2: RAN1 adopts A2-1 for CSI report configuration and A1-2 for NZP CSI-RS resource set configuration for channel measurement. Furthermore, for an CSI report configuration,

- NZP CSI-RS resource set configuration for channel measurement includes CSI-RS resources with the same number of antenna ports.

- Reduced NZP CSI-RS resource(s) for channel measurement corresponding to a spatial adaptation pattern are determined from the resource(s) in the configured CSI-RS resource set.

It should be noted that, with legacy CSI framework, the UE is configured with one or more CSI report configurations subject to UE capability. Each CSI report configuration indeed includes a single spatial adaptation pattern. The UE performs CSI measurement and provides a single CSI report for one configured CSI report settings. The CSI processing requirements (e.g., counting the number of simultaneous CSI processing units and counting the number of simultaneously active CSI-RS resources) follow such CSI report configuration and report framework. From UE implementation perspectives, grouping multiple CSI report configurations into a single CSI report configuration is only the change on configuration signaling. It does not change CSI processing requirements when comparing to the separate CSI report configurations. For instance, for scheme 4, when counting the number of CSI report(s) processed simultaneously in a CC, it still counts three instead of one. Such CSI-RS report configuration grouping as being done in scheme 4 does not solve the limitation of CSI-related UE capabilities if any. In other words, with this grouping, components in CSI-related UE capability feature groups (e.g., FG 2-33, FG 2-35, etc.) still needs to be scaled up linearly with the number of configured codebooks and the number of resources per a configured codebook. From RRC signaling design perspective, the CSI report configuration grouping (e.g., scheme 4) significantly change the legacy signaling design e.g., for CSI-ReportConfig with no reduction gain on the overall RRC signaling overhead compared to the separate CSI report configs (e.g., scheme 2). It should be noted that the overhead of scheme 2 can be reduced if needed e.g., one CSI report config can carry parameters common across the CSI report configs while other CSI report configs do not carry these common parameters.

Observation 6: Having single CSI report configuration associated with one or more spatial adaptation patterns (e.g., scheme 4) does not have lower RRC overhead than having separate CSI report configs where each CSI report config is associated with a spatial adaptation pattern.

- Note that overhead of separate CSI report configs can be reduced e.g., one CSI report config can carry parameters common across the CSI report configs while other CSI report configs do not carry these common parameters.

As we discussed above, motivation and benefit of going with scheme 4 is unclear when comparing with scheme 2. However, if RAN1 pursues scheme 4 for CSI report config and NZP CSI-RS resource configuration, the CSI processing requirements (e.g., CPU counting, counting of simultaneous active CSI-RS resources, etc.) should be scaled linearly by the number of codebook configurations and the number of CSI-RS resources for each configured codebook in the CSI report configuration that gNB requests CSI report. This is to ensure legacy CSI/CSI-RS capabilities are properly applied as noted in the WID that “Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements”.

Proposal 3: If RAN1 adopts A2-2 for CSI report configuration and A1-2 for NZP CSI-RS resource set configuration, the following aspects are included for an CSI report configuration.

- NZP CSI-RS resource set configuration for channel measurement includes CSI-RS resources with the same number of antenna ports.

- Reduced NZP CSI-RS resource(s) for channel measurement corresponding to a spatial adaptation pattern are determined from the resource(s) in the configured CSI-RS resource set.

- The CSI processing requirements (e.g., CPU counting, counting of simultaneous active CSI-RS resources, etc.) are scaled linearly by the number of codebook configurations and the number of CSI-RS resources for each configured codebook in the CSI report configuration.

For either scheme 2 or scheme 4, RAN1 should discuss how to determine reduced NZP CSI-RS resources for the spatial adaptation pattern for the configured NZP CSI-RS resource set.

Proposal 4: To determine a reduced NZP CSI-RS resource for a spatial adaptation pattern, one subset of CSI-RS antenna ports associated with a CSI-RS resource in the configured CSI-RS resource set is indicated.

- FFS: details on CSI-RS antenna port subset indication.

RAN1 #112 made the following agreements on CSI reports:

Agreement For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g. - FFS: gNB indicates to UE which CSI(s) the UE shall report - FFS: the UE selects which CSI(s) are reported - FFS: multiple CSI(s) are reported in a joint CSI report - FFS: Overhead reduction for multiple CSI(s) Note: UE complexity needs to be taken into account.

First of all, it should be noted that the current NR standards allow multiple CSI reports to be multiplexed and transmitted in the same PUCCH or the same PUSCH. Furthermore, CSI report overhead reduction can be achieved with separate CSI reports. For example, with scheme 2 on CSI report config and CSI-RS resource config, some CSI parameters in CSI reports can be relatively reported with respect to a CSI report.

Observation 7: The current specs allow multiple CSI reports to be multiplexed and transmitted in the same PUCCH or the same PUSCH.

Observation 8: CSI report overhead reduction can be achieved with separate CSI reports (e.g., some CSI parameters in CSI reports can be relatively reported with respect to a CSI report).

For multiple CSIs where each CSI corresponds to a spatial adaptation pattern, having multiple CSIs in a single CSI report will generate a new type of CSI report. Hence, it only makes sense when high overhead reduction at the minimal UE complexity can be achieved with such joint CSI approach. As a baseline, there should be a separate CSI report for each CSI corresponding to a spatial adaptation pattern that gNB requests to leverage UE implementation of legacy standards.

Observation 9: Having joint CSI report for multiple CSIs is only beneficial if CSI report overhead reduction is high compared to separate CSI reports.

Proposal 5: RAN1 only specifies joint CSI report for multiple CSIs if its CSI report overhead reduction is high compared to separate CSI reports.

- A separate CSI report for each CSI corresponding to a spatial adaptation pattern as baseline.

## Enhancements on Beam Management Procedure

Changing the number of active spatial elements (e.g., antenna ports, active transceiver chains) at gNB impacts shapes of Tx and/or Rx beams at gNB, especially at higher frequency bands where communication relies on analog beamforming. For example, when the number of active transceiver chains at gNB increases, more physical antennas will be active for beamforming, making the Tx and/or Rx beams at gNB narrower. On the other hand, when the number of active transceiver chains at gNB decreases, less physical antennas will be active for beamforming, making the Tx and/or Rx beams at gNB wider.

The beam shape impacts how many beams (i.e., the number of CSI-RS resources per set) gNB should configure for CSI-RS transmission to support Tx beam switching procedure and/or Rx beam switching procedure at UE. In particular, for a given SSB beam, gNB would need a smaller number of CSI-RS resources per set when the number of active spatial elements is small than the case when the number of active spatial elements is large. In the 3GPP standard, CSI-RS resources per set for beam management is RRC configured to the UE as part of NZP-CSI-RS-ResourceSet. Furthermore, the UE is RRC configured with one or more CSI report configs, each includes one or multiple CSI-RS resources per set and the associated CSI-RS antenna port configuration. When multiple CSI report configs are configured, 3GPP standard allows gNB to dynamically select one of the configured CSI report configs for beam management.

For Rx beam switching procedure, the parameter repetition in CSI-RS resource set is set to "ON". Furthermore, the UE may use FG 2-26 to indicate a recommended CSI-RS resource repetition number per resource set. In theory, the gNB may configure different CSI-RS resource repetition numbers for different CSI report configs. However, RAN4 specification (e.g., TS 38.101-2) defines CSI-RS based beam management based on the recommended repetition number. Therefore, it is expected that the configured repetition number is aligned with the recommended repetition number. For example, if the UE recommends 8 as CSI-RS resource repetition number per resource set, the gNB should configure CSI-RS resource sets having 8 CSI-RS resources with the same transmission beams.

| 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, | Mandatory with UE capability at least for FR2 Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |
| --- | --- | --- | --- |

In practice, the CSI-RS resource repetition number may depend on UE implementation on how it uses antenna elements for Rx beam switching procedure. For example, the total number of candidate Rx beams is 8 when UE uses 8 antenna elements, 5 when UE uses 4 antenna elements, and 3 when UE uses 2 antenna elements. Therefore, if UE can provide can report a list of candidate values for the CSI-RS resource repetition number per resource set, the gNB might be able to configure CSI report configs with the number of CSI-RS resources per set based on the candidate values reported by the UE. With this reporting, efficiency of CSI-RS resource usage and BS power consumption can be improved. In other words, the gNB does not need to transmit unnecessary CSI-RS (e.g., overlapping beams due to dynamic adaptation of spatial elements) in Rx beam switching procedure.

Observation 10: If UE can report a list of candidate values for the CSI-RS resource repetition number per resource set, the gNB might be able to configure CSI report configs based on the candidate values reported by the UE for improving efficiency of CSI-RS resource usage and power consumption during Rx beam switching procedure.

Proposal 6: UE reports a set of supported candidate values for the total number of CSI-RS resources per set for an CSI-RS resource set configured with repetition set to ‘on’.

# Adaptation of Power Offsets between PDSCH and CSI-RS

It has been shown during the SI that dynamic adaptation of the power offset values between PDSCH and CSI-RS is beneficial for network energy savings. 3GPP Rel-18 has the following objective for necessary enhancements on CSI to enable efficient adaptation of power offset values between PDSCH and CSI-RS.

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

The main motivation of this adaptation is that gNB may be able to compensate for some measurements e.g., L1-RSRP and CQI based on a transmission power difference between actual power offset and the configured power offset that the UE uses for the CSI report. This should work well with legacy 3GPP standards when the transmission power difference is not large. However, when the transmission power difference is large, the compensation at gNB may not be accurate for parameters such as RI and/or PMI. From our perspectives, the dynamic adaptation of the power offset values between PDSCH and CSI-RS can be realized by the following two main steps as illustrated in Figure 2:

Figure 2: Adaptation of power offset values between PDSCH and CSI-RS

- Step 1: CSF for adaptation of power offset values between PDSCH and CSI-RS

  - gNB configures multiple CSI report settings to the UE: CSI report settings have an identical CSI-RS resource configuration in time and frequency domain. However, the CSI-RS resource in each CSI report setting corresponds to a power offset value between PDSCH and CSI-RS. Table 9 provides an example of multiple CSI report configs for the adaptation.

  - UE measures and reports CSIs associated with CSI report settings.

- Step 2: gNB transmits PDSCH using a suitable transmit power

  - gNB decides a suitable transmit power for PDSCH transmission based on the reported CSIs

Table 10: Example of multiple CSI report configs for adaptation of power offset values

|  | CSI report config 1 | CSI report config 2 |
| --- | --- | --- |
| CSI-RS pattern (Row 17) | powerControlOffset = -4dB | powerControlOffset = 4dB |
| CSI-RS antenna port configuration | (N1,N2)=(8, 2) | (N1,N2)=(8, 2) |

In Step 1, the CSI reports are only useful for the adaptation when the difference between the corresponding power settings for the CSI-RS resource in CSI report settings are large. Furthermore, the goal is to reduce PDSCH transmission power to achieve network energy saving. Thus, two CSI report settings should be sufficient (e.g., one for normal operation and another one for PDSCH transmission power reduction).

Observation 11: The legacy CSI feedback framework is already very flexible to serve diverse use cases including adaptation of power offset values between PDSCH and CSI-RS. In particular, the gNB can obtains CSI associated with multiple power offset values by configuring multiple CSI report settings with the same CSI-RS resource setting but with different power offset values.

RAN1#112 made the following agreements for power domain adaptation:

Agreement For adaptation of power offset values between PDSCH and CSI-RS, further study the following - Where/how to configure multiple power offset values - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS - Whether other UE report content can be included Agreement Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that - Whether there is a need for transition time per adaptation (for UE) - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

Proposal 7: If RAN1 adopts a framework where UE performs CSI for more than one power offset value for a CSI report config, the following aspects are included.

- The CSI processing requirements (e.g., CPU counting, counting of simultaneous active CSI-RS resources, etc.) are scaled linearly by the number of power offset values and the number of CSI-RS resources in the CSI report configuration that gNB requests CSI report.

For multiple CSIs where each CSI corresponds to a spatial adaptation pattern, similar to discussion under adaptation of spatial elements, having multiple CSIs in a single CSI report will generate a new type of CSI report. Hence, it only makes sense when high overhead reduction at the minimal UE complexity can be achieved with such joint CSI approach.

Observation 12: Having joint CSI report for multiple CSIs in power adaptation is only beneficial if CSI report overhead reduction is high compared to separate CSI reports.

Proposal 8: RAN1 only specifies joint CSI report for multiple CSIs in power adaptation if its CSI report overhead reduction is high compared to separate CSI reports.

- A separate CSI report for each CSI corresponding to a power offset value that gNB requests as baseline.

For Step 2, legacy 3GPP standards allow gNB to dynamically update PDSCH transmission power which can be transparent to the UE. However, in practice, if gNB dynamically adjusts PDSCH transmission power, it will negatively impact the UE processing e.g., AGC and PDSCH performance accordingly. Therefore, from UE perspectives, the UE should know the PDSCH transmission power update to adjust its processing accordingly.

Proposal 9: The UE is provided with an indication on the PDSCH transmission power change.

FFS: Details on PDSCH transmission change indication

# Conclusion

We discuss our views on enhancements to enable efficient adaptation of spatial elements and efficient adaptation of power offset values between PDSCH and CSI-RS. In particular, the following observations and proposals are made:

On CSI enhancement for efficient adaptation of spatial elements

Observation 1: it is important to discuss whether the legacy CSI feedback framework can already enable efficient adaptation of spatial elements. If it cannot, which part of CSI feedback framework is limiting factor in enabling such adaptation.

Observation 2: CSI reports associated with different CSI-RS antenna port configurations should provide useful information for the gNB to dynamically adapt spatial elements. In particular, the reduced CSI-RS resource associated with a CSI report setting for CSI computation has a nested structure as follows:

- It is subset of the CSI-RS resource with all antenna ports being active,

- its resource pattern is one of the legacy patterns, and

- the antenna array corresponding to the reduced CSI-RS resource pattern is a uniform linear array with supported configuration in provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively.

Observation 3: CSI-RS resources of different CDM group sizes do not have a nested structure. For resources with the same CDM group size, only a few resources have a nested structure. In particular,

- For CDM8 (rows 15 & 18), 24-port CSI-RS resource in row 15 is nested within 32-port CSI-RS resource in row 18

- For CDM4 (rows 8, 10, 12, 14 and 17),

  - 24-port CSI-RS resource in row 14 is nested within 32-port CSI-RS resource in row 17

  - CSI-RS resources in other rows with CDM4 is not nested within 32-port CSI-RS resource in row 17 (one polarization is not usable).

- For CDM2 (rows 3-7, 9, 11, 13 and 16),

  - 2-port CSI-RS resource in row 3 is nested within 32-port CSI-RS resource in row 16

  - CSI-RS resources in other rows with CDM2 is not nested within 32-port CSI-RS resource in row 16: one polarization is not usable (row 11); invalid antenna array (rows 4-7, 9 and 13)

Observation 4: It is necessary to enhance CSI-RS patterns so that additional reduced CSI-RS patterns associated with subsets of CSI-RS antenna ports are nested inside the CSI-RS pattern with all CSI-RS antenna ports being active.

Observation 5: The following options can improve efficiency in selecting spatial elements for the adaptation of spatial elements:

- Option 1: Introduce additional CSI-RS time and frequency domain patterns.

- Option 2: Support CSI report based on a subset of CSI-RS antenna ports in the CSI-RS resource configured in an CSI report setting.

Observation 6: Having single CSI report configuration associated with one or more spatial adaptation patterns (e.g., scheme 4) does not have lower RRC overhead than having separate CSI report configs where each CSI report config is associated with a spatial adaptation pattern.

- Note that overhead of separate CSI report configs can be reduced e.g., one CSI report config can carry parameters common across the CSI report configs while other CSI report configs do not carry these common parameters.

Observation 7: The current specs allow multiple CSI reports to be multiplexed and transmitted in the same PUCCH or the same PUSCH.

Observation 8: CSI report overhead reduction can be achieved with separate CSI reports (e.g., some CSI parameters in CSI reports can be relatively reported with respect to a CSI report).

Observation 9: Having joint CSI report for multiple CSIs is only beneficial if CSI report overhead reduction is high compared to separate CSI reports.

Proposal 1: A spatial adaptation pattern includes a codebook configuration and reduced NZP CSI-RS resource(s) for channel measurement with the same number of antenna ports as that in the configured codebook.

- An antenna array corresponding to the spatial adaptation pattern is a uniform linear array with a supported configuration provided in Table 5.2.2.2.1-2 and Table 5.2.2.2.2-1 of TS 38.214 for Type-I single panel and Type-I multi-panel, respectively.

- FFS: how to determine the reduced NZP CSI-RS resource(s).

Proposal 2: RAN1 adopts A2-1 for CSI report configuration and A1-2 for NZP CSI-RS resource set configuration for channel measurement. Furthermore, for an CSI report configuration,

- NZP CSI-RS resource set configuration for channel measurement includes CSI-RS resources with the same number of antenna ports.

- Reduced NZP CSI-RS resource(s) for channel measurement corresponding to a spatial adaptation pattern are determined from the resource(s) in the configured CSI-RS resource set.

Proposal 3: If RAN1 adopts A2-2 for CSI report configuration and A1-2 for NZP CSI-RS resource set configuration, the following aspects are included for an CSI report configuration.

- NZP CSI-RS resource set configuration for channel measurement includes CSI-RS resources with the same number of antenna ports.

- Reduced NZP CSI-RS resource(s) for channel measurement corresponding to a spatial adaptation pattern are determined from the resource(s) in the configured CSI-RS resource set.

- The CSI processing requirements (e.g., CPU counting, counting of simultaneous active CSI-RS resources, etc.) are scaled linearly by the number of codebook configurations and the number of CSI-RS resources for each configured codebook in the CSI report configuration.

Proposal 4: To determine a reduced NZP CSI-RS resource for a spatial adaptation pattern, one subset of CSI-RS antenna ports associated with a CSI-RS resource in the configured CSI-RS resource set is indicated.

- FFS: details on CSI-RS antenna port subset indication.

Proposal 5: RAN1 only specifies joint CSI report for multiple CSIs if its CSI report overhead reduction is high compared to separate CSI reports.

- A separate CSI report for each CSI corresponding to a spatial adaptation pattern as baseline.

On beam management enhancements for adaptation of spatial elements

Observation 10: If UE can report a list of candidate values for the CSI-RS resource repetition number per resource set, the gNB might be able to configure CSI report configs based on the candidate values reported by the UE for improving efficiency of CSI-RS resource usage and power consumption during Rx beam switching procedure.

Proposal 6: UE reports a set of supported candidate values for the total number of CSI-RS resources per set for an CSI-RS resource set configured with repetition set to ‘on’.

On efficient adaptation of power offset values between PDSCH and CSI-RS

Observation 11: The legacy CSI feedback framework is already very flexible to serve diverse use cases including adaptation of power offset values between PDSCH and CSI-RS. In particular, the gNB can obtains CSI associated with multiple power offset values by configuring multiple CSI report settings with the same CSI-RS resource setting but with different power offset values.

Observation 12: Having joint CSI report for multiple CSIs in power adaptation is only beneficial if CSI report overhead reduction is high compared to separate CSI reports.

Proposal 7: If RAN1 adopts a framework where UE performs CSI for more than one power offset value for a CSI report config, the following aspects are included.

- The CSI processing requirements (e.g., CPU counting, counting of simultaneous active CSI-RS resources, etc.) are scaled linearly by the number of power offset values and the number of CSI-RS resources in the CSI report configuration that gNB requests CSI report.

Proposal 8: RAN1 only specifies joint CSI report for multiple CSIs in power adaptation if its CSI report overhead reduction is high compared to separate CSI reports.

- A separate CSI report for each CSI corresponding to a power offset value that gNB requests as baseline.

Proposal 9: The UE is provided with an indication on the PDSCH transmission power change.

- FFS: Details on PDSCH transmission change indication

# Reference

- RP-223540, New WID: Network energy savings for NR, December 2022.