3GPP TSG RAN WG1 Meeting #112-bis-e	R1-2303610

April 17th – 26th, 2023

Agenda item:	9.12.1

Source: 	Qualcomm Incorporated

Title: 	L1 Enhancements for Inter-Cell Beam Management

Document for:	Discussion/Decision

- Introduction

The agreements made in RAN1 #112 is summarized below.

Agreement - RAN1 shares the same understanding as RAN2 on agreement: - The LTM mobility trigger information is conveyed in a MAC CE - The same MAC CE is used for the LTM triggering. Agreement - The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following: - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2 Agreement - For L1-RSRP measurement RS configuration - For SSB based L1-RSRP measurement: - As a starting point, at least the following information needs to be provided to a UE, e.g. - For intra- and inter- frequency: PCI or logical ID (e.g., as being defined in R17 ICBM), time domain (e.g. SMTC or periodicity and SSB position in burst) - For inter-frequency: frequency domain location (e.g. center frequency), SCS - FFS: transmission power (for pathloss calculation) - Note: other parameters included in the configuration can be further discussed - Including above agreement into the LS - The detailed design of RRC structure is up to RAN2, and send an LS to RAN2 to request to work on the RRC structure design on the measurement configuration. - Following RAN1 understanding will be provided in the LS - RAN1 has discussed the following configuration options for L1 measurement configurations for SSB till RAN1#112: - Option 1) Configurations for L1 measurement RS is provided under ServingCellConfig for the serving cells - is useful to reuses the mechanism for Rel-17 ICBM and necessary information to support inter-frequency measurement will be added there. - Option 2) Configurations for L1 measurement RS is provided separately from ServingCellConfig for the serving cells and CellGroupConfig for the candidate cells - is useful to avoid the duplicated configurations for L1 measurement RSs, [and avoid UE to process configurations for L1 measurement RS provided under CellGroupConfig for the candidate cells] - Option 3) Configurations for L1 measurement RS is provided under CellGroupConfig for the candidate cells - can achieve the similar benefit as Option 2) by directly referring to the candidate cell configurations. - Note RAN2 has a full flexibility to design the whole RRC structure design. - RAN1 believes this is RAN2 expert region, and respectfully asks RAN2 to finalize the RRC structure design after RAN1 finalizes the discussion on RRC parameters. - It is noted that RAN1 foresees the necessity of similar discussions on TCI state pool for candidate cells and L1 measurement report configurations. Agreement - At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s) - FFS: beam indication for mTRP case

- Dynamic Switch Among Candidate Cells

To align the SpCell/CG update time at both sides, the application time for the update command should be specified as well. In addition, the application time can be differentiated for previously activated and deactivated new SpCell/CG.

Proposal 1: Application time should be specified for the SpCell/CG update command

- Application time can be different for previously activated and deactivated new SpCell/CG

Figure 1: Application time of SpCell/CG update command

Another issue is the DL/UL BWPs to be used for the new cell. This may depend on the UE traffic demand upon cell switch. For example, BWPs with larger BW can be used if the UE has large amount of buffered data. Otherwise, BWPs with smaller BW can be used to save the UE power consumption. Therefore, the DL/UL BWPs can be dynamically indicated in the cell update command based on the UE traffic demand at that moment. In addition, the used DL BWP should ideally include the DL RS for L1 measurement, since otherwise measurement gap may be needed depending on UE capability.

Proposal 2: DL/UL BWPs to be used for the new cell can be indicated in cell update command

Figure 2: SpCell update command triggered beam management

In R17 efficient SCell activation, SCell activation MAC-CE can also trigger A-TRS to reduce SCell activation latency. Similar concept can be extended to L1/L2 mobility. Specifically, the SpCell update command can also trigger TRS and beam management to reduce TCI state activation latency and beam refinement latency, respectively, especially when the new SpCell is previously deactivated. As for the triggered beam management, it can be for gNB and/or UE side beam refinement.

Proposal 3: To reduce TRS tracking and beam refinement latency after SpCell update, SpCell update command can also trigger TRS or beam refinement

Another issue is whether to reset HARQ processes after cell switch. In our view, maintaining HARQ continuity would be more efficient at least in certain scenarios, e.g. intra-DU cell switch. The detailed reasons are listed below.

- Maintaining HARQ continuity can reduce latency for URLLC traffic, since the reception combining can continue on the new cell for the same HARQ process

- Maintaining HARQ continuity can improve throughput for eMBB traffic, since the discarded packets in case of HARQ reset can be saved

- The above latency/throughput loss due to HARQ reset should be more pronounced in case of higher frequency band with small cell size due to the more frequent cell switch

- HARQ continuity across cells already exists at least in case of intra-frequency intra-DU cell switch, e.g. R17 ICBM

Proposal 4: To reduce latency/throughput impact due to HARQ process reset, support NW can maintain HARQ continuity after cell switch at least in certain scenarios, e.g. intra-frequency intra-DU cell switch

- Detailed procedure/signaling can be determined by RAN2

- L1 Measurement & Reporting for Cell Update

In RAN1 #110-bis-e, it is decided to further study whether CSI-RS based L1 measurement is supported and, if so, which type(s) should be supported. In our view, CSI-RS for tracking and beam management are critical to acquire time/frequency offset and beam refinement, which involve continuous tracking/maintenance. Therefore, they should be supported for both L1 intra/inter-frequency measurements.

Proposal 5: For CSI-RS based L1 measurement for candidate cell, support CSI-RS for tracking and beam management for both L1 intra/inter-frequency measurements

For the format of L1 intra-frequency measurement report, support UE to report X best candidate cells with Y best beams per reported candidate cell, where the X and Y can be configured subject to corresponding UE capabilities.

Proposal 6: For L1 intra-frequency measurement, support UE to report X best candidate cells with Y best beams per reported candidate cell in the same report

- X and Y can be configured subject to corresponding UE capabilities

For the format of L1 inter-frequency measurement report, support each report can carry measurement results for candidate cells on multiple frequencies. Specifically, support UE to report X best candidate cells across all measured frequencies with Y best beams per reported candidate cell in the same report, where the X and Y can be configured subject to corresponding UE capabilities.

Proposal 7: For L1 inter-frequency measurement, support UE to report X best candidate cells across all measured frequencies with Y best beams per reported candidate cell in the same report

- X and Y can be configured subject to corresponding UE capabilities

Instead of reporting every measured candidate cell, L1 cell-level metric can be introduced to rank the candidate cells and only report X best candidate cells to reduce the reporting overhead. The L1 cell-level metric can be defined as the linear average of Y best beams per cell with Y can be configured, e.g. Y=2 or 3.

Proposal 8: To reduce L1 report overhead, L1 cell-level metric can be introduced to rank and select X best measured candidate cells for reporting

- L1 cell-level metric can be linear average of Y>1 best beams per cell, e.g. Y=2 or 3

Figure 3: L1 cell-level metric saves report by only reporting cells with good cell metric

On the measurement configuration, in case that CSI-RS is used at least for L1 inter-frequency measurement, the corresponding TCI state for the CSI-RS should be provided by NW. The detailed TCI state configuration can be left to RAN2, e.g. whether the TCI state is configured within the same TCI state pool for the current serving cell or in a separate TCI state pool per candidate cell.

Proposal 9: Support TCI state configuration per CSI-RS resource at least for L1 inter-frequency measurement

- Detailed configuration can be up to RAN2

In RAN2 #109e, it has been agreed to use L1 measurement to trigger L1/L2 mobility. In case of large number of candidate cells for measurement, the corresponding reporting overhead and UL interference may not be negligible. To reduce the overhead, interference, and UE power consumption due to the L1 measurement reporting, it would be beneficial to consider event-triggered L1 report, which can be carried in MAC-CE to reduce reserved resource.

Proposal 10: Support event triggered report for candidate cells for LTM

- The report can be carried in MAC-CE, similar to existing event triggered L2 report

For the triggering condition, at least Event A3 for L3 report can be considered as starting point, e.g. candidate cell becomes amount of offset better than SpCell. Therefore, the report is sent only when the candidate cell has sufficiently good quality compared with the current SpCell.

Proposal 11: For event triggered report, at least a triggering condition similar to Event A3 for L3 report can be considered as starting point, e.g. candidate cell becomes amount of offset better than SpCell

For the metric computing the triggering condition, L1 cell-level metric can be introduced per candidate cell, e.g. as linear average of X best beams of the candidate cell. Compared with directly using best beam quality per candidate cell, this may save the overhead by reporting only when the candidate cell has sufficiently good cell-level metric to reduce the potential ping-pong handover effect, which should be minimized especially if the cell switch latency in L1/L2 mobility is much longer than the beam switch latency in R17 ICBM.

Proposal 12: For event triggered report, L1 cell-level metric can be used to quantify the quality per cell considered in the triggering condition

- L1 cell-level metric can be linear average of Y>1 best beams per cell, e.g. Y=2 or 3

In case of large number of candidate cells to be measured, UE may only be capable of simultaneously measuring a subset of them. To save reconfiguration latency, different subsets of L1 measurement report can be activated at different times, instead of using RRC reconfiguration each time. To further increase measurement/reporting flexibility, the activation MAC-CE can also update the config per L1 report, e.g. reporting periodicity, SSBs to be measured, etc.

Proposal 13: Support MAC-CE activation of L1 report for a set of PCIs

- Related measurement/reporting config may also be updated by the MAC-CE

In R17, measurement relaxation has been introduced for BFD/RLM such that UE does not need to measure the corresponding RSs if it is near the cell center or has low mobility. Similar concept can be extended to the event triggered report for LTM, where UE does not need to track the L1 measurement RS for candidate cells to save power at least when the serving cell quality is above a threshold.

Proposal 14: For event tiggered report, UE is not required to perform L1 measurements on candidate cell(s) if the serving cell quality is above a threshold value

In RAN1 #111, it has been agreed to support DL synchronization for candidate cell(s) based on at least SSB before cell switch command. To align the definition, the DL timing maintained by UE can be defined as that for receiving the 1st detected path of the corresponding DL frame from the candidate cell, as specified in RAN4 spec for TA adjustment.

Proposal 15: The maintained DL timing per candidate cell can be defined as that for the reception of the first detected path (in time) of the corresponding downlink frame from the candidate cell

To minimize cell switch latency, time-consuming L1 procedures can be performed before cell switch command. One of such L1 procedures can be the DL Rx timing acquisition, which can be achieved by tracking the earliest arrival time of SSBs from a candidate cell. The measurement can be performed by the UE. However, UE may have limited capability on maximum number of candidate cells to maintain the corresponding DL Rx timings. Therefore, NW should indicate a subset of promising candidate cell(s) based on their L1 report for UE to maintain corresponding DL Rx timing(s), subject to the UE capability.

Proposal 16: Support NW to indicate candidate cell(s) for UE to maintain corresponding DL Rx timing(s)

- Maximum number of candidate cell(s) to maintain the DL Rx timing can be up to UE capability

In case that the new cell supports mTRP operation, the mTRP can be enabled right after the cell switch to maintain good throughput and reliability. One major latency reduction factor is the group-based beam report to identify a best beam pair for simultaneous Rx/Tx. To save the corresponding latency, the group-based beam report can be enabled for promising candidate cells based on their L1 measurement results. The corresponding identified TCI states can be indicated in the cell switch command to facilitate the mTRP operation on the new cell.

Proposal 17: To facilitate mTRP operation on the new cell, R17 group-based beam report can be extended to LTM to identify a group of simultaneously receivable DL beams for a candidate cell

- Two CMR resource sets can be configured with each set including RS configured for LTM L1 measurement

- Beam Indication for Updated Cell

One remaining issue in RAN1 #111 is whether to support the scenario that at least one of the source and target cells can only support R15 TCI framework. In our view, it would be beneficial to only consider R17 unified TCI framework, since R15 TCI framework is inefficient in terms of beam indication due to separate beam indication mechanism per individual channel/RS. In addition, only R17 unified TCI framework is considered as beam indication scheme in R17 ICBM. Similar principle can be extended to LTM to simplify the discussion/design while maintaining all essential beam indication functionalities.

Proposal 18: On the TCI framework for LTM, R15 TCI framework is not supported

For the indicated DL/UL beams on the new cell to work well, UE needs to track the time/frequency offset for the DL beam and the path loss for the UL beam. According to RAN4 requirement [2], the activation latency for DL and UL applicable unified TCI states are plotted in Figure 4 and 5, respectively. For simplicity, it assumes that all TCI states to be activated are known and configured for a single cell. As shown in Figure 4 for DL applicable TCI activation latency, after the MAC-CE command application time, the TCI activation latency also includes the time to measure the 1st transmission of SSB as the QCL-Type A or C source in the TCI state to be activated and corresponding 2 ms processing time, after which the activated DL applicable unified TCI states are ready for DCI to indicate. Depending on the SSB time location, the SSB measurement and processing latency varies from 2 ms to 22 ms in case of 20 ms SSB periodicity. Similarly, as shown in Figure 5 for UL applicable TCI activation latency, after the MAC-CE command application time, the TCI activation latency also includes the time to measure the 5 occasions of PL RS associated with the TCI state to be activated and corresponding 2 ms processing time, after which the activated UL applicable unified TCI states are ready for DCI to indicate. The PL RS can be SSB or CSI-RS. If SSB serves as PL RS, depending on the SSB time location, the 5-occasion PL RS measurement and processing latency can be 102 ms in case of 20 ms SSB periodicity.

Figure 4: Activation latency for DL applicable unified TCI

Figure 5: Activation latency for UL applicable unified TCI

To save the above SSB/PL RS measurement latency, in L1/L2 mobility, TCI states can be activated for deactivated candidate SpCell and, hence, can be applied when this candidate SpCell is selected as the new SpCell. To truly save the latency, UE should maintain those TCI states before candidate SpCell is activated. Meanwhile, the TRS tracking for the candidate cell is achieved via activating the corresponding TCI state.

Proposal 19: Support NW to provide activated TCI state(s)/beam(s) for candidate cell before the cell switch command

- This should be applicable if the candidate cell is deactivated SCell, which currently does not allow TCI state activation prior to SCell activation

- The TRS tracking for candidate cell is achieved via activating the corresponding TCI state

In case of beam indication together with the cell switch command, a straightforward way is to use the TCI state configured for the new cell as the beam indication. Whether to support other options can be separately discussed.

Proposal 20: For the beam indication together with the cell switch command, at least support TCI state configured for the new cell as the beam indication

In R17 unified TCI, the indicated/activated TCI state in DCI/MAC-CE can be applied to multiple CCs in the same CC list to save the TCI indication overhead. The same mechanism can be extended to LTM, where the indicated TCI state in the cell switch command can be applied to multiple new cells in the same CC list. For example, if TCI state is indicated for a first new cell, the other new cells in the same CC list as the first new cell can use the same indicated TCI state without the need for individual TCI indication.

Proposal 21: For the beam indication together with the cell switch command, support that the same indicated TCI state can be applied to multiple new cells in the same CC list

Figure 6: Target DU may not know the beam indication for initial message due to long inter-DU latency

Another issue for beam indication is that in case of inter-DU HO, the source DU can send the beam indication for the new cell in the cell switch command as agreed before. However, if the source DU starts informing the target DU about the beam indication upon the cell switch command is sent, the target DU may need long latency to receive it due to large inter-DU delay, which could be tens of ms. Before that, the target DU may not be able to schedule the initial message with the UE due to lack of knowledge of beam indication. This may defeat the purpose of fast cell switch. Therefore, it would be beneficial to investigate this issue.

Proposal 22: In case of inter-DU LTM, study mechanism to reduce the latency for target DU to prepare the indicated beam after the cell switch command is sent

- Conclusion

In this contribution we discussed L1 enhancements for inter-cell beam management, including L1 measurement and reporting, beam indication, and dynamic switch mechanism among candidate serving cells. Below is the summary of the corresponding proposals.

Proposal 1: Application time should be specified for the SpCell/CG update command

- Application time can be different for previously activated and deactivated new SpCell/CG

Proposal 2: DL/UL BWPs to be used for the new cell can be indicated in cell update command

Proposal 3: To reduce TRS tracking and beam refinement latency after SpCell update, SpCell update command can also trigger TRS or beam refinement

Proposal 4: To reduce latency/throughput impact due to HARQ process reset, support NW can maintain HARQ continuity after cell switch at least in certain scenarios, e.g. intra-frequency intra-DU cell switch

- Detailed procedure/signaling can be determined by RAN2

Proposal 5: For CSI-RS based L1 measurement for candidate cell, support CSI-RS for tracking and beam management for both L1 intra/inter-frequency measurements

Proposal 6: For L1 intra-frequency measurement, support UE to report X best candidate cells with Y best beams per reported candidate cell in the same report

- X and Y can be configured subject to corresponding UE capabilities

Proposal 7: For L1 inter-frequency measurement, support UE to report X best candidate cells across all measured frequencies with Y best beams per reported candidate cell in the same report

- X and Y can be configured subject to corresponding UE capabilities

Proposal 8: To reduce L1 report overhead, L1 cell-level metric can be introduced to rank and select X best measured candidate cells for reporting

- L1 cell-level metric can be linear average of Y>1 best beams per cell, e.g. Y=2 or 3

Proposal 9: Support TCI state configuration per CSI-RS resource at least for L1 inter-frequency measurement

- Detailed configuration can be up to RAN2

Proposal 10: Support event triggered report for candidate cells for LTM

- The report can be carried in MAC-CE, similar to existing event triggered L2 report

Proposal 11: For event triggered report, at least a triggering condition similar to Event A3 for L3 report can be considered as starting point, e.g. candidate cell becomes amount of offset better than SpCell

Proposal 12: For event triggered report, L1 cell-level metric can be used to quantify the quality per cell considered in the triggering condition

- L1 cell-level metric can be linear average of Y>1 best beams per cell, e.g. Y=2 or 3

Proposal 13: Support MAC-CE activation of L1 report for a set of PCIs

- Related measurement/reporting config may also be updated by the MAC-CE

Proposal 14: For event tiggered report, UE is not required to perform L1 measurements on candidate cell(s) if the serving cell quality is above a threshold value

Proposal 15: The maintained DL timing per candidate cell can be defined as that for the reception of the first detected path (in time) of the corresponding downlink frame from the candidate cell

Proposal 16: Support NW to indicate candidate cell(s) for UE to maintain corresponding DL Rx timing(s)

- Maximum number of candidate cell(s) to maintain the DL Rx timing can be up to UE capability

Proposal 17: To facilitate mTRP operation on the new cell, R17 group-based beam report can be extended to LTM to identify a group of simultaneously receivable DL beams for a candidate cell

- Two CMR resource sets can be configured with each set including RS configured for LTM L1 measurement

Proposal 18: On the TCI framework for LTM, R15 TCI framework is not supported

Proposal 19: Support NW to provide activated TCI state(s)/beam(s) for candidate cell before the cell switch command

Proposal 20: For the beam indication together with the cell switch command, at least support TCI state configured for the new cell as the beam indication

Proposal 21: For the beam indication together with the cell switch command, support that the same indicated TCI state can be applied to multiple new cells in the same CC list

Proposal 22: In case of inter-DU LTM, study mechanism to reduce the latency for target DU to prepare the indicated beam after the cell switch command is sent

- References

- RP-213565 R18 WID on Further NR Mobility Enhancements

- TS 38.133 v17.5.0