| 3GPP TSG-RAN WG1 Meeting #112bis-e e-meeting, April 17th – 26th, 2023 |  | R1-2303617 |
| --- | --- | --- |

Agenda item:	9.14.3

Source: 	Qualcomm Incorporated

Title: 	Dynamic Switching between CP-OFDM and DFT-S-OFDM

Document for:	Discussion/Decision

# Introduction

This document discusses aspects related to dynamic waveform switching (DWS) between CP-OFDM and DFT-S-OFDM. In the previous RAN1 meetings, the following agreements were made on this topic:

Agreements made in R1-112 Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. Conclusion There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18. Agreement Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant. Conclusion The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant. Agreements made in R1-111

Agreements made in R1-110bis

In the following sections we discuss our views on the use cases and applicability of dynamic waveform switching, the signalling mechanism to enable DWS, and potential PHR enhancements that may be useful to facilitate DWS.

# Applicability of Dynamic Waveform Switching

In the previous meeting we agreed to specify DWS for DG-PUSCH scheduled by DCI Format 0_1 or 0_2. It was further discussed whether DWS should be enabled for DG-PUSCH scheduled by DCI Format 0_0 or not, but no agreements were made regarding this. DCI Format 0_0 serves as the fallback DCI and any dynamic behaviour associated with this format is certainly not desirable. Further, its best to avoid any change to this format’s size to accommodate DWS. The agreement to introduce an explicit bit to indicate the selected waveform in the scheduling DCI pretty much rules out its applicability to DCI Format 0_0. An alternative mechanism in parallel to the agreed mechanism is not desirable. Hence, we make the following proposal:

Proposal 1: DWS is not applicable to DG-PUSCH triggered by DCI Format 0_0.

Dynamic waveform switching for Msg3:

In the current specification, Msg3 waveform is determined by msg3-transformPrecoder. Some companies propose to also enable DWS for Msg3. Setting aside questions on the necessity/motivation, we are concerned that enabling this feature would require some indication of UE capability to support this feature. Early indication of a UE’s capability when it is not yet in connected state entails significant amount of work and it is probably best to avoid this in the absence of a strong motivation. Further, since Msg3 repetitions are now specified and latency is not a major concern, there is less motivation to introduce dynamic waveform shaping for Msg3. Finally, even if such a mechanism is introduced for Msg3, gNB may not be in a position to make an informed decision on when to switch from CP-OFDM to DFT-S-OFDM as its not aware of the additional power that a UE is able to deliver for DFT-S-OFDM.

Proposal 2: DWS is not applicable to Msg3 PUSCH transmission and retransmission.

# Switching Mechanism

In the last meeting it was agreed that the explicit bit included as part of UL scheduling DCI will only apply to the transmissions scheduled by the grant.  One remaining issue to resolve is the resulting DCI size when DWS is enabled and an explicit bit is included in the DCI to indicate the waveform to use.

Despite extensive discussions in the last meeting, there was no consensus on how to resolve the DCI size issue. Reproduced below are four options listed as part of the Feature Lead Summary from the last meeting:

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, if at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: Network perspective: This implies DWS field must be positioned before the first field that depends on waveform. - Note: UE perspective: This implies DWS field must be decoded before finding position of NDI field. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

The issue of DCI sizing arises since the bit-width of certain fields in the DCI are currently dependent on the type of waveform that is configured for the UE via RRC. Some examples include the TPMI field, FDRA field, etc.

There are two broad approaches to fix this issue --- either we can resize each field that is impacted or we can let the fields be as they are but align the resulting difference in DCIs for the two waveforms by padding bits at the end.

But the pros and cons of these two approaches need to be viewed in the context of other clauses in the specification that facilitate DCI size alignment across different features. With this in mind, we present certain relevant clauses in the specification related to DCI size alignment.

## Existing clauses on DCI size alignment

In the context of DCI scheduling a single PUSCH vs multiple PUSCHs, the DCI sizes are aligned by padding bits at the end of the DCI until the two sizes are equal.

From 38.212 Section 7.3.1.1.2

For DCIs with CRC scrambled by C-RNTI and CS-RNTI, it is required that no field for DCI with CRC scrambled by CS-RNTI be larger that the corresponding one for a DCI with CRC scrambled by C-RNTI. Additionally, when the bitwidths for a field are different depending on whether DCI’s  CRC is scrambled with C-RNTI or CS-RNTI, then padding bits are added to each such field with MSBs set to 0.

From 38.212 Section 7.3.1.1.2

When constructing a DCI with CRC scrambled by CS-RNTI, with NDI=1, the parameters in pusch-config are applied for PUSCH transmission except for certain parameters related to power control. Further, for DCI formats 0_1 and 0_2, if transformPrecoder is present in pusch-config then UE transmits based on this parameter and the DCI is expected to be constructed with this setting in mind.

From 38.214 Section 6.1 From 38.214, Section 6.1.3

## Discussion on the two options

Before choosing one of the four above options, we examine the impact of padding bits at the end of a DCI format and compare it to the impact of padding bits on a per-field basis. It is understood that the primary motivation for padding bits at the end comes from being able to amortize the overhead of padded bits across all the bit fields, i.e., if in a certain configuration a field is larger in size and another field is smaller in size, these two fields compensate each other and there may not be any need to add padded bits. In the case of dynamic waveform switching, such a benefit does not exist as all current fields for DFT-S-OFDM are smaller or the same size as that of CP-OFDM. A future change that results in a field with a larger size for DFT-S-OFDM may potentially benefit from this approach.

This approach however requires the UE to infer the dynamic waveform switching bit before proceeding to parse the DCI. These two operations have to be executed in a sequential manner. While this may not pose a significant problem, as the number of features that require dynamic DCI parsing grow, the complexity of DCI decoding on UE begins to grow and it requires further thought on how to bound this growing complexity.

In particular when a UE receives a DCI with CRC scrambled by CS-RNTI, the DCI could either be intended to activate or deactivate a configured grant or it could be intended to request a retransmission of a TB that was sent earlier using a configured grant transmission occasion. The intent behind the DCI is deciphered by the UE by looking at the NDI bit. This then requires the NDI bit to be at a location that is either at an invariant location or can be inferred based on the DWS bit. Thus, for a UE to be able to receive and parse DCIs with CRC scrambled by C-RNTI or CS-RNTI, the DWS bit needs to be placed before the NDI bit if the padding bits are placed at the end of the DCI.

On the other hand, if one were to pursue padding bits on a per-field basis, the DCI construction becomes invariant to the choice of the transmission waveform. For Fields whose bit width currently depends on the configured waveform type via the transformPrecoder parameter in RRC, the bit width is set to be the larger of the two values that results when transformPrecoder is enabled or disabled. This simplifies DCI construction and pairs well with other existing features in the specification that require DCI size alignment.

Admittedly, the second approach may not be efficient from a DCI size standpoint in case there arises a scenario where the DCI field for DFT-S-OFDM is larger than that for CP-OFDM. Such cases do not exist at this point and can be addressed when they present themselves in the future.

Option 4 suggests constructing DCI differently when DCI is addressed to CS-RNTI. This would then create a scenario where DG-PUSCH is triggered via two slightly different DCI formats. Such a fork seems unnecessary as it would end up creating two classes of DG-PUSCH. Additionally, in case DWS is disabled, DCI construction for CS-RNTI would fallback to legacy mode. This would require UE to effectively track a new DCI format and for this reason it is not preferred.

Option 3 take a hybrid approach and combines Options 1 and 2 to make certain field locations invariant to waveform type. While it does help resolve some of the concerns around dynamic DCI parsing, it seems to lead to a rather cumbersome approach to DCI construction. A simpler approach such as the one in Option 1 or 2 is preferred.

On the whole, its clear that both Options 1 and 2 are viable solutions. Given that CG-PUSCH is a feature that is already deployed in the field, going with Option 2 would be the least disruptive to UE implementation. With Option 1, more discussion on joint activation of other features that require dynamic DCI parsing such as support for CG-PUSCH, multi-PUSCH scheduling, multi-carrier scheduling, dynamic switching of TRP transmission modes, etc, will be required.

Proposal 3: For DCI size alignment when DWS is enabled, Option 2 (per-field alignment) is preferred. For approaches such as Option 1 that require per-format padding of bits, it is suggested to first discuss the issue of dynamic DCI parsing and its impact on existing features and UE implementation before proceeding to consider it for the specification.

# PHR Enhancements for Aiding Dynamic Waveform Switching

In a previous meeting, it was agreed to study potential enhancements to power headroom reporting to aid dynamic waveform switching. The goal of this objective is to aid the gNB to decide when to trigger a waveform switch based on how much additional power a UE can deliver using DFT-S-OFDM compared to CP-OFDM. There are two potential directions for enhancements that one could consider. The first is focused on trigger conditions that initiate the UE to report power headroom and the second is on the reporting structure itself.

Currently a UE triggers a PHR based on periodic timer or prohibit timer expiring, pathloss changing by a certain value, etc. To this list, it might be worthwhile to consider adding trigger conditions that get activated anytime a UE may be getting close to its maximum power, i.e., power headroom level falls below a threshold. These trigger conditions can be viewed by the gNB as a subtle indication from the UE to consider switching the waveform.

Proposal 4: Timely information regarding dynamic waveform switching is provided to a scheduler by introducing new trigger conditions to report power headroom.

- Allow a UE to trigger a PHR whenever power headroom falls below a certain threshold and another waveform with better headroom is available.

The second line of enhancements that might be worth considering is to make changes to the reporting structure and modify it appropriately so that the report carries the desired information for both waveforms. Currently a UE reports power headroom and maximum power (Pcmax) associated with a waveform. A possible option is to modify the current reporting framework to let the UE report two sets of values for power headroom and Pcmax assuming the same underlying transmission parameters. A reference PUSCH will then need to be assumed for the other waveform.

Proposal 5: Enhance the existing PHR structure to allow a UE to report Pcmax and power headroom for each waveform. A reference PUSCH is assumed for the waveform not currently in use. The RB allocation and modulation order are assumed to be the same as the current waveform. Allow MPR, A-MPR, and P-MPR to be taken into account when computing the power headroom.

# Conclusion

We make the following observations and proposals for dynamic switching between CP-OFDM and DFT-S-OFDM.

Proposal 1: DWS is not applicable to DG-PUSCH triggered by DCI Format 0_0.

Proposal 2: DWS is not applicable to Msg3 PUSCH transmission and retransmission.

Proposal 3: For DCI size alignment when DWS is enabled, Option 2 (per-field alignment) is preferred. For approaches such as Option 1 that require per-format padding of bits, it is suggested to first discuss the issue of dynamic DCI parsing and its impact on existing features and UE implementation before proceeding to consider it for the specification.

Proposal 4: Timely information regarding dynamic waveform switching is provided to a scheduler by introducing new trigger conditions to report power headroom.

- Allow a UE to trigger a PHR whenever power headroom falls below a certain threshold and another waveform with better headroom is available.

Proposal 5: Enhance the existing PHR structure to allow a UE to report Pcmax and power headroom for each waveform. A reference PUSCH is assumed for the waveform not currently in use. The RB allocation and modulation order are assumed to be the same as the current waveform. Allow MPR, A-MPR, and P-MPR to be taken into account when computing the power headroom.

# Reference

- RP-221858, Revised WID on Further NR Coverage Enhancements, 3GPP TSG RAN Meeting #96, RAN1 103-e, Budapest, Hungary, June 6-9, 2022