3GPP TSG-RAN WG1 Meeting #112bis	R1-2303619

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.15.1

Source:	Qualcomm, Thales

Title:		eMBB Spectral Efficiency SLS parameters and assumptions

Document for:	Discussion / Decision

# 1	Introduction

RAN#99 agreed on a Study Item for a Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology [1]. In this contribution we provide basic simulation assumptions to be used for the self-evaluation of eMBB Spectral Efficiency.

# 2	Simulation assumptions for SLS related to eMBB spectral efficiency

This section focuses on system-level simulation parameters and assumptions required to evaluate the following two Technical Performance Requirements, defined in ITU-R M.2514 [2]:

| Characteristic for evaluation | Assessment method | Requirement description sec. [2] | Applicable Usage Scenario / Test Environment | Required value [bit/s/Hz] |
| --- | --- | --- | --- | --- |
| 5th percentile spectral efficiency | Simulation (system-level) | § 7.2.4 | eMBB-s Rural | Uplink:; 0.003. Downlink:; 0.03. |
| Average spectral efficiency | Simulation (system-level) | § 7.2.5 | eMBB-s Rural | Uplink:; 0.1. Downlink:; 0.5. |

The following sections capture, in order:

- A basic set of evaluation parameters and values captured in the ITU-R Report [2], for handheld devices and eMBB-s Rural environment.

- Extra simulation parameters and values derived from the 3GPP IMT-2020 self-evaluation of 5G terrestrial NR [3]

- Other parameters derived from the 3GPP Rel-16 TR on NTN (38.821, [4])

The combined set of those parameters and assumptions is proposed to be used as baseline for the 3GPP self-evaluation of NR-NTN toward IMT-2020 satellite TPRs for 5th percentile and average spectral efficiency.

Note: the parameters and assumptions below would apply also to the evaluation of User data rate and Area traffic capacity, which can be derived from the 5%-ile and Avg Spectral Efficiency ([2]).

## 2.1	ITU-R evaluation parameters

The ITU-R Report M.2514 includes a certain set of example parameters related to UE configuration, satellite configuration and other specific parameters for the evaluation of TPRs in the eMBB-s Rural environment, including spectral efficiency TPRs (5th percentile and average spectral efficiency). In particular, the following characteristics and assumptions should be applied for the 3GPP self-evaluation:

Table 1. UE parameters

| Parameters | Values (Handheld terminal) |
| --- | --- |
| Antenna type and configuration | Omni-directional |
| Polarisation | Linear: ±45°X-pol |
| Antenna gain (dBi) | 0 |
| Antenna temperature (K) | 290 |
| Noise figure (dB) | 7 |
| Tx transmit power | 200 mW (23 dBm) |

Table 2. Parameters for evaluations of handheld TPRs in eMBB-s Rural test environment

| Parameters | Values/Types (Handheld terminal) |
| --- | --- |
|  | Rural-eMBB-s |
| Satellite orbit configuration | LEO, 600 km altitude |
| Spot beam pattern | Hexagonal pattern, at least 19 spot beams |
| Service link frequency | 2 GHz |
| Channel bandwidth | 30 MHz |
| 3 dB beam width | 4.41 degrees |
| Satellite EIRP density | 34 dBW/MHz |
| Satellite antenna gain | 30 dBi |
| Satellite G/T | 1.1 dB/K |
| Device deployment | 100% outdoor, randomly and uniformly distributed over the area |
| UE antenna height | 1.5 m |
| UE density | 10 UEs per spot beam |
| UE mobility model | For spectral efficiency evaluations: Stationary |
| Traffic model | Full buffer |

## 2.2	Other 3GPP simulation parameters and assumptions

2.2.1 	From 3GPP IMT-2020 self-evaluation of 5G terrestrial NR (eMBB SE SLS)

Based on simulation parameters and assumptions used for the 3GPP IMT-2020 self-evaluation of eMBB spectral efficiency for 5G terrestrial NR [3], the following additional parameters can be considered and adapted for NR-NTN spectral efficiency simulations.

Note: reference values are suggested for most parameters, rest is left for further discussion (TBD) or up to the simulation proponent.

eMBB SE Parameters

Table 3. eMBB SE Parameters and ref values

| Parameters | Reference value |  |
| --- | --- | --- |
|  | Donwlink | Uplink |
| Multiple access | OFDMA | OFDMA |
| Duplexing | FDD | FDD |
| Modulation | Up to 256 QAM | Up to 256 QAM |
| Coding on PDSCH | LDPC; Max CB size = 8448bit | LDPC; Max CB = 8448bit |
| Numerology | 15 kHz | 15 kHz |
| Guard band ratio | 4% (over 30MHz sim. BW) | 4% (over 30MHz sim. BW) |
| Transmission scheme | One layer/ No MIMO | One layer/ No MIMO |
| DL CSI measurement | One layer/ 1-port CSI-RS | Not Applicable |
| PRB bundling | Wideband | Not Applicable |
| Codeword (CW) | single CW | single CW |
| CSI feedback | TBD | Not Applicable |
| DL ACK/NACK delay | TBD [no HARQ, N+4] | Not Applicable |
| Re-transmission delay | TBD [no HARQ, N+8 (+RTT)] | TBD [no HARQ, N+4 (4ms after reception of the uplink transmission)] |
| TRXP Ant. configuration | single Tx per beam | single Rx per beam |
| UE Antenna configuration | 2 Rx | 1 Tx |
| Scheduling | PF | PF |
| Receiver | MMSE-IRC | MMSE-IRC |
| Channel estimation | Non-ideal | Non-ideal |
| UE attachment | Based on RSRP | Based on RSRP |
| Wrapping around method | see 38.821 | see 38.821 |
| Power control parameters | Not Applicable | N.A (assume max Pwr) |
| Power backoff model | Not Applicable | See [5] |

Overhead assumptions

Some overhead parameters/assumptions can also be considered and accounted for the evaluation of spectral efficiency results. The following table shows a subset of OH parameters used for 5G IMT-2020 self-evaluation, which apply to NR-NTN as well.

Table 4. OH parameters

| Downlink OH | Uplink OH |
| --- | --- |
| PDCCH | PUCCH |
| SSB | DMRS |
| CSI-RS for CM | SRS |
| DMRS |  |
| TRS |  |

It is for further discussion whether a set of default assumptions could be identified, or left up to the proponents (of the simulation results) and subject to later normalization.

2.2.2 	Other parameters/assumptions

In addition to the parameters described above, other extra assumptions and details were considered by RAN1 during Rel-16 NTN simulations work [4], which may be re-used for IMT-2020 self-evaluation, as needed, for example regarding specific satellite configuration parameters, such as:

Table 5. Other satellite specific parameters / assumptions

| Satellite antenna pattern | Ref to section 6.4.1 in [6]: Bessel function |
| --- | --- |
| Satellite antenna polarization configuration | Circular |
| Equivalent satellite antenna aperture | 2m |
| Satellite beam diameter | 50km |
| Beam layout definition | Ref to separate Table 6.1.1.1-4 (singles satellite simulation) |
| Frequency re-use factor | TBD (1 or 3) |
| Propagation conditions | Clear Sky, Line of sight |
| Channel model | Large scale model of [6] |

# 3	Conclusion

Based on the discussion above, we propose the following:

Proposal 1	The evaluation parameters and assumptions described in sec 2.1 and 2.2 should be endorsed, as baseline for the simulation of eMBB spectral efficiency TPRs for NR-NTN IMT-2020 self-evaluation.

# References

RP-230736, New SID, “Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology”, RAN#99, March 2023.

Report ITU-R M.2514: “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020”

RP-191521: 3GPP self-evaluation report for the IMT-2020 terrestrial RIT submission (attachment B.4.1)

TR 38.821 v16.1.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”, May 2021.

TS 38.101-5 v18.0.0, “User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)”, December 2022.

TR 38.811 v15.4.0, “Study on New Radio (NR) to support non-terrestrial networks (Release 15)”, September 2020.