3GPP TSG RAN WG1 #112bis-e                                                                             R1-2303624

e-Meeting, April 17th – April 26th, 2023

Source:	Qualcomm Incorporated

Title:	Views on MC-Enh, BWP without restriction, and eDSS

Agenda Item:	9.18

Document for: 	Discussion and Decision

# Background

In this contribution, we provide our views on the following Rel-18 topics:

- Rel-18 Multicarrier enhancements

- Rel-18 BWP without restriction

- Rel-18 DSS enhancements

# R18 Multicarrier enhancements

## 2.1	Multi-cell PDSCH/PUSCH scheduling with a single DCI format

### 2.1.1	Clarifications on BWP indication field

The UE BWP indicator was agreed as Type-1A field. Currently, interpretation of BWP indicator depends on the number of BWPs configured for the cell. In particular, following is the rule of how to interpret the BWP indication field according to TS38.212:

- The BWP indicator has log2(nBWP) bits, where;

  - nBWP = nBWP,RRC + 1 if nBWP,RRC <= 3, BWP indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

  - Otherwise, nBWP = nBWP,RRC and the BWP indicator is defined in Table 7.3.1.1.2-1;

With this, the configuration becomes weird if the number of BWPs configured for cells in a set of cells for multi-cell scheduling are different. For example, if cell-1 has one UE-specific BWP and cell-2 has four UE-specific BWPs and if the UE is configured with multi-cell scheduling for cell-1 and cell-2 by a DCI format 0_X/1_X with BWP-switching, the BWP indicator field interpretation becomes following: The field value ‘00’ indicates BWP#0 for cell-1 and UE-specific BWP#1 for cell-2, while the value ‘01’ indicates UE-specific BWP#1 for cell-1 and UE-specific BWP#2 for cell-2, as shown in the Table below. Note that the number of UE-specific BWPs is dependent on per-band UE capabilities and hence, this is a valid issue especially for multi-cell scheduling for inter-band CA.

| BWP indicator field | Cell 1 | Cell 2 |
| --- | --- | --- |
| 00 | BWP #0 | BWP with BWP-id = 1 |
| 01 | BWP with BWP-id = 1 | BWP with BWP-id = 2 |
| 10 | BWP with BWP-id = 2 | BWP with BWP-id = 3 |
| 11 |  | BWP with BWP-id = 4 |

Simple solution to fix this without RRC signalling could be to specify that the BWP indication field of a DCI format 0_X/1_X is interpreted in the same way as the above “Otherwise” case, no matter how many BWPs are configured on each cell in the set of cells. That is, BWP indication field in DCI format 0_X/1_X has log2(nBWP) bits where nBWP=nBWP_RRC and the indicator is defined in Table 7.3.1.1.2-1. Another approach to avoid this is to make sure that the same number of BWPs for all the cells in a set of cells for multi-cell scheduling is configured.

Proposal 2-1-1:

- Select either of the following options:

  - Opt.1: BWP indication field in DCI format 0_X/1_X has log2(nBWP) bits where nBWP=nBWP_RRC and the indicator is defined in Table 7.3.1.1.2-1

  - Opt.2: The number of configured BWPs for all the cells in a cell set of for multi-cell scheduling is the same

### 2.1.2	Clarifications on fields that are configurable between Type-1A and Type-2

Antenna port(s) (configurable between Type-1A and Type-2)

For DCI formats 1_1 and 1_2, the number of bits for antenna port(s) field depends on various RRC parameters, such as dmrs-Type and maxLength. Based on these RRC parameters, the UE refers to one of the multiple tables defined in TS 38.212 7.3.1.2.2 to interpret the Antenna port(s) field as follows.

When the antenna port(s) field in DCI format 1_X is configured as Type-1A, single field applies to all the scheduled cells by the DCI format. It does not make sense to consider that the cells scheduled by the same DCI format refers to different Tables for Antenna port(s). We propose to clarify that UE expects that Antenna port(s) field, when it is configured as Type-1A, refers to the common/same Table from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, or 7.3.1.2.2-4, for all the cells in the set of cells for DCI format 1_X.

Similarly, for DCI formats 0_1 and 0_2, the number of bits for antenna port(s) field depends on various RRC parameters, such as dmrs-Type, maxLength, transformPrecoder, and pi/2 BPSK. Based on these RRC parameters, the UE refers to one of the multiple tables defined in TS 38.212 7.3.1.1.2 to interpret the Antenna port(s) field as follows..

Same as for DL case, when the field is configured as Type-1A, it does not make sense to consider that the cells scheduled by the same DCI format refers to different Tables. We propose to clarify that UE expects that Antenna port(s) field, when it is configured as Type-1A, refers to the common/same Table from Tables 7.3.1.1.2-6, 7.3.1.1.2-6A, 7.3.1.1.2-7, 7.3.1.1.2-7A, 7.3.1.1.2-8, 7.3.1.1.2-9, 7.3.1.1.2-10, 7.3.1.1.2-11, 7.3.1.1.2-12, 7.3.1.1.2-13, 7.3.1.1.2-14, 7.3.1.1.2-14, 7.3.1.1.2-15, 7.3.1.1.2-16, 7.3.1.1.2-17, 7.3.1.1.2-18, 7.3.1.1.2-19, 7.3.1.1.2-20, 7.3.1.1.2-21, 7.3.1.1.2-22, 7.3.1.1.2-23, 7.3.1.1.2-24, and 7.3.1.1.2-25, for all the cells in the set of cells for DCI format 0_X.

TPMI (configurable between Type-1A and Type-2)

Similar to Antenna port(s), for DCI formats 0_1 and 0_2, the number of bits for TPMI field depends on various RRC parameters, such as tx-Config (codebook or non-codebook), ul-FullPower, maxRank, transformPrecoder, codebookSubset. Based on these RRC parameters, the UE refers to one of the multiple tables defined in TS 38.212 7.3.1.1.2 to interpret the TPMI field as follows..

Same as for Antenna port(s) field, when the TPMI field is configured as Type-1A, it does not make sense to consider that the cells scheduled by the same DCI format refers to different Tables. We propose to clarify that UE expects that TPMI field, when it is configured as Type-1A, refers to the common/same Table from Tables 7.3.1.1.2-2, 7.3.1.1.2-2A, 7.3.1.1.2-B, 7.3.1.1.2-3, 7.3.1.1.2-3A, 7.3.1.1.2-4, 7.3.1.1.2-4A, 7.3.1.1.2-5, and 7.3.1.1.2-5A, for all the cells in the set of cells for DCI format 0_X.

SRI (configurable between Type-1A and Type-2)

Similar to Antenna port(s) and TPMI, for DCI formats 0_1 and 0_2, the number of bits for SRI field depends on various RRC parameters, such as tx-Config (codebook or non-codebook), ul-FullPower, number of SRS resource sets, and maxMIMO-Layers. Based on these RRC parameters, the UE refers to one of the multiple tables defined in TS 38.212 7.3.1.1.2 to interpret the SRI field as follows.

Same as for Antenna port(s) field and TPMI field, when the SRI field is configured as Type-1A, it does not make sense to consider that the cells scheduled by the same DCI format refers to different Tables. We propose to clarify that UE expects that SRI field, when it is configured as Type-1A, refers to the common/same Table from Tables 7.3.1.1.2-28, 7.3.1.1.2-29, 7.3.1.1.2-30, 7.3.1.1.2-31, 7.3.1.1.2-32, 7.3.1.1.2-32A, and 7.3.1.1.2-32B, for all the cells in the set of cells for DCI format 0_X.

Proposal 2-1-2:

- For Antenna port(s) field in DCI format 1_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PDSCH receptions scheduled by the DCI format 1_X for any cell in the set for the DCI format 1_X are configured such that single Table from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, and 7.3.1.2.2-4 in TS38.212 is used.

- For Antenna port(s) field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-6, 7.3.1.1.2-6A, 7.3.1.1.2-7, 7.3.1.1.2-7A, 7.3.1.1.2-8, 7.3.1.1.2-9, 7.3.1.1.2-10, 7.3.1.1.2-11, 7.3.1.1.2-12, 7.3.1.1.2-13, 7.3.1.1.2-14, 7.3.1.1.2-14, 7.3.1.1.2-15, 7.3.1.1.2-16, 7.3.1.1.2-17, 7.3.1.1.2-18, 7.3.1.1.2-19, 7.3.1.1.2-20, 7.3.1.1.2-21, 7.3.1.1.2-22, 7.3.1.1.2-23, 7.3.1.1.2-24, and 7.3.1.1.2-25 in TS38.212 is used.

- For TPMI field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-2, 7.3.1.1.2-2A, 7.3.1.1.2-B, 7.3.1.1.2-3, 7.3.1.1.2-3A, 7.3.1.1.2-4, 7.3.1.1.2-4A, 7.3.1.1.2-5, and 7.3.1.1.2-5A in TS38.212 is used.

- For SRI field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-28, 7.3.1.1.2-29, 7.3.1.1.2-30, 7.3.1.1.2-31, 7.3.1.1.2-32, 7.3.1.1.2-32A, and 7.3.1.1.2-32B in TS38.212 is used.

### PDCCH overbooking for DCI format 0_X/1_X

According to TS38.213, currently, PDCCH overbooking is allowed only for search space sets monitored on the primary cell with cardinality of JUSS for scheduling on the primary cell. A DCI format 0_X or 1_X in this case can schedule any cell(s) in the set of cells including the primary cell. UE does not know if the DCI format 0_X/1_X to be detected in a monitored search space set schedules the primary cell or not until it completes the decode. Therefore, it does not make sense to follow the existing spec for a search space set for DCI format 0_X/1_X. Considering that a DCI format 0_X/1_X in a search space set monitored on the primary cell is for multi-cell scheduling and hence can schedule at least one secondary cell, no overbooking for search space set(s) for DCI format 0_X/1_X should be the straightforward approach.

Proposal 2-1-3:

- PDCCH overbooking is not supported for DCI format 0_X/1_X

### 2.1.4	Views on UE feature aspects

Nowadays, each UE now supports various DL/UL-CA configurations including intra/inter-bands, FDD/TDD, multiple SCSs, inter/intra-FR, etc. In addition, a UE supports various features specified in the legacy releases. It is important to specify UE features for multi-cell scheduling such that the multi-cell scheduling is enabled without unnecessary restrictions for implementation point of view.

First of all, multi-cell PDSCH scheduling and multi-cell PUSCH scheduling should be separate features, same as for Rel-16 DL/UL cross-carrier scheduling (FG18-5, FG18-5b). In addition, multi-cell scheduling should not prerequisite cross-carrier scheduling since multi-cell scheduling works even without cross-carrier scheduling. Further, it is necessary to enable supporting multi-cell scheduling for a limited set of bands in a DL/UL-CA configurations. We think a UE should be able to report support of multi-cell scheduling for one or multiple combination(s) of {a SCS for scheduling cell, a SCS for scheduled cells}.

Proposal 2-1-4:

- Introduce separate capabilities for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling

- Capabilities for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling do not prerequisite UE capabilities for R15/16 cross-carrier scheduling (FG6-10, 18-5, 18-5b)

- For a given DL-CA or UL-CA band combination, the UE reports support for:

  - Opt. 1: one or multiple combinations of {a band for scheduling cell, a set of band(s) for scheduled cells} for multi-cell scheduling by a single DCI format

  - Opt. 2: one or multiple combinations of {a carrier type for scheduling cell, a carrier type for scheduled cells} for multi-cell scheduling by a single DCI format

    - Carrier type: one from {FDD, TDD, Unlicensed, FR2}

  - Opt.3: one or more combination(s) of {a SCS for scheduling cell, a SCS for scheduled cells} for multi-cell scheduling by a single DCI format

Further details are presented in [1].

## 2.2	UL Tx switching for 3 or 4 bands

### 2.2.1 Switching period location on frequency domain

In RAN1 #112, Alt. 5 was approved for switching period location and gNB would configure the priority for each band. However, there are some scenarios we want to clarify which band(s) are configured with the switching period.

- Alt.5: gNB configures priorities to each carrier/band. - The gNB configures priority for each band. The UE determines the switching period location on either switching-from band(s) or switching-to band(s) that is involved in the UL Tx switching and is not with the highest priority band.

For simplicity, the below discussion is based on following assumptions:

- Network configures the band priority in descending order, i.e. A>B>C>D

- Band A is most important band.

Scenario 1: the highest priority band is not involved into the switch (i.e. B->C)

If the highest priority band is not involved into the switch, it’s reasonable to protect the higher priority band among the switch-from and switch-to band groups. For the switch of band B and C, band C should be configured with the switching period.

Proposal 2-2-1:

The switch-from or switch-to group with lowest priority band of the bands involved in the switch should be configured with the switching period.

Scenario 2: the same lower priority band transmits before and after switch.

Take switching from B+D -> C+D as an example, the switch-to and switch-from band group includes the same lower priority band. To determine which band group takes switching period location, we propose to remove this band (band D) from switch-to and switch-from band group and compare the priority of the rest bands. Following the principle to protect the higher priority band of the rest band(s), the switch-from or switch-to band group with the higher priority band should be protected and not take the switching period, which is switch-to band group (band C+D) of the above case.

Another example is B+D ->D which involves 3 bands. After removing the lower priority band, only switch-from band group has one band left (band B). In order to impact less band, the switch-to band group is preferrable to take the switching period.

Proposal 2-2-2:

If the same lower priority band transmits before and after switch and more than two bands are involved in the switch, remove the lower priority band from switch-to and switch-from band groups. Compare the rest band(s) of the switch-to and switch-from band groups, the group with lower priority band is preferable to take the switching period. If three bands are involved into the switch, the band group with the only one band take the switching period.

Scenario 3: the highest priority band transmits before and after switch. i.e. A+B->A+C.

In [4], RAN4 approved UE could report a “Tx chain unchanged” capability due to switch as below.

- R4-2220546 WF on UE Tx switching requirement for single TAG - For the band with the number of Tx chain unchanged due to switching, in addition to the baseline UE assumption agreed in RAN4 #104e, introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged

For the “unchanged UE”, the number of Tx chain on band A would not be outage by the switching from band B to C. If the switching gap is smaller than the required switching period, the lower priority band (band C) should be configured with switching period location and dropped.

For the “baseline UE”, transmission on band A would be outage during switching from B to C. Following the principle to protect the highest priority band (i.e. PCell), we propose network to avoid this case, and guarantee sufficient switch period if the highest priority band transmits before and after switch.

Otherwise, lower priority band (band C) of the rest bands should be dropped to create sufficient switching gap, and band A (transmission after switch) would be also outage.

Proposal 2-2-3:

If the highest priority band transmits before and after switch. i.e. A+B->A+C,

- for the “unchanged UE”, if the switching gap is smaller than the required switching period, the lower priority band (band C) should be configured with switching period and dropped;

- for the “baseline UE”, network should guarantee sufficient switch gap if the highest priority band transmits before and after switch.

Beyond above scenarios, if one of the bands is configured with two contiguous carriers, both carriers should be with same priority.

Proposal 2-2-4:

If one of the bands is configured with two contiguous carriers, both carriers should be configured with same priority.

### 2.2.2 Timeline

In RAN1 #112, timeline issue was discussed but without conclusion. We share our views on how to determine the overlapped transmission is one or two switches.

To illustrate the issue, we assume the band A and B transmit before switch and band C and D transmit after switch(es), and start of transmission on band C is no later than D. We use Figure 1 for illustration. As switching period time domain location is under discussion, the figure is back-to-back transmission between the switch-to and switch-from bands and UE needs to create the switching gap by dropping the transmission on the network configured bands. Here we assume the switch-from band group is configured with switching period location on frequency domain.

Figure 1 Illustration of overlap transmission after switch.

Case A) starts of transmission on band C and D are same; Case B) transmission on band C starts earlier.

As only one port is scheduled on C and D, the associated band of band C (first transmission after switch) is a key factor to determine the following switch. We would list below four possibilities for analysis.

- Alt. 1: Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band A. After switch, 1Tx is on band C and another Tx is on band A.

- Alt. 2: Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band B. After switch, 1Tx is on band C and another Tx is on band A.

- Alt. 3: Network configures twoT via uplinkTxSwitching-DualUL-TxState, both Tx chains are switched to band C.

- Alt. 4: Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band D. After switch, 1Tx is on band C and another Tx is on band D.

Alt. 1 Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band A. After switch, 1Tx is on band C and another Tx is on band A.

As the band C’s associated band is band A, the Tx would be on band A and C after switch. UE needs to switch the Tx chain from band A to D for the second transmission.

For Case A) the start of transmission on band C and D are same, UE could jointly create the switching gap before the start of the transmission (T0 in the figure). As in our companion paper [LS], we propose the switching gap to be summation of {{max(Tswitch_A_C, Tswitch_B_C), max(Tswitch_A_D, Tswitch_B_D)} where Tswitch_X_Y is the UE reported switching period for band pair X, Y.

For Case B) the start of transmission on band C is earlier than band D, the switch to band D may lead to outage on band C transmission. For the “unchanged” UE, the switching on band D would not cause the outage on band C and the switching period on band D could be anywhere. For the “baseline” UE, the switching on band D would lead the outage on band C if the switching gap overlapped with band C transmission. To avoid the outage, we propose following rules for transmission/switching on band D for one switch:

- The switch to band D is completed before T0, which is the start of transmission on band C.

- The start of transmission on band D should not be too late compared with T0. UE looks ahead the UL grants of band D transmission, and the look-ahead window is [X slot]. i.e. X = 1 reference slot.

- The UL grant of band D arrives before T0 – Toffset, Toffset is the PUSCH preparation time based on Tswitch of Summation of switching periods of {max (A->C, B->C), A->D}.

Alt. 2: Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band B. After switch, 1Tx is on band C and another Tx is on band B.

For Case A), this alternative is exactly same as Alt. 1.

For Case B), this alternative is similar as Alt. 1 and the only difference is on the switching period as the 2nd Tx chain is from band B to D.

Alt. 3: Network configures twoT via uplinkTxSwitching-DualUL-TxState, both Tx chains are switched to band C.

For Case A), this alternative is exactly same as Alt. 1

As the two Tx chains are on band C, the difference with above proposal is the switching period.

Alt. 4: Network configures oneT via uplinkTxSwitching-DualUL-TxState, band C’s associated band is band D. After switch, 1Tx is on band C and another Tx is on band D.

As the band C’s associated band is band D, the Tx chain is already on band D after the switch at T0. For both Case A) and B), UE is with one switch and the switching gap equals to summation of switching periods of {max (A->C, B->C), max (A->D, B->D)}.

Based on analysis above, we have following proposal for one switch.

Proposal 2-2-5:

When the switch-to bands are two and different with the switch-from band(s) (i.e. band A+B -> C+D), after switch start of band C transmission is no later than band D, 1 Tx chain switches to band C and another Tx chain is on one of {band A, B, or C} according configuration uplinkTxSwitching-DualUL-TxState. Once the following condition is met, UE is with one switch.

- If the two bands after switch start transmission at the same time, UE creates the switching gap before the start of the transmission for the two switches. The UL grants on both bands should be no later than T0-Toffset where Toffset is derived from switching gap to be summation of {max(Tswitch_A_C, Tswitch_B_C), max(Tswitch_A_D, Tswitch_B_D)} where Tswitch_X_Y is the UE reported switching period for band pair X, Y.

- If start of band C transmission is earlier than band D, 1 Tx chain switches to band C and another Tx chain is on one of {band A, B, or C}, the “baseline” UE is with one switch if following condition is met.

  - The switch to band D is completed before T0, which is the start of transmission on band C.

  - UE looks ahead the UL grants of band D transmission, the start of transmission on band D should be within a look ahead window from T0. The look-ahead window is [X slot]. i.e. X = 1 reference slot.

  - The UL grant of band D arrives before T0 – Toffset, Toffset is the PUSCH preparation time based on Tswitch of Summation of {{max(Tswitch_A_C, Tswitch_B_C), Tswitch_2BAND_D}

    - Tswitch_X_Y is the UE reported switching period for band pair X, Y.

    - Tswitch_2BAND_D is the switching period from the band with the 2nd Tx chain to band D.

Figure 2 is illustration of two switches. The Tx chains are on band A and B, and the band C transmission is ahead of band D. Similar as above analysis for single switch, the most important issue is the 2nd Tx chain status after one Tx chain switches to band C. If the band C’s associated band is band D, the 2nd Tx chain would switch to band D when the 1st Tx chain switches to band C, there is no more switch needed before band D transmission. Otherwise, UE needs to switch the 2nd Tx chain before T2 by following current timeline of UL grants.

### 2.2.3 Minimum separation time

In RAN#112 the minimum separation time was approved to resolve the back-to-back switching complexity. Some related issues were also discussed but without agreement due to lack of time, which are reporting granularity of UE capability and minimum separation time for MTAG. Below is the latest FL proposal which got majority support.

We propose to endorse the following proposal in this meeting.

For reporting granularity of UE capability on the X us ### Updated proposal 4-2: UE capability on the X us is reported per BC. For minimum separation time for MTAG case ### Updated proposal 4-3: For more than one TAG case, X us is subject to UE capability with a value set of {0us, 500us} as well as for one TAG case

Proposal 2-2-6: Endorse following for minimum separation time

- For reporting granularity of UE capability on the X us, UE capability on the X us is reported per BC.

- For more than one TAG case, X us is subject to UE capability with a value set of {0us, 500us} as well as for one TAG case.

# R18 BWP without restriction

At the RAN#99 meeting, support of options for BWP without restriction was agreed [1].

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4.

In this section, we provide our views on BWP without restriction.

## 3.1	Option A

We consider Option A does not have a RAN1 spec impact. Since there have been UE capabilities for CSI-RS based BM/RLM/BFD within active DL BWP already, new UE features for Option A (at least from RAN1 point of view) would not be necessary.

Observation 3-1:

- Option A does not require RAN1 spec impact or new UE feature.

## 3.2	Option B-1-1

In the past, UE features for BWP without restriction based on SSB outside active DL BWP have been proposed (e.g., in RP-222067). Considering that this is no longer R16/17 maintenance, there is no need to prerequisite the legacy UE feature for BWP without restriction (FG6-1a). It was observed in the past that companies have different understandings on FG6-1a, e.g., regarding whether or not CSI-RS based BM/RLM/BFD is enabled, etc. Specifying new FGs can resolve such issue in Rel-18.

In terms of the SSB location, it is necessary to clarify where it can actually be located (was not clear in FG6-1a). Obviously, the SSB shall still be within the carrier bandwidth configured for the UE. In addition, the case where the SSB is not within any DL BWP of the carrier must be invalid scenario. Therefore, we propose to clarify that the SSB is within a bandwidth of at least one DL BWP of the carrier for the UE. More specifically, we further clarify following: (i) If the UE has one UE-specific DL BWP configuration for the cell, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP; (ii) If the UE has more than one UE-specific DL BWP configurations for the cell, the SSB is within a bandwidth of either of the UE-specific DL BWPs. This clarification can be either in RAN1/RAN4 spec or in UE feature description. In [2], we propose to clarify it in the UE feature description.

Proposal 3-1:

- Option B-1-1 does not prerequisite support of FG6-1a ‘bwp-WithoutRestriction’

- For Option B-1-1, clarify that the SSB outside the active DL BWP is still within a bandwidth of at least one DL BWP of the carrier.

  - If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP

  - If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs

- Following could be the FG for Option B-1-1 as proposed in [2]

| Capability-bwp-wor-b-1-1 Indicates support of BM/RLM/BFD based on SSB outside the active DL BWP without interruptions. If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP. If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

As for specification impact, we consider following should be the starting point as presented in RP-222067.

Proposal 3-2:

- For Option B-1-1, consider following as the starting point of TS38.213 updates

# 5 Radio link monitoring The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. Except for SSB based radio link monitoring for a UE supporting [Capability-bwp-wor-b-1-1 or Capability-bwp-wor-b-1-2], tThe UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS. If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. Except for SSB based radio link monitoring for a UE supporting [Capability-bwp-wor-b-1-1 or Capability-bwp-wor-b-1-2], tThe UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell.

## 3.3	Option B-1-2

Same as for Option B-1-1, Option B-1-2 should not prerequisite FG6-1a. SSB location should be within a bandwidth of at least one DL BWP, same as for Option B-1-1. The only differences between Option B-1-2 and Option B-1-1 are (1) interruption related aspects and (2) CSI-RS based and NCD-SSB based BM/RLM/BFD have to be enabled. In our understanding, these basically do not appear in the RAN1 specifications. (1) should be up to RAN4. (2) can be clarified as part of UE feature description of Option B-1-2.

Proposal 3-3:

- Option B-1-2 does not prerequisite support of FG6-1a ‘bwp-WithoutRestriction’

- For Option B-1-2, clarify that the SSB outside the active DL BWP is still within a bandwidth of at least one DL BWP of the carrier.

  - If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP

  - If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs

- Following could be the FG for Option B-1-2 as proposed in [2]

  - Other details including potential other UE capabilities for interruptions are up to RAN4

| Capability-bwp-wor-b-1-2 Indicates support of BM/RLM/BFD based on SSB outside the active DL BWP with interruptions. If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP. If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs. The UE shall be allowed to use SSB outside the active DL BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s). The UE supporting this feature shall indicate support of Capability-bwo-wor-c (subject to IoDT availability). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

There is no additional RAN1 specification impact for Option B-1-2 compared to those for Option B-1-1.

Observation 3-2:

- RAN1 spec impact for Option B-1-2 is the same as for Option B-1-1

## 3.3	Option C

In Option C, a UE uses NCD-SSB within active DL BWP for BM/RLM/BFD in the same way as for RedCap UE. However, for Option C for non-RedCap UEs, there could be following cases:

- For a UE supporting Option C only, the active DL BWP contains both CD-SSB and NCD-SSB

- For a UE supporting both Option C and Option B-1-1/B-1-2, the active DL BWP contains NCD-SSB while does not contain CD-SSB which is located outside in the carrier.

For these cases, it is beneficial to make sure that the UE meets the requirements based on NCD-SSB as long as it is configured. The spec does not need to mandate UE to use the NCD-SSB. This can be clarified in either RAN1/RAN4 spec, or the corresponding UE feature descrition. Below, we take the option that clarifies this in the UE feature. If we take the other option (i.e., clarifying in the RAN1/RAN4 spec), the detailed discussion is necessary.

Proposal 3-4:

- Following could be the FG for Option B-1-2 as proposed in [2]

| Capability-bwp-wor-c Indicates support of BM/RLM/BFD based on NCD-SSB in the active DL BWP that is configured by [NonCellDefiningSSB] under BWP-DownlinkDedicated. If the UE is configured with NCD-SSB by [NonCellDefiningSSB] within the active DL BWP, the UE meets the requirements based on the NCD-SSB for BM/RLM/BFD. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3-5:

- For Option C, consider following as the starting point of TS38.213 updates

# 12 Bandwidth part operation […] If a UE is configured to operate with receiving an SS/PBCH block from the SS/PBCH blocks provided by [NonCellDefiningSSB] in BWP-DownlinkDedicated for an active DL BWP, the UE assumes that the active DL BWP includes the SS/PBCH blocks provided by [NonCellDefiningSSB]. The SS/PBCH blocks provided by [NonCellDefiningSSB-r18] and the SS/PBCH blocks that the UE used to obtain SIB1 have the same QCL properties, if they have the same index. For a UE indicated presence of SS/PBCH blocks within an active DL BWP by [NonCellDefiningSSB], collision handling between downlink receptions or uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. For monitoring of a PDCCH candidate by a UE configured with [NonCellDefiningSSB], if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by [NonCellDefiningSSB], the UE is not required to monitor the PDCCH candidate.

# R18 eDSS enhancements

## 4.1	NR PDCCH reception that overlaps with LTE CRS

RAN1 made following agreements:

Agreement If the feature “Reception of NR PDCCH candidates that overlap with LTE CRS REs “is supported, it is RAN1 understanding that the feature is supported by UE performing channel estimation with a regular legacy DMRS pattern in frequency dimension, i.e., no change to UE assumption on PDCCH DMRS RE positions/pattern in a symbol that are used for the purpose of channel estimation. Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET) Note: Restriction on the symbols and/or LTE CRS patterns applicable for above agreements can be considered during UE capability session. Conclusion RAN1 understands above agreements applies to 15kHz SCS only.

Since the legacy UEs that do not support NR-PDCCH reception overlapping LTE CRS REs, network should not configure Type-0/0A/2 CSS sets and Type-1 CSS set without dedicated configuration such that NR-PDCCH overlaps with LTE CRS REs. Therefore, we propose to limit the feature for USS set and Type-3 CSS set with dedicated configuration.

Proposal 4-1:

- NR PDCCH reception that overlaps with LTE CRS REs is limited to Type-1 CSS set with dedicated RRC configuration, Type-3 CSS set, and USS set

Regarding the restrictions on the symbols and/or LTE CRS patterns applicable for the agreements, we have the following views.

- For NR-PDSCH rate-matching around LTE-CRS REs, LTE-CRS pattern(s) or LTE-CRS pattern lists(s) can be configured using various parameters. For NR-PDCCH reception overlapping with LTE CRS REs, we assume no new parameter for indicating LTE-CRS pattern(s)/pattern list(s) is introduced, and the existing LTE-CRS pattern(s)/pattern list(s) parameters are re-used. The UE should be able to report which parameter(s) the UE can take into account for NR-PDCCH reception.

- In time-domain, NR-PDCCH reception overlapping with LTE-CRS REs should be limited to the first 3 (or 4) OFDM symbols. If a UE supports NR-PDCCH reception anywhere in a slot (e.g., FG3-2), the benefit of supporting NR-PDCCH reception overlapping with LTE CRS REs must be quite limited (or even be zero). The restriction can be part of UE features same as for FG3-1 and FG22-12.

Proposal 4-2:

- UE should be able to indicate support for one or multiple of the following:

  - R15 parameter for single LTE-CRS pattern (lte-CRS-ToMatchAround)

  - R16 parameter for single LTE-CRS pattern list (lte-CRS-PatternList1-r16)

  - R16 parameters for two overlapping LTE-CRS pattern lists for m-DCI m-TRP (lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16) with two different values of coresetPoolIndex

  - R18 parameters for two overlapping LTE-CRS pattern lists (lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) without two different values of coresetPoolIndex

  - R18 parameters for two overlapping LTE-CRS pattern lists for m-DCI m-TRP (lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) with two different values of coresetPoolIndex

- Separate UE feature groups for NR PDCCH reception that overlaps with LTE CRS REs for:

  - The first 3 OFDM symbols in a slot

  - Within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot (prerequisite: FG22-12)

A UE may support m-DCI m-TRP and maybe configured with CORESETs with different values of coresetPoolIndex. For PDSCH rate-matching, in this case the UE uses an LTE-CRS pattern list associated with the CORESET with a value of coresetPoolIndex. In other words, LTE CRS rate-matching pattern list is dynamically selected for PDSCH rate-matching. For NR-PDCCH reception with LTE CRS REs, the same rule can apply. More specifically, if a UE is configured with CORESETs with different values of coresetPoolIndex, for NR-PDCCH reception with a CORESET with one value of coresetPoolIndex, the UE assumes LTE-CRS pattern list associated with the coresetPoolIndex for NR-PDCCH reception. This should be clarified for the case where a UE is configured with CORESETs with different values of coresetPoolIndex, and LTE-CRS-PatternList1-r16 and LTE-CRS-PatternList2-r16, or LTE-CRS-PatternList3-r18 and LTE-CRS-PatternList4-r18.

Proposal 4-3:

- Clarify following:

  - If a UE is not configured with two different values of coresetPoolIndex for CORESETs, the UE takes into account all the LTE-CRS patterns/pattern lists configured by RRC

  - If a UE is configured with two different values of coresetPoolIndex for CORESETs, the UE takes into account LTE-CRS patterns/pattern lists configured by lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 for NR PDCCH reception in CORESET with coresetPoolIndex =0 and by lte-CRS-PatternList2-r16 or lte-CRS-PatternList4-r18 for NR PDCCH reception in CORESET with coresetPoolIndex = 1

The proposals 4-1, 4-2, and 4-3 can be addressed in either RAN1 spec or in UE feature descriptions. In [3], we propose to clarify them in the corresponding UE features. We are open to capture them in the RAN1 spec if it is a majority view.

# Conclusion

In this contribution, we provide our views on multi-carrier enhancements, BWP without restriction, and eDSS.

For multi-cell scheduling of multi-carrier enhancements, we have following proposals.

Proposal 2-1-1:

- Select either of the following options:

  - Opt.1: BWP indication field in DCI format 0_X/1_X has log2(nBWP) bits where nBWP=nBWP_RRC and the indicator is defined in Table 7.3.1.1.2-1

  - Opt.2: The number of configured BWPs for all the cells in a cell set of for multi-cell scheduling is the same

Proposal 2-1-2:

- For Antenna port(s) field in DCI format 1_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PDSCH receptions scheduled by the DCI format 1_X for any cell in the set for the DCI format 1_X are configured such that single Table from Tables 7.3.1.2.2-1, 7.3.1.2.2-2, 7.3.1.2.2-3, and 7.3.1.2.2-4 in TS38.212 is used.

- For Antenna port(s) field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-6, 7.3.1.1.2-6A, 7.3.1.1.2-7, 7.3.1.1.2-7A, 7.3.1.1.2-8, 7.3.1.1.2-9, 7.3.1.1.2-10, 7.3.1.1.2-11, 7.3.1.1.2-12, 7.3.1.1.2-13, 7.3.1.1.2-14, 7.3.1.1.2-14, 7.3.1.1.2-15, 7.3.1.1.2-16, 7.3.1.1.2-17, 7.3.1.1.2-18, 7.3.1.1.2-19, 7.3.1.1.2-20, 7.3.1.1.2-21, 7.3.1.1.2-22, 7.3.1.1.2-23, 7.3.1.1.2-24, and 7.3.1.1.2-25 in TS38.212 is used.

- For TPMI field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-2, 7.3.1.1.2-2A, 7.3.1.1.2-B, 7.3.1.1.2-3, 7.3.1.1.2-3A, 7.3.1.1.2-4, 7.3.1.1.2-4A, 7.3.1.1.2-5, and 7.3.1.1.2-5A in TS38.212 is used.

- For SRI field in DCI format 0_X, when the field is configured as Type-1A, the UE expects that RRC parameters for PUSCH transmissions scheduled by the DCI format 0_X for any cell in the set for the DCI format 0_X are configured such that single Table from Tables 7.3.1.1.2-28, 7.3.1.1.2-29, 7.3.1.1.2-30, 7.3.1.1.2-31, 7.3.1.1.2-32, 7.3.1.1.2-32A, and 7.3.1.1.2-32B in TS38.212 is used.

Proposal 2-1-3:

- PDCCH overbooking is not enabled for DCI format 0_X/1_X

Proposal 2-1-4:

- Introduce separate capabilities for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling

- Capabilities for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling do not prerequisite UE capabilities for R15/16 cross-carrier scheduling (FG6-10, 18-5, 18-5b)

- For a given DL-CA or UL-CA band combination, the UE reports support for:

  - Opt. 1: one or multiple combinations of {a band for scheduling cell, a set of band(s) for scheduled cells} for multi-cell scheduling by a single DCI format

  - Opt. 2: one or multiple combinations of {a carrier type for scheduling cell, a carrier type for scheduled cells} for multi-cell scheduling by a single DCI format

    - Carrier type: one from {FDD, TDD, Unlicensed, FR2}

  - Opt.3: one or more combination(s) of {a SCS for scheduling cell, a SCS for scheduled cells} for multi-cell scheduling by a single DCI format

For UL Tx switching for 3 or 4 bands of multi-carrier enhancements, we have following proposals.

Proposal 2-2-1:

The switch-from or switch-to group with lowest priority band of the bands involved into the switch should be configured with the switching period.

Proposal 2-2-2:

If the same lower priority band transmits before and after switch and more than two bands are involved into the switch, remove the lower priority band from switch-to and switch-from band groups. Compare the rest band(s) of the switch-to and switch-from band groups, the group with lower priority band is preferable to take the switching period location. If three bands are involved into the switch and only one the switch-to and switch-from band groups has a band left after removing the lower priority band, the band group with the two bands doesn’t take the switching period location.

Proposal 2-2-3:

If the highest priority band transmits before and after switch. i.e. A+B->A+C,

- for the “unchanged UE”, if the switching gap is smaller than the required switching period, the lower priority band (band C) should be configured with switching period and dropped;

- for the “Baseline UE”, network should guarantee sufficient switch gap if the highest priority band transmits before and after switch.

Proposal 2-2-4:

If one of the bands is configured with two contiguous carriers, both carriers should be configured with same priority.

Proposal 2-2-5:

When the switch-to bands are two and different with the switch-from band(s) (i.e. band A+B -> C+D), after switch start of band C transmission is no later than band D, 1 Tx chain switches to band C and another Tx chain is on one of {band A, B, or C} according configuration uplinkTxSwitching-DualUL-TxState. Once the following condition is met, UE is with one switch.

- If the two bands after switch start transmission at the same time, UE jointly creates the switching gap before the start of the transmission. The UL grants on both bands should be no later than T0-Toffset where Toffset is derived from switching gap to be summation of {max(Tswitch_A_C, Tswitch_B_C), max(Tswitch_A_D, Tswitch_B_D)} where Tswitch_X_Y is the UE reported switching period for band pair X, Y.

- If start of band C transmission is earlier than band D, 1 Tx chain switches to band C and another Tx chain is on one of {band A, B, or C}, the “baseline” UE is with one switch if following condition is met.

  - The switch to band D is completed before T0, which is the start of transmission on band C.

  - UE looks ahead the UL grants of band D transmission, the start of transmission on band D should be within a look ahead window from T0. The look-ahead window is [X slot]. i.e. X = 1 reference slot.

  - The UL grant of band D arrives before T0 – Toffset, Toffset is the PUSCH preparation time based on Tswitch of Summation of {{max(Tswitch_A_C, Tswitch_B_C), Tswitch_2BAND_D}

    - Tswitch_X_Y is the UE reported switching period for band pair X, Y.

    - Tswitch_2BAND_D is the switching period from the band with the 2nd Tx chain to band D.

Proposal 2-2-6:

Endorse following for minimum separation time:

- For reporting granularity of UE capability on the X us, UE capability on the X us is reported per BC.

- For more than one TAG case, X us is subject to UE capability with a value set of {0us, 500us} as well as for one TAG case.

For BWP without restriction, we have following proposals.

Proposal 3-1:

- Option B-1-1 does not prerequisite support of FG6-1a ‘bwp-WithoutRestriction’

- For Option B-1-1, clarify that the SSB outside the active DL BWP is still within a bandwidth of at least one DL BWP of the carrier.

  - If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP

  - If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs

- Following could be the FG for Option B-1-1 as proposed in [2]

| Capability-bwp-wor-b-1-1 Indicates support of BM/RLM/BFD based on SSB outside the active DL BWP without interruptions. If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP. If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3-2:

- For Option B-1-1, consider following as the starting point of TS38.213 updates

# 5 Radio link monitoring The downlink radio link quality of the primary cell is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. Except for SSB based radio link monitoring for a UE supporting [Capability-bwp-wor-b-1-1 or Capability-bwp-wor-b-1-2], tThe UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP, as described in clause 12, on the primary cell. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3, as described in clause 13, the UE is expected to perform RLM using the associated SS/PBCH block when the associated SS/PBCH block index is provided by RadioLinkMonitoringRS. If the UE is configured with a SCG, as described in [12, TS 38.331], and the parameter rlf-TimersAndConstants is provided by higher layers and is not set to release, the downlink radio link quality of the PSCell of the SCG is monitored by the UE for the purpose of indicating out-of-sync/in-sync status to higher layers. Except for SSB based radio link monitoring for a UE supporting [Capability-bwp-wor-b-1-1 or Capability-bwp-wor-b-1-2], tThe UE is not required to monitor the downlink radio link quality in DL BWPs other than the active DL BWP on the PSCell.

Proposal 3-3:

- Option B-1-2 does not prerequisite support of FG6-1a ‘bwp-WithoutRestriction’

- For Option B-1-2, clarify that the SSB outside the active DL BWP is still within a bandwidth of at least one DL BWP of the carrier.

  - If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP

  - If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs

- Following could be the FG for Option B-1-2 as proposed in [2]

  - Other details including potential other UE capabilities for interruptions are up to RAN4

| Capability-bwp-wor-b-1-2 Indicates support of BM/RLM/BFD based on SSB outside the active DL BWP with interruptions. If the UE has one UE-specific DL BWP configuration, the SSB is within a bandwidth of either initial DL BWP or UE-specific DL BWP. If the UE has more than one UE-specific DL BWP configurations, the SSB is within a bandwidth of either of the UE-specific DL BWPs. The UE shall be allowed to use SSB outside the active DL BWP with interruptions only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s). The UE supporting this feature shall indicate support of Capability-bwo-wor-c (subject to IoDT availability). | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3-4:

- Following could be the FG for Option B-1-2 as proposed in [2]

| Capability-bwp-wor-c Indicates support of BM/RLM/BFD based on NCD-SSB in the active DL BWP that is configured by [NonCellDefiningSSB] under BWP-DownlinkDedicated. If the UE is configured with NCD-SSB by [NonCellDefiningSSB] within the active DL BWP, the UE meets the requirements based on the NCD-SSB for BM/RLM/BFD. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 3-5:

- For Option C, consider following as the starting point of TS38.213 updates

# 12 Bandwidth part operation […] If a UE is configured to operate with receiving an SS/PBCH block from the SS/PBCH blocks provided by [NonCellDefiningSSB] in BWP-DownlinkDedicated for an active DL BWP, the UE assumes that the active DL BWP includes the SS/PBCH blocks provided by [NonCellDefiningSSB]. The SS/PBCH blocks provided by [NonCellDefiningSSB-r18] and the SS/PBCH blocks that the UE used to obtain SIB1 have the same QCL properties, if they have the same index. For a UE indicated presence of SS/PBCH blocks within an active DL BWP by [NonCellDefiningSSB], collision handling between downlink receptions or uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. For monitoring of a PDCCH candidate by a UE configured with [NonCellDefiningSSB], if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by [NonCellDefiningSSB], the UE is not required to monitor the PDCCH candidate.

For eDSS, we have following proposals.

Proposal 4-1:

- NR PDCCH reception that overlaps with LTE CRS REs is limited to Type-1 CSS set with dedicated RRC configuration, Type-3 CSS set, and USS set

Proposal 4-2:

- UE should be able to indicate support for one or multiple of the following:

  - R15 parameter for single LTE-CRS pattern (lte-CRS-ToMatchAround)

  - R16 parameter for single LTE-CRS pattern list (lte-CRS-PatternList1-r16)

  - R16 parameters for two overlapping LTE-CRS pattern lists for m-DCI m-TRP (lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16) with two different values of coresetPoolIndex

  - R18 parameters for two overlapping LTE-CRS pattern lists (lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) without two different values of coresetPoolIndex

  - R18 parameters for two overlapping LTE-CRS pattern lists for m-DCI m-TRP (lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18) with two different values of coresetPoolIndex

- Separate UE feature groups for NR PDCCH reception that overlaps with LTE CRS REs for:

  - The first 3 OFDM symbols in a slot

  - Within a single span of 3 consecutive OFDM symbols that is within the first 4 OFDM symbols in a slot (prerequisite: FG22-12)

# Reference

- R1-2303621, ‘UE features for MC enhancements,’ Qualcomm Incorporated

- R1-2303623, ‘UE features for BWP without restriction,’ Qualcomm Incorporated

- R1-2303622, ‘UE features for eDSS,’ Qualcomm Incorporated

- R4-2220546	, WF on UE Tx switching requirement for single TAG

- R1-2303562, Draft Reply to LS on RAN4 Rel-18 Multi-Carrier enhancement for NR, Qualcomm Incorporated