3GPP TSG RAN WG1 #112bis-e	R1-2303626

e-meeting, April 17th – 26th, 2023

Source:	Panasonic

Title: 	Discussion on simulation assumptions for self-evaluation of IMT-2020 satellite radio interface technology

Agenda Item:		9.15.1

Document for:	Discussion

Introduction

Study item on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology was agreed in RAN#99 [1]. The aim of SI is to provide the description of the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514. In this document, our view on the evaluation methodology and simulation conditions are provided.

Evaluation metric and requirement

The following are RAN1 related performance metric to be evaluated.

  - Peak data rate

  - Peak spectral efficiency

  - User experienced data rate

  - 5th percentile user spectral efficiency

  - Average spectral efficiency

  - Area traffic capacity

The definition of these metric and the minimum requirements are described in section 7 of ITU-R M.2514 and summarized below.

Peak data rate

Peak data rate is defined for a single mobile station. In a single band, it is related to the peak spectral efficiency in that band. Let Wa denote the assigned bandwidth and SEp denote the peak spectral efficiency in that band achieved at the assigned bandwidth. Then the user peak data rate Rp is given by:

Rp=Wa×SEp

The following Table indicates the required peak data rate.

| Peak data rate (DL) | 70 Mbit/s (1) |
| --- | --- |
| Peak data rate (UL) | 2 Mbit/s (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

Peak spectral efficiency

Peak spectral efficiency is the maximum data rate under ideal conditions normalized by the assigned bandwidth (in bit/s/Hz), where the maximum data rate is the received data bits assignable to a single mobile station, when up to all assignable radio resources for the corresponding link direction are utilized (i.e. excluding radio resources that are used for physical layer synchronization, reference signals or pilots and guard bands).

The required minimum peak spectral efficiencies are as follows:

| Peak spectral efficiency (DL) | 3 bit/s/Hz (1) |
| --- | --- |
| Peak spectral efficiency (UL) | 1.5 bit/s/Hz (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

User experienced data rate

User experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time.

Assuming one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate should be derived from the 5th percentile user spectral efficiency through equation (1). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser=W×SEuser

The requirements below are defined for the purpose of evaluation in the Rural-eMBB-s test environment applicable to handheld devices.

| User experienced data rate (DL) | 1 Mbit/s |
| --- | --- |
| User experienced data rate (UL) | 100 kbit/s |

5th percentile user spectral efficiency

The 5th percentile user spectral efficiency is the 5% point of the CDF of the normalized user throughput. The normalized user throughput is defined as the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time, divided by the channel bandwidth, and is measured in bit/s/Hz.

The requirements below are defined for the purpose of the evaluation in the Rural-eMBB-s test environment, applicable to handheld devices:

| 5th percentile user spectral efficiency (DL) | 0.03 bit/s/Hz |
| --- | --- |
| 5th percentile user spectral efficiency (UL) | 0.003 bit/s/Hz |

Average spectral efficiency

Average spectral efficiency is the aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time) divided by the channel bandwidth of a specific band divided by the number of TRxPs and is measured in bit/s/Hz/TRxP.

In the context of a satellite component radio interface, a TRxP (transmission and reception point) refers to a beam generated by the satellite.

The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor.

The following requirements are defined for the purpose of the evaluation in the Rural-eMBB-s test environment, applicable to handheld devices:

| Average spectral efficiency (DL) | 0.5 bit/s/Hz |
| --- | --- |
| Average spectral efficiency (UL) | 0.1 bit/s/Hz |

Area traffic capacity

Area traffic capacity is the total traffic throughput served per geographic area (in Mbit/s/km2). The throughput is the number of correctly received bits, i.e. the number of bits contained in the SDUs delivered to Layer 3, over a certain period of time.

This can be derived assuming one frequency band and one TRxP layer, based on the achievable average spectral efficiency, network deployment (e.g. TRxP (site) density) and bandwidth.

Let W denote the channel bandwidth and ρ the TRxP density (TRxP/m2). The area traffic capacity Carea is related to average spectral efficiency SEavg as follows:

Carea=ρ×W×SEavg

The requirements below are defined for the purpose of the evaluation in the Rural-eMBB-s test environment, applicable to handheld devices:

| Area traffic capacity (DL) | 8 kbit/s/km2 |
| --- | --- |
| Area traffic capacity (UL) | 1.5 kbit/s/km2 |

Simulation conditions

The following is described in section 7.2 of ITU-R M.2514.

In order to evaluate whether a proposed satellite radio interface technology meets the technical performance requirements for the Satellite Component of IMT-2020, demonstration of compliance assuming handheld terminals is necessary and sufficient. Other evaluations, e.g. for directional and MTD devices, may be provided by the proponent, though are not required.

Therefore, evaluation with handheld terminal should be prioritized. To meet the performance requirement, frequency band 2GHz and LEO 600km should be assumed for the evaluation.

Proposal 1: Evaluation with handheld terminal should be prioritized.

Proposal 2: Frequency band 2GHz and LEO 600km should be assumed for the evaluation.

Satellite configuration

In 3GPP TR38.821, satellite parameters used for simulation in the NTN study item are described. Satellite parameter set 1 is used for calibration among companies as higher priority than parameter set 2. Satellite parameter set 1 in Table 6.1.1.1-1 of TR38.821 should be used for the evaluation of IMT-2020 satellite radio interface.

Satellite parameter set 1 for LEO 600km in TR38.821 is copied below.

Table 1 Satellite characteristics (parameter set 1 of LEO600km, S-band)

| Parameters | Value |
| --- | --- |
| Satellite altitude | 600km |
| Satellite antenna pattern | Section 6.4 in TR38.811 [4] |
| Parameters for DL transmission |  |
| Equivalent satellite antenna aperture | 2 m |
| Satellite EIRP density | 34 dBW/MHz |
| Satellite Tx max Gain | 30 dBi |
| 3dB beamwidth | 4.4127 deg |
| Satellite beam diameter (Note 2) | 50 km |
| Parameters for UL reception |  |
| Equivalent satellite antenna aperture | 2 m |
| G/T | 1.1 dB K-1 |
| Satellite Rx max Gain | 30 dBi |

Proposal 3: Satellite parameter set 1 in Table 6.1.1.1-1 of TR38.821 should be used for the evaluation.

Terminal configuration

Characteristics of handheld terminal is described in TR38.821.

Table 2 Terminal characteristics (handheld terminal)

| Parameters | Value |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

Proposal 4: handheld terminal characteristics defined in Table 6.1.1.1-3 of TR38.821 should be used for the evaluation.

Channel model

It is mentioned in ITU-R M.2514 that the target scenario should be rural because NTN is complementary use of terrestrial network. Therefore, rural model defined in TR38.811 should be used.

System level simulation setup

System level simulation assumptions should be aligned with the one described in TR38.821. The following is proposed system level simulation assumptions based on TR38.821. Assumptions related to handheld terminal with S-band are picked up from TR38.821. Modified part is described in red character.

Table 3 system level simulation assumption

| Parameter | Value |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) |
| Maximum Bandwidth per beam (DL + UL) | S-band: DL 30 MHz and UL 30 MHz The bandwidth per beam must be adapted based on the frequency factor and the polarization re-use option considered. |
| Satellite polarization configuration | Circular |
| Frequency re-use factor | FRF=1 or 3 |
| Polarization re-use | Disable |
| Channel model | Large scale model of TR38.811[2] (Note 2) |
| Deployment scenarios | Rural |
| Propagation conditions | Clear Sky and Line of Sight |
| Beam layout | For FRF = 1, two additional tiers of beams are considered in the simulation surrounding the 19-beam layout. For FRF > 1, four additional tiers of beams are considered in the simulation surrounding the 19-beam layout. |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | 10 UEs per beam with uniform distribution in all the Voronoi cell area associated to each beam. The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE orientation | Random |
| UE antenna height | 1.5m |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Scheduler | To be reported by the companies |
| Traffic model | FTP 3 (see TR 38.802) packet size = 0.5 Mbyte Full buffer |
| Number of HARQ processes | Up to 32 (as in Rel.17) |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

Regarding the traffic model, although non full buffer model (FTP 3) is described in TR38.821, full buffer model should also be considered in order to satisfy the average spectral efficiency requirement which is defined as aggregate throughput of all users.

Proposal 5: System level simulation assumptions defined in TR38.821 should be used for the evaluation. Full buffer traffic should be considered in order to satisfy the average spectral efficiency requirement.

Conclusion

This contribution provides our view on the simulation assumptions for self-evaluation of IMT-2020 satellite radio interface technology.

Proposal 1: Evaluation with handheld terminal should be prioritized.

Proposal 2: Frequency band 2GHz and LEO 600km should be assumed for the evaluation.

Proposal 3: Satellite parameter set 1 in Table 6.1.1.1-1 of TR38.821 should be used for the evaluation.

Proposal 4: handheld terminal characteristics defined in Table 6.1.1.1-3 of TR38.821 should be used for the evaluation.

Proposal 5: System level simulation assumptions defined in TR38.821 should be used for the evaluation. Full buffer traffic should be considered in order to satisfy the average spectral efficiency requirement.

- Reference

- RP-230736, “New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology”, Ericsson, Qualcomm, Thales, MediaTek

- ITU-R M.2514

- TR38.821 V16.0.0

- TR38.811 V15.4.0