3GPP TSG RAN WG1 Meeting #112bis-e                                                            	     R1-2303627

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.9.3

Source:	Lenovo

Title:	Disabling of HARQ feedback for IoT NTN

Document for:	Discussion and decision

- Introduction

In the RAN#94 plenary meeting, an enhancement work item for Rel.18 IoT NTN was approved. One of the objectives is to specify the following HARQ enhancements to IoT NTN.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

The following agreements on disabling of HARQ feedback for IoT NTN were achieved:

Conclusion

For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling.

Agreement

For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process.

Agreement

For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation

- for the first SPS PDSCH after activation,

  - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise.

Conclusion

For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration.

Agreement

For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Agreement

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

In this contribution, detailed considerations of potential enhancement for IoT NTN, especially HARQ disabling enhancement are presented.

- Discussion

- Indication/configuration of disabling HARQ feedback in IoT NTN

For the indication and configuration of disabling HARQ feedback for IoT NTN, NR NTN configuration is assumed to be the baseline solution, and DCI based solution can also be supported for different scenarios. In previous RAN1 meeting, the working assumption with update have been agreed.

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Regarding the DCI-based overridden mechanism applied to both semi-statically HARQ enabled and disabled processes or only applied to semi-statically HARQ disabled processes or only applied to semi-statically HARQ enabled processes, it is better to leave more flexibility for gNB to update the HARQ enabling/disabling configuration for corresponding transmission for both types HARQ enabled and disabled processes if additional signaling enabled. Furthermore, if DCI-based overridden mechanism applied to both semi-statically HARQ enabled and disabled processes, it is easier to align the DCI based solution for override mechanism and DCI based solution for explicit HARQ enabling/disabling indication if RRC based HARQ enabling/disabling is not configured. Here overridden indication is to reverse the HARQ feedback enabling/disabling for the corresponding transmission from per-HARQ process RRC configuration, otherwise if overridden indication is DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration, it is equivalent that per-HARQ process bitmap signaling and DCI solution enabling/disabling signaling  can’t be configured simultaneously, which is not aligned with the original design for the overridden mechanism.

Regarding the DCI-based solution to override RRC based configuration for corresponding transmission for multiple TBs scheduled by single DCI, considering the DCI overhead for overridden indication, it is preferred that the overridden indication (e.g., 1-bit indication) in DCI is adopted to all scheduled HARQ processes (Note in case of mixed HARQ enabling/disabling scheduling by single DCI, it is preferred that the overridden indication in DCI is only adopted to all scheduled [enabled/disabled] HARQ processes, which avoid making the HARQ enabling/disabling configuration complicated). Furthermore, for the DCI-based solution to direct indication of HARQ enabling/disabling if RRC based default HARQ enabling/disabling is not configured, the indication of HARQ enabling/disabling (e.g., 1-bit indication) is also adopted to all scheduled HARQ processes.

Proposal 1: DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes for single TB scheduled by DCI.

Proposal 2: Indication to override RRC based configuration for corresponding transmission for multiple TBs scheduled by single DCI is adopted to all scheduled HARQ processes (Note in case of mixed HARQ enabling/disabling scheduling by single DCI, it is preferred that the overridden indication in DCI is only adopted to all scheduled [enabled/disabled] HARQ processes).

Proposal 3: Direct indication of HARQ enabling/disabling for multiple TBs scheduled by single DCI is adopted to all scheduled HARQ processes.

Regarding the Option 3 DCI based indication, the following potential indication methods were agreed for candidate.

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

In order to save the DCI size and align the design of RRC configuration with additional DCI overridden/indication and without additional DCI overridden/indication, Alt 2 that reusing/reinterpreting existing field in DCI is a good choice, e.g., reusing one state of HARQ ACK/NACK resource field (one of the state of the 2-bit HARQ-ACK resource offset field (for eMTC) or 4-bit HARQ-ACK resource field (for NB-IoT)) or reinterpreting other spare states of field (e.g., state “1110” of MCS field for NBIoT and any state “1010-1111” of MCS field for eMTC CEMode B) to indicate HARQ enabling/disabling or override the HARQ enabling/disabling configuration for corresponding transmission.

Proposal 4: For DCI based indication, reusing/reinterpreting existing fields in DCI can be supported.

- Multiple TB scheduling

Multiple TB scheduling is introduced in Rel.16 eMTC and NBIoT for peak data rate increase. With single DCI, multiple TBs are scheduled with interleaved or non-interleaved manner and corresponding feedback for each TB is performed after the data transmission with HARQ bundling or non-bundling manner.

For NR NTN HARQ disabling, two types of HARQ codebook are enhanced as:

- For Type-1 HARQ codebook in NR NTN, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

- For Type-2 HARQ codebook in NTN:

- Reduce codebook size with HARQ-ACK codebook only including HARQ-ACK of PDSCH with feedback-enabled HARQ processes

- For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes

Similar as enhancement of NR NTN HARQ codebook Type-1, ACK is assumed for a feedback-disabled HARQ process in the HARQ feedback for scheduling multiple TB scenario, which will introduce much unnecessary power consumption.

While similar as enhancement of NR NTN HARQ codebook Type-2, UE only reports the HARQ information for the HARQ enabled process, corresponding UE behavior and timing relationship as shown in Figure 1. That is, for a HARQ feedback transmission in PUCCH, the target transmission starting subframe of the PUCCH is 4ms after the reception of PDSCH and right after end of HARQ feedback transmission of the previous TB)

- For HARQ without bundling (e.g., NBIoT and eMTC), UE only reports the HARQ information for the HARQ enabled process, corresponding UE behavior and timing relationship follow legacy behavior.

- For HARQ with bundling (e.g., only eMTC), UE only reports the HARQ information for HARQ enabled process, HARQ-ACK for HARQ disabled processes is excluded in the HARQ bundle.

Figure 1 HARQ disabling in multiple TB scheduling

Proposal 5: For multiple TBs scheduled by single DCI with/without HARQ bundling, HARQ feedback is reported only for downlink transmission with HARQ process enabled (e.g., HARQ feedback is not reported for downlink transmission with HARQ process disabled).

- Conclusions

In this contribution, considerations of HARQ enhancement for IoT NTN are provided. The following proposals are present:

Proposal 1: DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes for single TB scheduled by DCI.

Proposal 2: Indication to override RRC based configuration for corresponding transmission for multiple TBs scheduled by single DCI is adopted to all scheduled HARQ processes (Note in case of mixed HARQ enabling/disabling scheduling by single DCI, it is preferred that the overridden indication in DCI is only adopted to all scheduled [enabled/disabled] HARQ processes).

Proposal 3: Direct indication of HARQ enabling/disabling for multiple TBs scheduled by single DCI is adopted to all scheduled HARQ processes.

Proposal 4: For DCI based indication, reusing/reinterpreting existing fields in DCI can be supported.

Proposal 5: For multiple TBs scheduled by single DCI with/without HARQ bundling, HARQ feedback is reported only for downlink transmission with HARQ process enabled (e.g., HARQ feedback is not reported for downlink transmission with HARQ process disabled).

- References

3GPP TR 36.763 V1.0.0 (2021-06)

RP-213596, “New WID on IoT NTN enhancements”, MediaTek Inc, RAN#94e

R1-2301804, “FLS#2 on disabling of HARQ feedback for IoT NTN”, Moderator (Lenovo)