3GPP TSG RAN WG1 Meeting #112bis-e                                                            	     R1-2303628

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.9.4

Source:	Lenovo

Title:	Improved GNSS operations for IoT NTN

Document for:	Discussion and decision

Introduction

In the RAN#94 plenary meeting, an enhancement work item for Rel.18 IoT NTN was approved. One of the objectives is to specify the following GNSS operations enhancements to IoT NTN.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption [RAN1]

The following agreements on improved GNSS operations for IoT NTN were achieved in RAN1-111 meeting:

Agreement

The following alternatives can be considered to inform eNB the success of GNSS measurement at UE side after GNSS measurement in RRC connected.

- Alt-1: The UE will report the new GNSS validity duration

- Alt-2: The reception of any UL transmission from the UE at eNB after the GNSS measurement

Agreement

On the length of GNSS measurement gap, which is aperiodically triggered by eNB, the gap duration should be equal to or larger than the latest UE reported GNSS position fix time duration.

FFS: whether the gap duration is configured by eNB, or the gap duration is equal to the latest reported GNSS position fix time duration.

Agreement

On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, RAN1 can down select one of the following alternatives:

- Alt 1: the start time should be at n+ X, where n is the end of MAC CE receiving subframe/slot

  - FFS: details of X, e.g. predefined value or configured value

- Alt 2: the start time should be based on the current GNSS validity duration with delay or without delay

Agreement

UE reports only one GNSS position fix time duration for GNSS measurement at least when moving to RRC connected state.

Agreement

At least for the case when frequency error is within frequency error requirements, study the mechanisms and conditions to allow UL transmission after original GNSS validity duration expires without GNSS re-acquisition for some duration.

- FFS: with legacy closed loop time correction or enhanced closed loop time correction

- This mechanism is enabled/configured by eNB

- FFS: whether such mechanism will be specified depends on the outcome of this study

In this contribution, detailed considerations of potential enhancement for IoT NTN, especially GNSS operation improvements are presented.

Discussion

Closed loop control of UL synchronization

Closed-loop time correction has been supported in IoT NTN after initial access procedure, which can improve power consumption with reduced use of GNSS module to re-acquire GNSS position fix, while closed-loop frequency correction hasn’t been specified and needs further study as discussed in previous meetings.

However, whether to support closed loop uplink synchronization, especially closed loop frequency correction needs further study. Firstly, UE timing and frequency adjustment via UE closed-loop corrections may be only applicable for stationary UEs or UEs with low speed not for high-velocity UEs, since the closed loop correction may not be able to track the variation. Secondly, for NBIoT and eMTC, PUSCH transmission is always configured with large repetition, UE may not have the opportunity to monitor the PDCCH, the closed loop frequency correction in MAC CE is not available and UE still may not have the opportunity to perform time/frequency synchronization and corresponding adjustment during the uplink transmission except the configured gap duration. Thirdly, if closed loop frequency correction is supported, it may involve some evaluations and requirements for RAN4 (e.g., frequency correction granularity, frequency correction range, accuracy requirement), the workload of WI may be reevaluated. Considering the workload and standard effort, it is better not to support of closed loop time and frequency correction enhancement during long connection.

Proposal 1：Considering the workload and standard effort, it is better not to support of closed loop time and frequency correction enhancement during long connection.

Schemes for GNSS measurement in connected

For Rel.18 IoT NTN, GNSS operation should be improved for long connection and high-velocity UEs. It was agreed that UE re-acquires GNSS position fix with a new gap and eNB at least aperiodically triggers UE to make GNSS measurement.

Regarding the triggering the GNSS measurement by network or by UE, since eNB has knowledge of the validity duration of UE’s GNSS information, triggering the GNSS measurement according to the validity duration can be controlled by network. Regarding the triggering the GNSS measurement for aperiodic and periodic way, both ways can be supported for different scenarios.

For aperiodic triggering GNSS measurement, the new scheduling gap can be configured for GNSS measurement in advance to the data scheduling based on the UE reported validity duration as shown in Figure 1. Aperiodic triggering by network is straightforward way to GNSS measurement, which is flexible and efficient for sporadic but LONG transmission. The GNSS measurement is be triggered by MAC CE. The new scheduling gap length is determined by the GNSS Time To First Fix (TTFF), the GNSS TTFF parameter is typically 1~2 seconds with hot start or around 5~9 seconds with warm or cold start, which can be configured by eNB based on the UE reported capability and should be equal to or larger than the latest UE reported GNSS position fix time duration.

Since aperiodic triggering GNSS measurement is configured when uplink transmission is really needed (e.g., in advance to the data scheduling), on when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be Xms after the end of MAC CE reception, and X is predefined as 3ms or 6ms.

Figure 1 Aperiodic and periodic triggering GNSS operation

Proposal 2: The length of GNSS measurement gap aperiodically triggered can be configured by eNB based on the UE reported capability.

Proposal 3: On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be Xms after the end of MAC CE reception, and X is predefined as 3ms or 6ms.

However, for one NPUSCH codeword transmission spans up to 40s, time/frequency synchronization can’t be maintained the same during NPUSCH transmission since UE’s reported validity duration of GNSS position fix may be less than 40s(e.g., 10s, 20s…) as specified in Rel.17 IoT NTN. UE can’t be triggered with aperiodic GNSS measurement by MAC CE during long uplink transmission. Before the uplink transmission, UE can be configured with GNSS measurement gap with periodic way by RRC configuration. Periodic GNSS measurement can be adopted during the long NPUSCH transmission (e.g., starting from the actual uplink transmission). Similar as NBIoT uplink transmission gap (e.g., with length of 40ms), during uplink transmission, a new periodic scheduling gap (e.g., much longer than legacy uplink transmission gap) after a transmission duration is configured to UE and the uplink transmission is punctured/postponed/suspended during the new scheduling gap. The transmission duration can be configured by eNB based on the UE reported validity duration of GNSS position fix.

During the new transmission gap, UE can refresh its GNSS position fix, may need to maintain DL synchronization, and may even need to perform contention-free/contention-based based RACH procedure to confirm the serving cell and corresponding TA. Some of the UE behaviors in the new transmission gap are up to UE implementation (e.g., GNSS position fix, DL SYNC).

Figure 2: Scheduling new gap for uplink transmission

Proposal 4: Triggering the GNSS measurement for both aperiodic and periodic way should be controlled by network.

Proposal 5: UE can be configured with GNSS measurement gap with periodic way by RRC configuration.

GNSS assistance information

It was agreed that UE reports at least GNSS position fix time duration for measurement and GNSS validity duration to gNB; UE reports GNSS position fix time duration for measurement at least during the initial access stage; UE may report GNSS validation duration with MAC CE in connected mode.

Regarding the indication for success/completion of GNSS position fix, GNSS validation duration is reported to gNB based on the UE capability, and eNB can configure the UE-specific GNSS measurement gap to guarantee the performance of GNSS operation, so there is no need to further explicitly indicate the success/completion of GNSS position fix, and the reception of any UL transmission from the UE at eNB side after the GNSS measurement can be an implicit way to report the success/completion of GNSS position fix.

On whether GNSS position fix time duration needs to be reported in connected, since UE is expected to report the GNSS position fix time duration for cold/warm/hot start in initial access procedure, eNB may configure the GNSS measurement based on the interval between two consecutive measurements, and the GNSS position fix time duration for each mode may not be expected to change, so there is no need to further indicate GNSS position fix time duration in connected.

Proposal 6: UE is not expected to further explicitly report indication for success/completion of GNSS position fix, and the reception of any UL transmission from the UE at eNB side after the GNSS measurement can be an implicit way to report the success/completion of GNSS position fix.

Proposal 7: UE is not expected to further update the GNSS position fix time duration in connected.

Conclusions

In this contribution, considerations of GNSS operation enhancement for IoT NTN are provided. The following proposals are given.

Proposal 1：Considering the workload and standard effort, it is better not to support of closed loop time and frequency correction enhancement during long connection.

Proposal 2: The length of GNSS measurement gap aperiodically triggered can be configured by eNB based on the UE reported capability.

Proposal 3: On when the GNSS measurement gap starts, which is aperiodically triggered by eNB with MAC CE, the start time should be Xms after the end of MAC CE reception, and X is predefined as 3ms or 6ms.

Proposal 4: Triggering the GNSS measurement for both aperiodic and periodic way should be controlled by network.

Proposal 5: UE can be configured with GNSS measurement gap with periodic way by RRC configuration.

Proposal 6: UE is not expected to further explicitly report indication for success/completion of GNSS position fix, and the reception of any UL transmission from the UE at eNB side after the GNSS measurement can be an implicit way to report the success/completion of GNSS position fix.

Proposal 7: UE is not expected to further update the GNSS position fix time duration in connected.

- References

3GPP TR 36.763 V1.0.0 (2021-06)

RP-213596, “New WID on IoT NTN enhancements”, MediaTek Inc, RAN#94e

R1-2302120, “Feature lead summary #1 of AI 9.11.4 on improved GNSS operations”, MediaTek Inc, RAN1#112