3GPP TSG RAN WG1 #112bis-e	        		   R1-2303633

# e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PUCCH resource determination for multiplexing multicast first and second HARQ-ACK reporting mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When UE multiplexes multicast first HARQ-ACK reporting mode with the multicast second HARQ-ACK reporting mode, it only states that the last DCI determining the PUCCH is last first reporting mode DCI, however, it does not mention which PUCCH configuration will be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided, otherwise, from pucch-Config/pucch-ConfigurationList” to clarify the PUCCH resource determination is based on the the multicast PUCCH associated with the first HARQ-ACK reporing mode, if provided, otherwise, based on the unicast PUCCH configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE’s behaviour is not clear which PUCCH configuration will be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

# 18	Multicast Broadcast Services

*** Unchanged text is omitted ***

If the UE multiplexes in a PUCCH only multicast HARQ-ACK information of same priority that is according to both the first and second HARQ-ACK reporting modes, the last DCI format that the UE uses to determine the PUCCH resource from pucch-ConfigMulticast1/pucch-ConfigurationListMulticast1, if provided, otherwise, from pucch-Config/pucch-ConfigurationList, as described in clause 9.2.3, is a last DCI format associated with multicast HARQ-ACK information that is according to the first HARQ-ACK reporting mode.

*** Unchanged text is omitted ***