3GPP TSG RAN1 #112bis-e	R1-2303638
e-Meeting, April 17th – 26th, 2023

Source:				Panasonic

Title:		UE complexity reduction for eRedCap

Agenda item:	9.6.1

Document for:			Discussion and Decision

Introduction

This document discusses the UE complexity reduction for Rel-18 eRedCap. The following topics are discussed:

- UE BB bandwidth reduction

  - Allocation for PDSCH SIB1/OSI

  - Timing gap between PDSCH RAR and PUSCH Msg3

  - Allocation for PDSCH msg4

  - Simultaneous unicast and P-RNTI triggered SIB PDSCH

  - SRS

  - Separate initial DL/UL BWP

- UE peak rate reduction

- Early indication via Msg1

Discussion

UE BB bandwidth reduction

Allocation for PDSCH SIB1/OSI

The agreement for PDSCH RAR made in RAN1 #111:

Agreement For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH

The agreement for PDSCH paging made in RAN1 #112:

Agreement: Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

As well as the agreements above for PDSCH RAR and paging, the agreements for SIB1 and OSI need the clarification that the scheduling of those PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. Otherwise, there can be an interpretation like that “just distributed PRB mapping larger than 5 MHz by VRB-to-PRB interleaving is allowed as in legacy operation,” which would not be the RAN1’s intention.

Our proposal is as follows:

For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation). The scheduling of SIB1 PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS. For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation). The scheduling of OSI PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 1:	Update the agreements for the PDSCH SIB1 and OSI with the clarification as follows:

The scheduling of SIB1/OSI PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Timing gap between PDSCH RAR and PUSCH Msg3

The agreement made in RAN1 #112:

Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs

In the previous tdoc [1], we proposed that the value of X should be NT,1 based on the analysis of the required processing time and the co-existence with the non-eRedCap UEs. For the down-selection given by the agreement above, we propose that the value X = 1/0.5 ms for 15/30 kHz SCS which is the nearest to NT,1, respectively, should be selected.

Proposal 2:	The value pair for X (the additional timing gap between PDSCH RAR and PUSCH Msg3 when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot) should be 1/0.5 ms for 15/30 kHz SCS.

Allocation for PDSCH Msg4

The working assumption made in RAN1 #112:

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

The agreement in RAN2 #121:

- Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not).

According to the RAN2 agreement, Msg3/MsgA PUSCH based early indication (EI) for Rel-18 eRedCap is introduced. This EI via Msg3 may always be given as well as that for Rel-17 RedCap, but it is up to RAN2 decision.

If the EI via Msg3 is always given, the network can always distinguish whether the UE having sent the Msg3 is eRedCap or not, and then the network can schedule the proper number of PRBs for Msg4 PDSCH for eRedCap UEs and non-RedCap UEs, respectively. Therefore, it is reasonable to confirm the RAN1 working assumption since there is no performance impact to non-eRedCap UEs. When the RO configuration is shared between eRedCap and non-eRedCap UEs, those UEs may happen to select the same preamble, and both correctly receive Msg2. Then both would send the same TC-RNTI value in a Msg3 transmission. When both are collided, the Msg3 decoding failure can continue until the maximum retransmission as HARQ soft buffer is collided. Or either of Msg3 transmission might successfully be decoded. Then Msg4 is sent based on eRedCap or non-eRedCap i.e., presence/absence of the EI in the received Msg3. For another UE, Msg4 is not required to correctly be received if Msg3 transmission is not successfully decoded at gNB. Therefore, this is not the issue.

If the EI via Msg3 is NOT always given, and when RO is shared by eRedCap and non-RedCap UEs, the network may not be able to distinguish whether the UE having sent the Msg3 is eRedCap or not. In this case, the working assumption may result in a performance loss for non-eRedCap UEs due to the limited number of PRBs. But the RAN1 discussion for this case can be deferred until RAN2 makes progress on the Msg3/A PUSCH early indication.

Based on the discussion above, we propose to defer confirming the working assumption and wait the RAN2 progress on the Msg3/A PUSCH early indication. If it is decided that the EI via Msg3 is always given, the working assumption can be confirmed.

Proposal 3:	Defer confirming the working assumption and wait the RAN2 progress on the Msg3/A PUSCH early indication.

Simultaneous reception of unicast and P-RNTI triggered SIB PDSCH

This subsection is the resubmission of [1].

In the current spec, the simultaneous reception of unicast PDSCH and P-RNTI triggered SIB PDSCH is required for a UE for FR1:

§5.1 [2]:

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.

In our view, SIB PDSCH utilize 20 MHz in many cases to keep the sufficient transmission power and maintain the coverage. Thus, it would not be a major case that the SIB PDSCH and unicast PDSCH for eRedCap are FDMed within 20 MHz. Also, precluding such simultaneous reception would be beneficial to avoid the complexity increase.

Therefore, we propose that the existing handling for FR2 is reused for the eRedCap (FR1). It means an eRedCap UE is not required to decode unicast PDSCH which is FDMed with P-RNTI triggered SIB. Exact description for FR2 is described as following.

§5.1 [2]:

On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Proposal 4:	The requirement for P-RNTI triggered SIB reception for eRedCap UE (FR1) is same as the solution used for FR2 i.e., the eRedCap UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

SRS

The FL proposal in RAN1 #112 [3]:

FL6 Medium Priority Question 2.10-1a: Should the SRS bandwidth be limited to 5 MHz as for other uplink transmissions?

It is proposed that the SRS bandwidth is reduced to 5 MHz. In our view, the following aspects should be discussed:

RF BW can be reduced to the BW corresponding to 25/16 PRBs for 15/30 kHz SCS (16 PRBs at least are required for maximum PUCCH scheduling). A small cost reduction is expected by the SRS bandwidth limitation because of the potential RF BW reduction.

There would be some performance loss on channel state estimation since the simultaneous SRS transmission spanning 20 MHz is impossible, although it can be recovered by the frequency hopping.

WID needs to be updated as that 5 MHz BB bandwidth limitation is applied for SRS as well as PDSCH/PUSCH.

Proposal 5:	The merit of SRS BW reduction to 5 MHz is not so large but the demerit is also not so large. RAN1 can continue the discussion.

Separate initial DL/UL BWP

Regarding the separate initial DL/UL BWP, we recommend that the following cases should separately be discussed for the better clarification:

- For a cell supporting both Rel-18 eRedCap UEs and Rel-17 RedCap UEs

- For a cell supporting Rel-18 eRedCap UEs and barring Rel-17 RedCap UEs

For a cell supporting both Rel-17 RedCap and Rel-18 eRedCap UEs

The current RAN1 situation is as follows:

Agreement: (RAN1 #110bis-e) For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs (replaced by later agreement) Conclusion: (RAN1 #112) There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”.

According to the agreement above, the case that the Rel-17 RedCap UEs and Rel-18 eRedCap UEs share the separate initial BWP is supported. On the other hand, there is no consensus to support the case that the separate initial BWP specific to the Rel-17 RedCap and that specific to Rel-18 eRedCap are simultaneous be configured.

The remaining case is that the separate initial BWP is configured only for the Rel-18 eRedCap UEs where Rel-17 RedCap UEs use the shared initial BWP with non-RedCap UEs. We propose to support this case as well. Allowing the case is beneficial for scheduling flexibility while the NW complexity or test effort would not increase so much. From RAN1 perspective, we propose that separate initial DL/UL BWP specific to Rel-18 eRedCap UEs can be configured only when the BWP specific to Rel-17 RedCap UEs is NOT configured, but the detail of the configuration should be up to RAN2.

Proposal 6:	For a cell supporting both Rel-17 RedCap and Rel-18 eRedCap UEs, separate initial DL/UL BWP specific to Rel-18 eRedCap UEs can be configured only when the separate initial DL/UL BWP specific to Rel-17 RedCap UEs is NOT configured. The detail of the configuration is up to RAN2.

For a cell supporting Rel-18 eRedCap UEs and barring Rel-17 RedCap UEs

In this case, there is no agreement yet whether separate initial BWP can be configured for Rel-18 eRedCap UEs. We propose that the separate initial BWP can be configured for Rel-18 eRedCap UEs. It is beneficial for the traffic offloading and separate scheduling between non-RedCap UEs and Rel-18 eRedCap UEs, like the case between non-RedCap UEs and Rel-17 RedCap UEs. The detail of the configuration can be as follows while the decision on this should also be up to RAN2:

- The configuration is given to the UEs via Rel-17 part of SIB signaling; it may be a bit tricky since Rel-17 RedCap is barred by the NW.

- The configuration is given to the UEs via Rel-18 part of SIB signaling.

Proposal 7:	For a cell supporting Rel-18 eRedCap UEs and barring Rel-17 RedCap UEs, the configuration of separate initial DL/UL BWP for Rel-18 eRedCap UEs should be supported. The detail of the configuration is up to RAN2.

UE peak rate reduction

This section is the update of [1].

The agreements made in RAN1 #110bis and #112:

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, (replaced by later agreement) - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement: Revise the earlier agreement by removing the square brackets like this: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL

The agreement made in RAN #99 [4]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Regarding “BW3/PR3 + PR1”, the Table 1 shows the supported max data rate for FR1 DL, calculated based on §4.1.2 [5], where the limited number of RB is assumed. To support target peak data rate 10 Mbps, (vLayers·Qm·f) needs to be 3 or more. Relaxation less than 3 would result in the existence of an eRedCap UE not supporting peak 10 Mbps, which is not desired. Therefore, the minimum acceptable value of X is 3. We can also accept X=3.2 to realize 10 Mbps for both SCS 15 kHz and 30 kHz.

Table 1: Calculation of supported peak data rate for FR1 DL

|  | vLayers·Qm·f | Supported max data rate [Mbps] for BW3/PR3 + PR1 |
| --- | --- | --- |
| NRB: 25 SCS: 15 kHz | 2 | 6.69 |
|  | 3 | 10.03 |
|  | 3.2 | 10.70 |
| NRB: 12 SCS: 30 kHz | 2 | 6.42 |
|  | 3 | 9.63 |
|  | 3.2 | 10.27 |

Regarding “20 MHz + PR1”, the Table 2 shows the supported max data rate for FR1 DL. To support target peak data rate 10 Mbps, (vLayers·Qm·f) needs to be 0.75 or more. We propose Y=0.75.

Table 2: Calculation of supported peak data rate for FR1 DL

|  | vLayers·Qm·f | Supported max data rate [Mbps] for BW3/PR3 + PR1 |
| --- | --- | --- |
| NRB: 106 SCS: 15 kHz | 0.4 | 5.46 |
|  | 0.75 | 10.63 |
|  | 0.8 | 11.34 |
| NRB: 51 SCS: 30 kHz | 0.4 | 6.42 |
|  | 0.75 | 10.23 |
|  | 0.8 | 10.91 |

Proposal 8:	The value of X (for the UE peak rate reduction for “BW3/PR3 + PR1”) is 3 or 3.2 (instead of 4).

Proposal 9:	The value of Y (for the UE peak rate reduction for “20 MHz + PR1”) is 0.75 (instead of 4).

Early indication via Msg1

The FL proposal in RAN1 #112 [3]:

High Priority Proposal 2.3-1g: From RAN1 perspective, support separate early indication from Rel-17 RedCap in Msg1, at least when separate early indication for Rel-17 RedCap is not configured: - Msg1 indication specific for Rel-18 RedCap UEs can be configured by the network (otherwise the Rel-17 RedCap UE behavior is used). …

In our view, Msg1 early indication (EI) specific to Rel-18 eRedCap is beneficial and necessary since the prolonged timeline between RAR and Msg3 specific to eRedCap has been supported. Also, we understand the concern on increased complexity and test effort due to further RO partitioning. This concern would be especially true when PRACH repetition is applied, where large number of PRACH resource is required. The proposal above provided by the FL, supporting Msg1 EI specific to Rel-18 eRedCap while avoiding the further RO partitioning, is a good middle ground. Furthermore, we propose the following clarification to avoid the further RO partitioning:

From RAN1 perspective, support separate early indication from Rel-17 RedCap in Msg1, at least when separate Msg1 early indication for Rel-17 RedCap is not configured: - Msg1 indication specific for Rel-18 RedCap UEs can be configured by the network (otherwise the Rel-17 RedCap UE behavior is used).

Whether to configure the Msg1 EI for Rel-17 or Rel-18 should be up to the NW. As shown in the figure below, when the Msg1 EI for Rel-17 is configured (e.g., in the Rel-17 part of SIB), the corresponding ROs should be shared by the RedCap and eRedCap UEs. When the Msg1 EI specific to Rel-18 is configured (e.g., in the Rel-18 part of SIB), the Rel-17 RedCap UEs should share the legacy ROs with non-RedCap UEs.

Proposal 10:	From RAN1 perspective, support separate early indication from Rel-17 RedCap in Msg1, at least when Msg1 separate early indication for Rel-17 RedCap is not configured:

- Msg1 indication specific for Rel-18 RedCap UEs can be configured by the network (otherwise the Rel-17 RedCap UE behavior is used).

Conclusion

Regarding the allocation for PDSCH SIB1/OSI:

Proposal 1:	Update the agreements for the PDSCH SIB1 and OSI with the clarification as follows:

The scheduling of SIB1/OSI PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Regarding the timing gap between PDSCH RAR and PUSCH Msg3:

Proposal 2:	The value pair for X (the additional timing gap between PDSCH RAR and PUSCH Msg3 when the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot) should be 1/0.5 ms for 15/30 kHz SCS.

Regarding the allocation for PDSCH Msg4

Proposal 3:	Defer confirming the working assumption and wait the RAN2 progress on the Msg3/A PUSCH early indication.

Regarding the simultaneous reception of unicast and P-RNTI triggered SIB PDSCH:

Proposal 4:	The requirement for P-RNTI triggered SIB reception for eRedCap UE (FR1) is same as the solution used for FR2 i.e., the eRedCap UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Regarding the SRS:

Proposal 5:	The merit of SRS BW reduction to 5 MHz is not so large but the demerit is also not so large. RAN1 can continue the discussion.

Regarding the separate initial DL/UL BWP:

Proposal 6:	For a cell supporting both Rel-17 RedCap and Rel-18 eRedCap UEs, separate initial DL/UL BWP specific to Rel-18 eRedCap UEs can be configured only when the separate initial DL/UL BWP specific to Rel-17 RedCap UEs is NOT configured. The detail of the configuration is up to RAN2.

Proposal 7:	For a cell supporting Rel-18 eRedCap UEs and barring Rel-17 RedCap UEs, the configuration of separate initial DL/UL BWP for Rel-18 eRedCap UEs should be supported. The detail of the configuration is up to RAN2.

Regarding the UE peak rate reduction:

Proposal 8:	The value of X (for the UE peak rate reduction for “BW3/PR3 + PR1”) is 3 or 3.2 (instead of 4).

Proposal 9:	The value of Y (for the UE peak rate reduction for “20 MHz + PR1”) is 0.75 (instead of 4).

Regarding the early indication via Msg1:

Proposal 10:	From RAN1 perspective, support separate early indication from Rel-17 RedCap in Msg1, at least when Msg1 separate early indication for Rel-17 RedCap is not configured:

- Msg1 indication specific for Rel-18 RedCap UEs can be configured by the network (otherwise the Rel-17 RedCap UE behavior is used).

Reference

[1] R1-2300852, Panasonic, RAN1 #112, Athens, Greece, Feb. 2023

[2] TS 38.214 V17.5.0

[3] R1-2301889, Ericsson, RAN1 #112, Athens, Greece, Feb. 2023

[4] RP-230778, CMCC, RAN #99, Rotterdam, Netherlands, Mar. 2023

[5] TS 38.306 V17.1.0