3GPP TSG RAN WG1 #112bis-e	                                                  R1-2303639

e-Meeting, April 17th – April 26th, 2023

Source:	CATT, CMCC, Samsung

Title:	TP on SBFD for TR 38.858

Agenda Item:	9.3

Document for:	Endorsement

- Introduction

Based on the agreements on SBFD made in RAN1 so far, TP is provided for TR 38.858 based on the latest TR agreed in R1-2300997.

- Text proposals

--------------------------------------------------------- Start of text proposal ---------------------------------------------------------

# 6	Subband non-overlapping full duplex (SBFD)

Editor's note: This section captures the schemes of SBFD and their feasibility from specification design point of view. gNB self-interference handling schemes (e.g., spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc.) and their feasibilities can be discussed in RAN4 and captured in section 9. Subsections may be added or removed under this section depending on the discussion in RAN1.

## 6.1	General aspects of SBFD schemes

Editor's note: This section captures the general aspects of SBFD schemes except self-interference, inter-gNB and inter-UE CLI handling schemes.

6.1.1	SBFD Operations

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction. For discussion purpose, SBFD symbol is defined as symbol with subbands that gNB would use for SBFD operation.

SBFD operation within a TDD carrier is studied and SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Whether SBFD operation in SSB symbols is supported or not is studied in RAN1.

RAN1 studies whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

The time and frequency location of subbands within a TDD carrier are not fixed in the specification. Regarding whether to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation, the following alternatives are studied with Alt 4 prioritized at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Among the four alternatives, SBFD operation Alt 4 is agreed as the baseline for SBFD operation at least for RRC_CONNECTED state.

6.1.1.1 Semi-static configuration of SBFD subbands

For indication of subband locations for SBFD operation, semi-static configuration of subband time and frequency location is studied as baseline. For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline. For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. Furthermore, for the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. For semi-static configuration of subband location, same subband frequency resources across different SBFD symbols are considered as baseline.

6.1.1.2	SBFD operation in symbols configured as DL in TDD-UL-DL-ConfigCommon

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

In addition, whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon for SBFD aware UEs are studied based on the following options:

- Option 1 (semi-static SBFD): DL receptions outside semi-statically configured DL subband(s) are not allowed

- Option 2: (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

6.1.1.3	SBFD operation in symbols configured as flexible in TDD-UL-DL-ConfigCommon

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following options are studied for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

In addition, whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in a symbol configured as flexible in TDD-UL-DL-ConfigCommon for SBFD aware UEs are studied based on the following options:

- Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

- Option 2 (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

  - UL transmissions outside the semi-statically configured UL subbands are not allowed

- Option 3 (dynamic SBFD): DL receptions outside semi-statically configured DL subband(s) are allowed

  - UL transmissions outside the semi-statically configured UL subbands are allowed

6.1.2	Impact and potential enhancements for transmissions and receptions

Impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following, are studied:

  - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

  - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

  - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

  - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

  - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

  - Resource allocation in frequency domain including frequency hopping

  - Resource allocation in time domain

  - Power domain

  - Spatial domain

RAN1 studies the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

  - RBG for PDSCH RA type 0

  - CSI reporting configuration

  - CSI-RS resource configuration

  - PRG of PDSCH

For SBFD-aware UEs, at least the following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands are studied. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

Frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs are studied considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

For SBFD-aware UEs, at least the following issues for PDSCH are studied:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following options are studied for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

For SBFD-aware UEs, the following options are studied for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, at least the followings are studied:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation

## 6.2	Inter-gNB CLI handling schemes specific for SBFD

Editor's note: This section captures the potential inter-gNB CLI handling schemes that are specific for SBFD.

## 6.3	Inter-UE CLI handling schemes specific for SBFD

Editor's note: This section captures the potential inter-UE CLI handling schemes that are specific for SBFD.

For inter-UE inter-subband CLI measurement, at least the following methods are studied:

  - Method#1: victim UE measures RSSI within DL subband

  - Method#2: victim UE measures RSRP of aggressor UE within UL subband

  - Method#3: victim UE measures RSSI within UL subband

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

--------------------------------------------------------- End of text proposal ---------------------------------------------------------

- Conclusions

In this contribution, text proposal on SBFD based on RAN1 agreements so far is provided or TR 38.858 and it is proposed to adopt the proposed TP.

- Appendix: RAN1 agreements of SBFD

RAN1#109-e

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Agreement

At least study SBFD operation within a TDD carrier

Conclusion

For discussion purpose only, SBFD symbols is defined as symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.

Guideline for future meetings

- Note: AI 9.3.3 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for dynamic TDD and schemes that are common for both SBFD and dynamic/flexible TDD.

- Note: AI 9.3.2 handles the potential inter-gNB and inter-UE CLI handling schemes that are specific for SBFD.

RAN1#110

Agreement

Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state.

- SBFD operation Alt 1:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side.

- SBFD operation Alt 2:

  - Time and frequency locations of subbands for SBFD operation are not known to UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

UE capability discussion is held in work item phase.

Agreement

For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

Agreement

For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline.

Working Assumption

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Agreement

For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

Agreement

Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes:

- Measurement resource/reporting configuration

- Measurement/reporting details (including UE processing delay)

- Relevant information exchange (between gNBs) if needed

- Usage of measurement at gNB

Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

RAN1#110bis-e

Agreement

For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline.

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

Agreement

For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required.

- FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Agreement

Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands.

Agreement

Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE

- If there are, whether/how to avoid/handle such collision cases (as second step)

Agreement

Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency.

Agreement

Study whether SBFD operation in SSB symbols is supported or not.

Agreement

For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

Agreement

The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1.

- The UL subband can be located at one side of the carrier.

- The UL subband can be located at the middle part of the carrier

Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible.

Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance.

Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above.

LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671.

Agreement

For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#111

Agreement

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

  - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- DL receptions within DL subband(s) are allowed in the symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Agreement

For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Agreement

Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols

- Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols

- Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition

- Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols

- Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols

Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered.

Examples of potential enhancements include:

- Resource allocation in frequency domain including frequency hopping

- Resource allocation in time domain

- Power domain

- Spatial domain

FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured.

Agreement

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs,

Option 1:

- UL transmissions within UL subband are allowed in the symbol

- UL transmissions outside UL subband are not allowed in the symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

- FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol

Option 2:

- UL transmissions within UL subband are allowed in the symbol

- The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same

  - FFS: SBFD aware UE behaviours

  - FFS: Whether or not signalling of guardband(s) is needed

- FFS: Whether or not the symbol can be converted to a DL-only symbol

- Frequency locations of DL subband(s) are known to the SBFD aware UE

- DL receptions within DL subband(s) are allowed in the symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously

Agreement

Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following:

- RBG for PDSCH RA type 0

- CSI reporting configuration

- CSI-RS resource configuration

- PRG of PDSCH

RAN1#112

Agreement

For dynamic SBFD,

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

  - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency.

For each option, additional conditions may apply to determine whether the option is applicable.

Agreement

Study whether or not a slot can consist of both SBFD and non-SBFD symbols including

  - Benefits

  - Use cases

  - Scheduling flexibility

  - Implementation complexity

  - Compatibility with legacy TDD DL/UL configuration

Agreement

For inter-UE inter-subband CLI measurement, study at least the following methods:

  - Method#1: victim UE measures RSSI within DL subband

    - FFS: Whether SINR can be measured

  - Method#2: victim UE measures RSRP of aggressor UE within UL subband

  - Method#3: victim UE measures RSSI within UL subband

  - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - Study the following options for SBFD-aware UEs:

    - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

    - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

  - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

    - PDSCH/PUSCH/PUCCH repetitions

    - SPS PDSCH/CG PUSCH

    - TBoMS

    - Multi-PUSCH/PDSCH scheduled by a single DCI

    - Periodic/semi-persistent SRS/CSI-RS/PUCCH

    - PDCCH

Agreement

For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Option 1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

FFS: The part of the RBG outside.

Agreement

For SBFD-aware UEs, study at least the following issues for PDSCH:

  - PRG(s) with size of 2 and 4 that overlaps with subband boundary

  - Wideband precoder in case of non-contiguous DL subbands

Agreement:

Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Agreement:

For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols):

  - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols

  - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols

Agreement:

Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots:

  - Whether/how to have separate resources

  - Whether/how to have separate FH parameters

  - Whether/how to have separate UL power control parameters

  - Whether/how to have separate beam/spatial relation