3GPP TSG RAN WG1 #112bis-e	R1- 2303643

e-Meeting, April 17th – 26th, 2023

Source:	Sharp

Title:	Views on Coverage enhancement for NR NTN

Agenda Item:	9.9.1

Document for:	Discussion and Decision

Introduction

In RAN1#112, the following agreements and working assumptions were made for the coverage enhancement for NR NTN [1]:

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design) Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

In this contribution, we share our views on coverage enhancement for NR NTN.

Discussions

Msg4 HARQ-ACK

Repetition request and capability report

- 2.1.1.1. Repetition request / Capability report via Msg1 or Msg3

In the RAN1#112 meeting, following working assumption regarding the Msg4 HARQ-ACK repetition was made.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH FFS: which signaling is used, e.g. DMRS ports

In the RAN1#112 meeting, for the working assumption above, whether either Msg1 or Msg3 is used for the Msg4 HARQ-ACK repetition request or capability report was discussed.

Although, for Rel-17 Msg3 repetition, a UE is able to request the Msg3 repetition only via the Msg1, for Rel-18 Msg4 HARQ-ACK repetition, either of Msg1 and Msg3 is available to use repetition request or capability report.

In case of the transmission of the Msg4 HARQ-ACK repetition request or capability report via the Msg1 (Option A), the dedicated RACH resources for the Msg4 HARQ-ACK repetition request or capability report which are separated with the RACH resources for other purpose is needed, and it means squeezing the property of the RACH resources for other purpose. To avoid it, one solution is linkage with the RACH resource for Msg3 repetition request. In general, a coverage of the Msg3 PUSCH is less than that of the Msg4 HARQ-ACK PUCCH. The UE that performs the Msg4 HARQ-ACK repetition is also likely to performs the Msg3 repetition. Therefore, the RACH resources for Msg3 repetition request can be reused for Msg4 HARQ-ACK repetition. (i.e. the indication of Msg3 repetition request also indicates Msg4 HARQ-ACK repetition) However, network need to differentiate between UE that supports Rel-17 Msg3 repetition and does not support Rel-18 Msg4 HARQ-ACK repetition and UE that supports both repetitions. Because, network that receives Msg3 repetition request that is transmitted by UE that supports only Msg3 repetition may expect to receive multiple Msg4 HARQ-ACK. Therefore, at least one additional dedicated RACH resource for the UE that does not support Msg4 HARQ-ACK repetition but does support Msg3 repetition would be needed.

Observation 1:

- If the RACH resources for Msg3 repetition request via Msg1 is used to also indicate Msg4 HARQ-ACK repetition request, the network need to differentiate between Rel-17 Msg3 repetition UE and Rel-18 UE that requests both Msg3 and Msg4 HARQ-ACK repetitions. For this differentiation, RACH resource segmentation for Rel-18 UE would be needed anyway.

In case of the transmission of the Msg4 HARQ-ACK repetition request or capability report via the Msg3 (Option B), using of reserved values of LCID is the one way to carry additional information. In this case, there are no issues of the differentiation and partitioning increasing instead of requiring an extension of Msg3. If using Msg3, although RAN1 need to ask RAN2 if they take this option, the repetition request or capability report can be indicated without impacting the other UEs. Considering pros/cons of Option A and Option B, we prefer to use Msg3 as indication of the Msg4 HARQ-ACK repetition request/capability report.

Observation 2:

- In case of the transmission of the Msg4 HARQ-ACK repetition request or capability report via the Msg3, the repetition request or capability report from Rel-18 UE does not have any impact on the legacy UEs.

Proposal 1:

- UE indicates gNB of whether to request Msg4 HARQ-ACK repetition via LCID value in MAC sub-header in Msg3.

  - Ask RAN2 if they take this option.

- 2.1.1.2. Necessity of RSRP threshold configuration

In the RAN1#112 meeting, following working assumptions regarding the Msg4 HARQ-ACK repetition was made.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

For the working assumption above, whether repetition request is used depending on whether the RSRP threshold is configured or not was discussed.

In Rel-17, the Msg3 repetition was introduced. In general, a coverage of the Msg3 PUSCH is less than that of the Msg4 HARQ-ACK PUCCH. Hence, a network that performs the Msg4 HARQ-ACK repetition is also likely to performs the Msg3 repetition.

Moreover, the network that operates Msg3 repetition is able to estimate UL shortage based on the measurement of the Msg1 and derive a repetition factor of the Msg3. The estimated UL shortage is also applicable to the derivation of Msg4 HARQ-ACK repetition factor. The request of the repetition of the Msg4 HARQ-ACK from the UE provides no additional value on top of the UL shortage estimated by the network.

Observation 3:

- The network that requires the Msg4 HARQ-ACK repetition is also likely to requires the Msg3 repetition.

Observation 4:

- The network that operates Msg3 repetition is able to estimate UL shortage based on the measurement of the Msg1 and the estimated UL shortage is used to derive a repetition factor of the Msg3. This estimated UL shortage is also applicable to the derivation of Msg4 HARQ-ACK repetition factor.

Observation 5:

- In terms of repetition factor determination at the network side, the request of the repetition of the Msg4 HARQ-ACK from the UE provides no additional value on top of the UL shortage estimated by the network.

In case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg1, the dedicated RACH resources for the Msg4 HARQ-ACK repetition request is need to be reserved. It causes squeezing other RACH resources. If the repetition request is transmitted via the Msg1, using the RSRP threshold would be a benefit for reducing the squeezing other resources.

In case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg3, since it has no impact for the other UEs, there is no benefit to use the RSRP threshold.

Therefore, whether either Msg1 or Msg3 is used for the Msg4 HARQ-ACK repetition request should be firstly discussed before the discussion of whether the working assumption is confirmed or not. If Msg3 is used, the second working assumption should not be confirmed.

Observation 6:

- Regarding the RSRP threshold,

  - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg1, the RACH resources partitioning based on the RSRP threshold has benefit for reducing the reservation of RACH resources for the Msg4 HARQ-ACK repetition.

  - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg3, the separation using RSRP threshold has no benefit.

Proposal 2:

- It should be discussed whether either Msg1 or Msg3 is used for the Msg4 HARQ-ACK repetition request, before discussing whether confirm the working assumption related to RSRP threshold configuration made in RAN1#112.

As the above section 2.1.1.1, we propose the Msg3 should be used to indicate for repetition request. Therefore, the second working assumption should not be confirmed.

Proposal 3:

- The working assumption related to RSRP threshold configuration made in RAN1#112 should not be confirmed.

- 2.1.1.3. Capability of repetition factors

For the Msg4 HARQ-ACK repetition capability, the UE support one of the following options:

- Alt 1: The UE has to support all of the configurable repetition factors (e.g. the UE support all of [1,2,4,8]). The UE indicates whether the repetition is supported or not to the gNB.

- Alt 2: The UE can support only part of the configurable repetition factors (e.g. the UE support part of [1,2,4,8]).

  - Alt 2-1: The UE indicates whether all the repetition factors broadcasted in the SIB are supported or not to the gNB.

  - Alt 2-2: The UE indicates which repetition factors the UE supports.

For the Alt 2-1, if the UE is not capable all of the repetition factors broadcasted in the SIB, even if the UE capable part of the broadcasted repetition factors, the UE cannot perform the Msg4 HARQ-ACK repetition. For the Alt 2-2, the number of bits for the capability report would be larger than 1, hence it would be an overhead. For Alt 1, in case of TDD configuration, it may be complicated to support all of the configurable repetition factors. For NTN, since all bands used for NTN are FDD, there is no issue related to TDD configuration. Therefore, the UE should support all of the configurable repetition factors.

Observation 7:

- For the Msg4 HARQ-ACK repetition capability, the UE support one of the following options:

  - Alt 1: The UE has to support all of the configurable repetition factors (e.g. the UE support all of [1,2,4,8]). The UE indicates whether the repetition is supported or not to the gNB.

  - Alt 2: The UE can support only part of the configurable repetition factors (e.g. the UE support part of [1,2,4,8]).

    - Alt 2-1: The UE indicates whether all the repetition factors broadcasted in the SIB are supported or not to the gNB.

    - Alt 2-2: The UE indicates which repetition factors the UE supports.

Observation 8:

- For the alt 2-1, if the UE is not capable all of the repetition factors broad casted in the SIB, even if the UE capable one of the broadcasted repetition factors, UE cannot perform the Msg4 HARQ-ACK repetition.

Observation 9:

- For the alt 2-2, increase of the bit for the capability report would be an overhead.

Proposal 4:

- The UE should support all of the configurable repetition factors.

Dynamic indication for the repetition

In the RAN1#112 meeting, the following agreement was made regarding Msg4 HARQ-ACK dynamic indication of repetition factor from gNB.

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

Firstly, for Alt 2, Msg3 is scheduled by RAR UL grant, not by DCI. Therefore, following correction should be needed.

Proposal 5:

- Regarding the agreement of RAN1#112 meeting, the following correction should be needed:

  - Alt 2: Field in DCI RAR UL grant scheduling Msg3 PUSCH

As we proposed in the above section, the UE should indicate the repetition request via Msg3. Therefore, the Alt 2 and Alt 4 should not be used. As for Alt 3, there is a complexity issue. Therefore, we prefer the Alt 1.

For dynamic indication of the repetition factor, it takes 2 bits to indicate multiple repetition factor. If part of the existing DCI field is reused for the dynamic indication, it must be considered impacts to the existing field.

In case of the PUCCH resource indicator field or PDSCH-to-HARQ feedback timing indicator field is used for the dynamic indication for the repetition, these fields have 3bits to indicate resource assignment and using 2 bits for the repetition factor indication would be serious restriction of flexibility of resource assignment for network.

In Rel-17 Msg3 repetition, MCS field in RAR UL grant is used to indicate repetition factor. The reason is that the channel to which the MCS field value in RAR UL grant applies and the channel that the repetition is needed are the same. In this situation, higher MCSs are not necessary for the UE which requested Msg3 repetition, because the Msg 3 repetition request means that the expected Msg3 reception power at the gNB side would not be sufficient. For Msg4 HARQ-ACK repetition, unlike Msg3 repetition, the channel to which the MCS field value applies and the channel that the repetition is needed are different. The MCS field indicates the MCS for Msg4 PDSCH but not PUCCH for Msg4 HARQ-ACK. Even though the quality of PUCCH for Msg4 HARQ-ACK is predicted to be insufficient, that does not necessarily mean only lower MCSs are feasible for Msg4 PDSCH. Rather, that may limit the network scheduling on Msg4 PDSCH.

The HARQ process number field can be used for indication of the repetition factor because the HARQ process number before the dedicated RRC configuration is not provided is not so large.

The downlink assignment indicator field (DAI field) can also be used to indicate the repetition factor because the DAI field in the DCI format 1_0 scrambled by TC-RNTI is reserved. We prefer to use reserved field rather than to use existing field (HARQ process number field).

In the last RAN1 meeting, some companies concerned that the HARQ process number field and DAI field cannot be used before dedicated RRC configuration is provided. Because HARQ process number would be large and the DAI field is not reserved for DCI1_0 that is not scrambled by TC-RNTI.

In the RAN#99 meeting, whether the PUCCH transmission when dedicated PUCCH resource configuration is not provided in addition to PUCCH transmission for Msg4 HARQ-ACK is to be specified or not was discussed. As a result of the discussion, RAN agreed to specify only PUCCH enhancement for Msg4 HARQ-ACK and excluded to specify the other PUCCH transmission. Therefore, the DAI field can be supported for the dynamic indication of the repetition factor.

Proposal 6:

- The DAI field is used for the dynamic indication of the Msg4 HARQ-ACK repetition factor.

DMRS bundling for PUSCH

In the last meeting, regarding following Proposal was discussed regarding DMRS bundling for PUSCH.

Proposal 2-3_v5 For NTN-specific PUSCH DMRS bundling, further discuss whether nominal/actual TDW determination defined in R17 can be reused for NTN or not, e.g., the following options. - Option 1: Actual TDW determination based on timing of TA pre-compensation update causing phase discontinuity and power inconsistency. UE determines the timing. - Nominal TDW is configured by gNB as in the existing spec. - UE reports information on the timing. FFS details - Actual TDW is determined by the reported information and events defined in R17. - New event is defined based on TA pre-compensation update. FFS details - Option 2: Nominal/actual TDW determination based on gNB configuration and/or indication - Option 2-1: Determined as in the existing spec. No spec change is assumed. - Nominal TDW is configured by gNB as in the existing spec. - Note: Based on gNB implementation, nominal TDW length can be configured to current timing drift, derived from satellite ephemeris, to allow UE to update time/frequency pre-compensation sufficiently often. - Actual TDW is determined by events defined in R17. - Note: no new event is defined for the TA pre-compensation update. - Option 2-2: Determined by new gNB configuration/indication. - FFS details - e.g., TA pre-compensation timing is indicated by gNB - e.g., further segmented configuration is provided by gNB - FFS: Whether new event is defined based on new gNB configuration/indication. - FFS: whether to define another new event to decide actual TDW (e.g., antenna switching, update of epoch time, etc.)

In Rel-17 DMRS bundling for CovEnh, the first nominal TDW starts at the first PUSCH repetition. In other words, the UE cannot be aware of boundaries of nominal TDW before the PUSCH is scheduled. As for the legacy NTN TA pre-compensation update, the timing of UE pre-compensation update is left to UE implementation. There could be two different implementations - one is that the timing of TA update depends on the start timing of PUSCH, and the other is that the timing of TA update does not depend on the start timing of PUSCH. For the latter case, if the timing of UE pre-compensation update is affected by the PUSCH scheduling, it would cause the new restriction on the Rel-18 NTN UE pre-compensation update timing. In other words, in case of the Option 2, since the timing of UE pre-compensation is determined by gNB, regarding TA pre-compensation update, implementation of the Rel-17 NR NTN chip-set may not be reused for that of the Rel-18 NR NTN chip-set. It means that Option 2 may force to change the UE implementation of the TA pre-compensation. This is not desirable. Therefore, it is suggested that actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

Proposal 7:

- Actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

Conclusion

In this contribution, we have the following proposals and observations:

Observation 1:

- If the RACH resources for Msg3 repetition request via Msg1 is used to also indicate Msg4 HARQ-ACK repetition request, the network need to differentiate between Rel-17 Msg3 repetition UE and Rel-18 UE that requests both Msg3 and Msg4 HARQ-ACK repetitions. For this differentiation, RACH resource segmentation for Rel-18 UE would be needed anyway.

Observation 2:

- In case of the transmission of the Msg4 HARQ-ACK repetition request or capability report via the Msg3, the repetition request or capability report from Rel-18 UE does not have any impact on the legacy UEs.

Proposal 1:

- UE indicates gNB of whether to request Msg4 HARQ-ACK repetition via LCID value in MAC sub-header in Msg3.

  - Ask RAN2 if they take this option.

Observation 3:

- The network that requires the Msg4 HARQ-ACK repetition is also likely to requires the Msg3 repetition.

Observation 4:

- The network that operates Msg3 repetition is able to estimate UL shortage based on the measurement of the Msg1 and the estimated UL shortage is used to derive a repetition factor of the Msg3. This estimated UL shortage is also applicable to the derivation of Msg4 HARQ-ACK repetition factor.

Observation 5:

- In terms of repetition factor determination at the network side, the request of the repetition of the Msg4 HARQ-ACK from the UE provides no additional value on top of the UL shortage estimated by the network.

Observation 6:

- Regarding the RSRP threshold,

  - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg1, the RACH resources partitioning based on the RSRP threshold has benefit for reducing the reservation of RACH resources for the Msg4 HARQ-ACK repetition.

  - in case of the transmission of the Msg4 HARQ-ACK repetition request via the Msg3, the separation using RSRP threshold has no benefit.

Proposal 2:

- It should be discussed whether either Msg1 or Msg3 is used for the Msg4 HARQ-ACK repetition request, before discussing whether confirm the working assumption related to RSRP threshold configuration made in RAN1#112.

Proposal 3:

- The working assumption related to RSRP threshold configuration made in RAN1#112 should not be confirmed.

Observation 7:

- For the Msg4 HARQ-ACK repetition capability, the UE should support one of the following options:

  - Alt 1: The UE supports all of the configurable repetition factors (e.g. the UE support all of [1,2,4,8]). The UE indicates whether the repetition is supported or not.

  - Alt 2: The UE supports part of the configurable repetition factors (e.g. the UE support part of [1,2,4,8]).

    - Alt 2-1: The UE indicates whether all of the repetition factors broadcasted in the SIB is supported or not.

    - Alt 2-2: The UE indicates all own repetition factors.

Observation 8:

- For the alt 2-1, if the UE is not capable all of the repetition factors broad casted in the SIB, even if the UE capable one of the broadcasted repetition factors, UE cannot perform the Msg4 HARQ-ACK repetition.

Observation 9:

- For the alt 2-2, increase of the bit for the capability report would be an overhead.

Proposal 4:

- The UE should support all of the configurable repetition factors.

Proposal 5:

- Regarding the agreement of RAN1#112 meeting, the following correction should be needed:

  - Alt 2: Field in DCI RAR UL grant scheduling Msg3 PUSCH

Proposal 6:

- The DAI field is used for the dynamic indication of the Msg4 HARQ-ACK repetition factor.

Proposal 7:

- Actual TDW should be determined based on when UE performs pre-compensation update causing phase discontinuity and power inconsistency.

References

“RAN1 Chairman’s Notes,” RAN1 #112, March 2022.