# 3GPP TSG-RAN-WG1 #112bis-e	R1-2303644

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.3

Source:	Google Inc.

Title:	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and decision

# 1. Introduction

In this document, we provide our views on mechanisms for dynamic switching between DFT-S-OFDM and CP-OFDM.

# 2. Discussion

## 2.1 DCI format

### 2.1.1 DCI format/field alignment

RAN1 #112 Agreement For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI. Note: no change of the current size alignment procedure between UL DCI and DL DCI. FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

In the last meeting, the group has agreed to use a 1-bit field in the UL scheduling DCI format to indicate the waveform switching. The group also discussed how the DCI format should be aligned (per-format or per-field) when dynamic waveform switching (DWS) is configured. Some options have been identified and summarised in the feature lead’s proposal 2-4r2. In general, the difference between these options are the total DCI size, location of the DWS field, and whether the UE has to read the DWS field before parsing other DCI fields (e.g., DWS independent and dependent fields). From our perspective, we do not presume the spec will introduce large waveform-dependent field in the near future, thus, the performance gain due to small DCI size difference is marginal.

Observation 1: The difference between different DCI size alignment options are the total DCI size, location of the DWS field, and whether the UE has to read the DWS field before other fields (e.g., DWS independent and dependent fields). We do not presume the spec will introduce large waveform-dependent field in the near future, thus, the performance gain due to small DCI size difference is marginal.

Proposal 1: For DCI size alignment as DWS is configured, support Option-2 in FL proposal 2-4r2 for less spec impact. To relax UE implementation, DWS field can be allocated at any position before waveform-dependent field(s).

### 2.1.2	FDRA field

As addressed by companies, FDRA Type-0 is not applicable for DFT-S-OFDM. Then there is an issue that if FDRA is configured as resourceAllocationType0 or dynamicSwitch for CP-OFDM, how the UE interprets FDRA field in a scheduling DCI format that indicates DFT-S-OFDM? First of all, because there is no ambiguity when Type-1 resource allocation is configured, the issue is valid only if the group agree to have FDRA Type-0 or dynamic switch for CP-OFDM when DWS is configured.

Proposal 2: If DWS is configured, support frequency domain resource allocation Type-0, Type-1, and dynamic switching for PUSCH transmission indicated with CP-OFDM waveform.

If the proposal above is valid, the solution is simple. Because the FDRA Type-0 is not applicable for DFT-S-OFDM, the UE interprets the FDRA field according to resource allocation type-1 when DFT-S-OFDM is indicated in the scheduling DCI format.

Proposal 3: UE interprets the FDRA field as resource allocation Type-1 when DFT-S-OFDM is indicated in the scheduling DCI format, and considers most significant bits in the FDRA field as padding bits, if the FDRA Type-0 or dynamic switching is configured to CP-OFDM.

## 2.2 Assistance information

RAN1 #111 Agreement Study the necessity of the following potential enhancements to assist the scheduler in determining waveform switching: - Reporting power headroom related information based on PCMAX,f,c applicable to a target waveform - Target waveform can be same or different from waveform of an actual PUSCH transmission - FFS target RB allocation and/or target modulation order can be same or different from respective properties of an actual PUSCH transmission - FFS determination of target waveform, target RB allocation, target modulation order - FFS details, e.g. report PCMAX,f,c or Type 1 power headroom for a waveform, or difference thereof between waveforms - PHR triggering enhancements, e.g. - Network-triggered PHR - PH becomes lower (higher) than a threshold - PHR triggered by waveform switching - Reporting of recommended waveform or request to switch waveform - Other solutions not precluded

According to the discussion in the last meeting, the majority of companies support to send assistance information to the base station, especially carried in the PHR. On the other hand, the opinions on whether to introduce new PHR triggering event and how to trigger it are diverse. In our understanding, this feature is an optimization, thus, unless the performance gain can be justified, it should be better to avoid high complexity implementation or large signalling overhead. In the legacy method (RRC configured waveform switching), the base station knows the UE is entering the cell edge and the signal strength becomes weak, then the base station configures the UE to transmit in another waveform. Likewise, dynamic waveform switching does not increase the coverage comparing to the legacy method, but it shortens the signalling latency. With the shortened latency and smaller overhead, the base station can easily evaluate the PHR difference, for example, in a region that both waveforms deliver similar performance. In this sense, we believe introducing a new PHR triggering event when the UE receives the DWS indication might be the simplest way (minimum overhead) to help base station better evaluates UE capability (e.g., PAPR reduction) under different waveforms.

Observation 2: Delivering assistance information is an optimization feature to dynamic waveform switching. The exact performance benefit has not been justified, thus introducing high complex implementation and large signalling overhead are not recommended.

Proposal 4: Support introducing a new PHR triggering event when a UE receives DWS indication in a scheduling DCI format.

# 3. Conclusion

Observation 1: The difference between different DCI size alignment options are the total DCI size, location of the DWS field, and whether the UE has to read the DWS field before other fields (e.g., DWS independent and dependent fields). We do not presume the spec will introduce large waveform-dependent field in the near future, thus, the performance gain due to small DCI size difference is marginal.

Proposal 1: For DCI size alignment as DWS is configured, support Option-2 in FL proposal 2-4r2 for less spec impact. To relax UE implementation, DWS field can be allocated at any position before waveform-dependent field(s).

Proposal 2: If DWS is configured, support frequency domain resource allocation Type-0, Type-1, and dynamic switching for PUSCH transmission indicated with CP-OFDM waveform.

Proposal 3: UE interprets the FDRA field as resource allocation Type-1 when DFT-S-OFDM is indicated in the scheduling DCI format, and considers most significant bits in the FDRA field as padding bits, if the FDRA Type-0 or dynamic switching is configured to CP-OFDM.

Observation 2: Delivering assistance information is an optimization feature to dynamic waveform switching. The exact performance benefit has not been justified, thus introducing high complex implementation and large signalling overhead are not recommended.

Proposal 4: Support introducing a new PHR triggering event when a UE receives DWS indication in a scheduling DCI format.

# 4. Reference

[1] 	Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0, Feb., 2023.

[2]	R1-2302222, ” Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM,” RAN1 #112, Feb., 2023.