3GPP TSG RAN WG1 #112bis-e			R1-2303646

e-Meeting, April 17th – April 26th, 2023

Title:	Draft reply LS on Comparison of SL-RSRP and SD-RSRP measurements

Response to:	R1-2302280/R2-2302234

Release:	Release 18

Work Item:	NR_SL_relay_enh-Core

Source:	Xiaomi

To:	RAN2

Cc:	RAN4

Contact person:

Email:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 would like to thank RAN2 for the LS (R1-2302280/R2-2302234) on Comparison of SL-RSRP and SD-RSRP measurements.

RAN1 would like to provide the following answers to RAN2 questions:

Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

[Answer]

Yes. The comparison can be used for the purpose of triggering a measurement report.

Sidelink based open loop power control has been supported for unicast, but not for broadcast. Therefore, the transmit power of the serving relay UE and the candidate relay UE can be different. If sidelink based open loop power control is used, the dependency of SL RSRP measurement result on pathloss will be reduced.

However, the power control parameters of serving relay UE are configured by the network. The network can take this intro account and can still set a suitable offset value to trigger the measurement report.

2	Actions

To RAN4 group

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

3	Dates of next TSG RAN WG 1 meetings

TSG-WG1 Meeting #113	22nd May – 26th May 2023			Incheon , KR