3GPP TSG RAN WG1 #112-bis-e							R1-2303649

e-meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.1

Source:	AT&T

Title:	General aspects of AI/ML framework for NR air interface

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. Some of the aspects of the study item include:

AI/ML model, terminology and description to identify common and specific characteristics for framework investigations:

- Characterize the defining stages of AI/ML related algorithms and associated complexity:

  - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable

  - Inference operation, e.g., input/output, pre-/post-process, as applicable

- Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g.,

  - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes]

  - Various levels of UE/gNB collaboration targeting at separate or joint ML operation.

- Characterize lifecycle management of AI/ML model: e.g.,  model training, model deployment , model inference, model monitoring, model updating

- Dataset(s) for training, validation, testing, and inference

- Identify common notation and terminology for AI/ML related functions, procedures and interfaces

- Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate

This contribution discusses the general framework of AI/ML applied to the NR air interface, specifically the remaining open issues regarding basic AI/ML model terminology, and the LCM procedures for one-sided and two-sided models.

# General AI/ML framework for NR air interface

Optimization is a fundamental challenge in deploying large-scale cellular networks as configuration and adaptation of system parameters can have significant impact on key performance indicators (KPIs) such as system capacity, user QoE, latency, reliability, coverage, and numbers of active users, etc.  This is especially critical for 5G networks as they are heterogenous in terms of frequency bands/ranges, macro and small cell deployments, diverse service offerings and traffic characteristics, and coexistence of different architectures including centralized virtual RAN functions and distributed nodes to support latency-sensitive edge computing and private networks. The same industry trends which enable network virtualization and deployment of low latency/high bandwidth services are also making application of Artificial Intelligence (AI) tools such as machine learning (ML) algorithms to 5G networks feasible and scalable.

These algorithms rely on historical data for deriving system models and training as well as real-time or near-real-time data collection to adapt to different network conditions. Furthermore, a variety of use cases can be supported by AI/ML techniques as noted in the SID including CSI feedback optimization, beam management, and positioning. Many of these use cases have common requirements in terms of data collection, KPIs for monitoring and LCM procedure. At the same time, different use cases can have vastly different requirements in terms of the impact on network nodes or functionalities. This implies that the appropriate implementation of different AI/ML techniques may involve multiple interfaces, signalling procedures, processing requirements (including requirements on data aggregation or co-location with different nodes/functions) and LCM requirements.  A common framework for model identification and signaling procedures is however important to be put in place to be used for all the sub use cases being considered in this study item, as well as any future use cases that may be considered for 5G-A.

Proposal 1: Study a common framework for model identification to indicate a common understanding between network and UE across all sub-use cases.

Model identification is a very important part of the LCM framework and agreeing on the terminology and the basic understanding of how to identify and signal an AI/ML model merited considerable debate in previous RAN1 meetings and post RAN1#112.

In RAN1#110-bis [2], the following agreements were made regarding AI/ML model identification.

Agreement

Study LCM procedure on the basis that an AI/ML model has a model ID with associated information and/or model functionality at least for some AI/ML operations when network needs to be aware of UE AI/ML models

- FFS: Detailed discussion of model ID with associated information and/or model functionality.

- FFS: usage of model ID with associated information and/or model functionality based LCM procedure

- FFS: whether support of model ID

- FFS: the detailed applicable AI/ML operations

Agreement

Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models.

Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Agreement

Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects:

- Procedure and assistance signaling for the AI model switching and/or selection

FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined)

In RAN1#111 [3], the following agreements were further made, related to defining model formats and structure, and model identification

Working Assumption

| Terminology | Description |
| --- | --- |
| Model identification | A process/method of identifying an AI/ML model for the common understanding between the NW and the UE Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification. |

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Note: whether and how to indicate Functionality will be discussed separately.

Working Assumption

| Terminology | Description |
| --- | --- |
| Model update | Process of updating the model parameters and/or model structure of a model |
| Model parameter update | Process of updating the model parameters of a model |

In RAN1#112[4], further agreements have been made on model/functionality identification and their relationship, and further study is still needed to understand model identification and the relation between functionality identification and model identification.

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

Agreement

- AI/ML-enabled Feature refers to a Feature where AI/ML may be used.

Agreement

- For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature.

Building on the previous meetings agreements on functionality and model identification, we propose a categorization of the AI/ML models and subsequently a framework for usage for model and functionality identification, that encompasses different model formats, model structures, signaling procedures for model identification, as well as location of the inference. The AI/ML model categorization is illustrated in Figure 1.

Level 3: Trained AI/ML Models

Level 2: Untrained Model Structure

Level 1: Functionality

Level 2.1: Open source AI/ML model structure

Level 2.2: Proprietary AI/ML model structure

Level 3.1.1: Proprietary-format models

Level 3.1.2: Open-format models

Level 3.2.1: Proprietary-format models

Level 3.2.2: Open-format models

Figure 1 AI/ML Model Categorization

- Model Level 1: functionality
Model level 1 describes the AI/ML model functionality i.e., use case of the AI/ML model (e.g., CSI prediction, Beam Management), possible scenario/configurations (e.g. indoor/outdoor, rank, prediction time, number of beams), performance requirements and/or input/output details (e.g. input/output number of bits). Typically, this will be used for one sided model where we are only interested in the functionality of AI/ML model. There may be multiple AI/ML models corresponding to a given functionality and the UE/network will decide the AI/ML algorithm to use for the functionality. It can also be potentially used for two sided models (if there is no model transfer). 
The functionalities can be standardized into a functionality ID which corresponds to a standardized signaling (to/from the network) used for functionality-based LCM such as model activation/deactivation/fallback, functionality update/switching, performance monitoring etc. It should be noted that multiple AI/ML functionalities can be defined for each sub-use case at each UE.

- Model Level 2: untrained model structure
Model level 2 describes the AI/ML algorithm (e.g. CNN, LSTM) and structure (number of layers, number of neurons in each layer) and input/outputs details (e.g. input/output number of bits). The input/output details are provided if they are not provided in the functionality. Model level 2 is required to have a common understanding between the gNB and UE regarding the UE capability. It is needed for two-sided models using collaboration level z where both gNB and UE are running a part of AI/ML model. It can also be used for collaboration level z with one sided model at UE to understand if the UE is able to run the AI/ML model of this complexity. Furthermore, it may provide a limited level of control in level y where the network may use it for model switching at the UE. A  “model structure ID” is thus required to determine the structure of AI/ML models. This level can further be divided into 2 sub-levels depending on whether the AI/ML model structure is open-source or proprietary.

- Level 3: Trained AI/ML model 
Model level 3 describes an AI/ML model that is trained or fine-tuned using a given dataset. This model can then be used for inference, directly or after compilation, for a single or multiple use cases. The trained AI/ML model can be labelled with a unique global model ID. The global model ID can be used to determine information such as version number (of the model), dataset ID, time stamp, source ID (e.g. device/network/chipset/software/service provider vendor), in addition to the functionality and the untrained model structure. It should be noted that any time when a new AI/ML model is generated or there is a model update (with the same model structure) using a new dataset for a given use case or functionality a new “time stamp” and/or “version number” and/or “dataset ID” is generated. For collaboration level z the trained AI/ML models can be in either proprietary format or open format with both of them having slightly different LCM. Furthermore, the LCM will be different if the model structure is open source or proprietary.

Proposal 2:  Characterize AI/ML model identification as a function of the associated information needed for the common understanding between the UE and the network, used for AI/ML LCM procedure

Proposal 3: Define three levels of AI/ML model identification categories: model functionality, AI/ML untrained model structure, and AI/ML trained model.

In RAN1#111, a working assumption was made on the model formats.

Working Assumption

Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion,

| Proprietary-format models | ML models of vendor-/device-specific proprietary format, from 3GPP perspective NOTE: An example is a device-specific binary executable format |
| --- | --- |
| Open-format models | ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspecive |

From RAN1 discussion viewpoint, RAN1 may assume that:

- Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared.

- Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared

The AI/ML model has different LCM depending on whether it is a proprietary format model, or open format model. The proprietary format models would likely consist of the AI/ML model structure, input/output, and weights(/parameter) compiled into a single file that can be used directly for inference. The open-format models would consist of the AI/ML model structure and/or input/output and/or weights/parameters compressed through a known format across vendors. This model information would likely need to be decompressed and complied before it can be used for model update and inference.

In addition to defining proprietary and open format models, it is beneficial to also define and differentiate between open and proprietary model structures. The knowledge of AI/ML model structure is important for an online (over-the-air) capability reporting of a UE device for a common understanding at the network if the UE can support the given AI/ML model. It will also result in a different LCM procedure than a proprietary structure AI/ML model.

For an open-source AI/ML model, the model update/transfer, would likely correspond to transmission of new/updated weights/parameters of the AI/ML model as the structure is already known. For a proprietary model format, the model transfer/update would likely result in compiling and transmission of the complete AI/ML model containing both model structure and weights. In the event of common understanding between the UE and network regarding the proprietary model structure (It is still proprietary to any third party), the model transfer/update can be performed by transmitting the new/updated weights/parameters.

We thus propose the following definitions for the open and proprietary model structures:

Proprietary AI/ML model structure: An AI/ML model that uses an AI/ML algorithm and/or input/outputs that are proprietary from 3GPP perspective.

Open-source AI/ML model structure: An AI/ML model that uses an AI/ML algorithm and input/outputs that are mutually recognizable across vendors and can allow interoperability, from 3GPP perspective.

Proposal 4: Consider “proprietary AI/ML model structure” and “open-source AI/ML model structure” as two separate AI/ML model structure categories for RAN1 discussion,

| Proprietary AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and/or input/outputs that are proprietary from 3GPP perspective. |
| --- | --- |
| Open-source AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and input/outputs that are mutually recognizable across vendors and can allow interoperability, from 3GPP perspective. |

# ML Model Life Cycle Management

The LCM procedure follows naturally from the AI/ML model categorization. Two LCM procedures were previously defined and are possible to use: functionality-based LCM procedure, and model-ID-based LCM procedure.

Model functionality is captured as one way to categorize the model, for a common understanding between the UE and the network about the model. Signaling of model structure and/or format or parameters is not always needed for the common understanding depending on the type of use case or AI/ML feature considered. We thus propose a hierarchical LCM procedure that always starts with a functionality based LCM, and can use a model-ID-based LCM when needed (depending on the use case), as the best way forward.

Proposal 4: Functionality based LCM procedure is used for both UE-sided and two-sided models.

Figure 2: Model Categorization and Life Cycle Management Procedure

Functionality-based LCM

Model-based LCM

Model Identification

Functionality

Model Structure ID

Additional ID

Model format ID

For UE-sided models, functionality ID can give a flexible, streamlined procedure on how to identify a model or a family of models used at the UE to the network, without the need to necessarily identify the model itself. When the functionality ID is signaled and encompasses information such as input/output parameters of the model, performance KPIs, pre-processing, and post-processing requirements, and associated information for enabling a certain feature, network does not need to be aware of the model that is used at the UE, and correspondingly, the model ID does not need to be signaled.

For one sided models, functionality based LCM can thus be sufficient.

Furthermore, for UE-sided models, model-ID-based LCM can be signaled for collaboration level z to signal if the UE is able to run a given AI/ML model for e.g. complexity purposes.

Proposal 5: For UE-side models and UE-part of two-sided models, functionality-based LCM is used.

Proposal 6: For UE-sided models, model-ID-based LCM procedure can be optional.

Proposal 7: For two-sided models, both functionality and model-ID-based LCM are used.

## UE-sided model Example sub-use case

An example that can be given to illustrate the functionality-based LCM procedure, is a UE-sided CSI prediction sub-use case. For such a use case, different functionalities can be defined, one functionality for each UE speed range (e.g. very high speed, high speed, medium speed, etc.). When a functionality-based LCM procedure is used, for a given model ID, a given functionality (e.g., for high speed UEs) is activated at a time. The network monitors that functionality and may request the UE to deactivate the model for that given functionality if the performance requirement is not met. Similarly, the UE can have different models for the same use case and for each of these models, a functionality corresponding to the UE speed range. For a given model, the functionality related to high speed will be activated by the network, and the network does not need to be aware of the models at the UE. The network is not aware of the model(s) used at the UE and is only concerned with activating/deactivating the functionality to achieve a certain KPI. For instance, for the CSI prediction we can have the following proposal regarding the functionality

Proposal 8: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Finally, it is important to discuss the requirement to study the model-ID based functionality in order to support any additional functionalities that are not specified within functionality-based LCM. These functionalities can be due to a UE or network wanting to support additional features (such as model transfer), configuration or granularities apart from ones specified within functionality-based LCM.

Proposal 9: Study the necessity for model-ID based LCM to support any additional functionalities that will not be supported through functionality-based LCM.

# Conclusion

In this contribution, we discussed the general framework of AI/ML applied to the NR air interface. The following proposals were made:

Proposal 1: Study a common framework for model identification to indicate a common understanding between network and UE across all sub-use cases.

Proposal 2:  Characterize AI/ML model identification as a function of the associated information needed for the common understanding between the UE and the network, used for AI/ML LCM procedure

Proposal 3: Define three levels of AI/ML model identification categories: model functionality, AI/ML untrained model structure, and AI/ML trained model

Proposal 4: Consider “proprietary AI/ML model structure” and “open-source AI/ML model structure” as two separate AI/ML model structure categories for RAN1 discussion

| Proprietary AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and/or input/outputs that are proprietary from 3GPP perspective. |
| --- | --- |
| Open-source AI/ML model structure | An AI/ML model that uses an AI/ML algorithm and input/outputs that are mutually recognizable across vendors and can allow interoperability, from 3GPP perspective. |

Proposal 5: For UE-side models and UE-part of two-sided models, functionality-based LCM is used.

Proposal 6: For UE-sided models, model-ID-based LCM procedure can be optional.

Proposal 7: For two-sided models, both functionality and model-ID-based LCM are used.

Proposal 8: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Proposal 9: Study the necessity for model-ID based LCM to support any additional functionalities that will not be supported through functionality-based LCM.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface.

[2] Chair’s notes, RAN1#110bis, e-meeting, October 2022

[3] Chair’s notes, RAN1#111, Toulouse, France, November 2022.

[4] Chair’s notes, RAN1#112, Athens, Greece, February 2023