3GPP TSG RAN WG1 #112e-bis						                       R1-2303655

e-meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.2.2

Source:	AT&T

Title:	Discussion on AI/ML for CSI feedback enhancement

Document for:	Discussion/Approval

# Introduction

As part of Rel-18 Study Item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], 3GPP has agreed to study the framework for AI/ML for air-interface corresponding to target use cases considering aspects such as performance, complexity, and potential specification aspects. One of the identified use cases include:

CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

For each of the use cases, one of the objectives is to

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In RAN#109e [2], the following agreements were reached.

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

In RAN1#110[3] the following agreements were reached.

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least

- CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input.

- CQI determination

- RI determination

Agreement

In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least

- Model output type/dimension/configuration and potential post processing

Agreement

In CSI compression using two-sided model use case, further discuss at least the following aspects, including their necessity/feasibility/potential specification impact, for data collection for AI/ML model training/inference/update/monitoring:

- Assistance signaling for UE’s data collection

- Assistance signaling for gNB’s data collection

- Delivery of the datasets.

In RAN1#110e-bis[4] the following conclusions were reached

Agreement

In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including:

- NW-side performance monitoring:  NW monitors the performance and make decisions of model activation/ deactivation/updating/switching

- UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching

Agreement

In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring.

Agreement

In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

Agreement

In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods:

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting

- Other monitoring solutions, at least including the following option:

  - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

Agreement

In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB:

- Alignment of the quantization/dequantization method and the feedback message size between Network and UE

# Potential specification impact for CSI compression with two-sided model

There are a few left-over issues from the last meeting regarding potential spec impact for CSI compression. For most of these proposals there was a general agreement among the companies so hopefully they can be agreed with minimum discussion.

## Training collaboration

In the previous meeting the Model ID and Model ID based LCM was still under discussion in 9.2.1 however in this meeting we expect discussion in 9.2.1 will have sufficient progress so that we can start discussion on some Model-ID based LCM.  Therefore, we should start discussion model transfer so that we can discuss any specification impact for the two-sided model use case with training collaboration type 1.

Proposal 1: In CSI compression using two-sided model use case with training collaboration type 1, further study potential specification impact on:

- Protocol and signalling mechanism to enable CSI compression specific model transfer.

It is important to discuss the necessity, feasibility, and any spec impact for sharing the training dataset, quantization/de-quantization related information or any other relevant information between the entities to support model training.

Proposal 2:  In CSI compression using two-sided model use case with training collaboration type 3, for sequential training, further study necessity, feasibility, and potential specification impact on:

- Training dataset and/or other information delivery from UE side to NW side for UE first training

- Training dataset and/or other information delivery from NW side to UE side for NW first training

- Data sample format/type and the dataset size

- Quantization/de-quantization related information

- Note: other aspects are not precluded.

## Performance monitoring, model update, activation/de-activation/switching and other topics

To support the performance monitoring of the AI/ML model we should study any specification impact required to enable performance monitoring using the existing CSI feedback scheme such as Rel-16 Type II as the reference to compare the performance of the existing CSI feedback scheme.

Proposal 3: In CSI compression using two-sided model use case, further study potential specification impact needed to enable model performance monitoring using an existing CSI feedback scheme as the reference to compare whether/how much AI/ML outperforms the existing CSI feedback scheme.

## Framework, UE capability, and other topics

We should study any AI/ML based CSI compression based on the legacy CSI feedback signalling framework. However, we should also study the requirement and benefit of specification enhancements regarding CSI-RS configuration, CSI reporting configuration and CSI processing procedure.

Proposal 4: The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study the necessity and benefits of any potential specification enhancement for

- CSI-RS configurations

- CSI reporting configurations

- CSI processing procedures.

- Other aspects are not precluded.

# Potential specification impact for CSI prediction with  UE-sided model

In RAN1#111[5] it was agreed that

Agreement

Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement.

Note: Continue evaluation discussion in 9.2.2.1.

Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer futher till the end of R18 AI/ML SI.

Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

In the previous meeting there has been good progress regarding the CSI compression sub use case. Therefore, in this meeting we propose to start the discussion on Spec impact for CSI prediction

Proposal 5: Resume the specification impact discussion for the CSI prediction using UE sided model.

We propose to study any spec impact needed for data collection to support model training/fine-tuning or model monitoring. While the model is deployed on the UE side however the model training or model monitoring can be performed at the network or at the UE. Therefore, we propose that

Proposal 6:

- In CSI prediction using UE sided model use case, study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least

  - Additional CSI configuration information

  - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

    - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

  - Signaling for triggering the data collection

- In CSI prediction using UE sided model use case, study the necessity, feasibility, and potential specification impact for NW side data collection including at least:

  - Additional CSI configuration information

  - Contents of the ground-truth CSI including:

    - Data sample type, e.g., precoding matrix, channel matrix etc.

    - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like).

    - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

  - Latency requirement for data collection

  - Signaling for triggering the data collection

In this meeting we expect to have a working assumption from 9.2.1 regarding the functionality-based LCM and Model-ID based LCM. It is understood that for the spec impact and details of the LCM discussion will be discussed within each use case. Therefore, we should start discussion on the spec impact regarding functionality and what features are to be reported/signaled through the functionality. We understand that different functionalities can be defined within a given use-case, for instance there is a separate functionality for each UE speed range (e.g. very high speed, high speed, medium speed, etc.). When a functionality-based LCM procedure is used, for a given model ID, a given functionality (e.g., for high speed UEs) is activated at a time. The network monitors that functionality and may request the UE to deactivate the model for that given functionality if the performance requirement is not met. Similarly, the UE can have different models for the same use case and for each of these models, a functionality corresponding to the UE speed range. For a given model, the functionality related to high speed will be activated by the network, and the network does not need to be aware of the models at the UE. The network is not aware of the model(s) used at the UE and is only concerned with activating/deactivating the functionality to achieve a certain KPI.

We propose that we discuss the parameter that will be controlled by functionality and the corresponding specification impact for functionality-based LCM

Proposal 7: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Finally, it is important to discuss if there is a requirement to study the model-ID based functionality in order to support any additional functionalities that are not specified within functionality-based LCM. These functionalities can be due to a UE or network wanting to support additional features (such as model transfer), configuration or granularities apart from ones specified within functionality-based LCM.

Proposal 8: For CSI prediction using UE sided model, study the requirement for model-ID based LCM to support additional functionalities that will not be supported through functionality-based LCM.

# Conclusion

In this contribution, we discussed a sub-use case on CSI feedback enhancements related to CSI compression. We made the following observations and proposals.

Proposal 1: In CSI compression using two-sided model use case with training collaboration type 1, further study potential specification impact on:

- Protocol and signalling mechanism to enable CSI compression specific model transfer.

Proposal 2:  In CSI compression using two-sided model use case with training collaboration type 3, for sequential training, further study necessity, feasibility, and potential specification impact on:

- Training dataset and/or other information delivery from UE side to NW side for UE first training

- Training dataset and/or other information delivery from NW side to UE side for NW first training

- Data sample format/type and the dataset size

- Quantization/de-quantization related information

- Note: other aspects are not precluded.

Proposal 3: In CSI compression using two-sided model use case, further study potential specification impact needed to enable model performance monitoring using an existing CSI feedback scheme as the reference to compare whether/how much AI/ML outperforms the existing CSI feedback scheme.

Proposal 4: The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study the necessity and benefits of any potential specification enhancement for

- CSI-RS configurations

- CSI reporting configurations

- CSI processing procedures.

- Other aspects are not precluded.

Proposal 5: Resume the specification impact discussion for the CSI prediction using UE sided model.

Proposal 6:

- In CSI prediction using UE sided model use case, study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least

  - Additional CSI configuration information

  - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc.

    - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side.

  - Signaling for triggering the data collection

- In CSI prediction using UE sided model use case, study the necessity, feasibility, and potential specification impact for NW side data collection including at least:

  - Additional CSI configuration information

  - Contents of the ground-truth CSI including:

    - Data sample type, e.g., precoding matrix, channel matrix etc.

    - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like).

    - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator)

  - Latency requirement for data collection

  - Signaling for triggering the data collection

Proposal 7: For CSI prediction using UE sided model study the following configurations and their granularity that will be signaled through the functionality, and the corresponding specification impact in functionality-based LCM

- UE speed

- Frequency PRB’s

- Prediction window

- Observation window

- Scenario (Uma etc.)

- Performance requirement/monitoring

- Other additional configurations

Proposal 8: For CSI prediction using UE sided model, study the requirement for model-ID based LCM to support additional functionalities that will not be supported through functionality-based LCM.

# Reference

[1] RP-213599 Study on AI/ML for Air-Interface, RAN#94e.

[2] RAN1#109-e Chair’s Notes, e-meeting May 9-20th, 2022

[3] RAN1#110 Chair’s Notes, August 22nd – 26th, 2022.

[4] RAN1#110bis-e Chair’s Notes, October 10th – 19th, 2022.

[5] RAN1#111 Chair’s Notes, November 14th – 18th, 2022.

[6] RAN1#112 Chair’s Notes, February 27th – Mar 3rd, 2023.