3GPP TSG-RAN WG1 #112bis-e	R1-2303656

e-Meeting, April 17th – April 26th, 2023

Source:                    	DENSO CORPORATION

Title:  	Discussion on UE complexity reduction

Document for:        	Discussion and Decision

Agenda Item:         	9.6.1	- UE complexity reduction

- Introduction

A work item on further NR RedCap UE complexity reduction has been approved [1]. RAN1#112 meeting discussed the WID and the following is part of agreement [2].

Agreement For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs Agreement For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number Agreement For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Working assumption: For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

At the RAN1#112 meeting, it was discussed on the timeline of RAR PDSCH reception and Msg.3 transmission, and it was agreed an additional time duration X could be fixed value. However, the value was FFS. In addition, separate early indication(s) for Rel.18 eRedCap UE and Msg.4/Msg.B PDSCH bandwidth were left as FFS. In this paper, we provide our view about these issues.

- Discussion

Msg.4 PDSCH bandwidth

RAN1#112 meeting made a working assumption on Msg.4 bandwidth. RAN1 has already made an agreement regarding unicast PDSCH as following:

- For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e.

Therefore, Msg.4 is covered by the exiting agreement. We confirm the following working assumption.

Proposal 1:	Confirm the working assumption that for UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Msg.B PDSCH bandwidth

An agreement to support Msg.3/Msg.A PUSCH based early indication for Rel.18 eRedCap was made at RAN2 [3]. Thus, the network can know Rel.18 eRedCap UE from Msg.A and treat Msg.B similar to Msg.4. If the working assumption regarding Msg.4 bandwidth is confirmed, same operation as Msg.4 would be applied to Msg.B.

Proposal 2:	For UE BB complexity reduction, a UE is not required to process a Msg.B PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Timeline of RAR and Msg.3

In the RAN1#112 agreement, the minimum time between RAR PDSCH reception and Msg.3 transmission was specified as NT,1 + NT,2 + 0.5 + X ms, where X is additional time duration corresponding to RAR PDSCH processing time. In addition, Msg.3 PUSCH transmission timing is specified as slot n + K2 + Δ, where the value of Δ depends on SCS of PUSCH and the value of K2 is indicated by PUSCH time-domain resource allocation (TDRA) table [4]. Here, Msg.3 PUSCH timing would be larger than the minimum time. If the value of X is large, the number of valid entries in the default TDRA table is reduced and it leads restriction of scheduling flexibility. Furthermore, considering that there is already margin in the legacy time NT,1 + NT,2 + 0.5 + X ms, we prefer that the value of X = 1/0.5 for 15/30kHz SCS is considered.

Proposal 3:	For the minimum time NT,1 + NT,2 + 0.5 + X ms, RAN1 to consider the value of X = 1/0.5 ms for 15/30kHz SCS.

Additional separate early indication

For Rel.17 RedCap UE, Msg.1 early indication is supported optionally. If Rel.18 supports separate Msg.1 early indication, PRACH resource may be configured for eRedCap UE separately from RedCap UE, and it causes PRACH resource fragment. This has impact on specifications and performance. However, if separate Msg.1 early indication is not supported, the network is restricted to transmit Msg.2 (RAR) within 5 MHz even for legacy UEs.

Rel.18 eRedCap UE requires additional time to process RAR compared to Rel.17 RedCap UE. Therefore, separate Msg.1 early indication is beneficial for the network to transmit/schedule the appropriate RAR/Msg.3 to Rel.18 eRedCap UE even in the case that Rel.17 RedCap UE early indication is used. Furthermore, considering that the behavior of eRedCap UE receiving RAR PDSCH larger than 5 MHz is also agreed, and it is up to the network whether to configure PRACH resource for eRedCap UE dedicated, separate Msg.1 early indication would be supported optionally for network flexibility.

Observation 1:	Rel.18 eRedCap UE early indication in Msg.1 is beneficial for Msg.2 transmission and network flexibility.

Proposal 4:	Support Rel.18 eRedCap UE early indication in Msg.1 optionally.

- Summary and proposal

In summary, the followings were observed and proposed:

Proposal 1:	Confirm the working assumption that for UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

- The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 2:	For UE BB complexity reduction, a UE is not required to process a Msg.B PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 3:	For the minimum time NT,1 + NT,2 + 0.5 + X ms, RAN1 to consider the value of X = 1/0.5 ms for 15/30kHz SCS.

Observation 1:	Rel.18 eRedCap UE early indication in Msg.1 is beneficial for Msg.2 transmission and network flexibility.

Proposal 4:	Support Rel.18 eRedCap UE early indication in Msg.1 optionally.

- References

[1] RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, December 2022.

[2] R1-2301885, “RAN1 agreements for Rel-18 NR RedCap”, Ericsson, RAN1#112, February-March 2023.

[3] R2-2301910, “Report from eRedCap breakout session”, February-March 2023.

[4] 3GPP TS 38.213, Physical layer procedures for control, (Release 17), V17.3.0 (2022-09).