3GPP TSG-RAN WG1 Meeting #112bis	R1-2303659

Electornic, 17 – 26th April, 2023

Agenda Item:	9.9.2

Source:	TCL Communication Ltd.

Title:	Discussion on Network Verified UE Location for NR NTN

Document for:	Discussion and Decision

# Introduction

In Release 17, a work item was carried out to define solutions enabling New Radio and NG-RAN to support Non-Terrestrial Networks (NTN). As part of Release 18, a new work item [1] is proposed to define enhancements aspects of NR NTN in terms of coverage, service continuity and UE location verification by the network to enable selection of an appropriate core network for UE attach and/or UE service access in accordance with the governing national or regional regulatory requirements applicable to specific UE location. Regarding network verified UE location, the work item objective as to

- Mandate the network operator to cross check the UE location reported by the UE accessing 3GPP network via satellite.

- Prioritize the specification of necessary enhancements to multi-RTT or DL-TDOA positioning methods to support the network verified UE location in NTN assuming a single satellite in view of UE at time.

- Reuse of the RAT only dependent positioning framework based on RAN1 conclusions (i.e., to avoid malicious UE from faking PLMN selection).

- Verify UE location with a target accuracy similar to terrestrial network macro cell size (i.e., 5-10 km granularity) taking into account take into account the mirror-image ambiguity issue.

During the RAN1#110 it was agreed that RAT dependent positioning methods Multi-RTT and DL/UL-TDOA to consider as starting point for the study on Network verified UE location. However, the performance of these methods is questionable under consideration of timing measurement error and the mirror-image ambiguity issue. In [2] we have discussed the feasibility of time-based positioning methods for UE location verification issue taking into account these considerations.  Lately, in RAN1#111 meeting, it was agreed for resolving mirror-image ambiguity issue that:

Agreements

- Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning:

Option 1: gNB or LMF implementation to solve the mirror error issue.

FFS: whether there is spec impact

Option 2: Reuse existing ECID method (e.g., combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

Option 3: NR NTN UE should report the Doppler calculated on the service link

Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight

Option 5: Reporting of cell coverage information (e.g., cell footprint and reference point, or antenna pattern) to the LMF

Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning

Other solutions are not precluded

And in RAN2#120 agreed that

Agreements

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the spec

In this contribution, we will rediscuss the UE location verification issue taking into account the above agreements.

# Discussion

Time-based positioning methods make use of the measured positioning signal timing difference or the time difference of arrival/departure of downlink/uplink signals received from/to different Transmission-Reception Points (TRPs) (i.e. a reference gNB and neighboring gNBs) at the same time. The resulting measurements are used along with other configuration information to locate the UE in relation to the neighboring TRPs. In case of single satellite in view [as per WID requirement], these existing methods cannot used in NTN case without enhancements because of the following details. Firstly, time-based positioning methods require very tight synchronization between UE and satellite which is hard to achieve due to the timing measurements error because of the satellite and possibly the UE movements, synchronization error and/or the clock drift between the UE and satellite. Such a timing measurement errors could lead to a positioning estimation error exceeding the required accuracy range of 5-10 km (Figure 1 and Table 1, [3, 4, 5, and 6]). Additionally, in the case of single satellite in view, the time-based measurement positioning methods are also not capable of resolving the ambiguity issue for a UE within a symmetric cell/beam with the respect to the satellite orbit plane item [Figure 2 and Table 2 [7, 8]. More specifically, without introducing some enhancements, it is very hard for time-based measurement positioning methods to verify correctly the location of a UE falling in a beam or cell that beneath a satellite’s orbital path (orbital plane).

Observation 1: Time-based measurement positioning methods cannot able to guarantee the targeted positioning accuracy of 5-10km under the considerations of timing differences measurement error and the mirror-image ambiguity issue.

Figure 1: Timing difference Measurement Error for Multi/RRT DL-TDOA Positioning Methods

Table 1 UE position uncertainty area size under time difference error multi-RRT

|  | Resulting UE position uncertainty area (km) |  |  |
| --- | --- | --- | --- |
|  | RTT error = 300ns | RTT error = 100ns | RTT error = 50ns |
| Scenario 1 | 224.7 | 74.9 | 37.4 |
| Scenario 2 | 38.2 | 12.7 | 6.3 |
| Scenario 3 | 24.5 | 8.1 | 4.0 |

Figure 3. Beams/cells underneath satellite’s orbital path suffer from mirror image ambiguity

Table 2 Positioning error performance for multi-RTT with consideration of ambiguity issue

|  | CDF=50% | CDF=67% | CDF=80% | CDF=90% | CDF=95% |
| --- | --- | --- | --- | --- | --- |
| δ=1s | 21.84 km | 29.56 km | 37.97 km | 47.00 km | 54.78 km |
| δ=5s | 19.50 km | 27.08 km | 35.56 km | 45.05 km | 53.35 km |
| δ=8s | 6.66 km | 12.32 km | 14.94 km | 20.64 km | 24.87 km |

In the following subsection of this report, we discuss in details timing difference measurement estimation error and the UE under beam/cell bellows a satellite orbital plane ambiguity issue

## Timing Difference Measurement Estimation Error

The major source for the timing difference estimation error for time-based positioning methods like Multi-RTT and DL/UL-TDOA is the issue of synchronization error due to a drift local clock of the positional signal sender and receiver. If no synchronization error is assumed, for DL/UL-TDOA positioning method; the value of the reference signal timing difference (RSTD) between a satellite at time instance t and a satellite at the time instance t+1 can be defined based on the observed time of arrival the of the positioning signals TOAt and TOAt+1 at the satellite at different instance (as shown in Figure 2),

RSTDt, t+1=TOAt+1−TOAt

Each RSTD value that is obtained for a pair of satellites at different time instance corresponds to a hyperbola equation on which the UE is assumed to be located, with foci located at these satellites at different time instance. To estimate the final location of the UE, a set of hyperbola equations can be deduced from a set of RSTD values, and the equations are solved to find the final location.

Figure 2: Time difference Measurement for DL-TDOA single-satellite case

Similarly, when no synchronization error is considered, the RTT measurements can be computed as the Rx-Tx time difference measurements of downlink signals received from the same satellite, measured by the wireless device at multiple different instants (t, t+1 …) and the measured Rx-Tx time difference measurements of uplink signals transmitted from wireless device and measured by the satellite/terrestrial node at time instants (t, t+1 …) as given

RTTt=Tst−Tdt

Each measured RTT is correspond to a circle, the intersection of all the circles will point at the UE location (assumed not moving), and in the case of moving UE, the intersections will describe a line, with the direction of movement (Figure 3).

Figure 3: Timing difference Measurement Error for Multi/RRT DL-TDOA Positioning Methods

In the case practical NTN deployment, both the wireless device and the satellite/ terrestrial node are likely to have a synchronization error due to a drift in their local clocks, which will degrade the precision of the RSTD/RTT measurement. To cope with RSTD/RTT measurement error. The difference between the drifts of the local clock of wireless device and the satellite/ terrestrial node can be taken into account to correct the measurement error due to clock miss synchronization issue. For example, for the RTT measurements can be corrected by addition or subtraction of timing error from the measured RTTt similarly for UL/DL-TDOA.

Proposal 1: Discuss a solution to mitigate the timing measurements error due the satellite movement, the synchronization error and/or the clock drift between the UE and satellite for time-based measurement positioning methods.

## Satellite Orbital Plane Ambiguity Issue

This issue happened when the wireless device which located in the beams/cells that are underneath the satellite’s orbital path. To resolve this, the UE can be provided from the gNB the time & frequency information of the synchronization signal block (SSB) of the beam in which the positioning signal is transmitted. Then, based on the SSB time & frequency information the UE can report the associated beam ID along with other assistance information such the satellite orbital trajectories (i.e., ephemeris parameters) and/or the angle of arrival (AoA) of the downlink or the direction of arrival (DoA) of the uplink positioning signal to the network to help determining whether the beam/cell where the UE is located is below a satellite orbital path. Then, determining the exact final UE location by utilizing either the satellite ephemeris parameters and/or angle of arrival (AoA) or the direction of arrival (DoA) of downlink/ uplink positioning signal (Figure 4,5 and Table 3).

Proposal 2: Investigate how to resolve the mirror image ambiguity for UE within a beam underneath a satellite’s path plane for time-based measurement positioning methods.

Table 3: Essential Elements of Ephemeris

| Orbital plane parameters | a | Square root of semi major axis（semi-major axis） |
| --- | --- | --- |
|  | e | Eccentricity（eccentricity） |
|  | i0 | Inclination angle at reference time（inclination） |
|  | Ω0 | Longitude of ascending node of orbit plane（right ascension of the ascending node） |
|  | ω | Latitude: Argument of perigee（argument of periapsis） |
| Satellite level parameters | M0 | Mean anomaly at reference time（true anomaly and a reference point in time） |
|  | t0e | Ephemeris reference time（the epoch） |

Figure 4: Satellite beams in which UE can be located at a given time measurement instance

Figure 5: Essential Parameters for Ephemeris

## Further discussion on the ambiguity issue

Regarding mirror image ambiguity issue, it was agreed during last RAN2#121 meeting, to assume that the issue can be resolved only by configuring neighbour cell measurement to UE, for example, measurement of two neighbour cells in the opposite side of a satellite beam [8].

Agreements

- RAN2 assumes that in general the mirror point issue can be resolved by properly configuring neighbor cell measurement to UE, for example, measurement of two neighbor cells in the opposite side of a satellite beam. FFS if there are any cases that require anything in the spec

In our view, configuration of neighbour cell measurement to UE may not completely address the ambiguity issue at least for some cases e.g., the case of moving cell because the cell change can happen frequently, in such situation; it is hard to determine the neighbours’ cells for a given UE. Additionally, such a configuration requires that the satellite beam where UE is located to be determined or known by the network for a given UE. On the other hand, RAN1 agreement on considering multiple solution options such as reporting of Doppler calculated on service link, beam identity or cell coverage information is more appropriate for addressing this issue. Therefore, we think the solution options being discussed in RAN1 for resolving the mirror ambiguity issue can be synchronized with RAN2

Agreements

Study the following options to resolve the mirror positions ambiguity for multi-RTT positioning:

Option 1: gNB or LMF implementation to solve the mirror error issue.

FFS: whether there is spec impact

Option 2: Reuse existing ECID method (e.g. combine UE neighbor measurements to solve the ambiguity between mirror positions), with potential enhancements

Option 3: NR NTN UE should report the Doppler calculated on the service link

Option 4: a VSAT UE should report its beam pointing in respect to satellite beam line of sight

Option 5: Reporting of cell coverage information (e.g. cell footprint and reference point, or antenna pattern) to the LMF

Option 6: Support and potentially enhance the optional Rel-17 UL-AoA measurements defined for multi-RTT positioning

Other solutions are not precluded

Based on the above discussion we propose:

Proposal 3: RAN1 synchornize the progress on the issues related to resolving the mirror image ambiguity and addressing the measurements error for time-based positioning methods.

# Conclusions

In summary, in this report we have discussed the feasibility of time-based positioning methods on verifying the reported UE location taking into account the timing measurement error at gNB and UE, and the mirror-image ambiguity issue. Based on the discussion we made following observations and proposals:

Observation 1: Time-based positioning methods cannot able to guarantee the targeted positioning accuracy of 5-10km under the considerations of timing differences measurement error and the mirror-image ambiguity issue.

Proposal 1: Discuss the solutions to mitigate the timing measurements error due the satellite movement, the synchronization error and/or the clock drift between the UE and satellite for time-based measurement positioning methods.

Proposal 2: Investigate how to resolve the mirror image ambiguity for UE within a beam underneath a satellite’s path plane for time-based measurement positioning methods.

Proposal 3: RAN1 synchornize the progress on the issues related to resolving the mirror image ambiguity and addressing the measurements error for time-based positioning methods.

# References

[1] RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales, RAN#98-e

[2] R1-2301653 , Discussion on Network UE Verified Location for NR NTN

[2] R1-2208389, Discussion on network verified UE location in NR NTN, Thales, RAN1#110-bis-e

[3] R1-2208835, Discussion on network verified UE location for NR NTN, OPPO, RAN1#110-bis-e

[4] R1-2208663, Discussions on network verified UE location for NR NTN, VIVO, RAN1#110-bis-e

[5] R1-2209265, Discussion on the network verified location, Xiaomi, RAN1#110-bis-e

[6] R1-2209751, Network verified UE location for NR NTN, Samsung, and RAN1 #110-bis-e

[7] R1-2208694, Discussion on network verified UE location for NR NTN, ZTE, RAN1#110-bis-e

[8] R2-2302401, Report of 3GPP TSG RAN WG2 meeting #121, ETSI MCC