3GPP TSG-RAN WG1 Meeting #112bis	R1-2303661

Online, April 17th – April 26th, 2023

Agenda Item:	9.12.1

Source:	Ericsson

Title:	Discussion on PRACH coverage enhancement

# Introduction

As is agreed in further NR coverage enhancement work item[1], coverage enhancements include multiple PRACH transmissions, including:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

In this contribution, we discuss the determination of the number of PRACH transmissions, PRACH multiplexing in frequency or code domain, RA-RNTI determination, RACH resource configuration, PRACH transmissions with different beams, and interplay with Msg3 transmission. Simulation results are provided.

# Discussion

## Determination of the number of PRACH transmissions

In RAN1#111 it was agreed that at least SSB-RSRP is used for a UE to determine the number of PRACH transmissions with the same Tx beam. Nevertheless, this agreement doesn’t take different UE transmission power into account, and will cause some problems, for which we provide our views in this section.

Agreement - For multiple PRACH transmissions with same Tx beam, at least SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt. - Note: whether to support multiple numbers of PRACH transmissions is separately discussed.

More and more UEs with different power classes (PC) from the traditional PC3 are put into market. UE power class defines the maximum output power, PPowerClass. In particular, FR1 UEs of PC1, PC1.5, PC2, or PC3 have corresponding PPowerClass of 31, 29, 26, or 23 dBm. With the same measured SSB RSRP and pathloss, UEs of different power classes may transmit a single PRACH with different output power, resulting in divergent PRACH performances. The PRACH performance difference will be exacerbated with Rel-18 multiple PRACH transmissions. Thus, it is reasonable to include UE transmission power in the determination of PRACH repetition factor.

- The wide range of PRACH transmission power results in divergent PRACH performance, which will be exacerbated with Rel-18 multiple PRACH transmissions. It is reasonable to include UE transmission power in the determination of PRACH repetition factor.

Now we discuss how to include UE transmission power in the determination of the number of PRACH transmissions. UE power headroom can be obtained according to Equation 1. As pathloss increases, a UE starts to trigger multiple PRACH transmissions when its power headroom is below a threshold, e.g., 0dB. In this sense, a UE would make full use of its transmission power before using RACH resources which are shared by all UEs in the cell.

UE power headroom = PCMAX,f,ci-(PPRACH,target,f,c+PLb,f,c) (Equation 1)

- If multiple PRACH transmissions are triggered when UE power headroom is below a threshold, a UE would make full use of all its transmission power first before using RACH resources which are shared by all UEs in the cell.

It has been argued that UE triggering multiple PRACH transmissions with a specific power headroom can be achieved by properly setting SSB RSRP threshold. In our view, it is true, only if all UEs in a cell have the same PCMAX. Denote the SSB RSRP threshold between a single PRACH and multiple PRACH transmissions by RSRP Th1. If it is set according to Equation 2, RSRP Th1 coincides with power headroom of zero. Nevertheless, the configured SSB RSRP threshold is specific to a particular PCMAX. UEs with a different PCMAX may trigger multiple PRACH transmissions with an unexpected power headroom.

RSRP Th1 = SSB Tx power – PCMAX + PPRACH target		(Equation 2)

UE power headroom = PCMAX - PPRACH target + measured SSB RSRP - SSB Tx power (Equation 3)

- With SSB RSRP thresholds configured, UEs with different PCMAX will determine different power headroom thresholds.

A simple solution is to let a UE determine the number of PRACH transmissions based on the configured power headroom threshold(s). As shown in Equation 3, UE power headroom is a comprehensive result of factors of PCMAX and the measured SSB RSRP. Therefore, UEs can calculate SSB RSRP thresholds based on the configured UE power headroom thresholds and their implementation PCMAX. In other words, SSB RSRP thresholds are implicitly represented by power headroom thresholds.

- SSB RSRP thresholds are implicitly represented by power headroom thresholds, in that UEs can calculate SSB RSRP thresholds based on the configured UE power headroom thresholds and their implementation PCMAX.

Regarding "at least SSB-RSRP threshold(s) are used" in the agreement, another solution is to configure both SSB RSRP thresholds and power headroom thresholds, a UE can determine a number of PRACH transmissions based on each of the two criteria and select the smaller value in an attempt to make sure the UE uses its transmission power completely before using cell-specific RACH resources. However, configuring two sets of thresholds is unnecessary, since one set can be derived from the other.

- Regarding "at least SSB-RSRP threshold(s) are used" in the agreement, gNB can configure both SSB RSRP thresholds and power headroom thresholds. Configuring two sets of thresholds is unnecessary, since one set can be derived from the other.

- A UE can determine the number of PRACH transmissions with the same Tx beam based on its power headroom and the configured UE power headroom threshold(s).

## Simultaneous PRACH transmissions with different Tx chains

PRACH transmissions multiplexed in time domain, also known as TDM approach, was agreed in RAN1#110b. What is to be studied is the simultaneous PRACH transmissions by a UE with FDM or CDM approach. With FDM approach, multiple PRACHs are respectively transmitted in different frequency domain ROs at the same time instance, each with a Tx chain. With CDM approach, a UE transmits different preambles in one RO. In this section, we discuss the different approaches and provide some simulation result.

Agreement - For multiple PRACH transmissions with same beam, at least ROs located at different time instances can be utilized for the transmissions. - FFS: whether/how the starting RB of ROs can be different at different time instances for multiple PRACH transmissions. - FFS: whether/how multiple PRACH transmissions located in the same time instance, e.g., for UEs with multiple Tx chains.

Though with two Tx chains transmitting simultaneously (each for a PRACH), a UE should conform with the RF requirements of its rated power class. That is to say, the total transmission power of the simultaneous PRACH transmissions should be same as that of a legacy single PRACH transmission. The total power of FDM and CDM in Figure 1(a) and 1(b) is the same as that illustrated in Figure 1(c). The use of multiple Tx chains in FDM or CDM approach could provide Tx diversity gains, compared with TDM approach using only one Tx chain. With FDM approach, the spatial diversity and frequency diversity naturally combine when Tx chains transmit in different frequency domain ROs, which can be beneficial in channels where frequency or spatial diversity is otherwise limited.

- Simultaneous PRACH transmissions have the total transmission power the same as that of a legacy single PRACH transmission.

- Simultaneous PRACH transmissions with different Tx chains by FDM or CDM has the potential to benefit from spatial diversity and frequency diversity, compared with TDM approach with a single Tx chain.

Figure 1: FDM, CDM, and TDM approaches of PRACH transmissions

The baseline for simulation comparison is TDM approach. Given different UE capabilities of supporting transparent Tx Diversity or antenna virtualization, a single PRACH can be transmitted in two ways. For a UE not supporting transparent Tx diversity, as illustrated in Figure 1(c), a PRACH is transmitted with one Tx chain. Such a UE needs at least one full power Tx chain in order to transmit a single PRACH at its rated power. A UE supporting Tx diversity can transmit the single PRACH from both Tx chains, each with half power, as shown in Figure 1(d). Note that not all UEs that have multiple Tx chains necessarily have transparent TxD capability, since this depends on UE implementation including antenna designs.

We configured a TDL-C channel and a gNB receive antenna configuration of two cross polarized elements in our simulation. Other simulation assumptions can be found in Appendix. The simulation result in Figure 2 is summarized in Table 1. The FDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1.5 dB and a single PRACH with two antennas by 0.5dB. It implies that the transmission with the second polarized antenna can bring about 1dB spatial diversity gain, and the transmission in a contiguous RO has a 0.5dB frequency diversity gain. The 1dB spatial diversity gain is also observed in CDM performance compared with that of a single PRACH transmission with one antenna. Note that two CDM PRACHs has nearly the same performance as a single PRACH transmitted with two Tx chains. This is expected, because the only difference between Figure 1(b) and 1(d) is the same or a different preamble is transmitted from the second antenna.

Figure 2: performance of FDM and CDM approaches vs. TDM approach

Table 1: performance of FDM and CDM approaches vs. TDM approach

| Baseline | Two FDM PRACHs, each transmitted with one antenna and half power | Two CDM PRACHs, each transmitted with one antenna and half power |
| --- | --- | --- |
| A single PRACH transmitted with one antenna and full power (for a UE not supporting Tx Diversity) | 1.5 dB | 1 dB |
| A single PRACH transmitted with two antennas, with half power for each antenna (for a UE supporting Tx Diversity) | 0.5 dB | 0 |

- Transmission with the second polarized antenna can bring about 1dB spatial diversity gain, and the transmission in a contiguous RO has a 0.5dB frequency diversity gain. In particular,

- FDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1.5 dB and a single PRACH with two antennas by 0.5dB.

- CDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1 dB and has nearly the same performance as a single PRACH transmission with two antennas.

Now we discuss the specification impact of FDM approach of multiple PRACH transmissions. Most of the agreements and conclusions for TDM PRACH transmissions can be reused for FDM. The differences between FDM and TDM include the determination of RA-RNTI and a group of frequency domain ROs for a RACH attempt.

The current proposal on RA-RNTI for TDM approach is that it is determined based on the first/last PRACH transmission. Similarly, for FDM approach, RA-RNTI can be determined based on the PRACH transmission in an RO with a small/large RO index.

Another specification impact is UE determination of a group of frequency domain ROs. With TDM approach, a UE determines the first RO, and the second RO is at a later time instance. This corresponds to the second RO with a larger RO index for FDM. Namely, with multiple frequency domain ROs associated with a selected SSB, after a UE determines the first RO, it determines an RO from the frequency domain ROs with a larger RO index for the second PRACH transmission. For example, with M frequency domain ROs associated with a selected SSB, if a UE selects RO#0 for the first PRACH transmission, it can select RO#M for TDM PRACH transmissions or RO#1 for FDM PRACH transmissions.

| M-1 | 2M-1 |
| --- | --- |
| … | … |
| 1 | M+1 |
| 0 | M |

- Most of the agreements and conclusions for TDM PRACH transmissions can be reused for FDM. The different specification impacts include the determination of RA-RNTI, e.g., based on an RO with a small/large RO index and a group of frequency domain ROs with different RO indexes.

Considering the 1dB spatial diversity gain and 0.5dB frequency diversity gain brought by simultaneous PRACH transmissions and the small specification impact, we have the following proposals.

- Support simultaneous PRACH transmissions for UEs with different Tx chains in Rel-18.

## RA-RNTI determination

Based on the agreement on one RAR window corresponding to multiple PRACH transmissions of one RACH attempt, only one RA-RNTI is needed for Msg2 RAR. It can be calculated based on the RO of a specific PRACH transmission, e.g., the first/last PRACH transmission. However, a UE transmits after handling the possible collisions. Collision handling affects PRACH transmissions and therefore RA-RNTI determination.

Agreement For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt. - FFS: the start position of the RAR window. - FFS: RA-RNTI.

More specifically, a UE determines a group of ROs for its determined number of PRACH transmissions before collision handling. If it cancels PRACH transmission in the first RO of the group, the second RO of the group actually carries the first PRACH transmission. Given ‘RA-RNTI is based on the RO of the first PRACH transmission’, gNB should calculate a RA-RNTI for this UE based on the second RO and another RA-RNTI for another UE based on the first RO, which has no collision concern. It requires that gNB is fully aware of the possible collision for all UEs, otherwise, there would be misunderstanding between gNB and UEs.

- If RA-RNTI is calculated based on the RO of a specific PRACH transmission, which is the outcome of collision handling, gNB is required to be fully aware of all UEs’ collision, otherwise there would be misunderstanding on RA-RNTI determination.

However, a gNB configures a cell-specific group of ROs for a number of PRACH transmissions without considering UEs’ possible collisions. If a UE cancels the PRACH transmission in the first RO of a group, the gNB is still able to determine the group of ROs for the UE based on any other detected PRACH transmission in the group. In this sense, RA-RNTI can be determined based on the RO of a specific PRACH transmission before collision handling. Even if gNB has no idea of UE transmission cancellation, there would be no misalignment between gNB and UE.

- RA-RNTI can be determined based on the RO of a specific PRACH transmission before collision handling.

## RACH resource configuration for PRACH transmissions

### Separate RO and shared RO

To differentiate multiple PRACH transmissions from single PRACH transmission, both separate ROs and separate preambles on shared ROs were agreed as working assumption in RAN1#112. In addition, the PRACH resources differentiation between different numbers of multiple PRACH transmissions is also needed.

Working Assumption For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs. - Note: Separate RO means that the RO is separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Working Assumption[ For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs. - Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. - FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning. Agreement For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions. - If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported. - FFS: details

An example of separate preambles in a shared RO and a separate RO for multiple PRACH transmissions is illustrated in Figure 3(a) and 3(b). Needless to say, adding a new preamble partition in an existing RO for a feature will reduce the number of preambles available for other features in the RO; configuring separate ROs for a feature has no impact on the users of existing ROs. Nevertheless, as can be seen in Figure 3(b), if all preambles in separate ROs are configured for only one feature, the RACH resource utilization is worth considering, for instance whether at a moment there are 64 UEs in a cell initiating Rel-18 multiple PRACH transmissions.

- If all preambles in separate ROs are configured for only one feature, the RACH resource utilization is worth considering.

Figure 3: shared RO, separate RO, and additional RO for multiple PRACH transmissions

To solve the issue of low utilization of separate ROs, Rel-17 RACH configuration supports the configuration of an additional RO with a Rel-17 IE additionalRACH-ConfigList, which is a separate RO (in addition to legacy ROs) and meanwhile a shared RO with preamble partitions configured for several features, including Rel-15 single PRACH transmission. Note that the Rel-17 IE is not visible to pre-Rel-17 UEs, and thereby the preamble partition of Rel-15 single PRACH transmission in the additional RO is available for UEs who recognize the IE.

- NR Rel-17 supports the configuration of an additional RO, which is a separate RO (in addition to existing ROs) and meanwhile a shared RO with preamble partitions configured for several features, including Rel-15 single PRACH transmission.

38.331 additionalRACH-ConfigList List of feature or feature combination-specific RACH configurations, i.e. the RACH configurations configured in addition to the one configured by rach-ConfigCommon and by msgA-ConfigCommon. The network associates all possible preambles of an additional RACH configuration to one or more feature(s) or feature combination(s). The network does not configure this list to have more than 16 entries. If both rach-ConfigCommon and msgA-ConfigCommon are configured for a specific FeatureCombination, the network always provides them in the same additionalRACH-Config.

The configuration of addition RO with a preamble partition for Rel-18 multiple PRACHs is illustrated in Figure 3(c). Preamble partitions of other features can also be configured in the additional RO for better preamble utilization. Actually, the configuration in Figure 3(b) can also be supported by Rel-17 RACH configuration, if gNB configures only one preamble partition in the addition RO, which is for multiple PRACH transmissions.

- With the Rel-17 RACH configuration, the configuration of an additional RO with partitions of multiple PRACHs and other features improves efficient preamble utilization and has no impact on legacy UEs.

- Confirm the working assumptions of separate ROs and separate preambles on shared ROs for multiple PRACH transmissions.

From RAN1’s perspective, configuration of RACH resources for multiple PRACH transmissions by separate preambles in a shared RO and by separate ROs is sufficient for gNBs to differentiate between single and multiple PRACH transmissions. It is beneficial from the perspective of configuration complexity and resource efficiency that RACH resources for legacy partitions and Rel-18 features, which require early UE indication, can be configured with a unified solution, compared with separate ROs configured only for multiple PRACH transmissions. It is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions and possibly other Rel-18 features.

- It is beneficial from the perspective of configuration complexity and resource efficiency that RACH resources for legacy partitions and Rel-18 features, which require early UE indication, can be configured with a unified solution, compared with separate ROs configured only for multiple PRACH transmissions.

- Send an LS to RAN2, informing RAN2 of RAN1’s decision on the support of separate preamble in a shared RO and separate ROs for multiple PRACH transmissions, and that it is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions and possibly other Rel-18 features.

### RO group

A term of RO group was agreed in RAN1#112. Since CDM approach of multiple PRACH transmissions may require only one RO, in our view, the discussion of RO group is mainly for TDM and FDM approaches. In this section, we discuss the issue of time span of an RO group.

- Focus the discussion of RO group on TDM and FDM PRACH transmissions.

Agreement For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions. Note 1: All ROs in one RO group is associated with the same SSB(s). Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission. Note 3: whether/how to define “RO group” in specification will be discussed separately Note 4: Valid RO(s) refers to what is defined in existing specification FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH. FFS: the time span of RO group. FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions. FFS: other details

A fundamental issue about RO group is whether an RO group consists of ROs for only one RACH attempt or multiple RACH attempts. This affects another issue on time span of RO group. In this section, we use the term ‘a set of ROs’ to refer to ROs for the multiple PRACH transmissions of one RACH attempt and discuss whether a set of ROs for a RACH attempt can cross multiple association periods.

According to 38.213, an association period is the smallest time span among those supported for at least one SS/PBCH block indexes to PRACH occasions mapping cycle. Figure 4 shows for PRACH configuration index 127 in FR2 TDD and 120KHz subcarrier spacing, there are 8 ROs in a PRACH configuration period of 10ms.

Figure 4: PRACH configuration index 127 in FR2 TDD

Let’s make some assumptions for an example:

- there are two SSBs, SSB1 and SSB2;

- onehalf is provided by ssb-perRACH-OccasionAndCB-PreamblesPerSSB, indicating one SSB is associated with two ROs;

- no FDM ROs

SSB to RO mapping is shown in Table 2. Since the ROs in the PRACH configuration period are sufficient for at least one SSB-RO mapping cycle, the association period is 10ms. For a selected SSB, e.g., SSB #1, one association period consists of two sets of ROs for two PRACH transmissions and one set of ROs for four PRACH transmissions, but it can’t accommodate a set of ROs for eight transmissions, which otherwise can be supported by ROs in two association periods. One may argue that a gNB can configure a smaller value for ssb-perRACH-OccasionAndCB-PreamblesPerSSB to support a larger number of PRACH transmissions in an association period. But with more consecutive ROs associated with one SSB, the price is a longer latency for UEs which select different SSBs to wait for PRACH transmissions. Especially, the number of SSBs in FR2 may be large. Therefore, it is beneficial that a set of ROs for multiple PRACH transmissions of a RACH attempt can span multiple association periods.

Table 2: SS/PBCH block indexes to PRACH occasions mapping with two SSBs

| RO index in a PRACH configuration period | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| associated SSB index | 1 | 1 | 2 | 2 | 1 | 1 | 2 | 2 |

- The number of ROs associated with an SSB in an association period may not be sufficient to support a large number of PRACH transmissions.

- A set of ROs for multiple PRACH transmissions of a RACH attempt can cross multiple association periods.

According to the agreement on one RAR for multiple PRACH transmissions mentioned in section 2.3, gNB and UE should know which several ROs form a set of ROs for a RACH attempt. In LTE eMTC, PRACH starting subframe is configured by eNB to indicate when the first PRACH transmission of a RACH attempt should happen. Although NR ROs may not spread evenly in time, a similar approach by RO index can be used. For an example of K PRACH transmissions, the index of the starting RO can be determined as e.g., mod(starting RO index, K)=0.

- For a RACH attempt, a UE can determine a set of ROs for the multiple PRACH transmissions with RO index.

## Multiple PRACH transmissions with different beams

Companies’ simulation results of multiple PRACH transmissions submitted to RAN1#112 were summarized as follows in the meeting. In short, a gain of 1dB, 1~3dB, 3.9~5dB and a loss of 1dB are observed for multiple PRACH transmissions with different beams, compared with multiple PRACH transmissions with the same wide beam.

Note: It is summarized by FL that for the same number of PRACH transmissions per source, - 1 source [Ericsson] shows that: Multiple PRACH transmitted by beam sweeping, where a UE has no prior knowledge of channel and sweeps Tx beams across 360 degrees horizontally and 180 degrees vertically, outperforms multiple PRACH transmissions with the same Tx wide beam (omni direction) by at least 1 dB, provided gNB configures only one SSB and receives PRACH with a wide beam. - 3 sources [ZTE, Nokia, vivo] show that: A gain from about 1~3 dB of beam sweeping is observed if a UE is able to direct at least one of its Tx beams in the right direction or to narrow down the azimuth and/or zenith range of 360 degrees and/or 180 degrees for beam sweeping compared with multiple PRACH transmissions with the same Tx wide beam. - 1 source [Huawei] shows that: compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9~5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam. - 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams far apart is 3 dB worse than PRACH repetition with single best beam - 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam is 1dB worse than PRACH repetition with single best beam. - 1 source [vivo] shows that: PRACH repetition via random beam directions performs 1 dB worse than PRACH repetition with omni beam.

Most simulations show a performance gain of PRACH transmissions with different beams, but a performance loss is also observed. A reason for divergent performances lies in the directions of different Tx beams. If a UE sweeps beams in a smaller angular sector than 360 degrees horizontally and 180 degrees vertically, the performance would depend on whether it directs at least one beam in the right direction. If yes, relatively large gain is observed. Otherwise, performance loss is possible, if all PRACH transmissions of a RACH attempt are in the wrong directions. However, according to 38.101-2, a UE should meet spherical coverage requirement, which is the minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE. The spherical coverage requirement guarantees a minimum performance of beam sweeping and can prevent a UE from sweeping beams only in the wrong directions, which EIRP could be lower than the minimum EIRP requirement. In contrast, without any down-selection of beams, a UE can sweep beams in all azimuth and zenith degrees covered by its panels. In this case, the 1dB gain provides the lower bound of beam sweeping's performance over PRACH repetitions with the same wide beam. Details can be found in section 5.

- Spherical coverage requirement guarantees the minimum performance of beam sweeping and can prevent a UE from sweeping beams only in the wrong directions, which EIRP could be lower than the minimum EIRP requirement.

- 1dB gain provides the lower bound of beam sweeping's performance over PRACH repetitions with the same wide beam.

- Support multiple PRACH transmissions with different Tx beams in Release 18

## Interplay with Msg3 repetition

Msg3 has been identified as a coverage bottleneck and was improved in Rel-17. We simulated Msg3 repetition with the same wide beam in FR2 and compare it with the baseline single PRACH transmission with a wide beam, and the simulation assumptions can be found in Appendix. It is observed in Figure 5 that even with a maximum of eight repetitions and inter-slot frequency hopping, the required SNR for Msg3 at 10% BLER is 1.7 dB higher than that of a single PRACH transmission at 1% mis-detection and is 4.5 dB more for 10% mis-detection rate. With Rel-18 PRACH enhancement, the performance gap between Msg1 and Msg3 would grow, and the overall success rate of RACH procedure is restricted by Msg3 performance. Therefore, Msg3 needs further enhancement to be on par with Rel-18 PRACH.

- In FR2, the required SNR for Msg3 with 8 repetitions and inter-slot frequency hopping at 10% BLER is 1.7 dB higher than that of a single PRACH transmission with a wide beam and 8 dB higher than a single PRACH transmission with the best beam for 1% missed detection. The gap could be 4.5 dB more for 10% mis-detection rate.

- With Rel-18 PRACH enhancement, the performance gap between Msg1 and Msg3 would grow. Msg3 needs further enhancement to be on par with Rel-18 PRACH.

Figure 5: Msg3 repetition and single PRACH transmission with a wide beam in FR2

The determination of UL Tx beam for Msg3 transmission is up to UE implementation. In Rel-18, the PRACH transmissions with different beams are received by gNB with different SNR. If a best UL beam can be indicated to UE for Msg3 transmission, it can benefit from the beamforming gain. More gains can be expected if Msg3 is repeated with the same best beam. The beam indication for Msg3 transmission can improve Msg3 performance for UEs incapable of beam correspondence. It helps UEs capable of beam correspondence as well, given the factors like time varying channel.

- Study how Msg3 performance can be improved by PRACH transmissions with different beams in Release 18

# Summary

In this contribution, we have discussed some issues related to multiple PRACH transmissions including:

- Determination of the number of PRACH transmissions

- Simultaneous PRACH transmissions with different Tx chains

- RA-RNTI determination

- RACH resource configuration

- PRACH transmissions with different beams

- Interplay with Msg3 transmission

In the previous sections we made the following observations:

Observation 1	The wide range of PRACH transmission power results in divergent PRACH performance, which will be exacerbated with Rel-18 multiple PRACH transmissions. It is reasonable to include UE transmission power in the determination of PRACH repetition factor.

Observation 2	If multiple PRACH transmissions are triggered when UE power headroom is below a threshold, a UE would make full use of all its transmission power first before using RACH resources which are shared by all UEs in the cell.

Observation 3	With SSB RSRP thresholds configured, UEs with different PCMAX will determine different power headroom thresholds.

Observation 4	SSB RSRP thresholds are implicitly represented by power headroom thresholds, in that UEs can calculate SSB RSRP thresholds based on the configured UE power headroom thresholds and their implementation PCMAX.

Observation 5	Regarding "at least SSB-RSRP threshold(s) are used" in the agreement, gNB can configure both SSB RSRP thresholds and power headroom thresholds. Configuring two sets of thresholds is unnecessary, since one set can be derived from the other.

Observation 6	Simultaneous PRACH transmissions have the total transmission power the same as that of a legacy single PRACH transmission.

Observation 7	Simultaneous PRACH transmissions with different Tx chains by FDM or CDM has the potential to benefit from spatial diversity and frequency diversity, compared with TDM approach with a single Tx chain.

Observation 8	Transmission with the second polarized antenna can bring about 1dB spatial diversity gain, and the transmission in a contiguous RO has a 0.5dB frequency diversity gain. In particular,

	FDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1.5 dB and a single PRACH with two antennas by 0.5dB.

	CDM approach of two PRACHs outperforms a single PRACH transmission with one antenna by 1 dB and has nearly the same performance as a single PRACH transmission with two antennas.

Observation 9	Most of the agreements and conclusions for TDM PRACH transmissions can be reused for FDM. The different specification impacts include the determination of RA-RNTI, e.g., based on an RO with a small/large RO index and a group of frequency domain ROs with different RO indexes.

Observation 10	If RA-RNTI is calculated based on the RO of a specific PRACH transmission, which is the outcome of collision handling, gNB is required to be fully aware of all UEs’ collision, otherwise there would be misunderstanding on RA-RNTI determination.

Observation 11	If all preambles in separate ROs are configured for only one feature, the RACH resource utilization is worth considering.

Observation 12	NR Rel-17 supports the configuration of an additional RO, which is a separate RO (in addition to existing ROs) and meanwhile a shared RO with preamble partitions configured for several features, including Rel-15 single PRACH transmission.

With the Rel-17 RACH configuration,

Observation 13	the configuration of an additional RO with partitions of multiple PRACHs and other features improves efficient preamble utilization and has no impact on legacy UEs.

Observation 14	It is beneficial from the perspective of configuration complexity and resource efficiency that RACH resources for legacy partitions and Rel-18 features, which require early UE indication, can be configured with a unified solution, compared with separate ROs configured only for multiple PRACH transmissions.

Observation 15	The number of ROs associated with an SSB in an association period may not be sufficient to support a large number of PRACH transmissions.

Observation 16	Spherical coverage requirement guarantees the minimum performance of beam sweeping and can prevent a UE from sweeping beams only in the wrong directions, which EIRP could be lower than the minimum EIRP requirement.

Observation 17	1dB gain provides the lower bound of beam sweeping's performance over PRACH repetitions with the same wide beam.

Observation 18	In FR2, the required SNR for Msg3 with 8 repetitions and inter-slot frequency hopping at 10% BLER is 1.7 dB higher than that of a single PRACH transmission with a wide beam and 8 dB higher than a single PRACH transmission with the best beam for 1% missed detection. The gap could be 4.5 dB more for 10% mis-detection rate.

Observation 19	With Rel-18 PRACH enhancement, the performance gap between Msg1 and Msg3 would grow. Msg3 needs further enhancement to be on par with Rel-18 PRACH.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A UE can determine the number of PRACH transmissions with the same Tx beam based on its power headroom and the configured UE power headroom threshold(s).

Proposal 2	Support simultaneous PRACH transmissions for UEs with different Tx chains in Rel-18.

Proposal 3	RA-RNTI can be determined based on the RO of a specific PRACH transmission before collision handling.

Proposal 4	Confirm the working assumptions of separate ROs and separate preambles on shared ROs for multiple PRACH transmissions.

Proposal 5	Send an LS to RAN2, informing RAN2 of RAN1’s decision on the support of separate preamble in a shared RO and separate ROs for multiple PRACH transmissions, and that it is up to RAN2 to decide how to configure RACH resources for Rel-18 multiple PRACH transmissions and possibly other Rel-18 features.

Proposal 6	Focus the discussion of RO group on TDM and FDM PRACH transmissions.

Proposal 7	A set of ROs for multiple PRACH transmissions of a RACH attempt can cross multiple association periods.

Proposal 8	For a RACH attempt, a UE can determine a set of ROs for the multiple PRACH transmissions with RO index.

Proposal 9	Support multiple PRACH transmissions with different Tx beams in Release 18

Proposal 10	Study how Msg3 performance can be improved by PRACH transmissions with different beams in Release 18

# References

RP-221858, “Revised WID on Further NR coverage enhancements”, China Telecom, 3GPP TSG RAN meeting #96, June 6th – 9th, 2022.

# Appendix

## Simulations of multiple PRACH transmissions with different beams

### Simulation assumptions

Although the working assumption provides some basic simulation assumptions, it doesn’t include how beams are generated and their orientation. We provide our rationale below.

In our simulation of beam sweeping, we assume the number of swept beams in the beam set is equal to the number of PRACH transmissions. The rationale is that with a higher number of PRACH transmissions, a UE sweeps narrower beams. More antenna elements are used to generate narrower beams for a larger number of PRACH transmissions. A wide beam is simulated with a dual-polarized antenna pair. For PRACH transmissions with the same best beam, the best beam is selected out of eight candidate beams. The UE antennas used in our simulation are as follows. To keep a fair comparison between PRACH transmissions with wide beam and narrow beam, the same TRP is assumed in our simulation.

PRACH transmissions with the same beam

- the same wide beam: 1x1x2 (1 dual polarized antenna pair)

- the same best beam: one of 8 candidate beams, which matches the channel best.

PRACH transmissions with beam sweeping

- with 2 beams: 2x1x2 (2 dual polarized antenna pairs)

- with 4 beams: 2x2x2 (4 dual polarized antenna pairs)

- with 8 beams: 2x2x2 (4 dual polarized antenna pairs), with a vertical oversampling rate of 2

Regarding beam orientation, according to the UE antenna configuration {2,2,2}, which is recommended for FR2 in TR38.830, in our simulation, there are always two vertical beams. To cover all the possible channel directions by beam sweeping, horizontal beams cover AOD -pi~pi evenly. We set the first horizontal beam with an angle of -pi. The angel sets are as follows.

- 2 PRACH transmissions

  - Azimuth angle set =a wide beam

  - Zenith angle set = [0, pi/2]

- 4 PRACH transmissions

  - Azimuth angle set = [-pi, 0], AOD degrees -180~180 evenly divided by 2 horizontal beams

  - Zenith angle set = [0, pi/2]

- 8 PRACH transmissions

  - Azimuth angle set = [-pi, -pi/2, 0, pi/2], AOD degrees -180~180 evenly divided by 4 horizontal beams

  - Zenith angle set = [0, pi/2]

### Simulation results

Since PRACH transmissions with different beams is the study part, we compare it with multiple PRACH transmissions with the same wide beam in Figure 1 and with PRACH transmissions with the same best beam in Figure 2. As observed in Figure 1, for a given number of PRACH transmissions, those with beam sweeping has about 1dB gain over those with the same wide beam.

- PRACH transmissions with different beams (beam sweeping) outperforms the transmissions with the same wide beam by about 1dB for the same number of transmissions.

Figure 1: beam sweeping vs. repeating the same wide beam

In Figure 2, a single PRACH transmission with the best beam has its performance between that of four PRACH transmissions with beam sweeping and eight PRACH transmissions with beam sweeping. This is consistent with our simulation assumption that the refined beam is the best beam from eight candidate beams. Another observation is that for the same number of PRACH transmissions, about 5dB loss is observed for beam sweeping option, compared with PRACH transmissions with the same best beam, which is generated by four times number of antennas.

- A single PRACH transmission with the best beam out of eight candidate beams has a performance between PRACH transmissions by sweeping four beams and eight beams.

- For the same number of PRACH transmissions, the transmissions with different beams (beam sweeping) has a loss of about 5dB compared with transmissions with the same best beam, which is generated by four times number of antennas.

Figure 2: beam sweeping vs. repeating the same best beam

In Figure 1 and Figure 2, about 2dB gain is observed when the number of PRACH transmissions doubles, and the gain slightly decreases when the number of PRACH transmissions increases. It is observed for PRACH transmissions with the same best beam, the same wide beam, or with different beams.

- About 2dB gain is observed when the number of PRACH transmissions doubles, and the gain slightly decreases when the number of PRACH transmissions increases. It is observed for PRACH transmissions with the same best beam, the same wide beam, or with different beams.

It should be noted that our simulation assumption of different Tx beams sweeping -pi~pi horizontally is based on an assumption that a UE has no any prior information of gNB direction. It is the worst case and provides the lower bound of beam sweeping's performance. However, in reality, if a UE has multiple panels, it is possibly able to select a panel with the best orientation before initiating random access. This is true even for a UE not supporting beam correspondence. Consider there are two antenna panels on the left and right size of a mobile phone, each generating Tx beams sweeping a 180-degree range, which is half of the angular range in our simulation assumption. After UE antenna panel selection, beam sweeping in a smaller angular range can reduce the unnecessary PRACH transmissions, which energy fail to reach gNB, and improve beam sweeping’s performance.

- Our simulation assumption of azimuth Tx beams sweeping a 360-degree angular section is the worst case and provides the lower bound of beam sweeping's performance. After UE antenna panel selection, beam sweeping in a smaller angular section can reduce the unnecessary PRACH transmissions, which energy fail to reach gNB, and improve beam sweeping’s performance.

## Simulation parameters

Table 1: Simulation parameters

| Frequency Range | FR2 |
| --- | --- |
| Carrier | 28GHz |
| TDD pattern | DDDSU |
| SCS | 120kHz |
| Channel model | CDL-A & TDL-C |
| Delay spread | 100 ns |
| UE antennas | [2 2 2] |
| ISD | 200 m |
| BS antennas | [1 2 2] |
| MCS index for Msg 3 | MCS=0 |
| Waveform | DFT-S-OFDM |
| FH offset | 50 PRBs |
| PRACH Format | B4 |
| Performance metric | Missed detection rate at 0.1% false alarm probability |
| UE speed | 3 km/h |