3GPP TSG-RAN WG1 #112 bis-e	R1-2303664

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Channel Structure for 2-step RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-Core |  |  |  |  |  | Date: |  |  | 2023-04-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 38.213 clause 8.1A, a mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH configuration. However, the IE MsgA-PUSCH-Config in TS 38.331 can configure two PUSCH resource groups simultaneously, and if two PUSCH resource groups are configured through this IE, according to the description in TS 38.213, these two PUSCH resource groups in one PUSCH configuration will share one mapping relationship for these two groups. However, in fact, according to the agreements in RAN1 #97 and #99 meeting below, if two groups are configured, the mapping is separately performed for each group and each preamble group mapped to one PUSCH resource configuration. Besides, according to the field descriptions of msgA-PUSCH-ResourceGroupA and msgA-PUSCH-ResourceGroupB in TS 38.331, if UE performs MsgA transmission using preambles group A PUSCH resources in group A should be used, and if UE performs MsgA transmission using preambles group B then PUSCH resources in group B should be used. To ensure this, the mapping for each group should be separately performed, that is the mapping unit for preamble and PRU is per PUSCH resource group, not per PUSCH configuration as one PUSCH configuration may configure two PUSCH resource groups. Therefore, the misunderstanding will be introduced if per PUSCH configuration is used for the mapping unit in the spec. MsgA-PUSCH-Config-r16 ::= SEQUENCE { msgA-PUSCH-ResourceGroupA-r16 MsgA-PUSCH-Resource-r16 OPTIONAL, -- Cond InitialBWPConfig msgA-PUSCH-ResourceGroupB-r16 MsgA-PUSCH-Resource-r16 OPTIONAL, -- Cond GroupBConfigured … } MsgA-PUSCH-Resource-r16 ::= SEQUENCE { msgA-MCS-r16 INTEGER (0..15), nrofSlotsMsgA-PUSCH-r16 INTEGER (1..4), nrofMsgA-PO-PerSlot-r16 ENUMERATED {one, two, three, six}, ... } msgA-PUSCH-ResourceGroupA MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group A. msgA-PUSCH-ResourceGroupB MsgA PUSCH resources that the UE shall use when performing MsgA transmission using preambles group B.. Agreements(#97) Support multiple msgA PUSCH configurations for a UE Agreements(#99) The preambles in a preamble group in each PRACH slot are mapped to the PRUs that are determined by the signaled single offset related to the PRACH slot per MsgA PUSCH configuration. |
| Summary of change: |  | Correct the preamble to PRU mapping unit to per PUSCH resource group configuration not PUSCH configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding will be caused, as according to the existing decription in the spec, there will be one common mapping relationship for group A and group B if two PUSCH resource groups are configured as these two groups in one PUSCH configuration, and these two groups share one mapping relationship, and it can not ensure preambles in group A mapped to PUSCH resource group A and preambles in group B mapped to PUSCH resource group B as the description in TS38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

8.1A	PUSCH for Type-2 random access procedure

For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. The PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the SCS configuration for the active UL BWP.

A UE does not transmit a PUSCH in a PUSCH occasion if the PUSCH occasion associated with a DMRS resource is not mapped to a preamble of valid PRACH occasions or if the associated PRACH preamble is not transmitted as described in clause 7.5 or clause 11.1. A UE can transmit a PRACH preamble in a valid PRACH occasion if the PRACH preamble is not mapped to a valid PUSCH occasion.

A mapping between one or multiple PRACH preambles and a PUSCH occasion associated with a DMRS resource is per PUSCH resource group configuration.

A UE determines time resources and frequency resources for PUSCH occasions in an active UL BWP from msgA-PUSCH-Config or separateMsgA-PUSCH-Config for the active UL BWP. If the active UL BWP is not the initial UL BWP and msgA-PUSCH-Config or separateMsgA-PUSCH-Config is not provided for the active UL BWP, the UE uses the msgA-PUSCH-Config or separateMsgA-PUSCH-Config provided for the initial UL BWP.

For mapping one or multiple preambles of a PRACH slot to a PUSCH occasion associated with a DMRS resource, a UE determines a first slot for a first PUSCH occasion in an active UL BWP from msgA-PUSCH-TimeDomainOffset that provides an offset, in number of slots in the active UL BWP, relative to the start of a PUSCH slot including the start of each PRACH slot. The UE does not expect to have a PRACH preamble transmission and a PUSCH transmission with a msgA in a PRACH slot or in a PUSCH slot, or to have overlapping msgA PUSCH occasions for a MsgA PUSCH configuration. The UE expects that a first PUSCH occasion in each slot has a same SLIV for a PUSCH transmission that is provided by startSymbolAndLengthMsgA-PO or msgA-PUSCH-timeDomainAllocation [6, TS 38.214].

<Unchanged parts are omitted>

where Npreamble=ceilTpreambleTPUSCH, Tpreamble is a total number of valid PRACH occasions per association pattern period multiplied by the number of preambles per valid PRACH occasion provided by rach-ConfigCommonTwoStepRA, and TPUSCH is a total number of valid PUSCH occasions per PUSCH resource group configuration per association pattern period multiplied by the number of DMRS resource indexes per valid PUSCH occasion provided by msgA-DMRS-Config.