3GPP TSG RAN WG1 #112bis-e										 	R1-2303667

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.1.4.1

Source:	NEC

Title:               	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

- Introduction

Rel-18 MIMO evolution for downlink and uplink will study and specify methods to support simultaneous multi-panel UL transmission. In the last meeting, the following conclusions/agreements have been achieved [1]. In this paper, we discuss UE panel management and UL precoding indication for multi-panel transmission.

Conclusion There is no consensus to support dynamic switching between STxMP SDM/SFN scheme and Rel-17 mTRP PUSCH TDM scheme. Conclusion There is no consensus to support 2 CWs in single-DCI based STxMP SDM scheme. Agreement Confirm the working assumption with the following updates: Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme with one CW - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH. Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN Agreement To support indicating DMRS ports in different CDM groups for layer combination {1+2} in SDM - Add new entry {0, 2, 3} to the DMRS table for the layer combination {1+2}; - This is optional UE capability for UE that supports sDCI based STxMP SDM Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner Agreement Among the two SRS resource sets configured for multi-DCI based STxMP PUSCH+PUSCH, the SRS resource set with lower set ID is the first SRS resource set.

- Discussion

  - UE panel capability reporting to support STxMP

UE needs to inform gNB that it is capable of simultaneous multi-panel transmission. In Rel-17, UE could provide, via UE capability reporting, its panel ‘type’ information to gNB by reporting capability value set which contains the maximum supported number of SRS antenna ports. For example, UE can report that it is equipped with one 2-port panel and one 4-port panel. Meanwhile, UE could report capability value set index along with beam report to indicate its panel ‘status’. For example, UE can report 4-port panel index along with CRI-RSRP report to let gNB be aware that 4-port panel is currently in use or to be used and is associated with reported CRI. Since no further details have been specified in Rel-17, panel management including panel switch might be heavily based on mutual understanding and implementations of gNB and UE.

Starting from what has been specified in Rel-17, two possible options can be considered to align the information between gNB and UE about whether UE could support STxMP.

- Option 1: simultaneous multi-panel can be treated as one special panel ‘type’, e.g., as one of capability value set contains the maximum supported number of SRS ports when multiple panels are involved in transmission at the same time. To be more specific, for UE with two 2-port panels, it can report capability value set 1: 2-port, capability value set 2: 4-port.

- Option 2: simultaneous multi-panel can be treated as a combination of multiple independent panels. To be more specific, for UE with two 2-port panels, it can report capacity value set 1: 2-port and a multiplication factor 2 to indicate that two panels can be used for transmission simultaneously.

Proposal 1: Support using UE capability value set to report panel information corresponds to simultaneous multi-panel.

  - CB/NCB based STxMP SDM PUSCH

Assuming that two SRS resource sets are configured for CB/NCB-based STxMP, it can be observed above that some restrictions might be not suitable to support simultaneous multi-panel transmissions and need to be removed, for example, the number of SRS resources in the two SRS resource sets could be different, e.g., NSRS,1 and NSRS,2, instead of mandating them to be the same.

Furthermore, SRI fields in DCI may need further enhancements, especially for the second SRI field for NCB. For example, for DCI scheduling NCB based STxMP SDM PUSCH, the following design can be considered.

- 1st SRI can be designed with a bitwidth: log2k=1min⁡(Lmax,SDM,  NSRS,1)NSRS,1k;

- 2nd SRI can be designed in a similar way: log2k=1min⁡(Lmax,SDM,  NSRS,2)NSRS,2k;

Where Lmax,SDM is single maximal number of layers (separate from Lmax for sTRP) that is applied to the first SRS resource set and the second SRS resource set, as agreed in the latest meeting [1]. When it is dynamically switched to S-TRP transmission, the corresponding SRI field may have a different bitwidth,

- SRI can be designed with a bitwidth: log2k=1min⁡(Lmax,STRP,  NSRS,i)NSRS,ik;

Where Lmax,STRP is single maximal number of layers for sTRP, and i-th SRS resource set is as indicated by the SRS resource set indicator. In addition, zero padding may be needed to control the length of DCI for SDM and S-TRP.

Proposal 2: The number of SRS resources in the two SRS resource sets can be different depending on UE panel capability.

Assuming two-port SRS resources are configured in both SRS resource sets for CB based STxMP SDM PUSCH, and a 2+2 layer combination is used for PUSCH transmission. PUSCH port(s) associated with the first SRS resource set is the same antenna port(s) as SRS port(s) in the first SRS resource indicated, for example, 1000 and 1001, but PUSCH port(s) associated with the second SRS resource set should be antenna port(s) in the second SRS resource indicated with a port ID offset, i.e., 1002 and 1003.

Proposal 3: Support an offset for the SRS port(s) in the SRS resource in the second SRS resource set.

For NCB, the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as pi = 1000 + i, according to current spec. When STxMP is applied, and two SRS resource sets (for example, each with 2 of 1-port SRS resources for 2-port panel) are configured, the port number determination needs some update, since it may be impacted by the order of SRS source index across two SRS resource sets. For example, if SRS resources 0 and 1 are associated with SRS resource set 1, and SRS resources 2 and 3 are associated with SRS resource set 2, it is possible that 4-port PUSCH (SDM scheme) is transmitted via port 1000, 1001, 1002, 1003, which is not aligned with current spec.

To support the NCB STxMP PUSCH port determination, we can consider that PUSCH port(s) associated with the second SRS resource set is the same antenna port(s) as SRS port(s) in the SRS resource indicated, where SRS port in (j+1)-th SRS resource in the second SRS resource set is Pj = 1000 + NSRS,1 + j, where NSRS,1 is the number of resources in the first SRS resource set.

Proposal 4: For NCB STxMP SDM, PUSCH port(s) associated with the second SRS resource set is the same antenna port(s) as SRS port(s) in the SRS resource indicated, where SRS port in (j+1)-th SRS resource in the second SRS resource set is Pj = 1000 + NSRS,1 + j, and NSRS,1 is the number of resources in the first SRS resource set.

  - Power control for STxMP PUSCH

According to recent RAN4 LS reply [4], both per-UE and per-panel power constraint assumptions can be considered.

Since RAN4 has not defined the concept of “panel”, following answers are provided based on earlier RAN1 assumption and the agreements in Multi-Rx WI as below: ‘Panel’ is defined as a group of antenna element that controls beam independently and has the following attributes: - Within a panel, one beam can be selected and used for DL reception or UL transmission. - Across different panels, multiple beams (each selected per panel) may be used for DL reception or UL transmission - ‘Beam’ is assumed to mean spatial filter associated with transmission or reception Regarding the questions, RAN4 is currently have the following answers: Question 1: From RAN4 perspective, is Assumption 1 is feasible? Answer: Yes. However, whether and how to introduce per-panel power limitation or similar concept and/or requirements in RAN4 is still under discussion. Question 2: From RAN4 perspective, is Assumption 2 is feasible? Answer: Yes. Question 3: In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? Answer: RAN4 confirm that existing UE RF requirements are framed so standards compliance implies regulation compliance (clause 6.5x in TS38.101-2). For any additional limitation like the sum over all panels of the per-panel power limitation for STxMP, would be defined in RAN4 if necessary. Question 4: If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can/shall be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? Answer: It is believed that both assumptions are feasible, and both assumptions shall be applied to a same UE. The per-panel power limitation would be defined if deemed necessary, and the per-UE power limitation would be applicable at all the time. ‘Limitation’ here applies to regulatory compliance rather than a configured power requirement. RAN4 can confirm that it can define the configured transmitted power requirement per-TCI for STxMP, but other solutions are not precluded. Additionally, RAN4 has agreed that ‘Panel’ shall not be explicitly used in a requirement to ensure maximum flexibility for different UE implementations. An Annex is also provided for some background information related to power limitation specified and discussed in RAN4 for FR2。

Observation 1: Both per-panel and per-UE power constraints are feasible, and both assumptions shall be applied to the same UE.

Since per-panel power limitation (or in RAN4 LS, per-TCI power requirement) might be introduced to support STxMP, the following aspects may be impacted.

- Pcmax configuration: per panel Pcmax might be configured to control the maximum UL power per panel

- PHR calculation: two Pcmax and two PHs might be needed in PHR, one for each panel

- DMRS-PUSCH power ratio: PUSCH-to-DMRS EPRE ratio is based on the number of DM-RS CDM groups without data per panel

- PUSCH-to-PTRS power ratio: PUSCH-to-PTRS power ratio per layer per RE is based on the number of PUSCH layers assigned to one panel

Proposal 5: Study STxMP power control with both per UE and per panel power constraints, including the possible impacts on Pcmax configuration, PHR, PUSCH-to-DMRS EPRE ratio, PUSCH-to-PTRS power ratio per layer per RE.

  - Consideration of CG based PUSCH for STxMP

It was agreed to support CG+CG STxMP PUSCH and for Type 1 CG-PUSCH, one SRS_resource_set_index value is configured in ConfiguredGrantConfig and srs-ResourceIndicator/precodingAndNumberOfLayers correspond to the SRS resource set. It seems that at least two CG configurations are needed to support CG+CG STxMP. Since multiple CG configurations may be configured in one BWP of a serving cell, the association between those two CG configurations which can be transmitted simultaneously needs to be indicated to UE, if more than two CG configurations are configured. For example, a CGForSTxMPLinkID can be provided for each configured grant configurations.

Proposal 6: Support to indicate the association between two CG configurations for CG based STxMP PUSCH+PUSCH.

- Conclusion

In this contribution, we provided our views on UL precoding indication for multi-panel transmission, and we have the following proposals:

Proposal 1: Support using UE capability value set to report panel information corresponds to simultaneous multi-panel.

Proposal 2: The number of SRS resources in the two SRS resource sets can be different depending on UE panel capability.

Proposal 3: Support an offset for the SRS port(s) in the SRS resource in the second SRS resource set.

Proposal 4: For NCB STxMP SDM, PUSCH port(s) associated with the second SRS resource set is the same antenna port(s) as SRS port(s) in the SRS resource indicated, where SRS port in (j+1)-th SRS resource in the second SRS resource set is Pj = 1000 + NSRS,1 + j, and NSRS,1 is the number of resources in the first SRS resource set.

Observation 1: Both per-panel and per-UE power constraints are feasible, and both assumptions shall be applied to the same UE.

Proposal 5: Study STxMP power control with both per UE and per panel power constraints, including the possible impacts on Pcmax configuration, PHR, PUSCH-to-DMRS EPRE ratio, PUSCH-to-PTRS power ratio per layer per RE.

Proposal 6: Support to indicate the association between two CG configurations for CG based STxMP PUSCH+PUSCH.

- References

- Chair's notes, RAN1#112, February 27th – March 3rd, 2023

- Chair's notes, RAN1#111, November 14th – 18th, 2022

- Chair's notes, RAN1#110bis-e, 10th – 19th October, 2022

- R1-2302265(R4-2303494), Reply LS on UE power limitation for STxMP in FR2, Athens, Greece, Feb. 27th – Mar. 3rd, 2023