3GPP TSG RAN WG1 #112bis-e										 	R1-2303668

e-Meeting, April 17th – 26th, 2023

Agenda item:	9.2.1

Source:	NEC

Title: 	Discussion on general aspects of AI ML framework

Document for:	Discussion and Decision

- Introduction

In RAN1 112 meeting, the following agreements and working assumptions are achieved [1]. In this contribution, we provide our views on general aspects of AI/ML framework, in particular on lifecycle management (LCM).

Agreement To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models. Case; Model delivery/transfer; Model storage location; Training location. y; model delivery (if needed) over-the-top; Outside 3gpp Network; UE-side / NW-side / neutral site. z1; model transfer in proprietary format; 3GPP Network; UE-side / neutral site. z2; model transfer in proprietary format; 3GPP Network; NW-side. z3; model transfer in open format; 3GPP Network; UE-side / neutral site. z4; model transfer in open format of a known model structure at UE; 3GPP Network; NW-side. z5; model transfer in open format of an unknown model structure at UE; 3GPP Network; NW-side. Note: The Case definition is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. Note: The Case definition is NOT intended to introduce sub-levels of Level z. Note: Other cases may be included further upon interest from companies. FFS: Z4 and Z5 boundary Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model Agreement - AI/ML-enabled Feature refers to a Feature where AI/ML may be used. Agreement - For functionality identification, there may be either one or more than one Functionalities defined within an AI/ML-enabled feature. Agreement For 3GPP AI/ML for PHY SI discussion, when companies report model complexity, the complexity shall be reported in terms of “number of real-value model parameters” and “number of real-value operations” regardless of underlying model arithmetic.

- Discussion

  - Lifecycle management (LCM)

LCM Framework

RAN1#112 [1] discussed several aspects about the architecture of the AI/ML LCM where several companies brought their understanding to the table. We think that following architecture diagram provides a substantial understanding of the AI/ML framework and does not complicate the discussion by unnecessary details.

The location of each of the indicated nodes can be defined based on the use case. For e.g., for the case of network-based monitoring and two-sided AI/ML model, the Model Inference and Model Management blocks are located at the gNB.

One point to note is that this is a functional framework of logical blocks where one or more logical blocks can be located within a single entity. For example, a UE may contain Data Inference, Data Collection, and Model Storage for the case of UE sided beam prediction. Further, some of these blocks may be out of 3GPP scope depending on the eventual agreement (e.g., Model Training).

Further, the interactions between the given blocks may or may not have impact on 3GPP signalling depending on the location of these blocks. For example, the “monitoring data” interaction between UE located Data Collection and gNB located Model Management may require 3GPP signalling enhancements. But for UE based monitoring, this may or may not require any standards assistance. Hence, further discussion may be required for the details of the framework per AI/ML use case.

Figure 1 AI/ML Framework Description

Proposal 1: Adopt Figure 1 as starting point for discussion for AI/ML functional framework and discuss the potential enhancements to support different use cases/variations of AI/ML models.

Model-ID-based LCM and Functionality-based LCM

If more than one AI/ML models are available for the same functionality, for example, different AI/ML models have been trained for different configurations or scenarios, LCM would be extended to multi-model management, including model activation, deactivation, selection, switching, and so on. Model-ID can be quite useful to control the signalling overhead. Even the AI/ML models are NW-sided models, model-ID is also needed to let UE know that the applied AI/ML model has been changed/updated and corresponding measurement and report for UE to provide for the input of model inference may also be updated.

Proposal 2: Support model-ID based lifecycle management also for NW-sided model.

Proposal 3: Model-ID can be explicitly included in LCM signaling to activate/deactivate/switch/select a specific AI/ML model

For a two-sided AI/ML model (e.g., for the use case of CSI compression), since NW part (e.g., CSI reconstruction part) and UE part (e.g., CSI generation part) may not be trained jointly, for example, via model training collaboration type 2 or 3, and multiple NW parts and UE parts may be trained even for one AI/ML model. In this case, in addition to model-ID, identification of NW parts and UE parts may also be useful, and before model inference, additional procedures may apply to select one of NW parts to be best matched with one of the UE parts.

Proposal 4: For a two-sided model, study methods to align NW part and UE part of one AI/ML model, e.g., assigning NW part ID and UE part ID in addition to model-ID.

Model training, model interference and data collection

While for offline AI/ML model training the datasets can be established and maintained in a non-real-time manner, it is our understanding that online training, or at least online fine-tuning should also consider online data collection. Therefore, methods to include real-world data into datasets for AI/ML model training or model update, particularly the testing datasets, should be also studied. At least two methods can be considered for dataset construction for online AI/ML model training, one is to collect measurement and reported data via legacy BM framework, CSI framework and positioning framework, the other is to adopt Generative Adversarial Networks (GAN)-like data sample generation and to validate the generated data sample by measurement and reporting via air-interface. Studies are needed to identify whether the legacy CSI/BM/positioning framework can be sufficient for online data collection.

Proposal 5: Study whether and how the legacy CSI framework, BM framework and positioning framework can provide sufficient data for model training (including fine-tuning) and model inference.

On the other hand, AI/ML model inference accuracy is one of the fundamental criteria for the performance of AI/ML models. To assess the accuracy, comparisons between AI/ML inference output and the ‘ground truth’ are needed. However, one reasonable assumption is that a reduced version of reference signals and correspondingly a reduced version of measurement and reports will be applied during the model inference stage, which may cause difficulties to obtain the ‘ground truth’. For example, in a compressed CSI feedback use case, there might be no original CSI report during model inference stage. Another example, in a beam selection use case, with less BM RS transmitted in the model inference stage, there might be no chance to measure the real optimal beam. Therefore, studies are needed to identify methods of ‘ground truth’ data collection to compare the model inference results and the real-world results. As a starting point, we may need to study whether and how the legacy CSI framework, BM framework and positioning framework can be used for ground truth data collection, for example, by configuring periodic measurement and report without AI/ML during model inference stage.

Proposal 6: For model monitoring based on inference accuracy, study methods of ‘ground truth’ data collection.

Multiple alternatives can be considered for the legacy CSI/BM/positioning framework to support AI/ML model. A hierarchical structure could be useful to link the AI/ML framework and the legacy CSI/BM/positioning framework. Take CSI as an example, one alternative is to configure one legacy CSI ReportConfig for each stage of LCM of an AI/ML model, as shown in the figure (a) below. The other is to use RRC to reconfigure the target AI/ML model and LCM stage for a legacy CSI ReportConfig, as shown in the figure (b) below.

Figure 2 using legacy CSI framework to collected report needed for an AI/ML model

(a) dedicated ReportConfig for LCM; (b) defining the target LCM for one ReportConfig

Clearly, above alternatives have their own pros and cons in terms of complexity, flexibility, requirement on UE capability, latency, as well as the spec impact. Therefore we have the following proposal.

Proposal 7: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

Model selection, activation, deactivation, switching, and fallback

As discussed, if more than one AI/ML models are available for the same functionality, they may be trained specifically for different configurations or different scenarios. Adaptive selection of applied AI/ML model can greatly reduce the required LCM signalling on explicit model selection, activation, deactivation, switching, and fallback. For example, if different AI/ML models are available for LOS/NLOS, high/low SINR, high/low velocity, more/less antenna ports/beams respectively, model switching can be adaptive to the change of configurations or the detection of change of the scenarios.

Proposal 8: Study adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling.

As most mechanisms currently assumed for model selection/activation/deactivation is by using UE dedicated signalling which can result in significantly high overhead and increased burden on network management, we might also consider the mechanisms by which some of AI/ML models can be activated autonomously e.g. based on broadcast signalling. For example, UE can get information about the AI/ML model to be activated through broadcast signalling (e.g., SIB) and can activate the model on its own if no additional information is required from the network for model selection. This would be beneficial for AI/ML models whose parameters are mainly derived based on network characteristics for e.g. beam prediction. Even though details of the signalling can be discussed in RAN2, RAN1 can still study and recommend applicability of autonomous AI/ML model activation.

Proposal 9: Study autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

Model monitoring

As discussed, if more than one AI/ML models are available for the same functionality, LCM would be extended to multi-model management. On the other hand, usually one of the AI/ML models is applied for model inference. When the applied AI/ML model cannot meet the performance requirement, for example, model failure is declared, model switching might be needed. To determine which model would be the better one to use, a test procedure can be done by comparing the model inference performance of multiple candidate models. To accelerate the procedure, monitoring of multiple AI/ML models for the same functionality can be considered. In this case, model switching can be done right after or even before the real model failure occurs, by comparing the monitoring results of multiple candidate models.

Proposal 10: Consider to support model monitoring of multiple AI/ML models for the same functionality.

For AI/ML model performance feedback, methods should be identified to support the monitoring of AI/ML model performance and the required feedback signalling. In RAN1 110-bis meeting [3], companies agreed to study the following methods.

Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system performance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE

It can be observed that each method has its own application scenario, since the cause of model failure may be different. Supporting more than one of monitoring methods seems inevitable. If the monitoring method is determined at NW, configuration information of model monitoring method should be provided to UE. If the monitoring method is determined at UE solely, together with the model monitoring results, it may report the applied monitoring method, or it may report the cause of model failure if the model monitoring results implying the model has been failed.

Proposal 11: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Moreover, it is expected that for UE sided monitoring, if the model fails or if model is not performing well then, the report may need to be provided to the network with minimal latency, especially for the AI/ML models which are delay sensitive (e.g., channel reporting). Hence, L1/L2 based mechanisms framework should be defined to allow reporting from UE to network about model failure or deteriorating model performance to reduce the delay as much as possible. If L1/L2 based mechanism is defined then signalling overhead is a serious concern and hence further discussion is required on the information content which can be included within L1/L2 signalling.

Proposal 12: Study L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

Model update

In the process of model monitoring, when the model performance (e.g., accuracy of model inference) is detected to deteriorate, it may be essential to perform model updating. For example, UE or gNB can optimize the existing model in combination with the latest local or field data (e.g., fine-tuning). Optionally, UE or gNB can switch to another model (e.g., for better generalization performance). Specifically, the first method requires longer processing delay, but the performance of the updated model may be better. For the second method, although the performance of the new model switched may not be as good as that of the optimized model, it can save the delay of model updating (e.g., fine-tuning). But multiple models may need to be allocated to the target case in advance for the second method. In addition, there may be other potential methods to achieve model updating. Therefore, the reasonable methods on model updating should be studied to ensure normal model inference and the updating of AI/ML model should cause as less interruption of AI/ML model inference as possible.

Proposal 13: Study the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

Model transfer

Three different collaboration levels are defined for now, mainly based on with/without model transfer. There has been significant discussion about the applicability of level z collaboration which requires model transfer within scope of 3GPP. Although, model transfer may not hold significance for the case when model training is performed at UE with minimal network involvement, we believe that model transfer with some 3GPP network assistance could be beneficial for the case of two-sided model or when some form of online training is performed for an AI/ML model which requires network involvement and control during training. For such cases, network is expected to be aware of the specifics of AI/ML model which would run at UE and hence a natural way to transfer the model to UE should have at least some form of network involvement. Hence, model transfer with 3GPP network assistance should be studied at least for two-sided AI/ML model or when online training is required for an AI/ML model.

Proposal 14: Study AI/ML model transfer with 3GPP network assistance at least for the case of two-sided AI/ML model or when online training is required for an AI/ML model.

Even though the signalling for model transfer should be handled by RAN2, RAN1 is expected to be involved in at least studying and agreeing on the format to be used for model transfer. For model transfer, there are several options which have been discussed till now including vendor specific format, executable format and any other open format e.g. Open Neural Network Exchange (ONNE) or 3GPP defined format. While vendor specific format and executable format allows usage of vendor specific proprietary algorithms, they do not allow any 3GPP network control on model development and fine tuning. Using an open format removes this disadvantage where 3GPP network has the understanding of model inputs and parameters and if any issue is identified by the gNB/3GPP network during model operation related to an input parameter then appropriate hyperparameters can be adjusted to allow for correct model (re)training. Hence, our preference is to support open AI/ML format for model transfer. The details for the format e.g. whether 3GPP defined or whether an existing format like ONNE should be used and supporting vendor specific proprietary algorithms, can be further discussed.

Proposal 15: Study AI/ML model transfer using open AI/ML format. FFS details of open format, support of vendor specific algorithms.

UE Capability

In the last meetings, companies concluded that RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. While those KPIs are needed for performance evaluation, they are also closely related to implementation. Especially for AI/ML model deployed at UE, it may not be proper to ask UE to reveal its computation capability in TOP/FLOP/MACs. UE capability of conducting AI/ML operations is obviously bounded by its implemented hardware, software and power consumption and so on. It is also possible that the AI/ML computation capability is shared among AI/ML for air-interface and AI/ML for other non-communication functions. Excessive AI/ML computations may drain UE battery and also cause the overheating issues on the device and therefore degrade the communication performance.

As discussed above, AI/ML models for different use cases may be implemented simultaneously on the same UE. For example, it is nature for a MIMO UE to support both AI/ML models for CSI and for BM. Studies are needed to assign the limited AI/ML capability to different use cases. Instead of exact values in terms of ML TOP/FLOP/MACs, a logic concept of AI/ML processing units (APUs) can be used in spec to reflect UE capability on AI/ML operations. UE could report the supported number of APUs via capability reporting. Each AI/ML model may occupy different number of APUs depending the size of AI/ML model and the total number of APUs occupied simultaneously cannot exceed the UE supported maximum number. The relationship between ML TOP/FLOP/MACs and one APU can be UE specific based on implementation and not shared with NW.

Proposal 16: Introduce AI/ML processing units (APUs) to reflect UE capability of AI/ML operations.

In addition, as discussed in Section 2.1.5 on model monitoring, considering multiple candidate AI/ML models to be monitored in parallel, UE capability on the supported number of models for parallel model monitoring is needed. Meanwhile, considering the different candidate monitoring methods, UE capability on the supported methods of model monitoring can also be reported.

Proposal 17: Support UE capability reporting on the supported number of AI/ML models for parallel model monitoring and on the supported methods for model monitoring.

  - Collaboration level between network and UE

Further, to our understanding, many use cases to be discussed in RAN1 can be categorized into level y: signalling-based collaboration without model transfer, which suggests that this definition of collaboration level may be too broad and cannot represent the different characteristics of AI/ML models applied for different use cases.

For the selected use cases, it can be observed that at least two collaboration (sub-)levels can be studied without consideration of model transfer (e.g., for the level y).

- One-sided AI/ML model. AI/ML models are deployed solely at gNB or at UE but exchange of assistance information is required. For example, in beam management use case, to predict future beams, AI/ML model might be deployed at gNB side and UE may need to feedback the correctness of predicted beams. This type of AI/ML operation requires relatively loose collaboration between UE and gNB.

- Two-sided AI/ML model. AI/ML models are split into multiple parts and both gNB and UE are involved in training the AI/ML model. For example, in the CSI feedback enhancement use case, to reduce CSI feedback overhead, autoencoder-like AI/ML model based compression and recovery can be applied, where UE is the encoder, gNB is the decoder and a joint AI/ML model training and a joint AI/ML model inference are expected. This type of AI/ML operation requires tight collaboration between UE and gNB since intermediate data (e.g., compressed CSI/PMI) needs to be exchanged.

Proposal 18: Support to define network-UE collaboration levels based on one-sided AI/ML model or two-sided AI/ML model.

- Conclusion

In this contribution, we provided our views on Rel-18 study on AI/ML for air-interface, and we have the following proposals:

Proposal 1: Adopt Figure 1 as starting point for discussion for AI/ML framework and discuss the potential enhancements to support different use cases/variations of AI/ML models.

Proposal 2: Support model-ID based lifecycle management also for NW-sided model.

Proposal 3: Model-ID can be explicitly included in LCM signaling to activate/deactivate/switch/select a specific AI/ML model

Proposal 4: For a two-sided model, study methods to align NW part and UE part of one AI/ML model, e.g., assigning NW part ID and UE part ID in addition to model-ID.

Proposal 5: Study whether and how the legacy CSI framework, BM framework and positioning framework can provide sufficient data for model training (including fine-tuning) and model inference.

Proposal 6: For model monitoring based on inference accuracy, study methods of ‘ground truth’ data collection.

Proposal 7: Study a hierarchical structure to configure the linkage between the AI/ML framework and the legacy CSI/BM/positioning framework.

Proposal 8: Study adaptive model selection, activation, deactivation, switching, and fallback without explicit LCM signaling

Proposal 9: Study autonomous model activation procedure for AI/ML models with assistance of network broadcast signaling.

Proposal 10: Consider to support model monitoring of multiple AI/ML models for the same functionality.

Proposal 11: Information of model monitoring methods can be provided to NW or UE. If model failure occurs, the cause of model failure may also be reported.

Proposal 12: Study L1/L2 based mechanism for UE reporting of model failure for UE sided model monitoring.

Proposal 13: Study the methods to update AI/ML model with minimum interruptions of AI/ML model inference.

Proposal 14: Study AI/ML model transfer with 3GPP network assistance at least for the case of two-sided AI/ML model or when online training is required for an AI/ML model.

Proposal 15: Study AI/ML model transfer using open AI/ML format. FFS details of open format, support of vendor specific algorithms.

Proposal 16: Introduce AI/ML processing units (APUs) to reflect UE capability of AI/ML operations.

Proposal 17: Support UE capability reporting on the supported number of AI/ML models for parallel model monitoring and on the supported methods for model monitoring.

Proposal 18: Support to define network-UE collaboration levels based on one-sided AI/ML model or two-sided AI/ML model.

- References

- Chair's notes, RAN1#112, February 27th– March 3rd 2023

- Chair's notes, RAN1#111, November 14th – 18th, 2022

- Chair's notes, RAN1#110bis-e, October 10th – 19th, 2022