3GPP TSG RAN WG1 #112bis-e		 R1-2303673

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.11.3

Source:	NEC

Title:              	Discussion on L1 signal design and procedure for LP-WUS

Document for:	Discussion and Decision

Introduction

In RAN#97e, a revised version of SID on low-power wake-up signal and receiver for NR was approved [1]. In RAN1#112, the following agreements on L1 signal and procedures are achieved:

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol) Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2 Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded. Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

In this Tdoc, we provide our views on the L1 signal design and the procedure for low power wake up signal.

Discussion

  - L1 signal design

In order to achieve extremely low power consumption (e.g., several µW) of the Low Power Wake-Up Receiver (LP-WUR), power hungry components (e.g., high-frequency and high-precision oscillator, ADC with high sampling rate and high resolution, advanced baseband processor, etc.) should be avoided. On-off Keying (OOK) is a promising candidate technique for LP-WUS design due to its very simple receiver (e.g., based on RF envelop detector) and the very low power consumption, which is already used in some systems, e.g., the RFID and IEEE 802.11ba.

In current NR, OFDM and DFT-s-OFDM based waveform are used for DL and UL transmissions. As indicated in the SID, this study will not require existing signals to be used as LP-WUS. However, it is beneficial to generate the OOK symbols based on the legacy OFDM or DFT-s-OFDM framework. For example, if the LP-WUS and normal DL channels (e.g., PDCCH, PDSCH) can be generated based on a same OFDM waveform framework, they will be orthogonal in frequency domain and guard band between LP-WUS and other channels may be avoided or reduced. In addition, in time domain, the boundaries of LP-WUS and other DL symbols can also be well aligned if OFDM based OOK generation is used, it is beneficial for resource utilization.

In IEEE 802.11ba, a multi-carrier OOK (MC-OOK) is used. In general, a MC-OOK on-symbol (i.e., the logical “1”) is a CP-OFDM symbol with non-zero power, and a MC-OOK off-symbol (i.e., the logical “0”) is a CP-OFDM symbol with zero power. However, the following issues should be considered if similar method is used in NR:

- The CP-OFDM symbol of NR is much longer than WiFi for some SCSs (e.g., ~71us for NR 15KHz SCS), using such a long OOK symbol will lead to low spectrum efficiency. Therefore, it should be supported that a signal OFDM symbol transmits M bits, and M >1.

- Multiple SCSs are supported in NR, however, in order to avoid increasing the complexity of OOK receiver, the OOK symbol length is better to be independent to OFDM SCSs, this require the OOK signal generation can be scaled easily for different SCSs. Based on the DFT-s-OFDM waveform generation framework, it is very flexible to generate OOK symbols with required symbol duration for different SCSs by adjusting the number of samples before the DFT transforming.

- For an OFDM receiver, CP should be removed at the receiver side since it is mainly used for the inter-symbol-interference (ISI) cancellation. However, for a LP-WUR, ISI is not a problem. If the similar CP removing is used, the complexity and power consumption may be increased. On the other hand, if CP is dropped at transmitter side, the benefit of orthogonality with legacy OFDM transmission will disappear. Therefore, it is beneficial to enhance the CP as a useful part of the OOK symbol, e.g., a CP with non-zero power can be seen as a part of an OOK-ON symbol, and a CP with zero power can be seen as a part of an OOK-OFF symbol. Fig. 1 gives an example of the utilization of CP part, wherein a CP-OFDM symbol transmits 4 bits with 4 OOK symbols, and CP is utilized as a prat of the first OOK symbol.

Fig.1 an example of 4 OOK symbols transmitted by a signal OFDM symbol

Observation 1: DFT-s-OFDM based OOK symbol generation is beneficial for the scaling of OOK symbol duration for different SCSs.

Observation 2: dropping CP at transmitter side may lose the orthogonality between LP-WUS signal and legacy OFDM symbol.

Observation 3: CP removing at receiver side may increase the complexity and power consumption of LP-WUR and lead to low spectrum efficiency.

Observation 4: CP can be enhanced as a useful part of OOK symbol, e.g., a CP with non-zero power can be seen as a useful part of an OOK-ON symbol, and a CP with zero power can be seen as a useful part of an OOK-OFF symbol.

Proposal 1: study DFT-s-OFDM based OOK symbol generation, to support transmit multiple bits by a single OFDM symbol.

Proposal 2: study CP enhancement to utilize the CP as a useful part of OOK symbols.

  - Procedures related to LP-WUS

For a UE in low power mode, the main radio may be switched-off and UE only needs to monitor the LP-WUS signal by a LP-WUR. Synchronization and mobility should be maintained for better receiver performance and connectivity robustness. There may be two options for UE to maintain its synchronization and mobility:

- Option 1: UE maintains synchronization and mobility based on SSB: In this case, UE may have to wake up and switch on main radio frequently to refine its time/frequency synchronization and perform RRM measurement. However, the power consumption may be increased significantly, therefore measurement relaxation should be considered.

- Option 2: UE maintains synchronization and mobility based on a low power synchronization signal (LP-SS): in this case, a new synchronization signal based on the OOK modulation (i.e., LP-SS) should be introduced. However, the requirement and design principle may be different from the synchronization signal for main radio (i.e., PSS or SSS):

  - UE may not need to maintain precise time synchronization like the SSB/TRS provides, since the OOK detection is more robust to time error and frequency offset than OFDM detection

  - UE may not need to acquire at least the slot index and OFDM symbol index (or even subframe index or SFN) through the LP-SS detection, since they may be useless for OOK detection.

  - Since the very simple modulation and receiver, the coverage of LP-SS may be not as good as other signals/channels associated with main radio, e.g., SSB or paging channel. Therefore, in order to avoid missing the wake-up indication, UE needs to be aware of whether it is still in coverage of LP-SS and LP-WUS signals.

Proposal 3: study synchronization design for UE in low power mode, consider low power synchronization signal (LP-SS) based synchronization.

Proposal 4: study RRM measurement for UE in low power mode, consider LP-SS based RRM and RRM relaxation for SSB based measurement.

For the design of wake up procedure, the wake-up mechanisms for CDRX in NR and the WUS/GWUS design in NB-IoT/eMTC can be references.

In NR, CDRX is defined for UE power saving in the connected mode, and DCI format 2-6 is used as WUS signal which is also called DCP (DCI with CRC scrambled by PS-RNTI). There are three cases UE should wake-up and start the on-duration timer:

- If UE receives a DCP, and the DCP indicates to start the timer;

- If all the occasions for DCP are not available, or a specific type of PDCCH has already been monitored (e.g., PDCCH for link recovery, random access response)

- If no DCP is received and the network configured a true value for the parameter ps-Wakeup

On the other hand, there are two cases UE should not wake-up and start the on-duration timer:

- If UE receives a DCP, and the DCP indicates to NOT start the timer;

- If no DCP is received and the network did NOT configure a true value for the parameter ps-Wakeup

In LTE eMTC and NB-IoT, wake-up signal is introduced for UE power saving in idle mode. The WUS for NB-IoT and eMTC is a ZC sequence which is generated based on the position of the associated paging occasion. If group WUS is configured, the WUS signal is also based on the group ID. When DRX or eDRX is used and the UE detects WUS, the UE shall monitor the following one or more POs. If the UE does not detect WUS the UE is not required to monitor the following PO(s). If the UE missed a WUS occasion (e.g. due to cell reselection), it monitors every PO until the start of next WUS or until the PTW ends, whichever is earlier.

For the wake-up procedure of LP-WUS scenario, similar procedures as NB-IoT/eMTC and the CDRX in NR can be considered. There may be three options for the wake-up procedure design:

- Option 1: a single sequence is used for wake-up indication for a UE or a UE group, UE determines whether to wake-up based on whether the sequence is detected.

- Option 2: two sequences are used for wake-up indication for a UE or a UE group, i.e., a positive LP-WUS sequence and a negative LP-WUS sequence, UE determines whether to wake up based on which sequence it detected.

- Option 3: a LP-WUS with a modulated code block is used for wake-up indication for a UE or a UE group. UE determines whether to wake-up based on whether its UE identity or group identity is carried in the code block.

In general, sequence based method may have better coverage than the code block based method, however it only conveys limited information. The three options should be evaluated and compared with multiple KPIs, e.g., coverage, bit rate and power consumption.

Furthermore, due to the OOK modulation and the very simple receiver, the miss-detection probability of LP-WUS signal may be higher than the ordinary transmissions by main radio. The impact of miss-detection should be considered into the wake-up procedure. In addition, a LP-WUS signal may occupy a considerable bandwidth and a relatively long time duration. Considering there may be multiple LP-WUS signals (e.g., group based LP-WUS) associated with a paging occasion, the overhead issue should also be considered.

Proposal 5: study wake-up procedure for UE in low power mode, consider sequence based LP-WUS (one or two sequences) and code block based LP-WUS for wake-up indication, and take the miss-detection and overhead issues into account.

Conclusion

In this contribution, we discussed the L1 signal design and procedure for LP-WUS, the following observations and proposals are made:

Observation 1: DFT-s-OFDM based OOK symbol generation is beneficial for the scaling of OOK symbol duration for different SCSs.

Observation 2: dropping CP at transmitter side may lose the orthogonality between LP-WUS signal and legacy OFDM symbol.

Observation 3: CP removing at receiver side may increase the complexity and power consumption of LP-WUR and lead to low spectrum efficiency.

Observation 4: CP can be enhanced as a useful part of OOK symbol, e.g., a CP with non-zero power can be seen as a useful part of an OOK-ON symbol, and a CP with zero power can be seen as a useful part of an OOK-OFF symbol.

Proposal 1: study DFT-s-OFDM based OOK symbol generation, to support transmit multiple bits by a single OFDM symbol.

Proposal 2: study CP enhancement to utilize the CP as a useful part of OOK symbols.

Proposal 3: study synchronization design for UE in low power mode, consider SSB based synchronization and low power synchronization signal (LP-SS) based synchronization.

Proposal 4: study RRM measurement for UE in low power mode, consider LP-SS based RRM and RRM relaxation for SSB based measurement.

Proposal 5: study wake-up procedure for UE in low power mode, consider sequence based LP-WUS (one or two sequences) and code block based LP-WUS for wake-up indication, and take the miss-detection and overhead issues into account.

References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, vivo, RAN meeting #97e, Sept. 12-16, 2022