3GPP TSG RAN WG1 #112b                                               R1- 2303675

Online, Apr. 17th – 26th, 2023

Source:	NEC

Title:	Discussion on AI/ML for positioning accuracy enhancement

Agenda Item:	9.2.4.2

Document for: 	Discussion and Decision

- Introduction

The study items related to the application of AI/ML to air interface were approved in RAN #94[1]. After multiple rounds of discussion within the NWM, the use cases for AI/ML application in RAN1 were narrowed down to enhancing CSI feedback enhancement, beam management, and positioning accuracy enhancements.

Below are listed agreements and conclusions reached during the RAN1 #112 meeting concerning the enhancement of positioning accuracy enhancements[2].

Agreement Regarding training data generation for AI/ML based positioning, - The following options of entity and mechanisms to generate ground truth label are identified - At least PRU is identified to generate ground truth label for UE-based positioning with UE-side model (Case 1) and UE-assisted positioning with UE-side model (Case 2a) - At least LMF with known PRU location is identified to generate ground truth label for UE-assisted/LMF-based positioning with LMF-side model (Case 2b) and NG-RAN node assisted positioning with LMF-side model (Case 3b) - At least network entity with known PRU location is identified to generate ground truth label for NG-RAN node assisted positioning with gNB-side model (Case 3a) - FFS whether and if so, applicable conditions and potential specification impact for the following options to generate ground truth label - UE generates ground truth label based on non-NR and/or NR RAT-dependent positioning methods - Network entity generates ground truth label based on positioning methods - The following options of entity to generate other training data (at least measurement corresponding to model input) are identified - For UE-based with UE-side model (Case 1) and UE-assisted positioning with UE-side (Case 2a) or LMF-side model (Case 2b) - PRU - UE - For NG-RAN node assisted positioning with Network-side model (Case 3a and Case 3b) - TRP - Note: transfer of training data from the entity generating training data to a different entity is not precluded and associated potential specification impact is for further study Agreement Regarding training data collection for AI/ML based positioning, study benefit(s) and potential specification impact (including necessity) at least for the following aspects - Associated information of training data - Quality indicator at least for ground truth label (if needed) - Other information associated with training data is not precluded. E.g., information related training dataset/samples, information related to scenario, resource configuration & mapping, timing for training data, information on implementation imperfections, etc. - Assistance signaling and procedure to facilitate generating/collecting training data - Potential determination of the UE/PRU/TRP which can provide the training data - Configuration of reference signal (for measurement and/or label) - Signaling other than above 2 for data collection - E.g., requested quality of training data Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on benefit(s), feasibility, necessity and potential specification impact for the following aspects - Entity to derive monitoring metric - UE at least for Case 1 and 2a (with UE-side model) - FFS PRU for Case 1 and 2a - gNB at least for Case 3a (with gNB-side model) - FFS gNB for Case 3b (with LMF-side model) - LMF at least for Case 2b and 3b (with LMF-side model) - Note1: companies are requested to report their assumption of entity to calculate monitoring metric if different from above options for each of the agreed cases (Case 1 to Case 3b) - If model monitoring does not require ground truth label (or its approximation). - Monitoring metric, e.g., statistics of measurement, relative displacement, inference output inconsistency, etc. - Assistance signaling and procedure, e.g., RS configuration(s) for measurement, measurement statistics as compared to the model input statistics of the training data, etc. - report of the calculated metric and/or model monitoring decision - If model monitoring requires and is provided ground truth label (or its approximation) - Monitoring metric, e.g., statistics of the difference between model output and ground truth label, etc. - Assistance signaling and procedure, e.g., from LMF to UE/gNB indicating ground truth label and/or measurement, etc. - report of the calculated metric and/or model monitoring decision - Note2: other options (of monitoring methods, monitoring metrics, assistance signaling) are not precluded Agreement Regarding AI/ML model inference, to study the potential specification impact (including the feasibility, and the necessity of specifying AI/ML model input and/or output) at least for the following aspects for AI/ML based positioning accuracy enhancement - For direct AI/ML positioning (Case 2b and 3b), type of measurement(s) as model inference input considering performance impact and associated signaling overhead - Potential new measurement: CIR/PDP - existing measurement: e.g., RSRP/RSRPP/RSTD - Note1: details of potential new measurement and/or potential enhancement to existing measurement is to be studied - Note2: study the impact of model input for other cases are not precluded - For AI/ML assisted positioning with UE-assisted (Case 2a) and NG-RAN node assisted positioning (Case 3a), measurement report to carry model output to LMF - new measurement report: e.g., ToA, path phase - existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP - enhancement of existing measurement report: e.g., soft information/high resolution of RSTD - Assistance signaling and procedure to facilitate model inference for both UE-side and Network-side model - RS configurations - Other assistance information is not precluded Note: Companies are encouraged to report their assumption of functionality and their assumption of information element(s) of AI/ML functionality identification for AI/ML based positioning with UE-side model (Case 1 and 2a).

According to the process of RAN1 #112 and previous meetings, the following three issues will be discussed in this contribution:

- Relation between the sub use case and collaboration level

- Dataset collection

- Reliability enhancement of felid data for model monitoring

- Relation between the sub use case and collaboration level

The RAN1 #111 meeting agreed to select direct AI/ML positioning and AI/ML assisted positioning as representative sub-use cases. The potential specification impact combining with UE-based/UE-assisted positioning defined in TS 38.305 are required to be clarified.

- Case 1: UE-based positioning with UE-side model, direct AI/ML positioning and AI/ML assisted positioning

According to the definition of TS 38.305[3], UE-based positioning refers to the positioning process where the UE is responsible for making the positioning calculation. When considering the collaboration level for this case,

- For direct AI/ML positioning, positioning process involves the UE calculating its location using an AI/ML model and transferring the result to network side if needed. In this case, collaboration level x is preferable because the AI/ML has the potential to replace traditional position calculation methods and has no impact on the network side.

- For AI/ML assisted positioning, positioning process involves the UE measuring/inferring the intermediate result using an AI/ML model and then using this intermediate result to calculate its location. In this case, collaboration level x is also preferable since the AI model has the potential to replace traditional position calculation method and has no impact on the network side.

Proposal 1: Collaboration level x is recommended for UE-based positioning with UE-side model (case 1) for both direct AI/ML positioning and AI/ML assisted positioning. This is because collaboration level x is more likely to support the use of AI models, which have the potential to replace traditional position calculation methods and do not require any changes on the network side.

-	If the AI model is deployed at network side (the terminologies is network-side model defined in AI 9.2.1), it is contradictory that UE based positioning refer to location calculation at UE side while direct AI positioning refer to location inference at network side which deploy the AI model. Thus NW side AI model is not support for this scenario.

- If the AI model is deployed at UE side, the technological process should be, inferring an intermediate result by AI model at UE side, calculating the UE positioning by the intermediate results via non-AI method at UE side. Similar as the above case, collaboration level x is preferable since it is more likely implemented by UE-self and transparent for network.

- If the AI model is deployed at network side, the normal technological process should be, inferring an intermediate result by AI model at network side, transferring the intermediate result to UE side, and calculating the UE positioning by the intermediate result via non-AI method. Although network side AI model can work, resource overhead, location delay and specification complexity will be introduced compared with current positioning mechanism.

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

According to the definition of TS 38.305, UE-assisted/LMF-based positioning refers to the process where UE provides measurements but does not make the positioning calculation, while the LMF makes the location calculation based on the reported measurements. When considering the collaboration level for this case,

- Since the AI/ML model is deployed at the UE side, the technological process should involve inferring an intermediate result using the AI/ML model on the UE side, transferring the intermediate result to LMF, calculating the UE’s location using mathematical methods rather than any AI/ML models. In this case, collaboration level y and collaboration level z are preferable because it is beneficial for the LMF to have access to the information from AI/ML model in order to use the output of the AI/ML model for further calculation.

Proposal 2: Collaboration level x and z is recommended for UE-assisted/LMF-based positioning with UE-side model. This is because collaboration level y and collaboration level z enables the LMF to access the information from the AI/ML model on the UE side, which can improve the accuracy of the positioning calculation.

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

In contrast to case 2a, case 2b involves deploying the AI/ML model at LMF side. To determine the appropriate collaboration level for this scenario,

- Since the AI/ML model is deployed at the LMF side, the process should involves reporting the measurement from the UE to the LMF, transferring them, and inferring the UE’s location using the AI/ML models. In this case, collaboration level x, y and z are all suitable as the accuracy of model inference and monitoring is not greatly affected by whether there is information interaction between the network and the UE regarding the AI/ML model.

Proposal 3: Collaboration level x, y and z all are suitable for case2b, UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning.

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

As per the definition provided in TS 38.305, NG-RAN node positioning refers to the gNB providing measurements of UL SRS but not performing the positioning calculation. The LMF then makes the location calculation based on the reported measurements. When determining the appropriate collaboration level for this case, the process of reporting measurements from the gNB to the LMF and performing the location calculation must be considered.

- Since the AI/ML model is deployed at the gNB side, the technological process should involve inferring an intermediate result using the AI/ML model on the gNB side, transferring the intermediate result to the LMF, and calculating the UE’s location using mathematical methods rather than any AI/ML models. In this case, collaboration level x, y and z are all appropriate since the accuracy of the model inference and monitoring is not significantly impacted by whether there is information interaction regarding the AI/ML model between the gNB and the UE.

Proposal 4: Collaboration level x, y and z all are suitable for case 3a, NG-RAN node positioning with gNB-side model, AI/ML assisted positioning.

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

In contrast to case 3a, case 3b involves the AI/ML model being deployed on LMF side. When determining the appropriate collaboration level for this scenario, the process of reporting measurements from the gNB to the LMF and performing the location calculation using the AI/ML model must be considered.

- Since the AI/ML model is deployed at the LMF side, the technological process should involve reporting the measurement of SRS from the gNB to the LMF, transferring the measurement, and inferring the UE’s location using the AI/ML models. In this case, collaboration level x, y and z are all appropriate since the accuracy of the model inference and monitoring will not greatly affected by whether there is information interaction regarding the AI/ML model between the network and the UE.

Proposal 5: Collaboration level x, y and z all are suitable for case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning.

- Dataset collection

As discussed in AI 9.2.4.1, the generalization performance of AI/ML models is crucial for their actual deployment. Existing evaluations have shown that the positioning performance of AI/ML-based positioning degrades when the model is trained with a dataset that has one set of drop/clutter parameters/network synchronization error/scenarios and tested with a dataset that has different drop/clutter parameters/network synchronization error/scenarios. Simulations have also shown that training AI/ML models with a mixed dataset can effectively improve model generalization performance. Additionally, fine-tuning can be used to improve generalization performance. However, the performance gain of fine-tuning can vary depending on the factors that impact generalization capability, even if fine-tuning with the same scale of field data. When the source domain and target domain are similar, fine-tuning the AI/ML model with a small amount of field data can approach the ideal positioning performance observed in simulations.

The accuracy of an AI/ML model for positioning improves with a suitable training dataset. Therefore, collecting a diverse set of field data for model training and fine-tuning is ideal when cost and availability are not a concern. However, in scenarios where data is restricted, it is important to determine how many field samples are needed for effective model training and fine-tuning, considering the varying impact of different factors. AI/ML-based positioning relies on learning features from large amounts of data and making inferences based on these features. When applied to wireless communication networks for positioning, an information exchange is necessary to assist entities (UE/PRU/gNB/LMF) in collecting a suitable and balanced dataset from other entities, with data being transferred in the physical or high layer

Proposal 6: RAN1 should specify an information interaction mechanism to assist the entities (UE/PRU/gNB/LMF) in collecting a suitable and balanced dataset from other entities where the data is transferred at the physical layer or higher layer.

- Reliability enhancement of felid data for model monitoring

To effectively monitor (and train) an AI/ML model for positioning, accurate measurement results and associated ground-truth labels are required. In the case of direct AI/ML positioning, the ground truth labels are the location of the targeted UE. For AI/ML assisted positioning, the ground truth labels are the ideal information of UE measurement/reporting, such as LOS/NLOS identification, RSTD, etc. Obtaining these ground truth labels is crucial, and typically requires non-AI/ML means such as base station or satellite positioning.

Currently, there are numerous positioning methods available, some within the framework of 3GPP and some beyond. Each positioning scheme has its own application scenario with an optimal positioning accuracy. To obtain field data for model monitoring efficiently and accurately, it is essential to select an appropriate scheme based on the condition that triggers the AI/ML model.

In previous meetings, two use cases (direct AI/ML positioning and AI/ML assisted positioning) were agreed upon, and each sub-use case requires UE location (or related) information for monitoring the AI/ML model. Unlike other use cases, it is not possible to monitor the positioning AI/ML model with field data collected offline since it is difficult to match the input (e.g., CIR/PDP) and output (e.g., location) when the UE is in an unpredictable physical environment. Although it is possible to obtain UE location (or related) information from RAT-dependent methods and RAT-independent methods such as OTDOA/Multi-RTT/GNSS, the reliability of model monitoring may be questionable due to the doubtful ground truth. This may be one of the reasons why the AI/ML model is triggered for inferring the UE location or for enhancing the positioning accuracy, unless the reason for triggering the AI/ML model is not due to the unreliability of non-AI model methods.

Figure 1. Relationship between model inference and verification

Observation 1: Indicating the reason for triggering the AI/ML model is beneficial for the entities that collect the field data used to monitor the AI/ML model.

Proposal 7: Provide a mechanism to improve the reliability of model monitoring by requiring the entities that collect field data for AI/ML model positioning to indicate the reason why the AI/ML model was triggered. This information can be used to initiate the appropriate positioning procedure for field data collection, ensuring that the collected data is relevant to the reason for triggering the AI/ML model.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observation and proposals are summarized as following:

Observation 1: Indicating the reason for triggering the AI/ML model is beneficial for the entities that collect the field data used to monitor the AI/ML model.

Proposal 1: Collaboration level x is recommended for UE-based positioning with UE-side model (case 1) for both direct AI/ML positioning and AI/ML assisted positioning. This is because collaboration level x is more likely to support the use of AI models, which have the potential to replace traditional position calculation methods and do not require any changes on the network side.

Proposal 2: Collaboration level x and z is recommended for UE-assisted/LMF-based positioning with UE-side model. This is because collaboration level y and collaboration level z enables the LMF to access the information from the AI/ML model on the UE side, which can improve the accuracy of the positioning calculation.

Proposal 3: Collaboration level x, y and z all are suitable for case2b, UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning.

Proposal 4: Collaboration level x, y and z all are suitable for case 3a, NG-RAN node positioning with gNB-side model, AI/ML assisted positioning.

Proposal 5: Collaboration level x, y and z all are suitable for case 3b, NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning.

Proposal 6: RAN1 should specify an information interaction mechanism to assist the entities (UE/PRU/gNB/LMF) in collecting a suitable and balanced dataset from other entities where the data is transferred at the physical layer or higher layer.

Proposal 7: Provide a mechanism to improve the reliability of model monitoring by requiring the entities that collect field data for AI/ML model positioning to indicate the reason why the AI/ML model was triggered. This information can be used to initiate the appropriate positioning procedure for field data collection, ensuring that the collected data is relevant to the reason for triggering the AI/ML model.

References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #94.

- 3GPP RAN1 #112 meeting, chairman’s notes.

- TS 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, v17.0.0.