3GPP TSG RAN WG1 #112bis-e						  R1-2303677

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.2

Source:	NEC

Title:               	Discussion on CSI enhancement

Document for:	Discussion and Decision

- Introduction

CSI enhancement for Coherent-JT targeting FR1 and up to 4 TRPs, CSI enhancement for high/medium UE velocities and TDCP reporting were agreed for Rel-18 MIMO evolution. And in RAN1#112 meeting [1], good progress was achieved for CSI enhancements. In this paper, we provided our views on CSI enhancements for CJT, high/medium velocities and TDCP reporting.

- Discussion

  - CSI enhancements for CJT

FD basis selection for mode-1 codebook

It was agreed to support TRP specific and TRP common FD basis selection for CJT codebook, and for TRP specific FD basis selection (i.e. mode-1), and two alternatives were selected for FD basis indication, the corresponding agreements are shown as follows.

Agreement For the Rel-18 Type-II codebook for CJT mTRP, support the following two modes: - Mode 1: Per-TRP/TRP-group SD/FD basis selection which allows independent FD basis selection across N TRPs / TRP groups. Example formulation (N = number of TRPs or TRP groups): W1,1W2,1Wf,1H⋮W1,NW2,NWf,NH - Mode 2: Per-TRP/TRP group (port-group or resource) SD basis selection and joint/common (across N TRPs) FD basis selection. Example formulation (N = number of TRPs or TRP groups): W1,1W2,1WfH⋮W1,NW2,NWfH - Striving for the two modes to share commonality in detailed designs such as parameter combinations, basis selection, TRP (group) selection, reference amplitude, W2 quantization schemes. - FFS: Depending on the decision on SCI design, whether additional per-TRP/TRP-group amplitude scaling and/or co-phase is needed or not, and whether they are a part of W2s Agreement On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes - Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources. - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources - Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset) For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis. Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

Based on the agreements achieved to support independent FD basis (i.e. mode-1), Alt 2 is naturally supported, Alt 1 is actually further restriction based on Alt 2, so if there is no consensus to agree on Alt 1, we think Alt 2 is the natural output. And in our understanding, the offset between delays should be independent for different TRPs, so Alt 1 should be excluded.

Proposal 1: Regarding FD basis selection for mode-1 CJT codebook, Alt 1 is actually additional restriction based on Alt 2, if no consensus, Alt 2 should be naturally supported per agreements. And we support Alt 2 only.

And regarding Alt 2, the details are shown as follows:

Table 1. FD basis selection for mode-1 codebook based on Rel-16 eTypeII codebook

| N3 | Alt 2 |
| --- | --- |
| N3≤19 (without FD window) | For reference TRP, Mv FD bases selected from N3 bases (log2N3-1Mv-1) (with FD basis remapping) |
|  | For other TRPs, Mv FD bases selected from N3 bases (log2N3Mv) |
| N3>19 (with FD window) | For reference TRP, Minitial indication (log22Mv) and Mv FD bases selected from 2Mv bases (log22Mv-1Mv-1) (with FD basis remapping) |
|  | For other TRPs, Minitial indication and Mv FD bases selected from 2Mv bases Minitial indication should cover larger range, e.g. {0,1,…N3-1} |

And for CJT based on Rel-17 port selection codebook, the details are shown as follows:

Table 2. FD basis selection for mode-1 codebook based on Rel-17 port selection codebook

| Number of FD basis | Alt 2 |
| --- | --- |
| M=1 | For reference TRP, no need of report |
|  | For other TRPs, one FD basis selected from N3 bases |
| M=2 and window N=2 | For reference TRP, no need of report |
|  | For other TRPs, starting of the window N indication is needed and no need of report for M FD basis selected from window N |
| M=2 and window N=4 | For reference TRP, second FD basis indication |
|  | For other TRPs, starting of the window N indication is needed and second FD basis indication |

Based on above discussion, we think Alt 2 should be supported, and for TRPs except the reference TRP associated with SCI, the Minitial for enhancement based on Rel-16 codebook or starting of the window N indication for enhancement based on Rel-17 codebook need to be updated.

Proposal 2: Support Alt 2 (Wf,n independently selected across N CSI-RS resources), and for CSI-RS resources except the reference CSI-RS resource associated with SCI, the M_initial for enhancement based on Rel-16 codebook or starting of the window N indication for enhancement based on Rel-17 codebook need to be updated.

Parameter configuration for NL and {pv, }

In last meeting, candidate values for {Ln} and {pv, } were agreed.

Conclusion On the Parameter Combination of Type-II codebook refinement for CJT mTRP, there is no consensus on adding a new (not previously agreed) codebook parameter, as well as replacing the legacy parameter L with a new (not previously agreed) parameter. - Note: Since dynamic {Ln} selection was agreed, this implies that the list of supported {Ln} combinations will be discussed separately from the list of supported {pv,} combinations - FFS: Whether/how the list of supported {Ln} combinations can be linked with the list of supported {pv,} combinations without introducing a new (not previously agreed) codebook parameter, e.g. via some UE capability Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {Ln} for the higher-layer-configured value of NTRP (FFS by RAN1#112: whether the bracketed permutations are also supported): - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement FFS: Whether the total number of Ln is a UE capability NTRP; {Ln} combination. 1; {2}. {4}. {6} (analogous to legacy, only for total # ports =32, rank 1-2, R=1. 2; {2,2}. {2,4}, [{4,2}]. {4,4}. 3; {2,2,2}. {2,2,4} [and its other permutations]. {4,4,4}. 4; {2,2,2,2}. {2,2,2,4} [and its other permutations]. {2,2,4,4} [and its other permutations]. {4,4,4,4}. Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, regarding the list of supported combinations of {Ln}, only support the following additional combinations: NTRP; {Ln} combination. 2; {4,2}. 3; {2,4,2}, {4,2,2}. No other permutations are supported. FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6 Agreement On the Type-II codebook refinement for CJT mTRP, for Rel-16-based refinement, support at least the following combinations of {pv,} from where the value of {pv,} is gNB-configured via higher-layer (RRC) signaling: - FFS by RAN1#112: whether other combinations can be supported FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement pv for layers 1-4; ; Condition(s). {1/8, 1/8, 1/16, 1/16}; ¼; --. ½; --. {1/4, 1/4, 1/8, 1/8}; ¼ (*); --. ½ (*); --. {1/4, 1/4, 1/4, 1/4}; ¾ (*); --. {1/2, 1/2, 1/2, 1/2}; ½; - Only applicable when NTRP≤3 and NL=1 - Optional. (*) Supported by legacy Rel-16

And it was agreed that NL combinations of {Ln} can be configured for a UE, we think one issue to be clarified firstly is whether the NL combinations should correspond to a same value of NTRP, in addition, it was agreed TRP selection can be dynamically determined by the UE, so another issue to be clarified is whether a corresponding subset of {Ln} with N < NTRP regarded as one of NL combinations? For example, if one combination is configured as {L1 , L2 , L3 , L4} for NTRP =4, and in case of TRP selection (e.g. first two TRPs selected), then the combination will be {L1 , L2} with N=2, in this case, whether {L1 , L2} should be counted as one of NL combinations?

Proposal 3: It should be clarified that whether NL combinations of {Ln} for one UE correspond to a same value of NTRP. And in our understanding, the NL combinations of {Ln} for one UE should correspond to a same value of NTRP.

Proposal 4: It should be clarified that whether a combination based on TRP selection with N < NTRP counted as one of NL combinations. And in our understanding, it should not be counted as one of NL combinations.

And regarding the large number of linkage between {Ln} and {pv, }, a further restriction on supported linkages should be discussed. One key target should be keep the CSI overhead in a reasonable level, e.g. not exceed maximum payload in current spec, especially in case of NTRP is large. In addition, as we agreed to support UE based TRP selection, the CSI payload may vary based on different number of selected TRPs, while the resource allocation has considered the largest overhead case, so if the number of selected TRPs is small (e.g. only one TRP selected), the CSI payload is much smaller considering the allocated resource (based on the small value of pv and which is configured for multi-TRP), which will cause waste of resource, and worse performance compared with legacy codebook. In this case, it’s better to configure multiple values of {pv, } for one combination of {Ln}, for example, {pv, } with smaller values for {Ln} with NTRP, and {pv, } with larger values for when N < NTRP, which can utilize the allocated resource efficiently and improve the performance.

For example, if one combination is configured as {L1 , L2 , L3 , L4} for NTRP =4, and in case of TRP selection (e.g. first two TRPs selected), then the combination will be {L1 , L2} with N=2, in this case, one supported set of {pv, } which can be linked with {L1 , L2} should be used.

Proposal 5: Support linkage between one of NL combinations and one set of {pv, }. And multiple sets of {pv, } can be configured with NL combinations, at least considering single-TRP hypothesis and multi-TRP hypothesis, which can utilize allocated resource for CSI report efficiently and improve performance for small number of selected TRPs.

Reference amplitude coefficient

Another issue is amplitude coefficient group design for W2, and 2 amplitude groups for CJT (i.e. Alt 1) was agreed and there was a working assumption to support (2N–1) amplitude groups for CJT (i.e. Alt 3), we think the relative amplitude coefficients per TRP (Alt 3) can be beneficial to provide finer granularity for amplitude without much overhead increasing.

Agreement On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer: - Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2) - FFS: Amplitude quantization table enhancement - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported - Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111) - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N) - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported - If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Alt 1 is simple while it’s only suitable when the relative amplitudes across TRPs are similar, and another issue for Alt 1 is that for the other TRPs except the reference TRP with SCI, the relative strongest amplitude coefficient in the other TRP may not on same polarization as the reference TRP. So which polarization in the other TRP except the reference TRP will apply the reported reference amplitude should be indicated. For example, SCI on reference TRP is on the first polarization, while the relative strongest amplitude coefficient in the other TRP may be on the second polarization, the possibility is around 50%. In this case, the same reference amplitude coefficient applied on the second polarization in the other TRP is not reasonable, especially when the reference amplitude coefficient is small, in other words, when the difference between weak polarization and strong polarization in reference TRP is large. As shown in Figure 1 and Figure 2, we showed the power gap between the strong polarization in other TRPs and the weak polarization in reference TRP when the strong polarization in reference TRP and in other TRPs are different.

Figure. 1. Power gap between strong polarization in other TRP and weak polarization in reference TRP (with SCI) when strong polarizations are different among TRPs (NTRP=2)

Figure. 2. Power gap between strong polarization in other TRP and weak polarization in reference TRP (with SCI) when strong polarizations are different among TRPs (NTRP=4)

From Figure.1 and Figure.2, we can see that in case of the strong polarizations in different TRPs are different (e.g. one polarization is strong in reference TRP and another polarization is strong in other TRP), with at least 50% possibility the strong polarization (different from the polarization with SCI in reference TRP) in other TRPs has high power than the weak polarization in reference TRP. If the reference amplitude is mainly derived based on the gap between SCI and weak polarization in reference TRP, this reference amplitude is not suitable to be applied to the strong polarization in the other TRP. Alt 3 can solve this issue, with (2N-1) reference amplitude per polarization per other TRP. So firstly, we’d like to confirm the WA to support Alt 3.

Proposal 6: Confirm the WA to support (2N–1) amplitude groups for CJT (i.e. Alt 3).

And for Alt 1, we think for the other TRP, polarization indication is needed to indicate which polarization will apply the reported reference amplitude. On the other hand, companies may argue that the amplitude difference between TRPs may be not that large in CJT, so Alt 3 is not needed, if so, there is no need of 3bits reference amplitude coefficient to cover 0 to -21dB with 3dB step size. For example, 1 or 2 bits of reference amplitude coefficient is sufficient.

Proposal 7: Regarding Alt 1 (2 amplitude groups for CJT), for the other CSI-RS resource except the reference CSI-RS resource (associated with SCI), polarization indication is needed to indicate which polarization is associated with the reported reference amplitude coefficient.

Proposal 8: Regarding Alt 1 (2 amplitude groups for CJT), the field size for reference amplitude coefficient report can be reduced from 3 bits to 1 or 2 bits.

CSI partitioning and priority

Furthermore, as more than one TRPs are involved for the enhanced codebook, CSI payload is increased, and consequently, the priority rules should be updated. Three alternatives were listed in last meeting as:

Agreement On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource): - Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n - Note: This implies that CSI-RS resource is designated the highest priority - Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+ - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis) - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI. - Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+ FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

In our understanding, CJT multi-TRP is applied for performance improvement, in case of UCI omission, one TRP (e.g. reference TRP) should have the highest priority to include as much information as possible for the whole TRP. So we support either Alt 2 or Alt 3, with Alt 2 preferred. And for Alt 2, Q(n)=0 if n corresponds to strongest TRP/SCI, which can ensure the strongest TRP/SCI can have highest priority.

Proposal 9: Regarding UCI omission for CJT, support either Alt 2 or Alt 3, and Alt 2 preferred, and for Alt 2, support Q(n)=0 if n corresponds to strongest TRP/SCI.

In addition, for CJT mode-1, FD basis is independently selected across TRPs, which means the power is focused on different FD basis indexes for different TRPs (which can be regarded as FD basis offset across TRPs), and in legacy priority rule, the priority for FD basis is based on permutation, where priority is concentrated around 0. While considering the FD basis offset across TRPs, for TRPs except the one associated with SCI, the priority should be concentrated around the offset value. So we think P(m) should be based on permutation as Rel-16 analogous (π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1)) for the CSI-RS resource associated with SCI, and the permutation for CSI-RS resources except the one associated with SCI should be π(f)=min⁡(2⋅mod(n3,lf-offset, N3), 2⋅N3-mod(n3,lf-offset, N3)-1), offset is the FD basis offset for the CSI-RS resource except the one associated with SCI.

Proposal 10: Priority function for FD permutation should consider the FD basis offset for CJT mode-1, as the power for CSI-RS resource except the reference CSI-RS resource is concentrated around the frequency domain basis offset. For example, P(m) should be based on permutation as Rel-16 analogous (π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1)) for the CSI-RS resource associated with SCI, and the permutation for CSI-RS resources except the one associated with SCI should be π(f)=min⁡(2⋅mod(n3,lf-offset, N3), 2⋅N3-mod(n3,lf-offset, N3)-1).

CBSR

It was agreed legacy CBSR will be reused for CJT mTRP,

Agreement On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction) - FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction - FFS: Whether CBSR can be configured to be off for a CSI-RS resource The same rank restriction is applied across NTRP CSI-RS resources

And regarding the amplitude restriction, we think it should be CSI-RS resource specific, and soft restriction should be applied. And considering UE based TRP selection, different CBSR configurations for different TRP selections can be studied. For example, in case of multi-TRP CJT, CBSR for one CSI-RS resource can be configured as off, as the signal from different TRPs are all signal not interference, while in case of single-TRP based on TRP selection, the CBSR for the CSI-RS resource should be on, as the signal should be considered as interference for other non-selected TRPs.

Proposal 11: Considering CBSR for CJT, support CSI-RS resource specific amplitude restriction, and soft restriction is supported. And support different CBSR rules considering TRP selection. For example, CBSR can be configured to be off for a CSI-RS resource in case of multi-TRP CJT, and in case of single-TRP based on the CSI-RS resource, the CBSR should be on.

And it was agreed rank restriction is applied across NTRP CSI-RS resources, in our understanding, this rank restriction is configured for multi-TRP CJT with all NTRP CSI-RS resources, in case of TRP selection, a separate rank restriction should be considered, especially for single-TRP hypothesis, which is similar as Rel-17 NCJT configuration. As single-TRP hypothesis, the number of layers may be different from CJT mTRP.

Proposal 12: Support separate rank restrictions for different hypotheses, e.g. for multi-TRP and single-TRP based on TRP selection.

In addition, considering the normalization of CJT codebook, if the normalization factor is calculated with all coefficients across TRPs, the power for each TRP is lower compared to single-TRP transmission, which may not fully utilize multiple TRPs. Another option is that normalization factor is based on the coefficients for one TRP (e.g. the maximum sum among TRPs).

Proposal 13: Normalization factor for CJT codebook can be studied, for example, based on coefficients summed for one CSI-RS resource instead of all CSI-RS resources.

  - CSI enhancements for high/medium velocities

Bitmap for NNZC

Regarding bitmap for non zero coefficients indication, three alternatives were listed as

Agreement For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, down-select one from the following alternatives (no later than RAN1#112bis-e): - Alt1. Q different 2-dimensional bitmaps where each bitmap reuses the legacy design i.e. the size of the bitmap for each selected DD basis vector is 2LMv - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4. A bitmap that includes bits associated with the set of {(f1, s1, t1)} with d(f1, s1, t1)<=d, where d is the threshold that can be configured by gNB, df1, s1,t1= minf1-f0, Mv-f1-f0+ mins1-s0, L – s1-s0+ mint1-t0, Q – t1-t0. s0, f0 and t0 denotes a reference SD basis index and a reference FD basis index and a reference DD basis index associated with SCI, respectively.

Alt 1 is simplest, and Alt 3A can further reduce overhead, while the precondition for Alt 3A is sparse FD and DD basis pair. We are fine with Alt 3A, while for some configurations, e.g. number of FD basis is only 1, we don’t think the first step bitmap (MQ, actually, the bitmap is Q) is needed, as all of the Q DD bases should be selected, in other words, there will be Q bits with value one, which is unnecessary.

Proposal 14: Regarding bitmap for NNZC, we are fine with Alt 3A in case of large value of M or at least when M is not 1.

Number of CQI for high/medium velocity

In last meeting, 2 CQIs were agreed as optional feature.

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding the time instance and/or PMI(s) in which a CQI is associated with, given the CSI reporting window WCSI (in slots), as well as the number of CQIs (=X) in one sub-band and one CSI reporting instance, support only the following: - Basic feature: X=1 and the CQI is associated with the first/earliest slot of the CSI reporting window and the first/earliest of the N4 W2 matrices - Optional features: - X=1 and the CQI is associated with: - the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - the last slot of the CSI reporting window (slot l+WCSI–1) and the N4-thW2 matrix - X=2 and - The 1st CQI is associated with the first/earliest slot of the CSI reporting window (slot l) and the first/earliest of the N4 W2 matrices, and - The 2nd CQI is associated with the middle slot of the CSI reporting window (slot l+WCSI/2) and the (N4 /2)-thW2 matrix - FFS: Whether/how to include CQI overhead reduction for X=2

And during the offline discussion, the CQI formats were listed, we think for the 2nd TD CQI, differential values are sufficient.

Proposal 15: Regarding CQI formats for 2nd TD CQI, we prefer either Alt 2 or Alt 3 with differential values.

And regarding the location of the 2nd CQI, UCI part 2 is sufficient.

Proposal 16: Regarding location of the 2nd TD CQI, support Alt 2 (in UCI part 2).

Doppler domain basis indication and SCI payload

It was agreed Q=2 DD bases can be indicated for high/medium velocities codebook refinement, and DD basis index 0 is always selected.

Agreement For the Type-II codebook refinement for high/medium velocities, for N4>2 and Q=2, the selection of Q out of N4 DD basis vectors is indicated by a log2N4-1-bit indicator in CSI part 2 - Analogous to FD basis selection, DD basis index 0 (representing DC) is always selected. Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, on the W2 quantization scheme when N4>1, for each layer: - One (common) SCI (Strongest Coefficient Indicator) applies across all Q selected DD basis vectors - One group comprises one polarization across all Q selected DD basis vectors (Cgroup,phase=1, Cgroup,amp=2) - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

In current spec, actually there are two cases considering one basis index always selected, for example, in case of Rel-16 type II codebook, FD basis index 0 is always selected, and FD basis remapping is specified in TS, and then SCI indication can be log22L. And in case of Rel-17 port selection codebook, FD basis index 0 is also always selected, while the FD basis remapping is not specified, and the SCI indication is log22L*M. So for the codebook refinement for high/medium velocities, we think the first thing to be clarified is whether DD basis remapping needs to be specified or not, and then the payload for SCI indication needs to be defined.

Proposal 17: Regarding DD basis indication (DD basis index 0 always selected), clarification on whether DD basis remapping (analogous to FD basis remapping in Rel-16 TypeII codebook) is needed or not. And SCI indication payload can be determined based on the clarification. For example, if DD basis remapping is not specified, the SCI payload should be log22L*Q and if DD basis remapping is specified, the SCI payload can be log22L.

Another issue to be clarified is SCI payload for rank =1 and rank >1, in current spec, SCI payload design for rank =1 and rank >1 is actually a little different, where SCI payload for rank =1 is log2KNZ and SCI payload for rank >1 per layer is log22L. In our understanding, the difference was actually based on discussion procedure, as rank =1 was agreed firstly, and then FD basis remapping was introduced, which reduce the payload to be log22L per layer for rank >1. And in Rel-18 codebook enhancement, we are wondering whether we can apply a unified solution for rank =1 and rank >1, for example, both based on log22L. Anyway, we think the SCI payload should be discussed to have a clear consensus.

Proposal 18: Support an agreement for SCI payload design (for both CJT and high/medium velocities). And it’s better to be clarified whether we can have a unified payload design for rank =1 and rank >1, for example, based on log22L instead of log2KNZ for rank 1.

CSI partitioning and priority

Regarding the priority rules, four alternatives were listed in last meeting as:

Agreement On the Type-II codebook refinement for high/medium velocities, regarding UCI omission, down-select between the following three alternatives (by RAN1#112bis-e where q denotes the q-th DD basis vector): - Alt1. Prio(,l,m,q)=2L. Q.RI.P(m)+Q.RI.l+Q.q - Note: This implies that DD basis is designated the highest priority - Alt2. Prio(,l,m,q)=2L.S(q).RI.N3+2L.RI. P(m)+RI.l+ - Note: This implies that DD basis is designated the lower priority (after FD basis) - FFS: S(q) maps the index q according to a rule - Alt3. Prio(,l,m,q)=2L.RI.Mv.q + 2L.RI.P(m)+ RI.l +  - Note: This implies that DD basis is designated the least priority - Alt4. Prio(,l,m,q)=2L.P(m).RI.Q+2L.RI.S(q)+RI.l+ - Note: This implies that DD basis is designated with lower priority (after SD basis) and higher priority (before FD basis) - FFS: S(q) maps the index q according to a rule FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m q=0,…,Q-1

In our understanding, Alt 2 and Alt 3 are similar, and regarding S(q), we think the index q associated with SCI should have higher priority, for example S(q) = 0 if q corresponds to SCI, otherwise S(q) = 1. And regarding the priority for DD bases, we think it should have higher priority at least higher than FD basis, which can guarantee doppler prediction to overcome CSI aging, so we prefer Alt 1 or Alt 4.

Proposal 19: Regarding UCI omission for high/medium velocities, support either Alt 1 or Alt 4. And regarding S(q), we think index q associated with SCI should have higher priority, for example, S(q) = 0 if q corresponds to SCI, otherwise S(q) = 1.

CBSR

For Rel-16 eType II codebook, codebook subset restriction is defined based on restriction of average amplitude for some spartial domain bases to reduce interference. For high/medium velocity codebook based on Rel-16 eTypeII codebook, we think the codebook subset restriction should be reused, at least as starting point. For example, the average amplitude restriction should be satisfied per time unit of N4.

Proposal 20: For high/medium velocity codebook based on Rel-16 eTypeII codebook, CBSR is supported. And the average amplitude restriction should be satisfied per time unit of N4.

  - CSI enhancements for TDCP

After hot discussion on TDCP, time-domain correlation profile was finally selected for TDCP reporting.

Agreement For aiding gNB determination of codebook switching and SRS periodicity with the Rel-18 TRS -based TDCP reporting, support reporting quantized wideband normalized amplitude/phase of the time-domain correlation profile with Y≥1 delay(s) as follows: - Basic feature: Y=1 with delay≤ Dbasic symbols, only wideband quantized normalized amplitude is reported - FFS: Candidate values for delay - Optional feature: Y=1 with delay>Dbasic symbols and Y≥1, wideband quantized normalized amplitude and phase for each delay are reported - For Y>1, the phase can be configured to be absent for all the Y delays - TBD: Whether the value of Y is configurable or following the delays from the configured TRS resource - TBD: Candidate value(s) for Y>1 - FFS: Value of Dbasic Agreement For the Rel-18 TRS-based TDCP reporting, the priority of the CSI report(s) associated with TDCP reporting is down-selected from the following alternatives: - Alt1. Lower than other CSI reports - Alt2. Same as CSI report(s) not carrying L1-RSRP or L1-SINR - Alt3. Higher than other CSI reports - Other alternatives are not precluded Agreement For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y for Y>1, down-select from the following alternatives: - Alt1. The value of Y is gNB-configured via higher-layer (RRC) signalling - Alt2. The value of Y follows the delays from the configured TRS resource - Alt3. The value of Y is UE-selected and reported The value of Y is a UE capability Agreement For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, by RAN1#112bis-e, decide between the following alternatives: - Alt1. Fully reuse legacy TRS - Alt2. Study enhancements on TRS (e.g. periodicities) Note. If there is no consensus on Alt2, Alt1 is the default outcome

First issue is to design the TRS pattern for TDCP measurement, which should consider both TRS overhead and requirement for TDCP measurement. In our understanding, the time domain correlation within one or two slots (one TRS occasion) is not that useful, as the correlation value is very near to one. So a typical pattern for TRS for TDCP measurement can be a periodic TRS set along with an aperiodic TRS set, note that aperiodic TDCP reporting has been agreed. And in this case, legacy TRS configuration can be reused, which have less spec effort. While the slot offset between the TRS resources in periodic and aperiodic sets should be based on the agreed values for delays for time domain correlation.

Proposal 21: For TRS based TDCP, periodic TRS set along with aperiodic TRS set with slot offset between TRS resources based on agreed delay values for TDCP reporting should be supported, which we think can be regarded as Alt 1, fully reuse legacy TRS.

Another issue is the priority for TDCP reporting, in our understanding, TDCP reporting is useful for network scheduling/configuration, so we don’t think a least priority is reasonable, and also considering the spec impact, we think categorizing TDCP reporting as the CSI reports not carrying L1-RSRP/L1-SINR is sufficient, which has no spec impact at all.

Proposal 22: Considering usefulness of TDCP reporting and spec impact, support Alt2 (Same as CSI report(s) not carrying L1-RSRP or L1-SINR) for priority of TDCP reporting.

- Conclusion

In this contribution, we provided our views on CSI enhancement for CJT and high/medium velocities, and we propose:

Proposal 1: Regarding FD basis selection for mode-1 CJT codebook, Alt 1 is actually additional restriction based on Alt 2, if no consensus, Alt 2 should be naturally supported per agreements. And we support Alt 2 only.

Proposal 2: Support Alt 2 (Wf,n independently selected across N CSI-RS resources), and for CSI-RS resources except the reference CSI-RS resource associated with SCI, the M_initial for enhancement based on Rel-16 codebook or starting of the window N indication for enhancement based on Rel-17 codebook need to be updated.

Proposal 3: It should be clarified that whether NL combinations of {Ln} for one UE correspond to a same value of NTRP. And in our understanding, the NL combinations of {Ln} for one UE should correspond to a same value of NTRP.

Proposal 4: It should be clarified that whether a combination based on TRP selection with N < NTRP counted as one of NL combinations. And in our understanding, it should not be counted as one of NL combinations.

Proposal 5: Support linkage between one of NL combinations and one set of {pv, }. And multiple sets of {pv, } can be configured with NL combinations, at least considering single-TRP hypothesis and multi-TRP hypothesis, which can utilize allocated resource for CSI report efficiently and improve performance for small number of selected TRPs.

Proposal 6: Confirm the WA to support (2N–1) amplitude groups for CJT (i.e. Alt 3).

Proposal 7: Regarding Alt 1 (2 amplitude groups for CJT), for the other CSI-RS resource except the reference CSI-RS resource (associated with SCI), polarization indication is needed to indicate which polarization is associated with the reported reference amplitude coefficient.

Proposal 8: Regarding Alt 1 (2 amplitude groups for CJT), the field size for reference amplitude coefficient report can be reduced from 3 bits to 1 or 2 bits.

Proposal 9: Regarding UCI omission for CJT, support either Alt 2 or Alt 3, and Alt 2 preferred, and for Alt 2, support Q(n)=0 if n corresponds to strongest TRP/SCI.

Proposal 10: Priority function for FD permutation should consider the FD basis offset for CJT mode-1, as the power for CSI-RS resource except the reference CSI-RS resource is concentrated around the frequency domain basis offset. For example, P(m) should be based on permutation as Rel-16 analogous (π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1)) for the CSI-RS resource associated with SCI, and the permutation for CSI-RS resources except the one associated with SCI should be π(f)=min⁡(2⋅mod(n3,lf-offset, N3), 2⋅N3-mod(n3,lf-offset, N3)-1).

Proposal 11: Considering CBSR for CJT, support CSI-RS resource specific amplitude restriction, and soft restriction is supported. And support different CBSR rules considering TRP selection. For example, CBSR can be configured to be off for a CSI-RS resource in case of multi-TRP CJT, and in case of single-TRP based on the CSI-RS resource, the CBSR should be on.

Proposal 12: Support separate rank restrictions for different hypotheses, e.g. for multi-TRP and single-TRP based on TRP selection.

Proposal 13: Normalization factor for CJT codebook can be studied, for example, based on coefficients summed for one CSI-RS resource instead of all CSI-RS resources.

Proposal 14: Regarding bitmap for NNZC, we are fine with Alt 3A in case of large value of M or at least when M is not 1.

Proposal 15: Regarding CQI formats for 2nd TD CQI, we prefer either Alt 2 or Alt 3 with differential values.

Proposal 16: Regarding location of the 2nd TD CQI, support Alt 2 (in UCI part 2).

Proposal 17: Regarding DD basis indication (DD basis index 0 always selected), clarification on whether DD basis remapping (analogous to FD basis remapping in Rel-16 TypeII codebook) is needed or not. And SCI indication payload can be determined based on the clarification. For example, if DD basis remapping is not specified, the SCI payload should be log22L*Q and if DD basis remapping is specified, the SCI payload can be log22L.

Proposal 18: Support an agreement for SCI payload design (for both CJT and high/medium velocities). And it’s better to be clarified whether we can have a unified payload design for rank =1 and rank >1, for example, based on log22L instead of log2KNZ for rank 1.

Proposal 19: Regarding UCI omission for high/medium velocities, support either Alt 1 or Alt 4. And regarding S(q), we think index q associated with SCI should have higher priority, for example, S(q) = 0 if q corresponds to SCI, otherwise S(q) = 1.

Proposal 20: For high/medium velocity codebook based on Rel-16 eTypeII codebook, CBSR is supported. And the average amplitude restriction should be satisfied per time unit of N4.

Proposal 21: For TRS based TDCP, periodic TRS set along with aperiodic TRS set with slot offset between TRS resources based on agreed delay values for TDCP reporting should be supported, which we think can be regarded as Alt 1, fully reuse legacy TRS.

Proposal 22: Considering usefulness of TDCP reporting and spec impact, support Alt2 (Same as CSI report(s) not carrying L1-RSRP or L1-SINR) for priority of TDCP reporting.

- References

- Chair's notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023