3GPP TSG RAN WG1 #112bis-e						  R1-2303678

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.3.1

Source:	NEC

Title:               	Discussion on increased number of orthogonal DMRS ports

Document for:	Discussion and Decision

- Introduction

Larger number of orthogonal DMRS ports for downlink and uplink was agreed to be specified, and good progress has been achieved. In RAN1#112 meeting [1], agreements were achieved as:

Working Assumption

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

In this contribution, we provided our views on increased number of orthogonal DMRS ports.

- Discussion

Antenna ports indication for Rel-18 eType1 DMRS ports has been well designed, and regarding 2CWs indication, we support to confirm the working assumption.

Proposal 1: Confirm the working assumption on antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for 2 CWs.

For Rel-18 eType1 DMRS ports with maxlength =2, and Rel-18 eType2 DMRS ports with maxlength = 1/2, at least the same existing Rel-15 DMRS ports indications as that for rank = 5,6,7,8 for PDSCH can still be reused, and some additional indications with new DMRS ports can be introduced. For example, the DMRS ports indication as shown in Table 1/2/3 can be applied.

Table.1 Rel-18 eType1 DMRS with maxLength=2

| Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- |
| 2 | 0,1,2,3,8 | 1 |
| 2 | 0,1,2,3,8,10 | 1 |
| 2 | 0,1,2,3,8,9,10 | 1 |
| 2 | 0,1,2,3,8,9,10,11 | 1 |
| 2 | 0-4 | 2 |
| 2 | 0,1,2,3,4,6 | 2 |
| 2 | 0,1,2,3,4,5,6 | 2 |
| 2 | 0,1,2,3,4,5,6,7 | 2 |

Table.2 Rel-18 eType2 DMRS with maxLength=1

| Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- |
| 3 | 0,1,2,3,4 |
| 3 | 0,1,2,3,4,5 |
| 2 | 0,1,2,3,12 |
| 2 | 0,1,2,3,12,13 |
| 2 | 0,1,2,3,12,13,14 |
| 2 | 0,1,2,3,12,13,14,15 |

Table.3 Rel-18 eType2 DMRS with maxLength=2

| Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- |
| 3 | 0,1,2,3,4 | 1 |
| 3 | 0,1,2,3,4,5 | 1 |
| 2 | 0,1,2,3,12 | 1 |
| 2 | 0,1,2,3,12,13 | 1 |
| 2 | 0,1,2,3,12,13,14 | 1 |
| 2 | 0,1,2,3,12,13,14,15 | 1 |
| 2 | 0,1,2,3,6 | 2 |
| 2 | 0,1,2,3,6,8 | 2 |
| 2 | 0,1,2,3,6,7,8 | 2 |
| 2 | 0,1,2,3,6,7,8,9 | 2 |

Proposal 2: For 2CWs with Rel-18 eType1/eType2 DMRS ports, existing Rel-15 DMRS ports indication as that for rank = 5,6,7,8 for PDSCH is reused, and some additional indications with new ports can be introduced.

- Conclusion

In this contribution, we provided our views on increased number of orthogonal DMRS ports, and we have the following proposals:

Proposal 1: Confirm the working assumption on antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for 2 CWs.

Proposal 2: For 2CWs with Rel-18 eType1/eType2 DMRS ports, existing Rel-15 DMRS ports indication as that for rank = 5,6,7,8 for PDSCH is reused, and some additional indications with new ports can be introduced.

- References

- Chair's notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023