3GPP TSG RAN WG1 #112bis-e						  R1-2303680

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.1.4.2

Source:	NEC

Title:               	Discussion on SRI/TPMI enhancement

Document for:	Discussion and Decision

- Introduction

Rel-18 MIMO evolution for downlink and uplink will specify enhancements to support 8 Tx UL operation. And in RAN1#112 meeting [1], agreements were achieved as:

Agreement

Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook

- Precoding matrices generated according to (O1, O2) = (1, 1) is supported

- Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1

  - Subject to UE capability

- FFS: Different O1, O2 values for different ranks

Agreement

For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1),

- Study whether/how to support (O1, O2) = (2,1), (2,2)

  - whether for all rank, or rank 1-2, or rank 3-8

  - applicability of different (O1, O2) values per agreed (N1, N2)

  - companies are encouraged to submit simulation results

Agreement

To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from,

- Alt1: First CW

- Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW)

Agreement

For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set,

- All SRS port combinations are supported

- For SRI indication, down-select from,

  - Option 1: Use an NSRS bit length bitmap

  - Option 2: Use a legacy-based solution

- Consideration of Lmax for SRI indication

For NSRS≤4, Rel-15 SRI indication is reused

Agreement

For partially coherent uplink precoding by an 8TX UE codebook,

- When Ng=2

  - Precoding design is based on Rel-15 UL 4TX codebook,

    - Full-coherent precoders are used

      - FFS whether partial-coherent precoders are needed

- When Ng=4, down-select from,

  - Alt1:

    - Precoding design is based on Rel-15 UL 2TX codebook,

      - Full-coherent precoders are used

  - Alt2:

    - Precoding design is based on Rel-15 UL 4TX codebook,

      - Partial-coherent precoders are used

Agreement

For partially coherent uplink precoding by an 8TX UE codebook, Ng=2,

- Following rank and layer splitting cases are supported

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 1 | (1,0), (0,1) |  |
| 8 |  | (4,4) |

- Select from the following cases based on the performance and overall DCI overhead

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (2,1), (1,2) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2), (3,1), (1,3) |
| 5 |  | (4,1), (1,4), (2,3), (3,2) |
| 6 |  | (4,2), (2,4), (3,3) |
| 7 |  | (4,3), (3,4) |

Note: Above is not relevant to how precoders are indicated.

In this contribution, we provided our views on TPMI enhancement.

- Discussion

For full coherent codebook, one remaining issue is whether to support (O1,O2) with value larger than 1. We think oversampling can achieve better performance, at least for advanced device with capability, as the target device include not only handsets, but also CPE, so we support (O1, O2) where O1>1 or O2>1, for example, (O1, O2) = (2,2) for (N1,N2) = (2,2) and (O1, O2) = (2,1) for (N1,N2) = (4,1).

Proposal 1: Support (O1,O2) where O1>1 or O2>1, for example, (O1, O2) = (2,2) for (N1,N2) = (2,2) and (O1, O2) = (2,1) for (N1,N2) = (4,1).

One key remaining issue for 8Tx is to design partial coherent codebook. As we already have Rel-15 2Tx and 4Tx codebook, so the partial coherent codebook for Ng=2 and Ng=4 can be separately discussed, Ng=2 can be based on 4Tx codebook, and Ng=2 can be based on 2Tx codebook. And we think one SRS resource set for codebook based uplink transmission is sufficient.

And for Ng = 2, for rank = 2/3/4, we think at least precoders with layer split should be supported, and considering the precoding design, we think there is no need to support all possibilities at least for rank = 2 (i.e. 16 precoders with rank = 1 * 16 precoders with rank =1), we can start from the 4Tx precoders with rank = 2, and each column is mapped on one antenna group, which can significantly reduce the signaling overhead.

And considering the layer split, the two antenna groups should have similar number of layers as much as possible, so we support layer split of (2,2) for rank = 4, layer split of (2,3), (3,2) for rank =5, and layer split of (3,3) for rank =6.

Proposal 2: For 8Tx partial coherent codebook with Ng=2, at least support percoders with layer split for rank = 2/3/4, specifically, support layer split (2,2) for rank =4, layer split of (2,3), (3,2) for rank =5, and layer split of (3,3) for rank =6.

Proposal 3: For 8Tx partial coherent codebook with Ng=2 and rank = 2/3/4, support at least 4Tx precoders with rank = 2/3/4 with subset of column(s) mapped on one of the two antenna groups, respectively.

And for Ng = 4, down-selection is needed between full-coherent precoders of 2Tx or partial-coherent precoders of 4Tx, actually, both cases are similar except 4Tx precoders with rank =3, we prefer clear design based on 2Tx full coherent precoders.

Proposal 4: For 8Tx partial coherent codebook with Ng=4, support precoding design based on 2Tx full-coherent precoders.

- Conclusion

In this contribution, we provided our views on TPMI enhancements, and we have the following proposals:

Proposal 1: Support (O1,O2) where O1>1 or O2>1, for example, (O1, O2) = (2,2) for (N1,N2) = (2,2) and (O1, O2) = (2,1) for (N1,N2) = (4,1).

Proposal 2: For 8Tx partial coherent codebook with Ng=2, at least support percoders with layer split for rank = 2/3/4, specifically, support layer split (2,2) for rank =4, layer split of (2,3), (3,2) for rank =5, and layer split of (3,3) for rank =6.

Proposal 3: For 8Tx partial coherent codebook with Ng=2 and rank = 2/3/4, support at least 4Tx precoders with rank = 2/3/4 with subset of column(s) mapped on one of the two antenna groups, respectively.

Proposal 4: For 8Tx partial coherent codebook with Ng=4, support precoding design based on 2Tx full-coherent precoders.

- References

- Chair's notes, RAN1#112, Athens, Greece, February 27th – March 3rd, 2023