3GPP TSG RAN WG1 #112bis-e										R1-2303686

e-Meeting, April 17th – April 26th,2023

Agenda Item:	9.4.1.1

Source: 	NEC

Title:	Channel Access of Sidelink on Unlicensed Spectrum

Document for:	Discussion/Decision

- Introduction

During the last meeting, several agreements and relevant options are concluded in [1], combined with previous agreements in [2] and [3], further analyses and views on sidelink on the unlicensed spectrum are presented in this contribution.

- Channel Access Issues

  - Channel Access for PSFCH

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

Besides Type 1 channel access procedure, as PSFCH resources are allocated following PSSCH symbols in a slot, for the case that a sidelink CO is initiated from PSCCH/PSSCH transmission and shared among UEs, using Type 2A channel access for PSFCH transmission may benefit the success ratio of sidelink A/N feedback information and further improve the sidelink resource efficiency. On the other hand, PSFCH resource allocation period may not meet the limitation of short control signaling according to the resource pool configuration on the unlicensed spectrum. Therefore, Type 2A for PSFCH without an SL CO should not be supported. Based on that, we prefer to use Type 2A for PSFCH transmission only within an SL CO.

- Type 2A channel access procedure can be used for PSFCH only within an SL CO.

  - Reference Duration for PSSCH with Groupcast Option 1

In the last meeting, the definition of reference duration for the case that sidelink ACK/NACK enabled has been agreed, the situation of groupcast option 1 needs to be further determined.

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

For the case that only PSSCH transmission(s) with groupcast option 1 (NACK only) enabled can be found in the latest SL CO, the PSSCH transmission UE can utilize the potential NACK feedback to determine the contention window. As PSSCH receiving UEs should report NACK on the same PSFCH resource, the receiving power on the PSFCH resource presents the ratio of accumulated NACK from Rx UEs to some extent, and it should be considered in the contention window adjustment procedure. Therefore, it is reasonable to define a reference duration for the case that only groupcast option 1 is enabled in the last SL CO, using a similar rule of the reference duration of A/N enabled, the end of a reference duration is the end of the first slot where at least one PSSCH with groupcast option 1 enabled is transmitted in the last SL CO.

- For the case that only PSSCH with groupcast option 1 enabled is transmitted in the latest SL CO, a reference duration should be defined as the end of the first slot where at least one PSSCH with groupcast option 1 enabled is transmitted.

  - Contention Window Adjustment

Before discussing the specific details, we should first clarify a basic principle of contention window determining, i.e., the value of contention window used in Type 1 channel access should be adjusted according to the priority of signal to be transmitted, not the cast type of sidelink. In other words, the contention window adjustment should be implemented for each priority including all the cast types with the same priority.

- Contention window adjustment should be determined per priority, i.e., the same value and adjustment steps of the contention window should be used for each priority and all cast types with the same priority.

In general, for the cases that sidelink HARQ feedback information is available, a similar procedure of NR-U should be reused for the contention window adjustment in SL-U.

According to sidelink HARQ feedback schemes, for a resource pool that contains PSFCH resources, the A/N available cases include:

- Case a) unicast with SL-HARQ enabled;

- Case b) groupcast with option 1 SL-HARQ enabled;

- Case c) groupcast with option 2 SL-HARQ enabled;

For case a), the same rule of NR-U contention window adjustment can be reused, i.e., for at least one “ACK” corresponding to PSSCH in the reference duration of the latest SL channel occupancy is received, and set CWp=CWmin,p, otherwise, CWp is increased.

For case b), as only NACK would be reported for groupcast with feedback option 1 and all feedback from receiving UEs should be overlapped in the same PSFCH resource, the receiving power on the associated PSFCH resource can be used for contention window adjustment as it presents the channel status and data receiving result for some extent. Specifically, a receiving power threshold can be configured for this case, when UE detects a signal on the associated PSFCH resource and the signal power is lower than the threshold, it can be considered as at least part of the target receiving UEs feedback as “ACK”, and set CWp=CWmin,p. Otherwise, if the receiving power on the PSFCH exceeds the threshold, CWp is increased.

For case c), as both ACK and NACK should be reported and can be identified respectively, a contention window can be adjusted by assessing the proportion of “ACK”. In detail, according to the member IDs in a group and associated feedback, when at least a ratio of “ACK” corresponding to PSSCH in the reference duration of the latest SL channel occupancy is received, set CWp=CWmin,p, otherwise, CWp is increased.

- For contention window adjustment in SL-U, if at least one PSSCH with HARQ feedback is enabled within a reference duration of the latest SL CO:

  - CWp=CWmin,p for every priority class p∈{1,2,3,4}, when any of the following cases is satisfied:

    - unicast with SL-HARQ feedback enabled, at least one ‘ACK’ is received;

    - groupcast with option 2 enabled (A/N), at least a ratio of ‘ACK’ is received;

    - groupcast with option 1 enabled (Nack only), receiving power on the associated PSFCH is lower than a threshold;

  - otherwise, CWp is increased for every priority class p∈{1,2,3,4};

For the case that sidelink A/N feedback is unavailable as shown below, SL reference duration is not used for contention window determining, and some other factors should be considered to perform contention window adjustment.

- no PSFCH resource is configured in a resource pool;

- for sidelink broadcast transmissions;

- SL-HARQ is disabled by Tx UE for unicast and groupcast;

The basic principle of contention window determination is to modify the contention window based on channel status, i.e., increasing the contention window while the channel tends to have more conflicts. Accordingly, sidelink CR/CBR may be used as an alternative to sidelink A/N, as it presents the ratio of sidelink channels being occupied. As sidelink CR/CBR is already defined and used in Rel-16 sidelink, a limited effort is needed to determine a sidelink CR/CBR based contention window adjustment scheme.

- For the case that SL-HARQ feedback is unavailable, sidelink CR/CBR should be used for contention window adjustment in SL-U.

  - CPE

The following agreement related to CPE is achieved in the last meeting, the further details of using option 1 or option 2 should be clarified.

Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

The main difference between option 1 and option 2 is the available number of symbols that can be used for CPE with 30/60 kHz SCS on sidelink. Considering that continuous sidelink slots should be involved in an SL CO and only one GP symbol is allocated before each AGC symbol, CPE should be within the GP symbol within an SL CO. Otherwise, CPE transmission with a length more than one symbol would conflict with PSSCH/PSFCH transmissions in an SL CO. On the other hand, without an SL CO, multiple CPE starting positions may provide more channel access possibilities. While higher SCSs lead to limited symbol length, CPE with a length of more than one symbol for 30/60 kHz can be used to support multiple CPE starting positions without an SL CO.

- Within an SL CO, a CPE can be transmitted within the symbol just before the next AGC symbol for 15/30/60 kHz.

- Without an SL CO, a CPE within at most 2 symbols can be used only for 30 or 60 kHz SCS.

For S-SSB, one or more UEs may transmit the same content on the same S-SSB resources, while multiple CPE starting positions may lead to an unexpected S-SSB transmission block. Therefore, only one CPE length for S-SSB is preferred.

For PSFCH transmissions, similar to the situation of S-SSB, more than one UE may try to access channel and transmit PSFCH on the same symbol, and multiple CPE starting positions may lead to transmission block. Therefore, only one CPE starting position should be used for PSFCH transmission.

- Support only one CPE starting position for S-SSB.

- Support only one CPE starting position for PSFCH.

Furthermore, multiple CPE lengths should be used for PSCCH/PSSCH transmission. A longer CPE length, i.e., an earlier CPE starting position, should be applied for sidelink transmission with higher priority. It may contribute to the success rate for higher priority services by using different CPE lengths for different priorities. Therefore, in the case that multiple CPE starting positions are configured for sidelink, the different CPE lengths should be used for different priorities.

- Multiple CPE lengths (starting positions) are supported for sidelink, and the different CPE lengths should be configured per priority.

- Mode 1

As described in [4] “Uu operation for mode 1 is limited to licensed spectrum only”, it means that gNB should schedule unlicensed spectrum resources for sidelink while it may not work on the unlicensed spectrum. According to the scheduling of gNB, UE should first perform a channel access procedure to obtain the scheduled sidelink resources, and then transmit sidelink signals on the assigned resources if channel access succeeds.

In the case that gNB does not work in the unlicensed spectrum, an assistant scheme may be mentioned that UE may report sensing information of the unlicensed spectrum to gNB on Uu for reference, and then gNB schedules sidelink resources in the unlicensed spectrum taking into account the assistant information. However, the sensing information reported by the UE introduces additional resource consumption on Uu, while the information may be with low timeliness by the way of Uu transmission and low accuracy with the limited receiving ability of UE. As a whole, the assistant information reporting may not be able to effectively help the scheduling of sidelink in the unlicensed spectrum. Therefore, it is unnecessary to exchange such information between UE and gNB.

- For mode 1 in SL-U, no assistant information related to the scheduling in the unlicensed spectrum needs to be exchanged between UE and gNB.

As COT sharing may benefit sidelink transmissions, it should also be supported in mode 1 resource allocation. According to the agreement that “gNB relaying/forwarding a UE initiated COT to another UE is not supported in Rel-18”, an intended SL CO may be considered to be scheduled by gNB and be shared for sidelink UEs to access channel with higher efficiency.

To execute mode 1 COT sharing on sidelink, gNB may indicate an intended SL CO for UEs which are scheduled on multiple consecutive slots, and an initial UE should also be assigned by gNB to perform the Type 1 channel access procedure to initial the intended CO. As the intended SL CO is indicated by gNB before the actual initiation, no additional indication is required to be relayed or forwarded by gNB from the initial UE to the responding UE(s). When the intended SL CO is successfully initiated, other UEs scheduled on the following slots could share the CO by using Type 2 channel access schemes. Besides, as no further indication from gNB after the intended SL CO is initiated, a common scheme for sidelink COT sharing could be designed for both mode 1 and mode 2 cases.

- The gNB schedules UEs in an intended SL CO which is to be initiated by a sidelink UE, and the SL CO may be shared by other scheduled UEs.

From the perspective of system allocation, it may be configured that mode 1 sidelink resource pools are assigned in both licensed spectrum and unlicensed spectrum. In other words, gNB may schedule sidelink resources for a UE in the licensed resources or the unlicensed resources. In this case, a specific issue to be considered is how to identify the DCI for legacy sidelink scheduling in the licensed spectrum and the ones for SL-U assignment. For example, as a dedicated RNTI is used for sidelink scheduling, another RNTI may be assigned for UE to identify DCI for SL-U scheduling.

- In the case that both licensed and unlicensed spectrum resources are configured for sidelink mode 1, it needs to be considered how to identify DCI for sidelink scheduling on the licensed spectrum or the unlicensed spectrum.

- Mode 2

  - Enhancement of Mode 2 Procedure for SL-U

For legacy sidelink, the mode 2 sensing and resource selection procedure is defined based on dedicated sidelink resource pools, and it can mitigate or avoid potential resource collisions among sidelink UEs. By using the mode 2 procedure, a UE can reserve resources for further transmissions and other UEs may exclude the reserved resources from the candidate resource set.

In the scenario of SL-U, channel resources may be occupied by not only sidelink UEs but also devices of other types of RATs, and then the resource collisions that are not caused by sidelink UEs cannot be identified and solved by using the legacy mode 2 procedure. Besides, the devices of other RATs cannot receive and understand the SCI, i.e., the resource reservation of a sidelink UE is invalid for other types of devices. Therefore, other types of devices may try to occupy the channel which may be overlapped with the reserved resources. It may lead to LBT failure of the sidelink UE and decrease the efficiency of using the legacy mode 2 procedure in SL-U.

The legacy mode 2 sensing and resource selection procedure is based on a dedicated sidelink resource pool in which potential resource collisions are only considered among sidelink UEs.

LBT failure may cause the resources selected by using the legacy mode 2 procedure cannot be used by the Tx UE, and further lead to lower resource efficiency of SL-U.

Considering the potential impact of LBT failure, it is necessary to discuss and determine how to improve the legacy mode 2 procedure for SL-U.

For example, in the legacy mode 2 procedure, a reserved resource indicated by UE A should be excluded from the candidate resource set of UE B according to relevant RSRP thresholds. In the case that the reserved resource is not actually used by UE A as LBT failure or other issues, the reserved resource may be available for UE B. In other words, the reserved resources may be considered to be retained in the candidate resource set. As there is some uncertainty about the resource reservation in SL-U, it should be considered to improve the mode 2 procedure for SL-U.

Besides, the RSRP threshold is a key parameter in mode 2 procedure which is used to evaluate whether a resource should be excluded from the candidate resource set. The RSRP threshold is semi-static configured per resource pool and may be increased if candidate resources are not enough. In SL-U, the channel status is affected by many factors and may be unstable, a semi-static configured RSRP threshold for mode 2 resource excluding cannot adapt well to the changeable channel condition. To make mode 2 procedure more suitable for SL-U, it should be considered to set the RSRP threshold with more flexibility according to the channel status.

Considering COT sharing and multi-consecutive slots transmission are supported in SL-U, UE may access the channel within a shared SL CO much easier than selecting resources out of an SL CO. With Rel-16 NR SL mode 2 resource selection, as each UE selects resources randomly in its candidate resource set, it cannot ensure using multi-consecutive slots for a single UE, nor does it prioritize using multi-consecutive slots among multiple UEs. Therefore, how to support MCSt in terms of mode 2 resource selection needs further study.

- Considering the potential improvement of mode 2 procedure to make it more appropriate for SL-U, the following factors may be considered:

  - uncertainty of the reserved resources indicated in SCI of UEs;

  - RSRP threshold used in excluding resources;

  - COT information;

  - Multi-consecutive Slots Transmission (MCSt).

According to the previous agreement on multi-consecutive slots transmission, what types of parameters and candidate resource sets should be provided in mode 2 resource selection is discussed, and potential options are listed in the agreement.

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different LsubCH sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different LsubCH sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

For SL-U, selecting multi-consecutive slots transmission may be more efficient as it helps to maintain channel occupancy. As a whole, to avoid introducing extra complexity and modification on standards, Rel-16/17 mode 2 procedure should be reused as a baseline.

With the legacy mode 2 procedure, the parameters related to a TB to be transmitted are given by MAC layer to PHY layer for resource selection per TB, including priority, the number of sub-channels, etc. Based on that, option 1 is preferred and the set of parameters should be associated with a single TB or more TBs with the same parameters. Accordingly, with a similar principle, option B is preferred for the candidate resource set report which has less impact on the legacy procedure.

Based on option 1 and option B, resource selection should be determined by MAC layer which obtains one or more candidate resource set reports, and MAC layer may further prefer to select consecutive slots if available for transmissions of a single TB or multiple TBs.

- For MCSt resource selection, option 1 is preferred, i.e., only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided to PHY layer.

- For MCSt resource selection, option B is preferred, i.e., PHY layer reports candidate single-slot resources to higher layer.

- COT Sharing in SL-U

According to the agreement of COT sharing as follows, some further details need to be discussed.

Agreement For UE-to-UE COT sharing, - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - FFS: UE forwarding/relaying information about a COT initiated by another UE. Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated - A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) - FFS: all other details and additional restrictions

When an SL CO is initiated and allowed to be shared by other UEs, the initial UE should indicate relevant information about the SL CO to potential responding UE(s) through SCI, including the CAPC level used for initiating the SL CO, the remaining duration of the CO, and the RB set(s) contained in the CO. Besides, based on the initial UE’s requirements, it can also disable the COT sharing, and a relevant indicator may also be included in the SCI.

- The information about an SL CO should be indicated by the initiating UE through SCI, including:

  - Whether the CO is allowed to be shared;

  - CAPC level used for initiating the CO;

  - The remaining duration of the CO;

  - RB set(s) in the CO;

  - Additional ID;

Considering the information overhead, especially when additional ID(s) are supported, it is hard to involve the information along with an SCI associated with a PSSCH transmission, and a dedicated SCI to indicate the information of an SL CO should be introduced.

- The information about an SL CO should be indicated through a dedicated SCI.

As each PSFCH resource is associated with relevant PSSCH transmission and there are no PSFCH resource conflicts for PSSCH transmissions without conflict. That is to say, within an SL CO, UEs can utilize relevant PSFCH resources respectively without additional conflicts. Therefore, besides the PSFCH transmission intended for the initiating UE of an SL CO, it should also be supported for the PSFCH transmission(s) to a destination UE other than the initiating UE.

- For sidelink UE-to-UE COT sharing, a responding UE can share the CO for PSFCH transmission(s) to a destination UE other than the initiating UE of the SL CO.

Considering the case that an SL CO may be successively shared by multiple responding UEs on consecutive slots, the COT sharing related SCI from the initial UE may be not detected by some UEs, and the relevant information of the SL CO should be relayed during the CO. That is to say, besides the initial UE, the responding UE may need to forward the information with an updated remaining duration of the CO.

- The information of a shared CO should be forwarded by the responding UE through SCI:

  - The remaining duration of the SL CO should be updated;

On the other hand, there are different understandings of gNB-to-UE COT sharing among companies. In our view, according to the description “Uu operation for mode 1 is limited to licensed spectrum only”, it aims to pursue a concise way to support gNB scheduling for SL-U and simplify the study of mode 1 without gNB operation in unlicensed spectrum. If gNB-to-UE COT sharing is supported, it may require gNB to operate in the unlicensed spectrum to share resources with sidelink UEs, the requirement “Uu operation for mode 1 is limited to licensed spectrum only” would instead lead to additional complexity for mode 1 discussion. Therefore, we propose to not support gNB-to-UE COT sharing at this stage.

- The gNB-to-UE COT sharing is not supported for SL-U.

- Multi-channel Operation

Regarding multi-channel operation in SL-U, the agreement is provided as follows.

Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

For a PSSCH transmission with multiple RB sets, the associated feedback transmission may not need to be transmitted on all the RB sets. In addition, in the case that the associated PSFCH resource is out of the SL CO used by the PSSCH transmission, Rx UE may have to perform independent channel access procedures to obtain the PSFCH transmission occasion. Hence, PSFCH transmission on a subset of RB sets for associated PSSCH transmission should be supported to facilitate the feedback transmission.

For a PSSCH transmission on multiple channels, the associated PSFCH should be transmitted on a subset of the multiple channels.

For a sidelink UE with multiple PSFCH transmissions on multiple channels, both NR-U DL Type A and Type B multi-channel access schemes should be considered, and the type of multi-channel access schemes may be determined by UE implementation or based on (pre-)configuration.

For dynamic channel access mode with multi-channel in SL-U, both Type A and Type B should be considered for PSFCH transmission.

- Conclusion

In this contribution, some issues related to the channel access procedure for Rel-18 SL-U are discussed, and the relevant observations and proposals are listed as follows:

Observation 1:	The legacy mode 2 sensing and resource selection procedure is based on a dedicated sidelink resource pool in which potential resource collisions are only considered among sidelink UEs.

Observation 2:	LBT failure may cause the resources selected by using the legacy mode 2 procedure cannot be used by the Tx UE, and further lead to lower resource efficiency of SL-U.

Proposal 1:	Type 2A channel access procedure can be used for PSFCH only within an SL CO.

Proposal 2:	For the case that only PSSCH with groupcast option 1 enabled is transmitted in the latest SL CO, a reference duration should be defined as the end of the first slot where at least one PSSCH with groupcast option 1 enabled is transmitted.

Proposal 3:	Contention window adjustment should be determined per priority, i.e., the same value and adjustment steps of the contention window should be used for each priority and all cast types with the same priority.

Proposal 4:	For contention window adjustment in SL-U, if at least one PSSCH with HARQ feedback is enabled within a reference duration of the latest SL CO:

−	CWp=CWmin,p for every priority class p∈{1,2,3,4}, when any of the following cases is satisfied:

	unicast with SL-HARQ feedback enabled, at least one ‘ACK’ is received;

	groupcast with option 2 enabled (A/N), at least a ratio of ‘ACK’ is received;

	groupcast with option 1 enabled (Nack only), receiving power on the associated PSFCH is lower than a threshold;

−	otherwise, CWp is increased for every priority class p∈{1,2,3,4};

Proposal 5:	For the case that SL-HARQ feedback is unavailable, sidelink CR/CBR should be used for contention window adjustment in SL-U.

Proposal 6:	Within an SL CO, a CPE can be transmitted within the symbol just before the next AGC symbol for 15/30/60 kHz.

Proposal 7:	Without an SL CO, a CPE within at most 2 symbols can be used only for 30 or 60 kHz SCS.

Proposal 8:	Support only one CPE starting position for S-SSB.

Proposal 9:	Support only one CPE starting position for PSFCH.

Proposal 10:	Multiple CPE lengths (starting positions) are supported for sidelink, and the different CPE lengths should be configured per priority.

Proposal 11:	For mode 1 in SL-U, no assistant information related to the scheduling in the unlicensed spectrum needs to be exchanged between UE and gNB.

Proposal 12:	The gNB schedules UEs in an intended SL CO which is to be initiated by a sidelink UE, and the SL CO may be shared by other scheduled UEs.

Proposal 13:	In the case that both licensed and unlicensed spectrum resources are configured for sidelink mode 1, it needs to be considered how to identify DCI for sidelink scheduling on the licensed spectrum or the unlicensed spectrum.

Proposal 14:	Considering the potential improvement of mode 2 procedure to make it more appropriate for SL-U, the following factors may be considered:

−	uncertainty of the reserved resources indicated in SCI of UEs;

−	RSRP threshold used in excluding resources;

−	COT information;

Proposal 15:	For MCSt resource selection, option 1 is preferred, i.e., only one set of parameters (prioTX, remaining PDB, LsubCH and Prsvp_TX) is provided to PHY layer.

Proposal 16:	For MCSt resource selection, option B is preferred, i.e., PHY layer reports candidate single-slot resources to higher layer.

Proposal 17:	The information about an SL CO should be indicated by the initiating UE through SCI, including:

−	Whether the CO is allowed to be shared;

−	CAPC level used for initiating the CO;

−	The remaining duration of the CO;

−	RB set(s) in the CO;

−	Additional ID;

Proposal 18:	The information about an SL CO should be indicated through a dedicated SCI.

Proposal 19:	For sidelink UE-to-UE COT sharing, a responding UE can share the CO for PSFCH transmission(s) to a destination UE other than the initiating UE of the SL CO.

Proposal 20:	The information of a shared CO should be forwarded by the responding UE through SCI:

−	The remaining duration of the SL CO should be updated;

Proposal 21:	The gNB-to-UE COT sharing is not supported for SL-U.

Proposal 22:	For a PSSCH transmission on multiple channels, the associated PSFCH should be transmitted on a subset of the multiple channels.

Proposal 23:	For dynamic channel access mode with multi-channel in SL-U, both Type A and Type B should be considered for PSFCH transmission.

- Reference

- Report of RAN1 #112

- Report of RAN1 #111

- Report of RAN1 #110bis-e

- RP-230077, WID revision: NR sidelink evolution