3GPP TSG RAN WG1 #112bis-e											R1-2303687

e-Meeting, April 17th – April 26th,2023

Agenda Item:	9.4.2

Source: 	NEC

Title:	Co-existence between LTE Sidelink and NR Sidelink

Document for:	Discussion/Decision

- Introduction

During previous meetings, agreements of co-channel co-existence of LTE sidelink and NR sidelink are achieved in [1]~[3]. Based on that, detailed analyses and schemes need to be discussed, and further views on related issues are presented in this contribution.

- Dynamic Resource Pool Sharing

First, the exact definition of “dynamic resource pool sharing” should be clarified.

As shown in Figure 1 case (a), LTE sidelink resource pool is partially overlapping with NR sidelink resource pool in the time domain, i.e., part of the subframes within the LTE sidelink are overlapped with slots of NR sidelink. Based on different subframe/slot allocation of the two RATs, it may lead to resource reservation period mismatch between LTE sidelink and NR sidelink, i.e., NR sidelink module cannot easily determine LTE SL resource reservation and exclude the candidate resources which are overlapped with reserved resources of LTE SL transmission.

In the case that an LTE sidelink resource pool and an NR sidelink resource pool are partially overlapped in time domain, the mismatch between LTE sidelink subframes and NR sidelink slots may introduce extra complexity for NR SL module resource selection, i.e., it is hard to exclude periodic reserved resource for LTE sidelink transmission due to the different sidelink subframe/slot allocation for LTE sidelink and NR sidelink.

In case (b), part of the RBs of the LTE sidelink resource pool are overlapped with NR sidelink resource pool, i.e., it is partially overlapping in the frequency domain of the two RATs. A similar issue needs to be considered as shown in case (a), while RBs and sub-channel allocation are different within resource pools for LTE sidelink and NR sidelink respectively, extra complexity and rules for excluding reserved resources of LTE sidelink transmission are needed to perform resource selection by NR SL module.

In the case that an LTE sidelink resource pool and an NR sidelink resource pool are partially overlapped in frequency domain, extra complexity and rules are needed to exclude reserved sub-channels for LTE sidelink transmission by NR SL module.

Figure 1 Cases for partial and full overlapping resource pools of LTE sidelink and NR sidelink

On the other hand, using a full overlapping resource pool as shown in case (c), a whole resource pool is configured as sharing resources for both LTE sidelink and NR sidelink, i.e., all the slots/subframes and RBs used for the two RATs are the same. Based on that, some more simple and general solutions for co-existence with a dynamic resource pool can be achieved.

Furthermore, considering the assistant information provided by LTE SL module to NR SL module for resource selection and collision avoidance, it is much simpler for both LTE SL module and NR SL module within the same shared resource pool. In detail, with the full overlapping resource pool, the LTE SL module can generate and provide the assistant information about LTE sidelink to NR SL module by using logical subframe index and sub-channel index in the resource pool directly. Meanwhile, the NR SL module can also apply the assistant information based on the shared resource pool without any conversion, such as determining the resources to be excluded according to reserved resources and relevant periods provided by LTE SL module. Otherwise, if partial overlapping resource pools are configured, it is more complex to understand and use the assistant information for NR SL module as the information is determined in an LTE sidelink resource pool.

Considering the complexity, we prefer to clarify that dynamic resource pool sharing should be discussed based on the full overlapping resource pool, i.e., a shared resource pool should be configured for both LTE sidelink and NR sidelink.

- A shared resource pool should be configured for both LTE sidelink and NR sidelink as a whole, i.e., partial overlapping of LTE sidelink and NR sidelink resources in time domain or frequency domain should not be supported.

According to the description in [4], 15/30 kHz are supported for dynamic resource pool sharing, further details of resource configuration of dynamic sharing resource pool should be clarified.

- RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe

As all symbols in a subframe are configured as sidelink resources in LTE, if only part of symbols in a slot are assigned as NR sidelink resources, it would introduce additional AGC issue on LTE sidelink. An example of NR sidelink with 7 symbols from the start of a slot is shown in Figure 2. In this case, symbol #6 of LTE sidelink would endure an inevitable power drop with 15 kHz of NR sidelink, and the same issue is on symbols #3 and #10 while NR sidelink with 30 kHz.

Figure 2 Partial overlapping of symbols for LTE sidelink and NR sidelink

To mitigate AGC impact on LTE sidelink within dynamic sharing resource pool, it is reasonable to assign all symbols in a slot for NR sidelink.

- Within a shared resource pool, all symbols in a slot should be configured as sidelink symbols for NR sidelink with 15/30 kHz.

In the frequency domain, the sub-channel size of NR sidelink should be configured by taking into account the size of LTE sub-channel. As shown in Figure 3, for the case that the sub-channel size of LTE sidelink and NR sidelink are independently configured, the misaligned sub-channel sizes would require more complexity for NR sidelink resource selection. For example, when sub-channel #0 and #1 of LTE sidelink are reserved by LTE UE, with misaligned sub-channel configuration, sub-channel #0 and #1 of NR sidelink cannot be used, while sub-channel #1 is available for the case that sub-channels are aligned between the two RATs. Obviously, it also benefits resource efficiency for co-existence.

Based on the above analyses, the sub-channel size of NR sidelink should be configured as an integer multiple of the sub-channel size of LTE sidelink in the shared resource pool, and the sub-channel boundaries of the two RATs should be aligned.

Figure 3 Sub-channel size for LTE sidelink and NR sidelink

- Within a shared resource pool, the sub-channel size of NR sidelink should be an integer multiple of the sub-channel size of LTE sidelink, and the boundary of NR sub-channel should be aligned with LTE sub-channel.

For the case that PSCCH resource pool and PSSCH resource pool are non-adjacent in LTE sidelink, in order to avoid impact on LTE PSCCH and simplify the study of the co-existence, it is better to restrict the shared resources as PSSCH resource pool of LTE, i.e., using the PSSCH resource pool of LTE sidelink as the shared resource pool for NR sidelink.

- For non-adjacent PSCCH/PSSCH resource configuration of LTE sidelink, only the PSSCH resource pool can be shared with NR sidelink.

- Resource (re)selection based on Assistant Information

In the previous meetings, relevant agreements of assistant information and resource excluding are achieved as follows.

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

According to the mode 2 resource selection mechanism, the NR SL module should exclude resources that are on the slots not sensed and reserved with relevant priority and RSRP threshold. With a similar principle, in order to exclude unavailable resources in a shared resource pool, the relevant assistant information should be provided by the LTE SL module. In detail, the assistant information should at least includes:

- a) subframes that are not sensed by the LTE SL module due to half-duplex;

- b) decoded LTE SCIs which indicate reserved resources, relevant period, and priority;

- c) LTE SL RSRP measurement results;

- d) reserved resources, relevant period, and priority of its LTE SL transmissions;

The information (a) is used to exclude subframes without sensing information, (b) and (c) are used for excluding reserved resources that meet the threshold, and item (d) aims to avoid transmission collision between its LTE SL module and NR SL module.

- The assistant information provided by LTE SL module to NR SL module should include:

  - subframes not monitored by the LTE SL module;

  - decoded LTE SCIs which indicate time and frequency locations of reserved resources, reservation periods and priorities;

  - LTE SL RSRP measurement results;

  - time and frequency locations of reserved resources, reservation periods and priorities of its LTE SL transmissions.

- For the resource (re)selection in a dynamic sharing resource pool, NR SL module should exclude candidate resources on the subframes that are not monitored by LTE module.

When excluding NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE, a relevant SL RSRP threshold should be (pre-)configured, and several options for the RSRP threshold are listed in the agreements above. To provide more flexibility and avoid impact on LTE sidelink transmission, it is preferred to use a dedicated RSRP threshold for dynamic resource sharing, i.e., Alt 3 is preferred.

For the NR SL candidate resources overlapping with resources selected for its own LTE SL transmissions, to avoid impact on LTE SL module, it is preferred to exclude the overlapping resources from NR SL candidate resource set, i.e. option 1 should be adopted.

- To exclude NR sidelink candidate resources overlapping with resources reserved by other LTE sidelink UEs, Alt 3 is preferred, i.e., an SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing should be introduced.

- For NR sidelink candidate resources overlapping with resources selected to be used for LTE SL module’s own LTE SL transmission, option 1 is preferred, i.e., the PHY layer of NR SL module should exclude the candidate resources.

- Conclusion

In this contribution, some analyses of sidelink co-channel co-existence for dynamic resource pool sharing are discussed, and the relevant observations and proposals are listed as follows:

Observation 1:	In the case that an LTE sidelink resource pool and an NR sidelink resource pool are partially overlapped in time domain, the mismatch between LTE sidelink subframes and NR sidelink slots may introduce extra complexity for NR SL module resource selection, i.e., it is hard to exclude periodic reserved resource for LTE sidelink transmission due to the different sidelink subframe/slot allocation for LTE sidelink and NR sidelink.

Observation 2:	In the case that an LTE sidelink resource pool and an NR sidelink resource pool are partially overlapped in frequency domain, extra complexity and rules are needed to exclude reserved sub-channels for LTE sidelink transmission by NR SL module.

Proposal 1:	A shared resource pool should be configured for both LTE sidelink and NR sidelink as a whole, i.e., partial overlapping of LTE sidelink and NR sidelink resources in time domain or frequency domain should not be supported.

Proposal 2:	Within a shared resource pool, all symbols in a slot should be configured as sidelink symbols for NR sidelink with 15/30 kHz.

Proposal 3:	Within a shared resource pool, the sub-channel size of NR sidelink should be an integer multiple of the sub-channel size of LTE sidelink, and the boundary of NR sub-channel should be aligned with LTE sub-channel.

Proposal 4:	For non-adjacent PSCCH/PSSCH resource configuration of LTE sidelink, only the PSSCH resource pool can be shared with NR sidelink.

Proposal 5:	The assistant information provided by LTE SL module to NR SL module should include:

−	subframes not monitored by the LTE SL module;

−	decoded LTE SCIs which indicate time and frequency locations of reserved resources, reservation periods and priorities;

−	LTE SL RSRP measurement results;

−	time and frequency locations of reserved resources, reservation periods and priorities of its LTE SL transmissions.

Proposal 6:	For the resource (re)selection in a dynamic sharing resource pool, NR SL module should exclude candidate resources on the subframes that are not monitored by LTE module.

Proposal 7:	To exclude NR sidelink candidate resources overlapping with resources reserved by other LTE sidelink UEs, Alt 3 is preferred, i.e., an SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing should be introduced.

Proposal 8:	For NR sidelink candidate resources overlapping with resources selected to be used for LTE SL module’s own LTE SL transmission, option 1 is preferred, i.e., the PHY layer of NR SL module should exclude the candidate resources.

- Reference

- Report of RAN1#112

- Report of RAN1 #111

- Report of RAN1 #110bis-e

- RP-230077, WID revision: NR sidelink evolution