3GPP TSG RAN WG1 #112bis			R1-2303691

e-Meeting, April 17th –26rd, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on value range mismatch of p0 for SRS/PUCCH in Rel-17 unified TCI framework

Agenda Item:	7.2

Document for: 	Discussion and Decision

- Introduction

In this contribution, we discuss an issue of value range mismatch of p0 for SRS/PUCCH in Rel-17 unified TCI framework.

- Discussion

In Rel-17 feMIMO WI, unified TCI framework was specified, which intends to apply a common beam to multiple signals/channels in an efficient configuration/indication manner.

In unified TCI framework, TPC parameters, as well as TCI state, can also be dynamically indicated. The basic approach is that: a set of TPC parameters (i.e., RS index 𝑞𝑑, p0, 𝛼 and power control adjustment state 𝑙) can be configured, and one set of the TPC parameters can be determined according to the indicated TCI-state. The corresponding specification is extracted from 38.213 [1], as below:

# 7 Uplink Power control […] In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State and for an indicated TCI-State or TCI-UL-State as described in [6, TS 38.214] - in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS - in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State - in clause 7.3.1, if p0AlphaSetforSRS is provided, - if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set […]

The yellow-highlighted part describes how p0, 𝛼 and power control adjustment state 𝑙 are provided and indicated, for each of PUSCH, PUCCH and SRS. When focusing on p0, PO_UE_PUSCH,b,f,cj, PO_PUCCH,b,f,cqu and PO_SRS,b,f,cqs are considered for PUSCH, PUCCH and SRS, respectively. The following is the corresponding RRC parameters specified in 38.331 [2], where, as can be seen, a single RRC parameter p0AlphaSet-r17 (p0-r17 for p0), whose value range is {-16..15}, is used for indication of p0 for each of PUSCH/PUCCH/SRS:

#### – Uplink-PowerControl The IE Uplink-PowerControl is used to configure UE specific power control parameter for PUSCH, PUCCH and SRS. Uplink-PowerControl information element -- ASN1START -- TAG-UPLINK-POWERCONTROL-START Uplink-powerControl-r17 ::= SEQUENCE { ul-powercontrolId-r17 Uplink-powerControlId-r17, p0AlphaSetforPUSCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforPUCCH-r17 P0AlphaSet-r17 OPTIONAL, -- Need R p0AlphaSetforSRS-r17 P0AlphaSet-r17 OPTIONAL -- Need R } P0AlphaSet-r17 ::= SEQUENCE { p0-r17 INTEGER (-16..15) OPTIONAL, -- Need R alpha-r17 Alpha OPTIONAL, -- Need S closedLoopIndex-r17 ENUMERATED { i0, i1 } } Uplink-powerControlId-r17 ::= INTEGER(1.. maxUL-TCI-r17) -- TAG-UPLINK-POWERCONTROL-STOP -- ASN1STOP

Observation 1:

- In Rel-17 unified TCI framework, the following TPC parameters can be configured by p0-r17, and determined based on the indicated TCI

  - PO_UE_PUSCH,b,f,cj for PUSCH transmit power control

  - PO_PUCCH,b,f,cqu for PUCCH transmit power control

  - PO_SRS,b,f,cqs for SRS transmit power control

Next, we show the exact UE behavior regarding transmit power control in NR, focusing on the difference between PUSCH and SRS. For PUSCH transmit power control, as described below from [1], the target power PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j).

### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, - PCMAX,f,c(i)is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier f of serving cell c in PUSCH transmission occasion i. - PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1. […] - For j∈2,…,J-1=SJ, a PO_NOMINAL,PUSCH,f,cj value, applicable for all j∈SJ, is provided by p0-NominalWithGrant, or PO_NOMINAL,PUSCH,f,cj=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant is not provided, for each carrier f of serving cell c and a set of PO_UE_PUSCH,b,f,cj values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP b of carrier f of serving cell c […]

PO_NOMINAL,PUSCH,f,cj is the “absolute” target power, while PO_UE_PUSCH,b,f,cj is the “differential” target power. PO_PUSCH,b,f,c(j) is derived from the sum of absolute target power and differential target power. The range of PO_UE_PUSCH,b,f,cj (i.e., p0 in P0-PUSCH-AlphaSet) is (-16..15), as specified in [2]:

P0-PUSCH-AlphaSet ::= SEQUENCE { p0-PUSCH-AlphaSetId P0-PUSCH-AlphaSetId, p0 INTEGER (-16..15) OPTIONAL, -- Need S alpha Alpha OPTIONAL -- Need S }

Meanwhile, the following is the corresponding part for SRS transmit power from [1]:

### 7.3.1 UE behaviour If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasion i as [dBm] where, - PCMAX,f,c(i) is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS 38.101-2] and [TS 38.101-3] for carrier f of serving cell c in SRS transmission occasion i - PO_SRS,b,f,c(qs) is provided by p0 for active UL BWP b of carrier f of serving cell c and SRS resource set qs provided by SRS-ResourceSet and SRS-ResourceSetId - MSRS,b,f,c(i) is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion i on active UL BWP b of carrier f of serving cell c and μ is a SCS configuration defined in [4, TS 38.211] […]

As can be seen above, unlike PO_PUSCH,b,f,c(j) for PUSCH, PO_SRS,b,f,c(qs) is provided by p0 only, where p0 is the absolute value.

In Rel.15 TCI framework, RRC parameter of p0 = {-202..24} can be configured per SRS-ResourceSet as below.

SRS-ResourceSet ::= SEQUENCE { srs-ResourceSetId SRS-ResourceSetId, srs-ResourceIdList SEQUENCE (SIZE(1..maxNrofSRS-ResourcesPerSet)) OF SRS-ResourceId OPTIONAL, -- Cond Setup resourceType CHOICE { aperiodic SEQUENCE { aperiodicSRS-ResourceTrigger INTEGER (1..maxNrofSRS-TriggerStates-1), csi-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook slotOffset INTEGER (1..32) OPTIONAL, -- Need S ..., [[ aperiodicSRS-ResourceTriggerList SEQUENCE (SIZE(1..maxNrofSRS-TriggerStates-2)) OF INTEGER (1..maxNrofSRS-TriggerStates-1) OPTIONAL -- Need M ]] }, semi-persistent SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... }, periodic SEQUENCE { associatedCSI-RS NZP-CSI-RS-ResourceId OPTIONAL, -- Cond NonCodebook ... } }, usage ENUMERATED {beamManagement, codebook, nonCodebook, antennaSwitching}, alpha Alpha OPTIONAL, -- Need S p0 INTEGER (-202..24) OPTIONAL, -- Cond Setup pathlossReferenceRS PathlossReferenceRS-Config OPTIONAL, -- Need M srs-PowerControlAdjustmentStates ENUMERATED { sameAsFci2, separateClosedLoop} OPTIONAL, -- Need S ..., [[ pathlossReferenceRSList-r16 SetupRelease { PathlossReferenceRSList-r16} OPTIONAL -- Need M ]], [[ usagePDC-r17 ENUMERATED {true} OPTIONAL, -- Need R availableSlotOffsetList-r17 SEQUENCE (SIZE(1..4)) OF AvailableSlotOffset-r17 OPTIONAL, -- Need R followUnifiedTCI-StateSRS-r17 ENUMERATED {enabled} OPTIONAL -- Need R ]] }

Based on above, definition of the value range of PO_UE_PUSCH,b,f,cj for PUSCH and PO_SRS,b,f,cqs for SRS are different. However, in TS38.331, the same RRC parameter of p0-r17 = {-16..15} in P0AlphaSet-r17 is used for the indication of both PUSCH and SRS. Consequently, the value range of p0 for SRS is unnecessary restricted based on the current specifications when unified TC framework is configured.

Observation 2:

- PO_UE_PUSCH,b,f,cj for PUSCH transmit power control is the differential target power in TS38.213.

  - The value range is (-16..15)

- PO_SRS,b,f,cqs for SRS transmit power control is the absolute target power in TS38.213.

  - The value range is (-202..24)

- The same RRC parameter of p0-r17 in P0AlphaSet-r17, whose value range is {-16..15}, is used for the indication of PO_UE_PUSCH,b,f,cj for PUSCH and PO_SRS,b,f,cqs for SRS in TS38.331.

There are two possible options to solve the above issue. Option 1 is that existing p0-r17 = {-16..15} in P0AlphaSet-r17 is not used for SRS, and to introduce new RRC parameter of p0Srs-r17 = {-202..24} in P0AlphaSet-r17 to indicate the absolute target power for SRS. Since it introduces new RRC parameter, we also need to send LS to RAN2 to inform the update, if agreed. Option 2 is to reuse existing RRC parameter p0-r17 = {-16..15} in P0AlphaSet-r17 for SRS, and RAN1 can discuss how to indicate the absolute target power for SRS. For example, the absolute target power for SRS is the sum of a component of absolute value (e.g., reuse PO_SRS,b,f,c(qs)) and a component of differential value (i.e. existing p0-r17). In this case, for the configuration of the absolute value, whether the existing other RRC parameter can be reused or new RRC parameter is needed can be discussed. Since the current TS38.213 is specified that the absolute value of p0 is configured for SRS, TS38.213 should be also updated for SRS in unified TCI framework. Considering Option 2 has larger impacts on RAN1 spec., we prefer Option 1.

Proposal 1:

- RAN1 to discuss how to resolve the issue of value range mismatch on p0 for SRS in Rel-17 unified TCI framework.

- Down select from the following options (Option 1 is preferred):

  - Option 1: Existing p0-r17 = {-16..15} in P0AlphaSet-r17 is not used for SRS, and introduce new RRC parameter of p0Srs-r17 = {-202..24} in P0AlphaSet-r17 to indicate the absolute target power for SRS.

    - Send LS to RAN2 to inform the above.

  - Option 2: Reuse existing p0-r17 = {-16..15} in P0AlphaSet-r17 for SRS, and discuss in RAN1 how to indicate the absolute target power for SRS.

Moreover, similar to SRS, Rel-17 unified TCI based p0 configuration/update for PUCCH also have an issue. According to section 7 of 38.213 [1], if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the values of PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State by using the same RRC parameter P0AlphaSet-r17, which means the value range (-16..15) is considered for p0. PO_PUCCH,b,f,cqu, however, is a parameter of composed of the sum of a component PO_NOMINAL,PUCCH (i.e., absolute target power) and a component PO_UE,PUCCH (i.e., differential target power). Therefore, the value range of p0-r17 doesn’t fit the actual range of PO_PUCCH,b,f,cqu.

The issue can be resolved more easily, compared to SRS described above, based on a straightforward modification in RAN1 specification in our view. For example, since PUCCH transmit power control considers differential target power PO_UE,PUCCH already, Rel-17 unified TCI based p0 configuration/update can just point to that parameter, instead of PO_PUCCH,b,f,c. This approach doesn’t cause any change for RAN2 RRC parameters, or any change for the behavior intended in RAN1 specification in principle.

Proposal 2:

- For the issue of value range mismatch on p0 for PUCCH in Rel-17 unified TCI framework, apply the CR proposed in R1-2303692 [3]

- Conclusion

In this contribution, we discussed an issue of value range mismatch of p0 for in Rel-17 unified TCI framework, and we made the following observations and proposal.

Observation 1:

- In Rel-17 unified TCI framework, the following TPC parameters can be configured by p0-r17, and determined based on the indicated TCI

  - PO_UE_PUSCH,b,f,cj for PUSCH transmit power control

  - PO_PUCCH,b,f,cqu for PUCCH transmit power control

  - PO_SRS,b,f,cqs for SRS transmit power control

Observation 2:

- PO_UE_PUSCH,b,f,cj for PUSCH transmit power control is the differential target power in TS38.213.

  - The value range is (-16..15)

- PO_SRS,b,f,cqs for SRS transmit power control is the absolute target power in TS38.213.

  - The value range is (-202..24)

- The same RRC parameter of p0-r17 in P0AlphaSet-r17, whose value range is {-16..15}, is used for the indication of PO_UE_PUSCH,b,f,cj for PUSCH and PO_SRS,b,f,cqs for SRS in TS38.331.

Proposal 1:

- RAN1 to discuss how to resolve the issue of value range mismatch on p0 for SRS in Rel-17 unified TCI framework.

- Down select from the following options (Option 1 is preferred):

  - Option 1: Existing p0-r17 = {-16..15} in P0AlphaSet-r17 is not used for SRS, and introduce new RRC parameter of p0Srs-r17 = {-202..24} in P0AlphaSet-r17 to indicate the absolute target power for SRS.

    - Send LS to RAN2 to inform the above.

  - Option 2: Reuse existing p0-r17 = {-16..15} in P0AlphaSet-r17 for SRS, and discuss in RAN1 how to indicate the absolute target power for SRS.

Proposal 2:

- For the issue of value range mismatch on p0 for PUCCH in Rel-17 unified TCI framework, apply the CR proposed in R1-2303692 [3]

References

- TS38.213 V17.5.0, Physical layer procedures for control, TSG RAN, March 2023

- TS38.331 V17.4.0, Radio Resource Control (RRC) protocol specification, TSG RAN, March 2023

- R1-2303692, Draft CR on value range mismatch of p0 for PUCCH in Rel-17 unified TCI framework, NTT DOCOMO, INC., April 2023