3GPP TSG-RAN WG1 Meeting #112-bis	R1-2303692

# e-Meeting,  April 17 - April 26

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XXX | Rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on value range mismatch of p0 for PUCCH in Rel-17 unified TCI framework |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In Rel-17 FeMIMO WI, unified TCI framework is supported. One of the related features is to configure/indicate TPC parameters (i.e., p0, alpha, power control adjustment state) for PUSCH/PUCCH/SRS according to a RRC parameter Uplink-powerControl-r17, where the value range (-16..15) dB is considered for p0 configuration. The value range fits for the configuration of “differential target power”, e.g., PO_UE_PUSCH,b,f,cj for PUSCH, or PO_UE_PUCCH,b,f,cqu for PUCCH. In Clause 7, however, it is now described that for PUCCH, “PO_PUCCH,b,f,cqu is provided by p0AlphaSetforPUCCH (configured in Uplink-powerControl-r17) associated with the indicated TCI-State or TCI-UL-State”. PO_PUCCH,b,f,cqu is nominal (absolute) target power, whose range is (-202..24) in general. In our understanding, the intention of p0 configuration/update based on Rel-17 unified TCI is differential value. Therefore, the value range of p0-r17 in p0AlphaSetforPUCCH specified in 38.331 seems correct, and Clause 7 of 38.213 aims at the configuration/update of a wrong TPC parameter. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change one of the TPC parameters configured/indicated for PUCCH in case Rel-17 unified TCI is configured, from PO_PUCCH,b,f,cqu to PO_UE_PUCCH(qu). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unintended transmit power is configured/indicated for PUCCH in case uTCI is configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of the CR. |  |  |  |  |  |  |  |  |

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by SRS-PosResourceSet as described in clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ+2μ∙kmac where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, μ  is the SCS configuration for the PUCCH or PUSCH, respectively, that is determined in the slot when the MAC CE command is applied and kmac is a number of slots for SCS configuration μ=0 provided by kmac or kmac=0 if kmac is not provided.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

In the remaining of this clause, if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State and for an indicated TCI-State or TCI-UL-State as described in [6, TS 38.214]

-	in clauses 7.1.1, 7.2.1, and 7.3.1, the RS index qd for obtaining the downlink pathloss estimate for PUSCH, PUCCH, and SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the indicated TCI-State or TCI-UL-State except for SRS transmission that is not provided followUnifiedTCI-StateSRS

-	in clause 7.1.1, if p0AlphaSetforPUSCH is provided, the values of PO_UE_PUSCH,b,f,cj, αb,f,cj, and the PUSCH power control adjustment state l are provided by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.2.1, if p0AlphaSetforPUCCH is provided, the values of PO_UE_PUCCH(qu) PO_PUCCH,b,f,cqu and the PUCCH power control adjustment state l are provided by p0AlphaSetforPUCCH associated with the indicated TCI-State or TCI-UL-State

-	in clause 7.3.1, if p0AlphaSetforSRS is provided,

-	if followUnifiedTCI-StateSRS is provided for a SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with the indicated TCI-State or TCI-UL-State

-	else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set and a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set

In the remaining of this clause, if a PDCCH reception by a UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1

-	a PDCCH monitoring occasion is the union of the PDCCH monitoring occasions for the two PDCCH candidates

-	the end of the PDCCH reception is the end of the PDCCH candidate that ends later

The PDCCH reception includes the two PDCCH candidates also when the UE is not required to monitor one of the two PDCCH candidates as described in clauses 10 (except clause 10.4), 11.1, 11.1.1 and 17.2.