3GPP TSG RAN WG1 #112bis-e	R1-2303695

# e-Meeting, April 17th – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | Draft | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to support NACK-only feedback with multiple G-RNTIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Multiple G-RNTIs were introduced e.g., to support G-RNTI per multicast service. Appropriate communicastion parameters are different among G-RNTIs, and multiple G-RNTIs enables to configure parameters per multicast service. Then the case where one UE is involved with multiple G-RNTIs is a typical situation, but the current specification does not support to multiplex HARQ-ACK information for more than one G-RNTIs when NACK-only mode 2 is used. This reduces scheduling flexibility significantly. Using NACK-only mode 1 instead of mode 2 is not appropriate solution for multiple G-RNTIs since NACK-only mode 1 requires much more PUCCH resources than those for mode 2. The case where NACK-only mode 1 is unavailable due to insufficient PUCCH resources is not rare case. This correction allows the currently excluded situation as above and each UE just multiplexes them as in section 9.1.3.1 (type 2 HARQ-ACK CB construction). Issue would be that if different UEs are associated with different G-RNTI combinations, but gNB can provide appropriate PUCCH configuration. For example, UE group-1 monitor G-RNTI-A and UE group-2 monitor G-RNTI-A and G-RNTI-B. In this case, gNB can provide separate PUCCH configurations per UE group. UE group-1 use PUCCH resource set 1 and UE group-2 use set 2. Those resources can be orthogonal and thus no conflict issue between UE group-1 and UE group-2 is not found in this way. At the same time, each UE’s behavior is just to follow section 9.1.3.1. Neither additional UE impact nor higher layer spec impact is assumed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the limitation that moreThanOneNackOnlyMode is indicated for only one G-RNTI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scheduling flexibility for NACK-only mode 2 is significant low for multple G-RNTIs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

*** Unchanged text is omitted ***

A UE that is indicated the second HARQ-ACK reporting mode, and for the case when the UE reports more than one HARQ-ACK information bits, the UE can be indicated to provide the HARQ-ACK information bits in a PUCCH either according to the first HARQ-ACK reporting mode when the UE is not provided moreThanOneNackOnlyMode or, for only one G-RNTI or only one G-CS-RNTI, according to the second HARQ-ACK reporting mode by selecting a PUCCH resource from a set of PUCCH resources for the PUCCH transmission based on the values of the HARQ-ACK information bits as described in Table 18-1 when the UE is provided moreThanOneNackOnlyMode. The UE generates HARQ-ACK information bits for the second HARQ-ACK reporting mode according to a Type-2 HARQ-ACK codebook as described in clause 9.1.3.1. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0.

*** Unchanged text is omitted ***