3GPP TSG RAN WG1 #112bis-e	R1-2303697

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC

Title:	Discussion on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are three objectives for extension of unified TCI framework, as shown below.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. […] - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss unified TCI framework related enhancements.

- Unified TCI framework extension for multi-TRP (M-TRP)

  - Single-DCI based M-TRP

- Remaining issues of default beams for PDSCH

In RAN1#112, the following agreement was made.

Agreement On unified TCI framework extension for S-DCI based MTRP, a 2-bit [TCI selection field] can be configured by RRC to be present in a DCI format 1_1/1_2 that schedules/activates PDSCH reception (including dynamic PDSCH and SPS PDSCH) according to the followings: - If the DCI format 1_1/1_2 indicates codepoint "00" for the [TCI selection field], the UE shall apply the first one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "01" for the [TCI selection field], the UE shall apply the second one of two indicated joint/DL TCI states to all PDSCH DMRS port(s) of corresponding PDSCH transmission occasions(s) scheduled/activated by the DCI format 1_1/1_2 - If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the PDSCH reception scheduled/activated by the DCI format 1_1/1_2 - FFS: Whether and how to use the codepoint "11" of the [TCI selection field] If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2 regardless of threshold, above apply to PDSCH reception(s) scheduled/activated by the DCI format 1_1/1_2. - Note: If the UE supports the capability of two default beams for S-DCI based MTRP in FR2, UE uses both indicated joint/DL TCI states to buffer the received signal before a threshold. If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above apply to the scheduled/activated PDSCH reception when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold - FFS: How to apply the indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is less than a threshold in FR2 FFS: Detail of the capability of two default beams for S-DCI based MTRP FFS: The threshold value

Regarding to the 1st FFS point, if the UE doesn’t support the capability of two default beams for S-DCI based M-TRP in FR2, UE cannot buffer the received signal with two default beams in case the scheduling offset less than threshold. Hence, we need to select one of the indicated joint/DL TCI states for PDSCH scheduled with the scheduling offset less than threshold. The following options can be considered:

- Opt.1: Always 1st (or 2nd) joint/DL TCI state is applied to the PDSCH.

- Opt.2: Higher layer signaling can select 1st or 2nd joint/DL TCI state applied to the PDSCH.

Potential advantage of Opt.2 is that Opt.2 has higher flexibility to indicate one of the two joint/DL TCI states (e.g. 1st or 2nd). However, even in Opt.1, gNB can update order of the two joint/DL TCI states (e.g. {TCI#1, TCI#2} or {TCI#2, TCI#1}) by MAC CE. Hence, we believe Opt.1 is enough. Note that UE behavior of buffering for the scheduling offset less than threshold should be common for all DCI format 1_0/1_1/1_2 and regardless of the configuration of [TCI selection field], because UE cannot distinguish DCI format before finishing the DCI decoding.

Regarding to the 2nd FFS point, it highly depends on whether the threshold is the same as timeDurationForQCL in Rel.15 UE capability signaling or not. If the threshold is the same as timeDurationForQCL, we can reuse the following UE capability to indicate whether UE supports to buffer two default beams for the scheduling offset less than threshold for single DCI-based multi-TRP. If not, we should define another UE capability to report the support of two default beams in R18.

| defaultQCL-TwoTCI-r16 Indicates whether the UE supports default QCL assumption with two TCI states using single-DCI based multi-TRP. The UE can include this field only if simultaneousReceptionDiffTypeD-r16 is present. Otherwise, the UE does not include this field. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

Regarding to the 3rd FFS point, one possible option is to reuse the timeDurationForQCL in Rel.15 as the threshold. However, one issue of reusing the timeDurationForQCL is that the minimum reported value is too large. For example, for SCS 120kHz, the minimum value is 14-symbol. If we reuse timeDurationForQCL, DCI based dynamic switching between single-TRP and multi-TRP is not applied to self-slot scheduling in FR2, and the use-case of the dynamic switching will be limited. Hence, we prefer to introduce new values of the threshold, which have at least one value smaller than timeDurationForQCL.

FeatureSetDownlink ::= SEQUENCE { […] timeDurationForQCL SEQUENCE { scs-60kHz ENUMERATED {s7, s14, s28} OPTIONAL, scs-120kHz ENUMERATED {s14, s28} OPTIONAL } […] }

Proposal 2-1:

  - On unified TCI framework extension for S-DCI based MTRP, regardless of the configuration of [TCI selection field],

    - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, for PDSCH scheduled by DCI format 1_0/1_1/1_2 with the scheduling offset less than threshold,

      - If MAC CE/DCI indicates one indicated joint/DL TCI state, the one joint/DL TCI state is applied to the PDSCH.

      - If MAC CE/DCI indicates two indicated joint/DL TCI states, always 1st joint/DL TCI state is applied to the PDSCH.

    - The threshold is reported by UE capability. Select one from the following options (Opt.1 is preferred).

      - Opt.1: Introduce new values of the threshold, which contain at least one value smaller than the values of timeDurationForQCL.

        - Introduce new UE capability to report whether to support buffering with the two default beams for the scheduling offset < thleshold.

      - Opt.2: Reuse UE capability of timeDurationForQCL as the threshold.

        - Reuse UE capability of defaultQCL-TwoTCI-r16 to report whether to support buffering with the two default beams

- DCI format 1_1/1_2 configured w/o the [TCI selection field]

In RAN1#112, the following agreement was made.

Agreement: On unified TCI framework extension for S-DCI based MTRP, down-select at least one of the followings for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field]: - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt2: The UE shall apply the first one of two indicated joint/DL TCI state(s) to the scheduled/activated PDSCH reception - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception - Alt3A: The UE shall apply the same joint/DL TCI state(s) that is applied to the PDCCH reception with the scheduling/activation DCI to the scheduled/activated PDSCH reception - Alt4: Which indicated joint/DL TCI state(s) is/are applied to the scheduled/activated PDSCH reception is determined according to the existing TCI field of the most recently applied beam indication DCI Above applies at least if the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold (if the threshold is needed)

In Rel.17 SFN-PDCCH/PDSCH, even if PDSCH is scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without TCI state field, one or two TCI state(s) of the scheduling CORESET can be applied to the scheduled SFN-PDSCH in the following case. It means that even if TCI state field does not exist in the scheduling DCI, SFN-PDSCH can be scheduled by SFN-PDCCH. Hence, for SFN-PDSCH in Rel.18, for SFN scenario, one or two TCI states of the scheduling CORESET should be applied to the scheduled SFN-PDSCH.

- 5.1.5 Antenna ports quasi co-location […] When a UE is configured with both sfnSchemePdcch and sfnSchemePdsch scheduled by DCI format 1_0 or by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable: - if the UE supports sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support sfn-SchemeA-DynamicSwitching-r17 or sfn-SchemeB-DynamicSwitching-r17, the UE should be activated with the CORESET with two TCI states. - else if the UE does not support sfn-DefaultDL-BeamSetup-r17 for DCI scheduling without TCI field, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2.

Except for SFN-PDCCH/PDSCH, we believe higher layer signaling can indicate one or both of the indicated joint/DL TCI state to the scheduled PDSCH. In RAN1#112, some companies proposed Alt.3A/Alt.4 which enable dynamic switching between single-TRP and multi-TRP without [TCI selection field]. However, in our view, if gNB wants dynamic switching, gNB can configure [TCI selection field], and there is no use-case to enable dynamic switching without [TCI selection field]. Between Alt.1 and Alt.2, Alt.1 is more flexible that gNB can use RRC to indicate one or two of the indicated joint/DL TCI state(s). Hence, we support Alt.1.

Proposal 2-2:

  - On unified TCI framework extension for S-DCI based MTRP, for PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without [TCI selection field],

    - if both sfnSchemePdcch and sfnSchemePdsch are configured, for PDSCH scheduled with the scheduling offset equal to or larger than threshold, if applicable, support Alt.3.

      - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

    - Else, support Alt.1.

      - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- Mapping of indicated TCI state to PUSCH

For PUSCH, we have the following agreement in RAN1#112 meeting.

Agreement On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG): - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s) - If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator: - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214) - FFS: SDM and SFN based PUSCH Tx schemes FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

In Rel.17 TDM based PUSCH repetition, SRS resource set indicator field in DCI format 0_1/0_2 can indicate the dynamic switching between TDM based PUSCH repetition and single TRP transmission. Codepoint “00” and “01” can indicate single TRP transmission, where codepoint “00” means “1st SRS resource set only” and codepoint “01” means “2nd SRS resource set only”, respectively. Codepoint “10” and “11” can indicate TDM based PUSCH repetition, where both indicate 1st and 2nd SRS resource sets with different mapping order.

For Rel.18 SDM based STxMP PUSCH, similar to Rel.17 TDM based PUSCH repetition, existing SRS resource set indicator field in DCI format 0_1/0_2 is reused for the dynamic switching between SDM based STxMP PUSCH and single TRP PUSCH transmission. For the DCI codepoint of the SRS resource set indicator, codepoint “00” and “01” can indicate “1st SRS resource set only” and “2nd SRS resource set only” (i.e. single TRP transmission). For codepoint “10”, it can indicate SDM based STxMP PUSCH. For codepoint “11”, in our view, there is no benefit to support dynamic indication of TRP switching order for SDM based STxMP PUSCH, because both TCI states are mapped to the same CW of a PUSCH. Hence, we believe codepoint “11” should be reserved.

Proposal 2-3:

  - On unified TCI framework extension for S-DCI based MTRP, if SDM STxMP PUSCH is configured, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

    - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 1st SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the 2nd indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to the 1st PUSCH antenna port(s)/port(s) associated with the 1st SRS resource set for CB/NCB, and the UE shall apply the 2nd indicated joint/UL TCI state to the 2nd PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

For Rel.18 SFN based STxMP PUSCH, similar to Rel.17 TDM based PUSCH repetition, existing SRS resource set indicator field in DCI format 0_1/0_2 is reused to indicate the dynamic switching between SFN based STxMP PUSCH and single TRP PUSCH transmission. For the DCI codepoint of the SRS resource set indicator, codepoint “00” and “01” can indicate “1st SRS resource set only” and “2nd SRS resource set only” (i.e. single TRP transmission). For codepoint “10”, it can indicate SFN based STxMP PUSCH. For codepoint “11”, in our view, there is no benefit to support dynamic indication of TRP switching order for SFN based STxMP PUSCH, because both TCI states are mapped to the same RE of a PUSCH in SFN. Hence, we believe codepoint “11” should be reserved.

Proposal 2-4:

  - On unified TCI framework extension for S-DCI based MTRP, if SFN PUSCH is configured, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

    - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 1st SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the 2nd indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator:, the UE shall apply the 1st indicated joint/UL TCI state to the all PUSCH antenna port(s)/port(s) associated with the 1st SRS resource set for CB/NCB, and the UE shall apply the 2nd indicated joint/UL TCI state to the all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

Regarding to the last FFS (The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission), in out view, it can be handled by gNB implementation. For example, gNB can indicate the 1st indicate joint/UL TCI state to the 1st SRS resource set for CB/NCB and the 2nd indicate joint/UL TCI state to the 2nd SRS resource set for CB/NCB. If gNB believes the same spatial Tx filter(s) should be applied to both PUSCH and the corresponding SRS, gNB can do so by gNB implementation. Hence, there is no need to specify special handling for this case.

Proposal 2-5:

  - On unified TCI framework extension for S-DCI based MTRP, there is no need to specify special handling for the following case.

    - FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

  - Multi-DCI based M-TRP

- Mapping of indicated TCI state to PDCCH/PDSCH/CSI-RS

In RAN1#111, the following agreement was made for PDCCH/PDSCH.

Agreement On unified TCI framework extension for M-DCI based MTRP, the same configuration/rule used in Rel-17 unified TCI framework (for determining whether the UE shall apply the indicated joint/DL TCI state to PDCCH on a CORESET and respective PDSCH) is reused to determine whether the UE shall apply the indicated joint/DL TCI state specific to a coresetPoolIndex value to PDCCH on a CORESET associated with the same coresetPoolIndex value and PDSCH scheduled/activated by the PDCCH.

For A-CSI-RS, considering a DCI may trigger multiple CSI-RS resources and the CSI-RS resources in a triggered CSI-RS resource set may be transmitted from different TRPs, it is beneficial that coresetPoolIndex can be configured per CSI-RS resource for A-CSI-RS to indicate which indicated joint/DL TCI state is applied to the CSI-RS resource. Similarly, for P/SP CSI-RS, coresetPoolIndex can be configured per CSI-RS resource.

Proposal 2-6:

  - For unified TCI extension for multi-DCI based M-TRP, support the following explicit association between coresetPoolIndex value and each CSI-RS resource to determine the applied indicated joint/DL TCI state.

    - For A-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

    - For P/SP-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

- Mapping of indicated TCI state to PUSCH/SRS/PUCCH

In RAN1#112, the following agreement was made for PUCCH.

Agreement On unified TCI framework extension for M-DCI based MTRP, down-select from the following options for PUCCH transmission: - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission - Opt2: An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively. - Opt3: For a PUCCH transmission triggered by PDCCH on a CORESET when the UCI in the PUCCH transmission carries HARQ-ACK information only, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is determined from the one associated with the CORESET. Otherwise, either Opt1 or Opt2 is adopted. - FFS: Whether Opt3 applies only when the UE is not provided with ackNackFeedbackMode = joint - Opt4: For a PUCCH transmission with an LRR trigged for either the first BFD-RS set (q0,0) or the second BFD-RS set (q0,1) when the UE is provided only one or two schedulingRequestID-BFR configuration, the UE shall apply the indicated joint/UL TCI state specific to a coresetPoolIndex value to the PUCCH transmission, where the coresetPoolIndex value is 1 when the LRR is trigged for the first BFD-RS set (q0,0) and the coresetPoolIndex value is 0 when the LRR is trigged for the second BFD-RS set (q0,1). Otherwise, either Opt1 or Opt2 is adopted. Note: Either Opt1 or Opt2 must be supported

In the above agreement, Opt.1-3 discuss the mapping of indicated joint/UL TCI states to PUCCH resources for general cases, and Opt.4 discuss the mapping for BFR. We believe enhancement of BFR is necessary but can be discussed later.

In Opt.1-3, the difference between Opt.1/2 and Opt.3 is whether a PUCCH with HARQ should be transmitted to the TRP which transmits the scheduling DCI. Since the use-case of M-DCI based M-TRP is for non-ideal backhaul, it is preferable to enable a TRP transmits the scheduling DCI also receives a PUCCH with the corresponding HARQ. Hence, in Rel.16, gNB can indicate different spatial relation to different PUCCH resource, so that the PRI field in the scheduling DCI can select spatial relation along with the PUCCH resource. Any of Opt.1-3 allows this operation. For example, in Opt.1/2, gNB can configure coresetPoolIndex or TRP ID per a PUCCH resource, and the 1st and 2nd indicated joint/UL TCI states can be applied to different PUCCH resource. Then, the PRI field in the scheduling DCI can select 1st or 2nd indicated joint/UL TCI states along with a PUCCH resource. For Opt.3, 1st or 2nd indicated joint/UL TCI states of a PUCCH with HARQ is selected by coresetPoolIndex of the scheduling CORESET. In our view, there is no need to introduce restriction of Opt.3, and Opt.1/2 is enough.

For Opt.1/2, we believe the difference between Opt.1 and Opt.2 is small, both options configure RRC parameter, per a PUCCH resource/resource group, to indicate 1st indicated joint/UL TCI state associated with coresetPoolIndex = 0 or 2nd indicated joint/UL TCI state associated with coresetPoolIndex = 1. Since coresetPoolIndex is always used for M-DCI based M-TRP, we can directly use coresetPoolIndex for the indication, and we support Opt.1.

Proposal 2-7:

  - For unified TCI extension for multi-DCI based M-TRP, support Opt.1 for PUCCH transmission:

    - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

For A-SRS, considering a DCI may trigger multiple SRS resource sets and the SRS resources in the multiple SRS resource sets may be transmitted to different TRPs, it is beneficial that CORESETPoolIndex can be configured per SRS resource for AP SRS.

Proposal 2-8:

  - For unified TCI extension for multi-DCI based M-TRP, support the following implicit/explicit association between coresetPoolIndex value and each applied channel/RS to determine the applied indicated joint/UL TCI state.

    - For dynamically scheduled/activated PUSCH, CORESETPoolIndex associated with the scheduling/activating PDCCH is utilized.

    - For Type 1 CG, P/SP-SRS, CORESETPoolIndex is RRC-configured.

    - For AP-SRS, CORESETPoolIndex is RRC-configured per SRS resource.

  - TRP specific BFR for unified TCI

In RAN1#110bis-e meeting, the following agreement was made.

Agreement On unified TCI framework extension, study the following enhancements for TRP-specific BFR: - Implicit BFD-RS determination based on the indicated joint/DL TCI states for S-DCI based MTRP - Enhancement to beam update after NW response to TRP-specific BFR request

We believe it is beneficial to support TRP specific BFR for unified TCI state. For example, implicit BFD-RS determination rule should be updated so that the two indicated joint/DL TCI states for Single-DCI based M-TRP can be used for BFD-RS for the two TRPs. For implicit BFD-RS for multi-DCI based M-TRP, no spec. enhancement would be needed to determine two indicated joint/DL TCI states for two TRPs, while we are open to discuss if any spec. enhancement is needed.

After the completion of the BFR for both Single-DCI based and Multi-DCI based M-TRP, indicated joint/DL/UL TCI state(s) and TPC assumptions should be updated.

Proposal 2-9:

  - For unified TCI extension for single/multi-DCI based M-TRP, support the following enhancement for TRP specific BFR.

    - For implicit BFD RS determination of single-DCI based M-TRP, use two joint/DL TCI states for BFD RS for two TRPs.

      - Note: For implicit BFD RS determination of multi-DCI based M-TRP, use each joint/DL TCI state of each CoresetPoolIndex for each BFD RS set.

    - After NW response of TRP-specific BFR request for both Single-DCI based and Multi-DCI based M-TRP, joint/DL/UL TCI state(s) and TPC assumptions should be updated.

- Power control for M-TRP operation

Agreement On unified TCI framework extension, if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall determine UL Tx power for the PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) based on the UL PC parameter setting for PUSCH/PUCCH/SRS, if any, and the PL-RS included in the indicated joint/UL TCI state - FFS: For STxMP, the maximum Tx power when the UE determines UL Tx power for the PUSCH/PUCCH transmission occasion(s) or antenna port(s) (discussed after receiving RAN4 reply on UE power limitation for STxMP in FR2) - FFS: Default UL PC parameter setting(s) if one or both of indicated joint/UL TCI states applied to PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s) does/do not include the UL PC parameter setting(s) for PUCCH/PUSCH/SRS Agreement On UE power limitation for STxMP for FR2, send LS to RAN4 to check the followings: - Whether it is feasible to assume power limitation per panel for STxMP (Assumption 1) - Whether it is feasible to assume a total power limitation per UE over all UE panels used for STxMP (Assumption 2) - In either of Assumption1 or Assumption 2, whether the total power limitation per UE over all UE panels used for STxMP or the sum of per-panel power limitation for STxMP can be different from (greater than) the existing power limitation for a given power class? - If both Assumption 1 and Assumption 2 are feasible, whether both assumptions can be applied to a same UE, and what is the relationship between the per-panel power limitation and total power limitation if both are applied (e.g., the sum of per-panel power limitation can be larger than the total power limitation per UE, or should be always the same)? FFS: Detail of exact LS if agreed Note: Scenarios of above include at least single carrier scenario for FR2 Note: Above power limitation includes both total radiated power and EIRP LS to RAN4 (approved on May 24th, see below). R1-2205639 LS on UE power limitation for STxMP in FR2 RAN1, MediaTek Decision: As per email decision posted on May 24th, the LS is approved.

It was agreed that if an indicated joint/UL TCI state(s) applies to a PUSCH/PUCCH/SRS transmission occasion(s) or antenna port(s), the UE shall apply the UL PC parameter setting included in the indicated joint/UL TCI state. Besides PC parameter setting, PCMAX in STxMP needs to be studied. UE configured maximum output power PCMAX is defined that UE UL Tx power at a transmission occasion cannot exceed PCMAX. For example, PUSCH transmission power is determined in the following equation.

[dBm] (1)

In legacy spec. the configured UE maximum output power PCMAX shall be set such that the EIRP and TRP are within power limitation bounds.

[38.101-2-h50] The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax

For power limitation in STxMP, following two assumptions were discussed in the previous meeting, and an LS was sent to RAN4 regarding the feasibility of the two assumptions.

- Assumption1: power limitation per panel is assumed. UE UL Tx power of each panel cannot exceed the per panel limitation.

- Assumption2: power limitation per UE is assumed. Sum of UE UL Tx power of two panels cannot exceed the per UE limitation.

According to RAN4 Reply LS [2], both assumptions are feasible and both assumptions shall be applied to a same UE.

For STxMP, PC parameter setting is configured per panel. PCMAX shall also be set per panel, so that similar as the above equation (1) of UL Tx power control in single panel Tx, the UL Tx power is determined as the minimum of the Tx power calculated based on per panel PC parameter setting and the per panel PCMAX. With both assumptions applied to a UE, UE shall set PCMAX for each panel so that both power limitation per panel and power limitation per UE are met.

Furthermore, with per panel PCMAX and per panel PC parameter setting, per panel PHR shall be reported.

Proposal 3-1:

  - For STxMP, PCMAX is set per panel. UL Tx power is determined as minimum of Tx power calculated based on per panel PC parameter setting and per panel PCMAX.

Proposal 3-2:

  - For STxMP, support per panel PCMAX and per panel PHR report.

For M-TRP PUSCH/PUCCH, the applied UL PC parameter setting if one or both indicated joint or UL TCI state(s) is not associated with an UL PC parameter setting (including P0, alpha for PUSCH, and closed loop index) for PUCCH/PUSCH was discussed. Default power control parameter setting was supported in Rel-17 unified TCI. In Rel-17, a default power control parameter setting can be configured in BWP-UplinkDedicated and the configured default power control parameter setting can be used when joint or UL TCI is not associated with an UL PC parameter setting. Furthermore, per TRP default power control parameter was supported in Rel-17 S-DCI M-TRP PUSCH repetition. In Rel-17 M-TRP PUSCH, per TRP default power control parameter can be determined and used when SRI field is not included in DCI or association between PC parameter and SRI is not provided. Thus, from our perspective, per TRP default power control parameter setting should also be supported with Rel-18 unified TCI framework.

A straightforward enhancement is to configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting. For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control parameter can be applied to the PUSCH/PUCCH transmission.

Proposal 3-3:

  - Configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting.

  - For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control can be applied to the PUSCH/PUCCH transmission.

Beam reporting for STxMP

To support STxMP, gNB needs to be aware of whether two beams can be transmitted simultaneously by UE. One option is to report correspondence between beams and UE panels. The beams corresponding to different UE panels can be transmitted simultaneously by UE. In Rel-17, UE capability value index can be reported for each beam in beam reporting. However, in Rel-17, each UE capability value refers to unique UE capability. For STxMP, to report correspondence between beams and panels and considering two panels may have same capability, enhancement based on Rel-17 UE capability value reporting framework can be considered. Based on Rel-17 UE capability value reporting framework, UE capability values with same capability can be supported.

Another option is to directly report pair(s) of beams that can be transmitted simultaneously by UE. In Rel-15/17, group-based beam reporting is supported for DL Rx, i.e., UE can report pair(s) of beams that can be received simultaneously by UE. For STxMP, to support UE reporting pair(s) of beams that can be transmitted simultaneously by UE, enhancement based on Rel-15/17 group-based beam reporting framework can be considered. Based on Rel-15/17 group-based beam reporting framework, if STxMP is supported, UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE. Alternatively, new UL group-based beam reporting which is independent from Rel-15/17 group-based beam reporting can be considered. In the new UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

Proposal 4-1:

  - To support STxMP, following options of enhancement on beam reporting can be considered.

    - Option1: reuse Rel-17 UE capability value reporting framework with enhancement that when STxMP is supported UE can report UE capability values with same capability. Two beams with different UE capability value set index correspond to different panel and can be transmitted simultaneously by UE.

    - Option2: reuse Rel-15/17 group-based beam reporting framework with enhancement that when STxMP is supported UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE.

    - Option3: introduce new UL group-based beam reporting. In UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

- Conclusion

In this contribution, we discussed unified TCI framework related enhancements. Based on the discussion, we made the following proposals.

Unified TCI framework extension for M-TRP

Proposal 2-1:

  - On unified TCI framework extension for S-DCI based MTRP, regardless of the configuration of [TCI selection field],

    - If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, for PDSCH scheduled by DCI format 1_0/1_1/1_2 with the scheduling offset less than threshold,

      - If MAC CE/DCI indicates one indicated joint/DL TCI state, the one joint/DL TCI state is applied to the PDSCH.

      - If MAC CE/DCI indicates two indicated joint/DL TCI states, always 1st joint/DL TCI state is applied to the PDSCH.

    - The threshold is reported by UE capability. Select one from the following options (Opt.1 is preferred).

      - Opt.1: Introduce new values of the threshold, which contain at least one value smaller than the values of timeDurationForQCL.

        - Introduce new UE capability to report whether to support buffering with the two default beams for the scheduling offset < thleshold.

      - Opt.2: Reuse UE capability of timeDurationForQCL as the threshold.

        - Reuse UE capability of defaultQCL-TwoTCI-r16 to report whether to support buffering with the two default beams

Proposal 2-2:

  - On unified TCI framework extension for S-DCI based MTRP, for PDSCH scheduled by DCI format 1_0 or by DCI format 1_1/1_2 without [TCI selection field],

    - if both sfnSchemePdcch and sfnSchemePdsch are configured, for PDSCH scheduled with the scheduling offset equal to or larger than threshold, if applicable, support Alt.3.

      - Alt3: The UE shall apply both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

    - Else, support Alt.1.

      - Alt1: Using RRC configuration to inform that the UE shall apply the first one, the second one, or both of two indicated joint/DL TCI states to the scheduled/activated PDSCH reception

Proposal 2-3:

  - On unified TCI framework extension for S-DCI based MTRP, if SDM STxMP PUSCH is configured, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

    - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 1st SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the 2nd indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to the 1st PUSCH antenna port(s)/port(s) associated with the 1st SRS resource set for CB/NCB, and the UE shall apply the 2nd indicated joint/UL TCI state to the 2nd PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

Proposal 2-4:

  - On unified TCI framework extension for S-DCI based MTRP, if SFN PUSCH is configured, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

    - If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the 1st indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 1st SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the 2nd indicated joint/UL TCI state to all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

    - If the DCI format 0_1/0_2 indicates codepoint "10" for the existing SRS resource set indicator:, the UE shall apply the 1st indicated joint/UL TCI state to the all PUSCH antenna port(s)/port(s) associated with the 1st SRS resource set for CB/NCB, and the UE shall apply the 2nd indicated joint/UL TCI state to the all PUSCH antenna port(s)/layer(s) associated with the 2nd SRS resource set for CB/NCB.

Proposal 2-5:

  - On unified TCI framework extension for S-DCI based MTRP, there is no need to specify special handling for the following case.

    - FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

Proposal 2-6:

  - For unified TCI extension for multi-DCI based M-TRP, support the following explicit association between coresetPoolIndex value and each CSI-RS resource to determine the applied indicated joint/DL TCI state.

    - For A-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

    - For P/SP-CSI-RS, coresetPoolIndex is RRC-configured per CSI-RS resource.

Proposal 2-7:

  - For unified TCI extension for multi-DCI based M-TRP, support Opt.1 for PUCCH transmission:

    - Opt1: A coresetPoolIndex value can be provided per PUCCH resource/resource group, and the UE shall apply the indicated joint/UL TCI state specific to the coresetPoolIndex value to the corresponding PUCCH transmission

Proposal 2-8:

  - For unified TCI extension for multi-DCI based M-TRP, support the following implicit/explicit association between coresetPoolIndex value and each applied channel/RS to determine the applied indicated joint/UL TCI state.

    - For dynamically scheduled/activated PUSCH, CORESETPoolIndex associated with the scheduling/activating PDCCH is utilized.

    - For Type 1 CG, P/SP-SRS, CORESETPoolIndex is RRC-configured.

    - For AP-SRS, CORESETPoolIndex is RRC-configured per SRS resource.

Proposal 2-9:

  - For unified TCI extension for single/multi-DCI based M-TRP, support the following enhancement for TRP specific BFR.

    - For implicit BFD RS determination of single-DCI based M-TRP, use two joint/DL TCI states for BFD RS for two TRPs.

      - Note: For implicit BFD RS determination of multi-DCI based M-TRP, use each joint/DL TCI state of each CoresetPoolIndex for each BFD RS set.

    - After NW response of TRP-specific BFR request for both Single-DCI based and Multi-DCI based M-TRP, joint/DL/UL TCI state(s) and TPC assumptions should be updated.

Power control for M-TRP operation

Proposal 3-1:

  - For STxMP, PCMAX is set per panel. UL Tx power is determined as minimum of Tx power calculated based on per panel PC parameter setting and per panel PCMAX.

Proposal 3-2:

  - For STxMP, support per panel PCMAX and per panel PHR report.

Proposal 3-3:

  - Configure two default power control parameter settings in BWP-UplinkDedicated for two TRPs. The default power control parameter settings can be used when joint or UL TCI is not associated with an UL PC parameter setting.

  - For M-TRP PUSCH/PUCCH, the first/second default power control parameter can correspond to first/second “indicated TCI” respectively. If the first/second “indicated TCI” applies to a PUSCH/PUCCH transmission and is not associated with a PC parameter setting, the corresponding default power control can be applied to the PUSCH/PUCCH transmission.

Beam reporting for STxMP

Proposal 4-1:

  - To support STxMP, following options of enhancement on beam reporting can be considered.

    - Option1: reuse Rel-17 UE capability value reporting framework with enhancement that when STxMP is supported UE can report UE capability values with same capability. Two beams with different UE capability value set index correspond to different panel and can be transmitted simultaneously by UE.

    - Option2: reuse Rel-15/17 group-based beam reporting framework with enhancement that when STxMP is supported UE shall report pair(s) of beams that can be received simultaneously by UE and can also be transmitted simultaneously by UE.

    - Option3: introduce new UL group-based beam reporting. In UL group-based beam reporting, UE shall report pair(s) of beams that can be transmitted simultaneously by UE.

- Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

[2] 3GPP R4-2303494, “Reply LS on UE power limitation for STxMP in FR2, RAN4, Mar. 2023