3GPP TSG RAN WG1 #112bis-e			R1-2303699

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on CSI enhancement

Agenda Item:	9.1.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for CSI enhancements. In this contribution, we discuss CSI enhancement for high/medium UE velocities and CSI enhancement for M-TRP CJT for Rel-18 MIMO.

- CSI enhancement for coherent JT (CJT)

2.1 Codebook mode and parameters

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding the codebook parameter R, the supported value(s) from the legacy specification are reused.

- FFS: whether additional value 4 can also be added

In last RAN1 meeting, several agreements were made regarding codebook parameters. And there are still FFS for some parameters, as shown below.

For parameter R, legacy values of 1 and 2 have been supported. If R=4 is supported, the CSI feedback overhead and UE complexity will be further increased, which is not preferred.

Proposal 2-1

- For codebook parameters, not support R = 4.

Agreement

FFS: Whether the total number of Ln is a UE capability

Agreement

FFS: For NTRP>1, in addition to the supported combinations/permutations, whether to support at least one additional combination where at least one of the Ln values (n=1, …, NTRP) is 6

Agreement

On the Parameter Combination of Type-II codebook refinement for CJT mTRP, support linkage between the list of supported {Ln} combinations and list of supported {pv,} combinations via pairing each combination for {pv,} with at least one combination for {Ln}, for each NTRP value.

- FFS (by RAN1#112bis-e): The exact list of supported pairs/linkage, or restriction of {Ln} when paired to each of {pv,}

- FFS (by RAN1#112bis-e): Whether/How to support configuration signalling for indicating the linkage

- Note: While no additional codebook parameter will be introduced, the total number of SD basis vectors across CSI-RS resources can still be used as a criterion for choosing the supported pairs/linkage

On the parameter combination configuration, for Rel-16-based refinement, the supported combinations of {Ln} and supported combinations of {pv,} have been defined in last RAN1 meeting. And there are following FFS points for further discussion.

For NTRP=1, it is reasonable to support no more than 8 linkage pairs, like legacy. And the parameter combinations supported in legacy should be considered, as shown in Table 2-1. We’re open to consider one additional linkage for NTRP=1. For NTRP>1, we understand the performance and reporting overhead are the most important metrics to determine the supported pairs/linkages. There was an email discussion before this meeting, and some companies provided simulation results for different linkages. Based on the simulations, FL had a following proposal. After checking the simulation results from companies, we could support FL proposal 1.C.1.

Proposal 1.C.1: On the Parameter Combination of Type-II codebook refinement for CJT mTRP, only the following linkages are supported (marked ‘x’)

- For NTRP =1,

  - For Rel-16 eType-II based: fully reuse seven out of the eight Parameter Combinations from Rel-16 eType-II as indicated in the table below

    - FFS (by RAN1#112bis-e): whether to add one more Parameter Combination for L=4 based on the legacy Rel-16 eType-II FD combo {½, ½, ¼, ¼; ½} or the agreed FD combo {½, ½, ½, ½; ½}, or not to add from the indicated seven below

  - For Rel-17 FeType-II based, fully reuse the eight Parameter Combinations from Rel-16 eType-II

- For NTRP >1, only the following linkages are supported (marked ‘x’)

| NTRP | SD combo | FD combo {pv}, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | {1/8, 1/8, 1/16, 1/16}, ¼ | {1/8, 1/8, 1/16, 1/16}, ½ | {1/4, 1/4, 1/8, 1/8}, ¼ | {1/4, 1/4, 1/8, 1/8}, ½ | {1/4, 1/4, 1/4, 1/4}, ¾ | {1/2, 1/2, 1/2, 1/2}, ½ |
| 1 | 2 |  |  | X | X |  |  |
|  | 4 |  |  | X | X | X |  |
|  | 6 w restriction |  |  |  | X | X |  |
| 2 | {2,2} | x |  |  |  |  |  |
|  | {2,4} {4,2} | x |  |  |  |  |  |
|  | {4,4} |  | x |  | x |  | x |
| 3 | {2,2,2} | x | x |  |  |  |  |
|  | {2,2,4} {2,4,2} {4,2,2} | x | x |  | x |  |  |
|  | {4,4,4} | x | x | x | x | x | x |
| 4 | {2,2,2,2} | x |  |  |  |  | N/A |
|  | {2,2,2,4} | x |  |  |  |  | N/A |
|  | {2,2,4,4} |  |  |  | x | x | N/A |
|  | {4,4,4,4} |  | x |  | x | x | N/A |

Table 2-1: The supported linkages for NTRP=1

| NTRP | SD combo | FD combo {pv},  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | {1/8, 1/8, 1/16, 1/16}, ¼ | {1/8, 1/8, 1/16, 1/16}, ½ | {1/4, 1/4, 1/8, 1/8}, ¼ | {1/4, 1/4, 1/8, 1/8}, ½ | {1/4, 1/4, 1/4, 1/4}, ¾ | {1/2, 1/2, 1/2, 1/2}, ½ |
| 1 | 2 |  |  | Y | Y |  |  |
|  | 4 |  |  | Y | Y | Y |  |
|  | 6 w/ restriction |  |  |  | Y | Y |  |

Proposal 2-2

- On the Parameter Combination, for NTRP=1, at least support the legacy parameter combinations.

- Support FL proposal 1.C.1.

For supported combinations of {Ln}, when NTRP > 1, currently only Ln=2, 4 are agreed. But for NTRP =1, currently Ln=2, 4, 6 are agreed. The configuration seems not aligned for single-TRP case. Even when NTRP > 1, since UE can perform and report TRP selection, it is possible that UE reports single-TRP CSI to NW. But based on current supported {Ln} combinations, Ln=6 is not supported when single-TRP is selected/reported by UE while Ln=6 is supported when single-TRP is configured by NW. We think this mismatch per configuration does not make sense, and enhancement is needed to make consistent support of Ln=6 for single-TRP. Thus, we support additional {Ln} combination with Ln=6 for each Ln value, but this {Ln} combination can be selected by UE only when UE reports TRP selection as single-TRP.

Proposal 2-3

- Support additional {Ln} combination with Ln=6 for each Ln value, but this {Ln} combination can be selected/reported by UE only when UE reports TRP selection indication as single-TRP.

Agreement

FFS (by RAN1#112bis-e): Whether/how the supported combinations of {n} for Rel-17-based refinement are derived from the supported combinations of {Ln} for Rel-16-based refinement

Agreement

FFS (by RAN1#112bis-e): Whether/how the supported combinations of {M} for Rel-17-based refinement are derived from the supported combinations of {pv ,} for Rel-16-based refinement

In last RAN1 meeting, the parameter combination configurations were mainly discussed for Rel-16 based refinement. For Rel-17 based refinement, it is not discussed yet and we have following FFS.

For supported combinations of {n}, it could have 3 candidate values of 1/2, 3/4, and 1. For n =1, all the CSI-RS ports are selected and reported. We think the meaning of n =1 is quite different from Ln=6 in Rel-16 based refinement. Thus, the supported combinations of {n} for Rel-17-based refinement should not be derived from the supported combinations of {Ln} directly. And n =1 should be considered for combinations of {n} when NTRP > 1. Similarly, the meaning of {M} for Rel-17-based refinement is also different from {pv ,} in Rel-16 based refinement and separate discussion is needed. For M, layer-common configuration is sufficient.

Proposal 2-4

- The supported combinations of {n} for Rel-17-based refinement is not directly derived from the supported combinations of {Ln} for Rel-16-based refinement.

  - Separate discussion is needed. n =1 is supported for combinations of {n} when NTRP > 1.

- The supported combinations of {M} for Rel-17-based refinement is not derived from the supported combinations of {pv ,}.

  - Separate discussion is needed. {M} configuration is layer-common.

In last RAN1 meeting, candidate values for NL were agreed, but there is an FFS on restriction.

Agreement

On the Type-II codebook refinement for CJT mTRP, only support NL ={2,4} as additional candidate values to NL=1.

- FFS: Additional restriction(s) depending on the configured value for NTRP

Considering the complexity increases with larger TRP number, it seems reasonable to limit the supported NL value for larger TRP number case. However, TRP selection can be performed by UE, so the complexity may be decreased. In addition, more {Ln} combinations are beneficial for performance, especially when UE reports smaller TRP number for CJT. Thus, we prefer not to add additional restrictions depending on the configured value for NTRP for supported candidate values for NL.

Proposal 2-5

- Prefer not to add additional restrictions depending on the configured value for NTRP for supported candidate values for NL.

2.2 Codebook design details

FD basis

In legacy Rel-16 eType II CSI, different FD basis selection methods are used for the cases of N3>19  and N3≤19.  For N3≤19, UE determines the FD index 0 and reports the M-1 FD basis. For N3>19, UE reports Minit and M-1 FD basis.  For mode 2 in Rel-18 CJT CSI, FD basis selection is common across TRPs, thus, legacy mechanism can be reused. While for mode 1, since FD basis selection is per-TRP, how to report FD basis for multiple TRPs needs to be clarified. The latest agreement on this issue is as follows.

Agreement

On the Type-II codebook refinement for CJT mTRP, for mode-1, down select (in RAN1#112) only one from the following schemes

- Alt1. The use of per-CSI-RS-resource FD basis selection offset (relative to a reference CSI-RS resource) for independent FD basis selection across N CSI-RS resources.

  - Example formulation: Wf,n=diag(1 ej2πN3φn…. ej2πN3(N3-1)φn)Wf where φn is the FD basis selection offset for CSI-RS resource n relative to a reference CSI-RS resource n with φn=0, and Wf is commonly selected across N CSI-RS resources

- Alt2. Wf,n independently selected across N CSI-RS resources (without any per-CSI-RS-resource FD basis selection offset)

For all the above alternatives, the legacy FD basis selection indication scheme is applied on each selected FD basis.

Note: Per previous agreements, the number of selected FD basis vectors (Mv/pv or M) is gNB-configured via higher-layer signaling and common across the N CSI-RS resources

The most flexible method is to report independent FD basis per TRP as shown in Alt2, but the reporting overhead is large. We also have doubt about whether such flexibility is really needed or not. In Alt1, it is proposed to report per-TRP FD basis offset relative to a reference TRP to implement independent FD basis selection, to unify the design for mode 1 and mode 2 and to reduce FD basis reporting overhead. Such a method is acceptable to us as long as no obvious performance loss is observed.

Proposal 2-6

- On FD basis selection/reporting for mode 1, we’re open to support Alt1 or Alt2.

  - Alt1 is preferred if no obvious performance loss is observed compared with Alt2.

W2 design

In RAN1#110b-e meeting, following was agreed regarding W2 quantization and SCI.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding W2 quantization group, for each layer:

- Support the following: (Alt1) One group comprises one polarization across all N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2)

  - FFS: Amplitude quantization table enhancement

  - For the amplitude group other than the group associated with the SCI, the reference amplitude is reported

- Working assumption: Alt3 is supported in addition to Alt1 (to be confirmed in RAN1#111)

  - (Alt3). One group comprises one polarization for one CSI-RS resource with a common phase reference across N CSI-RS resources (Cgroup,phase=1, Cgroup,amp=2N)

    - For each of the (2N–1) amplitude groups (other than the group associated with the SCI), the reference amplitude is reported

- If the support Alt3 in addition to Alt1 is confirmed, only one of the two schemes will be a basic feature for UEs supporting Rel-18 Type-II CJT codebook

Alt1 has been supported and can be applied to both codebook mode 1 and codebook mode 2. Alt3 is mainly for codebook mode 1 in our understanding, where multiple TRPs can be deployed non-co-located and the power/pathloss difference among multiple TRPs could be large. If Alt3 is supported in addition to Alt1, Alt3 can be applied to codebook mode 1 only, and Alt1 can be basic feature for Rel-18 CJT codebook.

Proposal 2-7

- Regarding W2 quantization group, for each layer, if Alt3 is supported in addition to Alt1, Alt3 is applied to codebook mode 1 only, and Alt1 is basic feature.

2.3 CMR configuration

Agreement

For the Rel-18 Type-II codebook refinement for CJT mTRP with NTRP>1 TRP/TRP-groups, the following is supported:

- The CMR comprises K>1 NZP CSI-RS resources, where one resource corresponds to one TRP/TRP-group (i.e. K=NTRP)

  - Each of the CSI-RS resources has a same number of CSI-RS ports

- Note: The terms TRP and TRP-group are used for discussion purposes only (no spec impact is implied).

In RAN1#110 meeting, following agreement was made for CMR configuration. Some further restrictions on the K CMRs should be discussed.

For example, the max number of ports per CMR resource, and max number of total ports should be discussed. In addition, to ensure the measurement accuracy, similar as NCJT CSI, the configured K NZP CSI-RS resources associated with a CSI-ReportingConfig for CJT should be restricted within the same DRX Active Time, and within X continuous slot(s) without DL/UL switch. At least X=1 should be supported.

Proposal 2-8

- The configured K NZP CSI-RS resources associated with a CSI-ReportingConfig for CJT should be restricted within the same DRX Active Time, and within X continuous slot(s) without DL/UL switch. At least X=1 should be supported.

2.4 CBSR configuration

In last RAN1 meeting, the CBSR configuration on SD basis restriction is agreed to be per-TRP. But the CBSR configuration on amplitude restriction is still FFS. In legacy, signalling for amplitude restriction for 4 indicated vector groups can be configured and 2N1N2 bits are needed for a vector group. The signaling overhead on amplitude restriction is much larger than the signalling overhead on SD basis restriction. If this configuration signaling is extended to per-TRP directly, the signalling overhead becomes large, thus, it is not preferred. On the other hand, since the CBSR configuration on amplitude restriction has been supported for Rel-16 eType II CSI, it is straightforward to support it at least when NTRP = 1 CSI-RS resource is configured. When NTRP  > 1, the signalling overhead is acceptable if TRP-common signalling is configured on amplitude restriction.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding CBSR, at least for restricting SD basis selection, the legacy CBSR scheme is fully reused for each of the RRC-configured NTRP CSI-RS resources (resulting in CSI-RS-resource-specific SD beam group restriction)

- FFS: Whether amplitude restriction is CSI-RS-resource-common or specific, and soft vs hard restriction

- FFS: Whether CBSR can be configured to be off for a CSI-RS resource

The same rank restriction is applied across NTRP CSI-RS resources

Proposal 2-9

- Support CBSR configuration on amplitude restriction when NTRP = 1 CSI-RS resource is configured.

- Support TRP-common CBSR configuration on  amplitude restriction when NTRP > 1 CSI-RS resources are configured.

2.5 CSI mapping order and CSI omission

In last RAN1 meeting, following agreement regarding UCI omission was agreed.

Agreement

On the Type-II codebook refinement for CJT mTRP, regarding UCI omission, down-select between the following three alternatives (by RAN1#112-bis where n denotes the n-th CSI-RS resource):

- Alt1. Prio(,l,m,n)=( k=0n-12Lk) .N.RI.P(m)+N.RI.l(n)+N.n

  - Note: This implies that CSI-RS resource is designated the highest priority

- Alt2. Prio(,l,m,n)=2L’.Qn).RI.N3+2L’.RI. P(m)+RI.l(n)+

  - Note: This implies that CSI-RS resource is designated the lowest priority (after FD basis)

  - Note: L’ denotes the max value of Ln from all selected N CSI-RS resources

  - FFS: Q(n) maps the index n according to a rule, e.g., Q(n)=n, or Q(n)=0 if n corresponds to strongest TRP/SCI.

- Alt3. Replace SD basis index l in legacy Prio calculation with k=0n-12Lk+ln, i.e., SD basis index over all resources: Prio(,l,m,n) = 2Ltot.RI.P(m)+ RI.k=0n-12Lk+RI.l(n)+

FFS: FD permutation P(.) as Rel-16-analogous, or no permutation i.e. P(m)=m

In our understanding, CJT is beneficial in joint operation of mTRPs and UE measures the composite channel of MTRP. Thus, when CSI omission happens, we do not prefer to drop all the information for a specific TRP. Instead, we’d like to keep the channel information of multiple TRPs for joint operation. Thus, Alt2 is not preferred. In addition, we realize that the difference among different TRPs/CSI-RS resources is mainly SD basis selection. Therefore, it is good to handle CSI-RS resources in SD basis. Thus, Alt3 is preferred.

Proposal 2-10

- For UCI omission, Alt3 is preferred.

For CJT CSI, semi-persistent and aperiodic CSI reporting on PUSCH have been supported. For CSI part 1, new CSI reporting contents of indication for TRP selection and indication for L parameter combination have been agreed. The mapping order of those new CSI fields should be discussed. In addition, for the bitmap to indicate non-zero coefficients, we can also discuss whether to support bitmap omission if the reported number of non-zero coefficients is equal to the max number, like bitmap omission in Rel-17 PS CSI.

Proposal 2-11

- For CSI part 1, define mapping order of indication for TRP selection and indication for L parameter combination.

  - Study whether to support bitmap omission for indicator of non-zero coefficients.

For CSI part 2, PMI information are put into three groups for potential CSI omission. For reporting contents in each group, if a reporting parameter is per-TRP basis, the mapping order of CSI fields for each group should be also discussed. In this case, the CSI omission rule based on three groups can be kept.

Proposal 2-12

- For CSI part 2, discuss mapping order of per-TRP parameters for each group.

- Type-II CSI enhancement in doppler domain

3.1 CQI format and location for the 2nd CQI

The following Offline proposal was proposed by Moderator during pre-tdoc offline discussion:

Proposal 2.F.1: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI is located in UCI part 2 Proposal 2.F.2: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting, the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQIs calculated independently from the 1st CQI

We support both Proposal 2.F.1 and Proposal 2.F.2. Regarding the 2nd CQI location, we understand the Proposal 2.F.1 is trying to follow the legacy rule for Type-I CSI, which seems reasonable. Regarding the 2nd CQI format, the offline discussion focused on the choice between independent CQI or differential CQI. We think independent CQI for each of the 1st and 2nd CQI would be sufficient, given no large benefit is obtained by supporting differential CQI in time domain.

Proposal 3-1: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting,

- Support the 2nd CQI is located in UCI part 2

- Support the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQI calculated independently from the 1st CQI

3.2 CSI reporting window

There was also a good offline discussion on NNZC bitmap design. Moderator kindly drew the following Proposal 2.G.1

Proposal 2.G.1: For the Type-II codebook refinement for high/medium velocities, regarding the bitmap(s) for indicating the locations of the NZCs, - When the UE is configured with Q=1: for each layer, one 2-dimensional bitmap of size-2LM reusing the legacy design is used - When the UE is configured with Q=2: for each layer, - Basic feature: two 2-dimensional bitmaps, each of size-2LM reusing the legacy design for each of the two selected DD basis vectors, are used - Optional feature (for higher CSI overhead, FFS: definition), if the following down-selection succeeds: down-select from the following two alternatives in RAN#112bis-e: - Alt3A: A single 2-dimensional bitmap of size MQ to report the selected S pairs of FD basis vector and DD basis vector and a single 2-dimensional bitmap of size 2LS for indicating the location of the NZCs, where each row corresponds to a selected SD basis vector and each column corresponds to one of the selected S pairs of FD basis vector and DD basis vector. - Alt4’: Q different bitmaps are supported for each layer, each of the Q bitmaps corresponds to DD basis q = 0 or 1. - For each polarization, each of the Q bitmaps contains bits included in a set of SD basis and FD basis pairs {(s, f)}, satisfying min(f,Mv-f)+ min(¦s-sref ¦, L-¦s-sref ¦)≤D, where - s∈0,…,L-1, f∈0,…,M-1 - sref∈{0,…,L-1} is the SD basis indicated by SCI - Two polarizations have same set of {(s, f)} in the bitmap

We support Proposal 2.G.1. For FFS of optional feature part, we are open for further discussion.

Proposal 3-2

- Support Proposal 2.G.1

- Support of TDCP

During the offline discussion, the following proposal was made:

Proposal 3.A: For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, - KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’ - No further spec enhancement on TRS is supported - FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, RE location, relation with resource set used for legacy usage Proposal 3.B: For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, - At least the following size-Q quantization alphabet is supported: 1-2-N-qs, q=0,1,…,2Q-1 - TBD: supported value(s) of N (e.g. 2Q-1), Q, s (e.g. ½, ¼, 1/8, …), whether a center threshold is also supported (and if so, higher-layer configured) - FFS: Whether different schemes can be supported for different use cases

We are generally supportive of Proposal 3.A and 3.B above.

For the last FFS in 3.A, there was a suggestion to have such restriction(s) on the corresponding TRS resource set(s). We agree that those restrictions would be reasonable to be specified. For example, restricting QCL relationship, power control and/or RE location to be the same could be beneficial for UE to implement this feature, and for NW to understand TDCP better based on the reported metric.

For the proposal 3.B, FFS captures whether to support different schemes for different use cases. We are open to consider other schemes, if there is a valid use case which may not be suitable for the one captured in the proposal 3.A.

Proposal 4-1

- Support Proposal 3.A and Proposal 3.B

- Support to add further restriction on the TRS resource set(s) on e.g., QCL relationship, power control, RE location

Another point may be the definition of CPU for TDCP reporting. As TRS is not associated with any CSI reporting until Rel-17, CPU=0 is defined when TRS is configured. When we support TDCP reporting, the exact CPU value needs to be discussed in our view. This aspect could be a subsequent issue after concluding the metric in TDCP reporting. If a simpler metric is supported, we believe CPU=1 is sufficient here.

Proposal 4-2

- Regarding TDCP reporting, discuss the exact value for CPU

- Conclusion

In this contribution, we discussed CSI enhancement for high/medium UE velocities and CSI enhancement for M-TRP CJT for Rel-18 MIMO. Based on the discussion, we made following proposals.

For M-TRP CJT:

Proposal 2-1

- For codebook parameters, not support R = 4.

Proposal 2-2

- On the Parameter Combination, for NTRP=1, at least support the legacy parameter combinations.

- Support FL proposal 1.C.1.

Proposal 2-3

- Support additional {Ln} combination with Ln=6 for each Ln value, but this {Ln} combination can be selected/reported by UE only when UE reports TRP selection indication as single-TRP.

Proposal 2-4

- The supported combinations of {n} for Rel-17-based refinement is not directly derived from the supported combinations of {Ln} for Rel-16-based refinement.

  - Separate discussion is needed. n =1 is supported for combinations of {n} when NTRP > 1.

- The supported combinations of {M} for Rel-17-based refinement is not derived from the supported combinations of {pv ,}.

  - Separate discussion is needed. {M} configuration is layer-common.

Proposal 2-5

- Prefer not to add additional restrictions depending on the configured value for NTRP for supported candidate values for NL.

Proposal 2-6

- On FD basis selection/reporting for mode 1, we’re open to support Alt1 or Alt2.

  - Alt1 is preferred if no obvious performance loss is observed compared with Alt2.

Proposal 2-7

- Regarding W2 quantization group, for each layer, if Alt3 is supported in addition to Alt1, Alt3 is applied to codebook mode 1 only, and Alt1 is basic feature.

Proposal 2-8

- The configured K NZP CSI-RS resources associated with a CSI-ReportingConfig for CJT should be restricted within the same DRX Active Time, and within X continuous slot(s) without DL/UL switch. At least X=1 should be supported.

Proposal 2-9

- Support CBSR configuration on amplitude restriction when NTRP = 1 CSI-RS resource is configured.

- Support TRP-common CBSR configuration on  amplitude restriction when NTRP > 1 CSI-RS resources are configured.

Proposal 2-10

- For UCI omission, Alt3 is preferred.

Proposal 2-11

- For CSI part 1, define mapping order of indication for TRP selection and indication for L parameter combination.

  - Study whether to support bitmap omission for indicator of non-zero coefficients.

Proposal 2-12

- For CSI part 2, discuss mapping order of per-TRP parameters for each group.

For Type-II CSI enhancement in doppler domain:

Proposal 3-1: For the Type-II codebook refinement for high/medium velocities, when a UE is configured with X=2 for CQI calculation and reporting,

- Support the 2nd CQI is located in UCI part 2

- Support the 2nd CQI includes 4-bit wideband CQI and 2-bit sub-bands CQI calculated independently from the 1st CQI

Proposal 3-2

- Support Proposal 2.G.1

For TDCP reporting:

Proposal 4-1

- Support Proposal 3.A and Proposal 3.B

- Support to add further restriction on the TRS resource set(s) on e.g., QCL relationship, power control, RE location

Proposal 4-2

- Regarding TDCP reporting, discuss the exact value for CPU

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.