3GPP TSG RAN WG1 #112bis-e			R1-2303706

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on other aspects on AI/ML for CSI feedback enhancement

Agenda Item:	9.2.2.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of potential specification impacts as following.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements.

- Discussion on sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements

At the RAN1#109-e meeting, spatial-frequency domain CSI compression using two-sided AI models, as known as auto-encoder of CSI feedback was agreed to be a representative sub use case as following [2].

Agreement

Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case.

- Note: Study of other sub use cases is not precluded.

- Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

Fig. 1 illustrates the framework of CSI compression with two-sided models. As shown in Fig. 1, UE is equipped with an AI/ML encoder to compress CSI into encoded bits, while the corresponding AI/ML decoder is deployed on gNB to reconstruct CSI from encoded bits. In CSI compression with two-sided models, UE calculates downlink CSI, such as channel matrix or precoding matrix, and feeds the CSI into the encoder for compression. After the AI/ML encoder extracts essential features and outputs the encoded bits, UE reports the encoded bits to gNB where CSI can be reconstructed from encoded bits with the AI/ML decoder. In this contribution, the inputs of encoder and outputs of decoder are assumed to be the same.

Figure 1. The framework of spatial-frequency domain CSI compression with two-sided models.

With this AI/ML-based compression technique, accuracy improvements under a certain overhead of CSI reports and overhead reduction for CSI reports achieving a certain performance are observed [3].

At the RAN1#112 meeting, it was agreed that the potential specification impacts with the precoding matrix as input/output are further studied while the study of explicit channel matrix depends on the performance evaluation as following [4].

Agreement

In CSI compression using two-sided model use case, further study potential specification impact of the following output-CSI-UE and input-CSI-NW at least for Option 1:

- Option 1: Precoding matrix

  - 1a: The precoding matrix in spatial-frequency domain

  - 1b: The precoding matrix represented using angular-delay domain projection

- Option 2: Explicit channel matrix (i.e., full Tx * Rx MIMO channel)

  - 2a: raw channel is in spatial-frequency domain

  - 2b: raw channel is in angular-delay domain

- Note: Whether Option 2 is also studied depends on the performance evaluations in 9.2.2.1.

- Note: RI and CQI will be discussed separately

Along with this agreement, we discuss the spatial-frequency domain CSI compression with precoding matrix as input/output in the subsequent sections.

  - Model training for CSI compression

Agreement

In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, repectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

- Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes).

- Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW

- Other collaboration types are not excluded.

Conclusion

In CSI compression using two-sided model use case, training collaboration type 2 over the air interface for model training (not including model update) is deprioritized in R18 SI.

At the RAN1#110 and the RAN1#111 meeting, the following potential training procedures were identified to deploy an encoder and a decoder at UE and NW, and type 2 training procedure was agreed to be deprioritized except for model update [5] [6].

・Type 1: Joint training of two-sided model at a single side/entity

・Type 2: Joint training of the two-sided model at network side and UE side, respectively.

・Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

In this contribution, we assume that separate training is sequential training, because the only sequential training has been observed as a feasible separate training procedure with simulation results as far as we know.

In sequential training, the model on one side is trained after the model training at the other side. In our view, the sequential training can be categorized into two types as follows.

・Type 3-A: training based on the dataset collected from the trained model on the other side.

・Type 3-B: training via the exchange of forward and back propagation with the trained model on the other side.

In the type 3-A training procedure, the joint training is performed at one side first. After that, the other side receives the dataset produced by the trained encoder or decoder. Then, the other side trains the encoder or decoder based on the delivered dataset. In this training procedure, the only dataset delivery is necessary as the collaboration between UE and NW. Fig. 2 illustrates type 3-A training procedure when the NW side model is firstly trained in the sequential training.

Figure 2. Type 3-A training procedure, where the UE side model is trained based on the dataset collected from the trained encoder at NW side.

Similarly to type 3-A training procedure, the first step of type 3-B training procedure is the joint training performed at one side. After that, UE side and NW side exchange the common dataset for the training over UE and NW. With that common dataset, model is trained over UE and NW via the exchange of the forward propagation and the back propagation. During the training over UE and NW, the model at one side is frozen. Type 3-B training procedure can be illustrated as shown in Fig.3, when the NW side model is firstly trained in the sequential training.

The advantage of type 3-B training procedure is the extendibility. In our companion contribution, the performance comparison between type 3-A training and type 3-B training is provided [3]. When it comes to training the model compatible with the model in use at the other side, type 3-B training can provide better performance. Also, the input or output of model does not need to be revealed to the other side. If companies have the concern on the proprietary aspect regarding the model input or model output, type 3-B training procedure can be a solution. Thus, as the type 3-A training procedure and type 3-B training procedure has pros and cons, it is better to categorize the type 3 training into type 3-A training and type 3-B training for the further study.

Figure 3. Type 3-B training procedure, where the UE side model is trained with frozen decoder at NW side via forward propagation and back propagation exchange between UE and NW.

Proposal 1: Categorize type 3 training procedure (sequential training) as follows.

- Type 3-A: sequential training via the dataset delivery

  - Step 1 Joint training at one side

  - Step 2 Delivery of the dataset produced by trained encoder/decoder to the other side

  - Step 3 Training encoder/decoder based on the delivered dataset within the other side

- Type 3-B: sequential training via the gradient exchange

  - Step 1: Joint training at one side

  - Step 2: Share the common dataset for training at Step 3

  - Step 3: Training encoder/decoder at the other side via FP/BP exchange with the frozen decoder/encoder

Joint training is preferable rather than sequential training from the performance perspective, as the performance of joint training is the upper bound of sequential training. However, it was observed that the performance gap between sequential training and joint training are negligible when pre-/post-processing, such as quantization/dequantization method, is aligned [4].

Observation 1: The performance of joint training is the upper bound of sequential training.

Observation 2: Three type of training procedures provides the similar performance, when the pre-/post-processing is aligned.

On the other hand, the required overhead is different according to the training procedure. In Table 2, the necessary interaction between NW and UE in each training procedure is summarized. Type 2 and type 3 training procedures require the dataset delivery or/and the exchange of forward propagation and back propagation from one side to the other, which results in the large signaling overhead as shown in our companion contribution [3]. Hence, type 1 training procedure is preferred than type 2 and type 3 in terms of the overhead aspect.

Observation 3: Type 2 and type 3 training procedure requires large signalling overhead due to the dataset transfer or the exchange of forward/back propagation from one side to the other.

Table 2.  Necessary interaction between NW and UE in each training procedure.

| Training procedure | Type 1 training | Type 2 training | Type 3 training (sequential training) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Type 3-a training | Type 3-b training |
| Dataset delivery between UE and NW | - | Necessary | Necessary | Necessary |
| Model delivery | Necessary | - | - | - |
| Forward/back propagation exchange | - | Necessary |  | Necessary |

From performance and overhead perspective, type 1 training procedure is the most reasonable one. However, type1 training procedure requires the disclosure of model information to the other entity, which is viewed as the important issue by several companies. In addition, there is the feasibility concern of deploying the model trained by the other entity. In the practical scenario, AI/ML model processing could be so large burden that the hardware needs to be optimized for certain AI/ML models especially at the UE side. In that case, it is difficult for an entity to adapt the model trained by another entity without considering the hardware constraint of the entity adapting the model. If these aspects cannot be ignored, it is reasonable to consider model delivery without 3GPP signalling as the first step for type 1 training procedure. Specifically, it can be assumed that the offline agreements are made between multiple UE and NW vendors outside 3GPP before the model delivery. In that case, the proprietary issues can be solved without specification enhancement efforts.

Observation 4: Type 1 training procedure can provide good performance and requires less overhead signalling. However, the feasibility of model transfer is questionable in terms of the proprietary and hardware aspects.

As the type 2 training procedure requires overhead, the disclosure of model information, and the complicated interaction over UE and NW, it was deprioritized in Rel-18 except for the model update. In our view, these issues exist even for the model update in type 2 training. Hence, type 2 training procedure can be deprioritized even for model update.

Proposal 2: Deprioritize type 2 training procedure even for the model update.

  - Life cycle management for CSI compression

At the RAN1#111 and the RAN1#112 meetings [7][8], it was agreed to study two LCM procedures: functionality-based LCM and model ID-based LCM.

Agreement

For UE-part/UE-side models, study the following mechanisms for LCM procedures:

- For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality

  - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality.

  - FFS: Whether or how to indicate Functionality

- For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs

Agreement

For UE-side models and UE-part of two-sided models:

- For AI/ML functionality identification

  - Reuse legacy 3GPP framework of Features as a starting point for discussion.

  - UE indicates supported functionalities/functionality for a given sub-use-case.

    - UE capability reporting is taken as starting point.

- For AI/ML model identification

  - Models are identified by model ID at the Network. UE indicates supported AI/ML models.

- In functionality-based LCM

  - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI).

  - Models may not be identified at the Network, and UE may perform model-level LCM.

    - Study whether and how much awareness/interaction NW should have about model-level LCM

- In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID.

FFS: Relationship between functionality identification and model identification

FFS: Performance monitoring and RAN4 impact

FFS: detailed understanding on model

In functionality-based LCM procedure, UE is expected to receive the indication regarding which functionality should be activated/deactivated/monitored. When there are multiple models associated with one functionality, UE could control model level LCM (e.g., which model to be activated/deactivated/monitored) within one functionality. On the other hand, NW indicates the individual model via model ID in the model-ID-based LCM.

    - Model ID in two-sided model

In case of one-sided model at UE side, model ID should represent the identity of model at UE side. However, it is unclear what model is identified by the model ID in two-sided model. For example, the model ID may identify encoder model, decoder model, or paired model. According to that difference, the procedure for model-ID-based LCM could be different. For example, if UE receives the model ID representing the encoder model in the activation indication, UE would activate that indicated encoder model. On the other hand, if UE receives the model ID representing the decoder model in the activation indication, UE would activate one among encoder models paired to the indicated decoder model. Thus, it is better to make consensus on what model is identified by model ID in the two-sided model for the clarification. Unless the clarification could be smoothly made regarding it, we think it is better to introduce paired model ID, encoder model ID, and decoder model ID for the discussion purpose.

Proposal 3: Clarify what model is identified by model ID in the two-sided model. Until the clarification is made, it is better to introduce paired model ID, encoder model ID, and decoder model ID for the discussion purpose.

    - Paring of two-sided model

NW-UE collaboration level y does not support the model transfer. If the model transfer is supported, NW and UE can align the paired models by delivering one of paired model via 3GPP framework. However, in case of NW-UE collaboration level y, some mechanisms to align the paired trained models are necessary for two-sided models.

Proposal 4: Study the mechanism to align the paired trained models for two-sided models.

One possible solution is to exchange the information of available models at each side between NW and UE before and during the model/functionality identification. Fig. 4 illustrates the example of how NW and UE exchange the information of available two-sided models, where the model/functionality identification enables NW to be aware of the supported models/functionalities at UE side. In this example, UE firstly receives the assistance information including the decoder model ID before the model/functionality identification. If models paired to the reconstruction models in the assistance information are available at UE side, UE initiates the model/functionality identification. During the model/functionality identification, UE reports the information of generation models paired to the reconstruction models available at NW side. Then, NW can operate the generation models at UE side by model activation/deactivation. This approach can be performed without disclosing the proprietary model information. For instance, the ID representing the model can be used to identify the generation and reconstruction model. In such cases, UE and NW can discern whether certain model is the model paired to the available model at their side.

Observation 5: Model and assistance information can be used for paring of trained two-sided models in CSI compression.

Figure 4. Example to pair the two-sided models via assistance information and model registration.

  - Reporting framework

When CSI compression is applied, the encoded bits are reported. As the encoded bits represent CSI, it is reasonable to reuse the CSI reporting mechanism. However, RAN1 has not sufficiently discussed the feasibility of reusing the CSI reporting mechanism for encoded bits reporting. In this section, the feasibility of reusing legacy CSI reporting framework for CSI compression is discussed. If there is no technical issue, the existing CSI reporting framework should be reused for CSI compression.

Proposal 5: Reuse legacy CSI reporting principle, unless technical issue is observed.

    - CSI payload size

At the RAN1#112 meeting, the agreement related to the CSI payload size determination was made as follows [4].

Agreement

In CSI compression using two-sided model use case, further study the following aspects for CSI configuration and report:

- NW configuration to determine CSI payload size, e.g., possible CSI payload size, possible rank restriction and/or other related configuration.

- How UE determines/reports the actual CSI payload size and/or other CSI related information within constraints configured by the network.

Basically, NW configuration determines the possible CSI payload size. However, when the possible CSI payload size are multiple values, UE could determine and report one of them. In the existing framework, CSI payload size is not explicitly configured or reported. Instead, CSI payload size can be implicitly calculated from the NW configuration and the reported information by UE. For example, NW configuration determines what information is reported via the combination of parameters, and UE reports the rank information of reported precoding matrix in the same reporting instance. This principle could be reused for CSI compression as well. Instead of the explicit indication/configuration of CSI payload size, the possible CSI payload size can be calculated implicitly from the parameter configured by rank restriction and the possible list of encoder models/functionalities. Within the constraints configured by NW, UE determines and reports the rank and functionality/encoder model ID. Then, NW can implicitly calculate the reported CSI payload size. One potential concern of this approach could be the adaptive model. When one physical model is the adaptive model that can output different payload sizes, encoder model ID is not enough to identify the payload size. However, this issue can be solved by assigning the different logical model IDs or functionalities per payload size within one physical adaptive model.

Proposal 6: It is unnecessary to explicitly indicate/configure the CSI payload size. Instead, CSI payload size can be implicitly calculated based on the rank and model ID/functionality information.

    - CQI calculation

Even though the most of the existing CQI frameworks can be reused when the reconstructed CSI type is a precoding matrix, some enhancements are necessary so that CQI is applicable to the CSI compression. In the current framework, the CQI is calculated by UE assuming that the precoding matrix derived from the reported PMI is applied. If PMI is replaced by the encoded bit information, how to assume a precoding matrix for CQI calculation needs to be specified for CSI compression scenarios. Also, unless the decoder is deployed at UE in addition to the encoder, the reconstructed CSI is not available at UE. In that case, UE is not fully aware of the precoding matrix reconstructed at gNB. At the RAN1#112 meeting, the agreement related to CQI determination was made [4].

Agreement

In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured.

- Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 1a: CQI is calculated based on target CSI with realistic channel measurement

  - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

  - Option 1c: CQI is calculated based on legacy codebook

- Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including

  - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment

    - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW.

  - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder.

- Other options are not precluded

- Note1: feasibility of different options should be evaluated

- Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated

- Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Option 1a is the simplest approach to calculate CQI based on the target CSI. The drawback of this approach is that the calculated CQI could be exaggerated due to the gap between the target CSI and the reconstructed CSI. The same issue exists in Option 1b and Option 1c. On the other hand, Option 2a and Option 2b can provide the CQI calculated based on the reconstructed CSI. However, Option 2a requires the additional processing and additional model only for the CQI calculation, and Option 2b results in the large latency to obtain the CQI. Thus, all Options have pros and cons.

If the accurate CQI is obtained, the proper MCS can be selected. However, CQI is not the only reference for the MCS selection. For example, HARQ-ACK mechanism can help MCS selection via closed-loop mechanism. Hence, the accurate CQI is not essential in the operation. As long as reported CQI can be useful as the rough reference for MCS selection, it is sufficient from the operation perspective. From this point of view, Option 1a seems sufficient enough as CQI for CSI compression.

Observation 6: There is another mechanism to help MCS selection, such as HARQ-ACK mechanism, in addition to CQI reporting.

Proposal 7: CQI calculated based on target CSI is sufficient for CSI compression.

    - Channel for CSI reporting

In the existing 5G NR framework, subband type II codebook can be reported only on PUSCH due to the expected overhead. Even though CSI compression can reduce the signalling overhead, the signalling overhead is expected to be larger than wideband type II codebook. Therefore, it is reasonable to reuse the same constraint as subband type II codebook about channel for CSI reporting.

Observation 7: For CSI compression, the constraint on channel for CSI reporting can be the same as subband type II codebook.

    - Priority rule and mapping order

CSI reporting consists of two parts, where CSI part 1 (e.g., RI, CQI) has a fixed payload size to identify CSI part 2 payload (e.g., subband information in PMI). The same principle can be reused in CSI compression. For example, the encoder model information and rank information are included in CSI part 1, while compressed bits are reported in CSI part2. With this approach, the compression ratio or payload size of compressed bits can be identified based on CSI part 1.

Observation 8: For CSI compression, CSI reporting can consist of two parts; CSI part 1 including RI/encoder model ID/CQI, and CSI part 2 including compressed bits.

Also, the priority rules for CSI collision handling and CSI omission can be basically reused for CSI compression. For instance, the priority values associated with the CSI reports can be calculated with the existing formula specified in 5.2.5 in 38.214. One incompatible mechanism to CSI compression is the priority reporting level within the compressed bits. As the compressed bits are not dividable, the priority reporting level cannot be defined in the same manner as other type 2 codebooks. Hence, it is better to study the priority reporting level for CSI part 2 compatible with CSI compression.

Observation 9: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits.

Proposal 8: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits.

  - Model monitoring

At the RAN1#110bis-e meeting and the RAN1#112 meeting, the agreements related to performance monitoring were made [4][9].

Agreement

In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods:

- Intermediate KPIs as monitoring metrics (e.g., SGCS)

- Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK).

- Legacy CSI based monitoring: schemes using additional legacy CSI reporting

- Other monitoring solutions, at least including the following option:

  - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

Agreement

Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance

    - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance)

    - Overhead (e.g., signaling overhead associated with model monitoring)

    - Complexity (e.g., computation and memory cost for model monitoring)

    - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

    - FFS: Power consumption

    - Other KPIs are not precluded.

Note: Relevant KPIs may vary across different model monitoring approaches.

FFS: Discussion of KPIs for other LCM procedures

Agreement

In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least:

- NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side.

- UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- UE-side monitoring based on the output of the CSI reconstruction model at the UE-side

  - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side.

  - Network may configure a threshold criterion to facilitate UE to perform model monitoring.

- FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE.

Note: Monitoring approaches not based on intermediate KPI are not precluded

Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

As captured in the above agreements, there are several performance monitoring metrics/methods in CSI compression. In this sub-section, each performance monitoring approach is analysed from the accuracy, relevance, overhead, complexity and latency perspectives.

One approach to monitor the performance is based on model accuracy by comparing the target CSI and the reconstructed CSI. This approach can monitor the purely model performance without other factors. Also, since the model accuracy can be calculated at each time instance, it is possible to immediately detect the temporal performance degradation. In that case, the proper model/functionality operation (e.g., model/functionality switching and fallback operation) can be taken soon after the model failure. In CSI compression, the performance monitoring based on model accuracy can be mainly categorized into three types as captured above: NW side monitoring with target CSI reporting, UE side monitoring with NW indication of reconstructed CSI, and UE side monitoring with reconstruction model at UE side.

NW side monitoring with target CSI reporting can be depicted as Fig. 5. In this approach, UE reports results of model inference and the quantized target CSI, so that NW can monitor how accurate model inference is based on comparison between reconstructed CSI and the quantized target CSI. As it requires non-AI-based CSI feedback calculation and reporting in addition to AI-based CSI feedback, the overhead can be the disadvantage. Furthermore, the quantization error of non-AI-based CSI feedback could result in the low accuracy of the monitored performance.

Observation 10: NW side monitoring with target CSI reporting suffers from the signalling overhead and quantization error of target CSI reporting.

Figure 5. NW side monitoring with target CSI reporting.

Fig.6 shows UE side monitoring with NW indication of reconstructed CSI. This model monitoring approach also suffers from the signalling overhead and quantization error due to the NW indication of reconstructed CSI report. On top of that, the additional signalling overhead by reporting the calculated performance metric is caused, as it needs to be reported to NW for NW functionality/model scale upcoming operation decisions.

Observation 11: UE side monitoring with NW indication of reconstructed CSI suffers from the signalling overhead and quantization error of reconstructed CSI indication.

Figure 6. UE-side model monitoring with NW indication of reconstructed CSI.

UE side monitoring with reconstruction model at UE side is illustrated in Fig. 7. In this approach, UE is expected to have the paired decoder to output the reconstructed CSI. Hence, the additional model storage and processing due to the reconstruction model at UE side is the drawback. However, the accuracy of model is high by comparing the actual reconstructed CSI with target CSI.

Observation 12: The feasibility of UE side monitoring with reconstruction model at UE side is questionable due to the additional model storage and processing for the reconstruction model at UE side.

Figure 7. UE side monitoring with reconstruction model at UE side.

Empirical system performance is another model monitoring metric. When the model is monitored based on this performance, the system performance can be increased by the proper model management. This metric can be obtained by observing the signals transmitted with the precoder based on the reconstructed CSI. The biggest advantage of this approach is the simplicity. Neither additional signalling nor measurement is needed to acquire the monitored performance. However, even if the monitored empirical system performance is low, it is difficult to discern if this deterioration is due to the active model.

Observation 13: Empirical system performance does not require the additional signalling and measurement. However, the relevance to the model performance is low compared to the inference accuracy KPI.

Input/output data distribution can be useful to discern if the data characteristic is the same between the model training stage and the model inference stage. If the statistical data characteristic is changed, the model performance also might change according to it. However, as the relevance of the data distribution to the model/system performance is questionable, the feasibility of the model monitoring based on input/output data distribution in CSI compression should be discussed before studying the specification impacts related to it.

Proposal 9: Discuss the feasibility of the model monitoring based on the input/output data distribution in CSI compression, before the specification impact discussion related to it.

In Table 3, the characterization of each model monitoring approach is summarized.

Table 3.  Characterization of each model monitoring approach.

| Model monitoring approach | Relevance to the model performance | Signaling overhead | Complexity | Latency |
| --- | --- | --- | --- | --- |
| NW side monitoring with target CSI reporting | High relevance | Large | Low | Low latency to detect the model failure |
| UE side monitoring with NW indication of reconstructed CSI | High relevance | Large | Low | Low latency to detect the model failure |
| UE side monitoring with reconstruction model at UE side | High relevance | Low | High due to the reconstruction model at UE side | Low latency to detect the model failure |
| empirical system performance | Low relevance | Low | - | Large latency to detect the model failure |
| input/output data distribution | Low relevance | Low | High due to the drift detection | Large latency to detect the model failure |

- Conclusion

In this contribution, we discussed the sub use-cases and potential specification impacts on AI/ML for CSI feedback enhancements. Based on the discussion we made the following observations and proposals.

Observation 1: The performance of joint training is the upper bound of sequential training.

Observation 2: Three type of training procedures provides the similar performance, when the pre-/post-processing is aligned.

Observation 3: Type 2 and type 3 training procedure requires large signalling overhead due to the dataset transfer or the exchange of forward/back propagation from one side to the other.

Observation 4: Type 1 training procedure can provide good performance and requires less overhead signalling. However, the feasibility of model transfer is questionable in terms of the proprietary and hardware aspects.

Observation 5: Model and assistance information can be used for paring of trained two-sided models in CSI compression.

Observation 6: There is another mechanism to help MCS selection, such as HARQ-ACK mechanism, in addition to CQI reporting.

Observation 7: For CSI compression, the constraint on channel for CSI reporting can be the same as subband type II codebook.

Observation 8: For CSI compression, CSI reporting can consist of two parts; CSI part 1 including RI/encoder model ID/CQI, and CSI part 2 including compressed bits.

Observation 9: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits.

Observation 10: NW side monitoring with target CSI reporting suffers from the signalling overhead and quantization error of target CSI reporting.

Observation 11: UE side monitoring with NW indication of reconstructed CSI suffers from the signalling overhead and quantization error of reconstructed CSI indication.

Observation 12: The feasibility of UE side monitoring with reconstruction model at UE side is questionable due to the additional model storage and processing for the reconstruction model at UE side.

Observation 13: Empirical system performance does not require the additional signalling and measurement. However, the relevance to the model performance is low compared to the inference accuracy KPI.

Proposal 1: Categorize type 3 training procedure (sequential training) as follows.

- Type 3-A: sequential training via the dataset delivery

  - Step 1 Joint training at one side

  - Step 2 Delivery of the dataset produced by trained encoder/decoder to the other side

  - Step 3 Training encoder/decoder based on the delivered dataset within the other side

- Type 3-B: sequential training via the gradient exchange

  - Step 1: Joint training at one side

  - Step 2: Share the common dataset for training at Step 3

  - Step 3: Training encoder/decoder at the other side via FP/BP exchange with the frozen decoder/encoder

Proposal 2: Deprioritize type 2 training procedure even for the model update.

Proposal 3: Clarify what model is identified by model ID in the two-sided model. Until the clarification is made, it is better to introduce paired model ID, encoder model ID, and decoder model ID for the discussion purpose.

Proposal 4: Study the mechanism to align the paired trained models for two-sided models.

Proposal 5: Reuse legacy CSI reporting principle, unless technical issue is observed.

Proposal 6: It is unnecessary to explicitly indicate/configure the CSI payload size. Instead, CSI payload size can be implicitly calculated based on the rank and model ID/functionality information.

Proposal 7: CQI calculated based on target CSI is sufficient for CSI compression.

Proposal 8: For CSI compression, the legacy priority rules for CSI collision handling and CSI omission can be reused except for the priority reporting level within the compressed bits.

Proposal 9: Discuss the feasibility of the model monitoring based on the input/output data distribution in CSI compression, before the specification impact discussion related to it.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (Apple), R1-2205556, “Email discussion on other aspects of AI/ML for CSI enhancement”, May. 2022.

- NTT DOCOMO, INC., R1-2303705, “Discussion on evaluation on AI/ML for CSI feedback enhancements”, Apr. 2023.

- Moderator (Apple), R1-2301912, “Email discussion on other aspects of AI/ML for CSI enhancement”, Feb. 2023.

- Moderator (Apple), R1-2208077 “Summary #4 on other aspects of AI/ML for CSI enhancement”, Aug. 2022.

- Moderator (Apple), R1-2210611, “Summary #4 on other aspects of AI/ML for CSI enhancement”, Oct. 2022.

- Moderator (Qualcomm), R1-2210661, “Summary#5 of General Aspects of AI/ML Framework”, Oct. 2022

- Moderator (Qualcomm), R1-2213003, “Summary#1 of General Aspects of AI/ML Framework”, Nov. 2022

- Moderator (Huawei), R1-2212966, “Summary#7 of [110bis-e-R18-AI/ML-02]”, Nov. 2022