3GPP TSG RAN WG1 #112bis-e			R1-2303708

e-Meeting, April 17th – April 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on other aspects AI/ML for beam management

Agenda Item:	9.2.3.2

Document for: 	Discussion and Decision

- Introduction

At the RAN#94-e meeting, a new SID [1] on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” was approved. This SID captures the objective of SI in terms of potential specification impacts as follows.

- Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework:

  - PHY layer aspects, e.g., (RAN1)

    - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases

    - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback

  - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1

    - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference),  and management of data and AI/ML model, per RAN1 input

    - Collaboration level specific specification impact per use case

  - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2

    - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable

    - Consider the need and implications for AI/ML processing capabilities definition

In this contribution, we discuss sub use-cases and potential specification impacts on AI/ML for beam management.

- Discussion on sub use-cases and potential specification impacts on AI/ML for beam management

At the RAN1#109-e meeting and the RAN1#110bis-e, it was agreed that the only spatial-domain DL beam prediction (BM-case 1) and temporal DL beam prediction (BM-case 2) are studied for characterization and baseline performance evaluations as follows [2] [3].

Conclusion

For AI/ML based beam management, RAN1 has no consensus to support on studying any other sub use case in addition to BM-Case1 and BM-Case2

Note: this conclusion is independent of the discussion on the alternatives of AI/ML model inputs for BM-Case1 and BM-Case2

Agreement

For AI/ML-based beam management, support BM-Case1 and BM-Case2 for characterization and baseline performance evaluations

- BM-Case1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

- FFS: details of BM-Case1 and BM-Case2

- FFS: other sub use cases

Note: For BM-Case1 and BM-Case2, Beams in Set A and Set B can be in the same Frequency Range

  - General aspects for DL beam prediction

    - Beam prediction scenario

At the RAN1#110 meeting, the following beam prediction types for spatial domain beam prediction (BM-Case 1) and temporal beam prediction (BM-Case 2) were agreed to be studied [4].

Agreement

For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams:

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

- Note1: DL Rx beam prediction may or may not have spec impact

As shown in the Note1 in the above agreement, Rx beam prediction could be up to implementation without any specification impacts. Also, as the number of Rx beams is generally smaller than that of Tx beams in DL, the expected overhead reduction gain of beam prediction is relatively small. For those reasons, we focus on DL Tx beam prediction and beam pair prediction in this contribution.

Currently, RAN1 AI/ML study covers a variety of scenarios/options for the beam prediction. Given that it is important to consider all possibilities, some studied scenarios could be unpractical or minor cases. For the efficacy of the study, it is reasonable to put the focus on the study of practical scenarios if possible. As the first step, we think RAN1 should discuss and identify the practical scenario for Tx beam prediction and beam pair prediction in addition to the performance gain.

Proposal 1: Identify the practical scenario for Tx beam prediction and beam pair prediction.

Fig. 1 illustrates three scenarios for Tx beam prediction and beam pair prediction. Since the beam prediction requires Set B measurements as inputs for AI/ML models, P1 measurement is inevitable for both Tx beam prediction and beam pair prediction. When Tx beam prediction is applied, the beam quality of dense Tx beams can be estimated based on Set B measurements. As a result, P2 measurement can be replaced by the beam prediction as shown in Fig. 1.

Depending on whether Rx beam refinement is performed after Tx beam prediction or not, Tx beam prediction can be categorized into two scenarios. When Rx beam refinement is not performed, UE determines the DL Rx beam based on P1 measurement. On the other hand, P3 measurement is necessary if Rx beam refinement is performed after beam prediction, as depicted in scenario 2 of Fig.1. In case of beam pair prediction, Rx beam can be predicted in addition to Tx beam. Hence, P2 and P3 beam measurements can be replaced by the beam pair prediction. Also, it should be noted that additional K measurements are necessary after beam prediction, if top K beam prediction is applied.

Observation 1: After the beam prediction, additional beam measurements are necessary for the beam determination in the following case

・Top K beam prediction is applied

・Rx beam refinement is performed after Tx beam prediction

Figure 1. Necessary measurements for Tx beam prediction and beam pair prediction. (a) Tx beam prediction (b) beam pair prediction

At the RAN1#111 meeting, it was agreed to report the assumption on T1 and T2 patterns in temporal beam prediction [5].

Agreement

- For the evaluation of the overhead for BM-Case2, adoption the following metrics:

  - RS overhead reduction,

    - Option 2: RS OH reduction%=1-NM

      - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable

      - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme

      - Companies report the assumption on additional measurements

    - FFS: Option 3: RS OH reduction%=1-NLM

      - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance

      - where M is the total number of beams (pairs) to be predicted for each time instance

      - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction

    - Companies report the assumption on T1 and T2 patterns

In temporal beam prediction, there are two patterns for T1 and T2 as shown in Fig.2: prediction of beam quality between each measurement/reporting (pattern A) and prediction of beam quality instead of measurement/reporting (pattern B). In case of pattern A, the time sequence of inputs for AI/ML model in T1 has different periodicity from one of outputs in T2. Since the beam prediction can be used to interpolate the beam quality between each measurement, the periodicity of beam measurement/reporting can be configured to be large compared to one without the beam prediction. On the other hand, the time sequence of inputs for AI/ML model has the same periodicity from one of outputs in pattern B. In this pattern, beam measurement/reporting can be skipped after certain number of measurements, because the beam prediction can compensate them based on the historical beam measurements. Thus, even though both patterns can reduce the overhead of measurement/reporting, one approach can lead to the large periodicity of measurement/reporting, while the other approach could enable skipping of measurement/reporting for a while. As both approaches bring the practical gain and the desired pattern can be different according to NW operation, it is beneficial to study both patterns for T1 and T2.

Figure 2. T1 and T2 patterns in temporal beam prediction. (a) prediction of beam quality between each measurement/reporting (b) prediction of beam quality instead of measurement/reporting.

Proposal 2: Study the following two patterns for T1 and T2 in temporal beam prediction.

・prediction of beam quality between each measurement/reporting

・prediction of beam quality instead of measurement/reporting

    - Beam Set for DL beam prediction

At the RAN1#110bis-e and the RAN1#112, the definition of Set B was clarified, and the concept of Set C was introduced as follows, respectively [3] [5].

Conclusion

For the sub use case BM-Case1 and BM-Case2, Set B is a set of beams whose measurements are taken as inputs of the AI/ML model

Agreement

- Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable)

  - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C

- Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

In the above agreement, Set C is defined as the measured beams (pairs). In the DL beam prediction at NW side, Set B is selected from the reported beams. However, the reported beams are not always the same as measured beams. For example, the reported beams are ones achieving the largest L1-RSRP among the measured beams in the current L1-RSRP reporting. As the Set B and Set C are not always the same, it is better to differentiate them. For the same reason, we think it is better to define Set D as a set of beams which could be potentially measured by UE. Fig.3 illustrates the example of Set A, Set B, Set C, and Set D.

Figure 3. Example of Set A, Set B, Set C, and Set D. (a) DL beam prediction at UE side (b) DL beam prediction at NW side.

In the practical scenario, UE cannot detect some signals due to the measurement sensitivity when the SNR is lower than certain level. Then, it is meaningless to measure the beam achieving very low SNR, such as the beam whose boresight is not aligned with UE direction. As one of the biggest motivations of the beam prediction is to reduce the number of beam measurements, the useless beam measurements should be avoided as much as possible in the beam prediction. Then, it is reasonable that UE does not measure all beams when some beams are expected to be not detectable. In other words, Set C should be chosen smartly among the potentially measured beams (Set D). Considering the situation where Set C is a subset of Set D, Set D should be defined to facilitate the discussion.

Proposal 3: Define Set C and Set D as follows to facilitate the discussion

・Set C is a set of beams whose measurements are obtained by UE for model inputs (Set B ⊂ Set C)

・Set D is a set of beams which could be potentially measured by UE  (Set C ⊂ Set D)

Proposal 4: Study the following scenario for the beam prediction

・Fixed Set C. UE measure the same beam per model inference.

・Variable Set C. UE may measure the different beam per model inference.

    - Performance monitoring for DL beam prediction

At the RAN1#112, the agreement related to the metric for the performance monitoring in DL beam prediction was made as follows [5]

Agreement

Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection:

- Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy

- Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER

- Alt.3: Performance metric based on input/output data distribution of AI/ML

- Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP

- Other alternatives are not precluded

- Note: At least the performance and spec impact should be considered

Performance metric is an important aspect in performance monitoring. Beam prediction accuracy related KPI can accurately evaluate the model inference performance. This metric can be calculated based on the inference results (predicted Set A beam RSRP/indication) and the target values of model inference (ground truth data). Fig. 3 shows example of how to obtain the beam prediction accuracy, when the input of model inference and the target value of model inference are L1-RSRP of Set B and L1-RSRP of Set A, respectively. The drawback of this approach is the measurement overhead. As it is necessary to search out the actual topK/1 beam in this approach, the beam measurements of all Set A beams are required.

Observation 2: Beam prediction accuracy related KPI requires the large measurement overhead for searching out the actual top 1/K beam(s).

Figure 3. Performance monitoring via beam prediction accuracy related KPI, when the input of model inference and the target value of model inference are L1-RSRP of Set B and L1-RSRP of Set A, respectively.

Link quality related KPI is another metric in performance monitoring. As these KPIs are anyway necessary for the NW operation, this metric can be obtained without the additional signalling and measurements for performance monitoring. One drawback of the performance monitoring based on link quality related KPI is the relevance to the model performance. Even if the monitored link quality related KPI (e.g., throughput and L1-RSRP) is low, it is difficult to discern if this deterioration comes from the active model performance.

Observation 3: Link quality related KPI is affected by various factors other than model performance.

Input/output data distribution is also one approach to monitor the model performance. The drift detection based on the data distribution can discern if the data characteristic is the same between the model training stage and the model inference stage. If the statistical data characteristic is changed, the model performance also might change according to it. However, as the relevance of the data distribution to the model/system performance is questionable, the accuracy of the performance monitoring based on input/output data distribution in beam prediction should be discussed before studying the specification impacts related to it.

Proposal 5: Discuss the feasibility of the performance monitoring based on the input/output data distribution in the beam prediction, before the specification impact discussion related to it.

Predicted L1-RSRP difference is one of the attractive performance metrics. This approach can evaluate the model performance by comparing the predicted value with the actual value of the same beam. As opposed to the beam prediction accuracy related KPI, the predicted L1-RSRP does not require searching out the actual top1/K beam. Instead, the only measurements of the top 1/K predicted beams are necessary in addition to the inference results. Hence, the additional measurement overhead is lower than the beam prediction accuracy related KPI.

Observation 4: The required measurement overhead for performance monitoring is relatively low for the monitoring based on the predicted L1-RSRP difference.

Agreement

For the sub use case BM-Case1, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

Agreement

For the sub use case BM-Case2, consider both Alt.1 and Alt.2 for further study:

- Alt.1: AI/ML inference at NW side

- Alt.2: AI/ML inference at UE side

For the spatial domain beam prediction and temporal beam prediction, it was agreed to study the beam prediction at UE side and NW side at the RAN1#109-e meeting [2]. Since the potential specification impacts heavily depend on where model inference is performed, we discuss the potential specification impacts related to spatial domain beam prediction and temporal beam prediction for NW side model and UE side model individually in the subsequent sections, respectively.

  - DL beam prediction with NW side model

Since gNB is expected to have more computational resources, memory storage, and high-frequent exposure to communications compared with UE, DL beam prediction with NW side model is one of the promising scenarios.

In Rel-17, beam measurement reporting supports reporting UE panel used for the corresponding measurement. However, Rx beam ID is still determined by UE implementation and not included in the current beam measurement reports. The lack of DL Rx beam information at NW side makes it difficult to perform beam prediction with NW side model. DL Rx beam prediction and DL Tx-Rx beam pair prediction at NW are apparently hard without Rx beam information. However, there is a concern about reporting DL Rx beams due to the proprietary aspects. If RAN1 can make the consensus that the DL Rx beam information cannot be reported to NW, DL Tx-Rx beam pair prediction with NW side model should be deprioritized due to the feasibility.

Observation 5: In DL Tx-Rx beam pair prediction with NW side model, some mechanisms to report Rx beam ID used for beam measurements are necessary.

Proposal 6: If RAN1 can make the consensus that the DL Rx beam information cannot be reported to NW, DL Tx-Rx beam pair prediction with NW side model should be deprioritized due to the feasibility.

In addition, Tx beam prediction is also difficult when DL Rx beam is frequently changed without NW awareness, because the measurement value is different even with the same Tx beam according to the DL Rx beam. In Fig.4, two Rx beam determination mechanisms are illustrated. When the Rx beam for each Tx beam measurement is determined to achieve the largest RSRP for each Tx beam (Alt 1 in Fig. 4), Tx beam prediction performance deteriorates compared to the case where the particular Rx beam is used for all Tx beam measurements (Alt 2 in Fig. 4), as shown in our companion contribution [6]. Thus, as the DL Rx beam determination is important for the beam prediction, configuring the certain Rx beam determination policy should be discussed for NW side beam prediction.

Figure 4. Example of Rx beam determination policy. Alt1) Set B measurement is obtained from the best Rx beam for each Tx beam. Alt2) Set B measurement is obtained from the best Rx beam for the best Tx-Rx beam pair among Set C.

Proposal 7: In DL Tx beam prediction with NW side model, configuring Rx beam determination policy, e.g., the same Rx beam for Set B beam measurements, can be considered as potential specification impacts.

    - Spatial domain DL beam prediction with NW side model

Since spatial domain DL beam prediction can estimate the best beam based on CSI reports with sparse beam measurements, CSI report does not necessarily include the beam measurements associated with the best beam. Moreover, covering a wide direction with a small number of beam measurements could be important for spatial domain beam prediction. On the other hand, measurement sensitivity should be considered. If the SNR is lower than certain level, the measurement values are not reliable for model inputs. Thus, there are several aspects to be considered regarding what beams should be reported for the beam prediction. In the current L1-RSRP reporting, RSRP of the CRI/SSBRI achieving the largest RSRP is reported. However, the selection policy on CRI/SSBRI whose RSRP is reported could be optimized for this sub use case.

Observation 6: Enhancements on beam selection policy in CSI reports might be potential specification impacts for spatial domain beam estimation.

    - Temporal DL beam prediction with NW side model

Historical beam measurements in one reporting instance can help AI/ML to provide high performance in beam management [6]. In the current specification, multiplexed CSI report can consist of historical beam measurements. For example, CSI report at one uplink transmission occasion can include multiple CSI where each CSI corresponds to different NZP CSI-RS occasions by configuring multiple CSI report with overlapped uplink transmission occasions. However, the conventional beam reporting mechanism is not optimized for the historical beam measurement report. For instance, current L1-RSRP reporting conveys RSRP of the CRI/SSBRI achieving the largest RSRP. As a result, each CSI report might include the different CRI/SSBRI over time. But for the purpose of tracking beam quality, the data showing how the quality of same beam changes in time domain is useful. In that sense, common CRI/SSBRI of beam reporting over time domain might be beneficial for time domain beam prediction. In addition to it, UCI payload reduction can be expected, if common CRI/SSBRI for multiple CSI is supported as shown in Fig. 5. Thus, L1 beam reporting mechanism should be optimized to improve the performance of temporal beam prediction.

Proposal 8: L1 beam reporting should be enhanced to facilitate the model inference of temporal beam prediction with the overhead reduction.

Figure 5. Example of enhanced CSI report for time-domain beam prediction.

    - Performance monitoring of NW side models

At the RAN1#110bis-e meeting, the agreements related to the performance monitoring of beam prediction were made as follows [3].

Agreement

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring:

- NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement

Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects

- Beam measurement and report for model monitoring

- Note: This may or may not have specification impact.

In case of NW side model, it was agreed to study performance monitoring metrics and making decisions of model operation at NW side. Since the ground truth data is required to evaluate the model accuracy, the mechanism to report the ground truth data to NW should be studied. Given that some AI/ML models in BM-Case 1 and BM Case2 outputs the top-1 or top-N/1 probability, the ground truth data corresponding to that information should be reported for NW side model monitoring. For this purpose, the only actual top-1 or top-N beams are required for model monitoring. Hence, the overhead reduction could be considered by top-1 or top-N beam indication without L1-RSRP/SINR.

Proposal 9: Study the overhead reduction of L1 signalling to report SetA beam measurements for NW-based model monitoring.

  - DL beam prediction with UE side model

When DL beam prediction is performed at UE side, the DL Rx beam information is fully available. Also, the DL beam measurement values can be obtained without beam measurement reporting unlike DL beam prediction at NW side. Thus, DL beam prediction at UE side is also as promising as well as beam prediction at NW side.

    - Spatial domain DL beam prediction with UE side model

Agreement

Regarding the sub use case BM-Case1 and BM-Case2, study the following alternatives for AI/ML output:

- Alt.1: Tx and/or Rx Beam ID(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.2: Tx and/or Rx Beam ID(s) of the N predicted DL Tx and/or Rx beams and  other information

  - FFS: other information (e.g., probability for the beam to be the best beam, the associated confidence, beam application time/dwelling time, Predicted Beam failure)

  - E.g., N predicted beams can be the top-N predicted beams

- Alt.3: Tx and/or Rx Beam angle(s) and/or the predicted L1-RSRP of the N predicted DL Tx and/or Rx beams

  - E.g., N predicted beams can be the top-N predicted beams

  - FFS: details of Beam angle(s)

- FFS: how to select the N DL Tx and/or Rx beams (e.g., L1-RSRP higher than a threshold, a sum probability of being the best beams higher than a threshold, RSRP corresponding to the expected Tx and/or Rx beam direction(s))

- Note1: It is up to companies to provide other alternative(s)

- Note2: Beam ID is only used for discussion purpose

- Note3: All the outputs are “nominal” and only for discussion purpose

- Note4: Values of N is up to each company.

- Note5: All of the outputs in the above alternatives may vary based on whether the AI/ML model inference is at UE side or gNB side.

- Note 6: The Top-N beam IDs might have been derived via post-processing of the ML-model output

At the RAN1#110 meeting, several AI/ML outputs were agreed to be studied for DL beam prediction [4]. As can be seen in the above agreement, top-N predicted beams are also considered as the candidate of model outputs. When the model output is top-N predicted beams, two-stage beam measurements should be considered to select one applied beam as shown in Fig.6. Two-stage beam measurement with top-N predicted beams consists of the following steps.

- UE measures RS with Set B beams and feed the measurement values into AI/ML models that outputs top-N predicted beams.

- UE measures top-N predicted beams from Set A based on the outcome of Step.1.

- UE reports K beam measurements in Step2.

Figure 6. Two-stage beam measurements with top-N predicted beams.

When the two-stage beam measurement is applied, the number of measurements depends on Set B and N. Even if the number of beams in Set A is large, it is possible to save the number of measurements. Also, the probability that top-1 genie aided beam is one of top-N predicted beam gets large, as N increases [4]. It implies that two-stage beam measurements increase the beam selection accuracy compared to the top-1 beam prediction. Moreover, two-stage beam measurements make it possible to select the beam based on the actual measured values instead of the predicted values. It might help UE obtaining the actual QCL relations. As many gains can be expected in two-stage beam measurement, two-stage beam measurements should be considered for DL beam prediction with UE-side models.

Proposal 10: Study two-stage beam measurements with top-N predicted beams, since it reduces RS measurement overhead and increases the reliability of beam selection compared to top-1 beam prediction.

    - Temporal DL beam prediction with UE side model

At the RAN1#110bis-e, the agreement to study the potential specification impact of L1 signalling including multiple time instance(s) was agreed as follows [3]

Agreement

For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW

- The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference

  - FFS: value of N

- FFS: Predicted L1-RSRP corresponding to the beam(s)

- Information about the timestamp corresponding the reported beam(s)

  - FFS: explicit or implicit

- FFS: other information

In general temporal beam prediction, the predicted time instance is shifted from the measured RS occasion by the time offset specific to active models. If the measured RS occasion is determined by CSI reference resource in the 5G NR framework, the time instance corresponding to the reported beam(s) can be implicitly determined when the time offset is known at NW. In that case, it is not necessary to report the time instances corresponding to the reported beam(s) in the beam reporting.

Observation 7: Time instances corresponding to the reported beam(s) does not need to be reported when CSI reference resource is aligned between UE and NW, since it can be implicitly determined based on the measured RS occasion and the time offset.

In Rel-18 MIMO, however, there is a discussion about whether CSI reference resource can be qualified to be a source to determine the valid timing. Some companies have the concern that CSI reference resources may not be aligned between UE and gNB due to the miss-detection of DCI and the incomplete specification description of CSI reference resource. If so, it is beneficial to report the explicit predicted time instances in the reporting so that NW can make sure which predicted time instance is associated with each reported beam.

Observation 8: It is beneficial to report the explicit predicted time instances in the reporting, if CSI reference resource is not always aligned between UE and NW.

    - Performance monitoring of UE side models

At the RAN1#110bis-e meeting and the RAN1#112 meeting, the agreements related to the performance monitoring were agreed [3] [5].

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection:

- Atl1. UE-side Model monitoring

  - UE monitors the performance metric(s)

  - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation

- Atl2. NW-side Model monitoring

  - NW monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- Alt3. Hybrid model monitoring

  - UE monitors the performance metric(s)

  - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility:

- Indication/request/report from UE to gNB for performance monitoring

  - Note: The indictation/request/report may be not needed in some case(s)

- Configuration/Signaling from gNB to UE for performance monitoring

- Other aspect(s) is not precluded

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity:

- Configuration/Signaling from gNB to UE for measurement and/or reporting

- UE reporting to NW (e.g., for the calculation of performance metric)

- Indication from NW for UE to do LCM operations

- Other aspect(s) is not precluded

- Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered

To cover the functionality-based LCM in addition to model-ID-based LCM, “performance monitoring” is captured instead of “model monitoring” in the agreements at the RAN1#112 meeting [5]. From the above agreements, the model monitoring of UE side models can be categorized into three types: UE side model monitoring, NW side model monitoring, and hybrid model monitoring. On the other hand, only UE side performance monitoring and NW side performance monitoring were agreed to be studied, and hybrid performance monitoring was excluded in the agreements. As the hybrid performance monitoring can reduce the signalling overhead compared to NW side performance monitoring, the hybrid performance monitoring should also be studied.

Proposal 11: Study the hybrid performance monitoring, where NW obtains the performance matric calculated at UE.

- Conclusion

In this contribution, we discussed the sub use-cases and potential specification impacts on AI/ML for beam management. Based on the discussion we made the following observations and proposals.

Observation 1: After the beam prediction, additional beam measurements are necessary for the beam determination in the following case

・Top K beam prediction is applied

・Rx beam refinement is performed after Tx beam prediction

Observation 2: Beam prediction accuracy related KPI requires the large measurement overhead for searching out the actual top 1/K beam(s).

Observation 3: Link quality related KPI is affected by various factors other than model performance.

Observation 4: The required measurement overhead for performance monitoring is relatively low for the monitoring based on the predicted L1-RSRP difference.

Observation 5: In DL Tx-Rx beam pair prediction with NW side model, some mechanisms to report Rx beam ID used for beam measurements are necessary.

Observation 6: Enhancements on beam selection policy in CSI reports might be potential specification impacts for spatial domain beam estimation.

Observation 7: Time instances corresponding to the reported beam(s) does not need to be reported when CSI reference resource is aligned between UE and NW, since it can be implicitly determined based on the measured RS occasion and the time offset.

Observation 8: It is beneficial to report the explicit predicted time instances in the reporting, if CSI reference resource is not always aligned between UE and NW.

Proposal 1: Identify the practical scenario for Tx beam prediction and beam pair prediction.

Proposal 2: Study the following two patterns for T1 and T2 in temporal beam prediction.

・prediction of beam quality between each measurement/reporting

・prediction of beam quality instead of measurement/reporting

Proposal 3: Define Set C and Set D as follows to facilitate the discussion

・Set C is a set of beams whose measurements are obtained by UE for model inputs (Set B ⊂ Set C)

・Set D is a set of beams which could be potentially measured by UE  (Set C ⊂ Set D)

Proposal 4: Study the following scenario for the beam prediction

・Fixed Set C. UE measure the same beam per model inference.

・Variable Set C. UE may measure the different beam per model inference.

Proposal 5: Discuss the feasibility of the performance monitoring based on the input/output data distribution in the beam prediction, before the specification impact discussion related to it.

Proposal 6: If RAN1 can make the consensus that the DL Rx beam information cannot be reported to NW, DL Tx-Rx beam pair prediction with NW side model should be deprioritized due to the feasibility.

Proposal 7: In DL Tx beam prediction with NW side model, configuring Rx beam determination policy, e.g., the same Rx beam for Set B beam measurements, can be considered as potential specification impacts.

Proposal 8: L1 beam reporting should be enhanced to facilitate the model inference of temporal beam prediction with the overhead reduction.

Proposal 9: Study the overhead reduction of L1 signalling to report SetA beam measurements for NW-based model monitoring.

Proposal 10: Study two-stage beam measurements with top-N predicted beams, since it reduces RS measurement overhead and increases the reliability of beam selection compared to top-1 beam prediction.

Proposal 11: Study the hybrid performance monitoring, where NW obtains the performance matric calculated at UE.

References

- Moderator (Qualcomm), RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Dec. 2021.

- Moderator (OPPO), R1-2205454, “Discussion summary#4 for other aspects on AI/ML for beam management”, May. 2022.

- Moderator (OPPO), R1-2210356, “Summary#4 for other aspects on AI/ML for beam management”, Oct. 2022.

- Moderator (OPPO), R1-2207874, “Summary#4 for other aspects on AI/ML for beam management”, Aug. 2022.

- Moderator (OPPO), R1-2301896, “Summary#5 for other aspects on AI/ML for beam management”, Feb. 2023.

- NTT DOCOMO, INC., R1-2303707, “Discussion on evaluation on AI/ML for beam management”, Apr. 2023.